3GPP TSG-RAN5 Meeting #106	R5-250471

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1578 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Invite message for CEN alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Default message contents of SIP INVITE message for MT Call A.2.9 needs update to align with CEN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated contents of A.2.9 Invite for MT Call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WI will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, A.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 34.123-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

…

[157]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS"

[158]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma".

[159]	IETF RFC 7090: "Public Safety Answering Point (PSAP) Callback".

<End of modified section>

<Start of modified section>

## A.2.9	INVITE for MT Call

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | INVITE |  |  |
| Request-URI | A4 | UE’s registered contact address in SIP URI form, as provided in the Contact header of the REGISTER message |  |  |
|  | A5 | UE’s contact address in SIP URI form, as provided in the Contact header within any response or request within the dialog |  |  |
|  | A10 | UE’s registered contact address in SIP URI form, as provided in the Contact header of the REGISTER message with the tag user = phone |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1 | The SS P-CSCF address and the SS protected server port |  |  |
|  | A2 | The SS P-CSCF address and the SS unprotected server port (optional) |  |  |
| via-branch |  | Value starting with ‘z9hG4bK’ |  |  |
| Via |  | In addition to the via-parm entry for the SS, the following via-parm entries are included: |  | RFC 3261 [15] |
| via-parm |  | SIP/2.0/UDP scscf1.3gpp.org;branch=z9hG4bK…, SIP/2.0/UDP scscf2.3gpp.org;branch=z9hG4bK..., SIP/2.0/UDP pcscf2.3gpp.org;branch=z9hG4bK..., SIP/2.0/UDP caller.3gpp.org:6543;branch=z9hG4bK... (NOTE 2) |  |  |
| Record-Route |  |  |  | RFC 3261 [15] |
| rec-route | A1 AND A4 | <sip: SS P-CSCF address: protected server port of SS ;lr> |  |  |
|  | A2 AND A4 | <sip: SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |
| Record-Route |  | In addition to the rec-route entry for the SS, the following rec-route entries are included: |  | RFC 3261 [15] |
| rec-route |  | <sip:term@scscf1.3gpp.org;lr>, <sip:orig@scscf2.3gpp.org;lr>, <sip:pcscf2.3gpp.org;lr> |  |  |
| Record-Route |  |  |  | RFC 3261 [15] |
| rec-route | A1 AND A5 | MO call established: same value as in 183 Session Progress for INVITE, condition A1, in reverse order MT call established: <sip: SS P-CSCF address: protected server port of SS ;lr> |  |  |
| Record-Route |  |  |  | RFC 3261 [15] |
| rec-route | A2 AND A5 | MO call established: same value as in 183 Session Progress for INVITE, condition A3, in reverse order MT call established: <sip: SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec | A4 | SIP URI of the SS representing the calling UE |  |  |
|  | A11 | urn:service:sos.ecall.manual | Rel-19 | RFC 7090 [159] |
|  | A12 | urn:service:sos.ecall.automatic | Rel-19 | RFC 7090 [159] |
|  | A13 | The Test URI as per the generic “eCall test URI” which uses EFSDNURI from table 4.9.3.5-1 for “eCall capable” UEs or EFFDNURI from table 4.9.3.5-2 for “eCall only” UEs as specified in 3GPP TS 36.508 [94] | Rel-19 | TS 24.229 [10] |
| tag | A4,A11, A12,A13 | any value |  |  |
| addr-spec | A5 | SIP URI of the SS representing the calling UE as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in To header within requests sent by the UE and in From header within requests sent by the SS) |  |  |
| tag | A5 | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec | A4 | SIP URI of the UE’s default public user id |  |  |
| tag | A4 | not present |  |  |
| addr-spec | A5 | SIP URI of the UE as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in From header within requests sent by the UE and in To header within requests sent by the SS) |  |  |
| tag | A5 | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid | A4 | a random text string generated by the SS |  |  |
|  | A5 | value of Call-ID as in any previous request in the same dialog |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value | A4 | any value (e.g. 4711) |  |  |
|  | A5 | value of CSeq sent by the SS within its previous request in the same dialog but increased by one |  |  |
| method |  | INVITE |  |  |
| Priority | A11,A12 | psap-callback | Rel-19 | TS 24.229 [10] |
| Supported |  | The option tags defined below shall be included additionally to any option tags defined in any specific message content, unless specified otherwise in this specific message content. |  | RFC 3261 [15] |
| option-tag |  | 100rel |  |  |
|  |  | timer |  | RFC 4028 [146] |
| P-Called-Party-ID |  | One of the UE’s registered, non-barred public ID |  | RFC 7315 [132] |
| P-Asserted-Identity | A13 |  | Rel-19 | TS 24.229 [10] |
| addr-spec |  | The Test URI as per the generic “eCall test URI” which uses EFSDNURI from table 4.9.3.5-1 for “eCall capable” UEs or EFFDNURI from table 4.9.3.5-2 for “eCall only” UEs as specified in 3GPP TS 36.508 [94]. |  |  |
| uri-parameter |  | lr |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec | A1 AND A4 | SIP URI with IP address or FQDN and protected server port of the calling UE, for example “sip:caller@3gpp.org:6543” |  |  |
|  | A2 AND A4 | SIP URI with IP address or FQDN and unprotected server port of the calling UE |  |  |
|  | A5 | same contact information for the SS as used before in this dialog |  |  |
| feature-param | A3 | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (NOTE 1) |  |  |
|  | A7 | video |  |  |
|  |  | audio |  |  |
| Feature-Caps |  |  |  |  |
| feature-param | A9 | g.3gpp.srvcc-alerting |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| Accept |  |  | Rel-7 | RFC 3261 [15] |
| media-range | A4 | application/sdp, application/3gpp-ims+xml |  |  |
|  | A11,A12,A13 | application/EmergencyCallData.Control+xml | Rel-19 | TS 24.229 [10] |
| Recv-Info | A11,A12,A13 |  | Rel-19 | TS 24.229 [10] |
| Info-package-type |  | EmergencyCallData.eCall.MSD |  |  |
| P-Asserted-Service |  |  |  | RFC 6050 [68] |
| Service-ID | A3 AND A4 | urn:urn-7:3gpp-service.ims.icsi.mmtel |  |  |
| Accept-Contact |  |  |  | RFC 3841 [64] |
| ac-value | A3 AND A4 | *;+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (NOTE 1) |  | TS 24.173 [65] |
| ac-value | A8 | video |  | IR.94 [134] NG.114 [157] |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/sdp |  |  |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |
| A3 | UE supports MTSI (A.3A/50 3GPP TS 34.229-2 [5]) |
| A4 | INVITE creating a dialog |
| A5 | re-INVITE within a dialog |
| A6 | Void |
| A7 | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A8 | INVITE for creating a video call and UE supports video media feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A9 | INVITE for creating a voice or video call and UE supports g.3gpp.srvcc-alerting media feature tag (A.12/34 3GPP TS 34.229-2 [5]) |
| A10 A11 A12 A13 | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) INVITE for PSAP Callback when a manual eCall was placed before and UE supports national regulations in CEN EN 17184:2024 (A.4.5/67 3GPP TS 34.229-2 [5] or A.4.5/68 3GPP TS 34.229-2 [5]) INVITE for PSAP Callback when an automatic eCall was placed before and UE supports national regulations in CEN EN 17184:2024 (A.4.5/67 3GPP TS 34.229-2 [5] or A.4.5/68 3GPP TS 34.229-2 [5]) INVITE for PSAP Callback when an eCall to URI for test services was placed before and UE supports national regulations in CEN EN 17184:2024 (A.4.5/67 3GPP TS 34.229-2 [5] or A.4.5/68 3GPP TS 34.229-2 [5]) |

NOTE 1:	URN is the outcome of the URL encoding (“Percent-Encoding” according to RFC 3986 [129]) of urn:urn-7:3gpp-service.ims.icsi.mmtel.

NOTE 2:	Branch parameter values sent by SS are different within a test case execution.

<End of modified section>