3GPP TSG-RAN5 Meeting #106	R5-250475

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5417 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to EPS test eCall test case 11.3.2 for CEN alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | EPS test eCall test case 11.3.2 needs update to align with CEN requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added MSD transfer and PSAP Callback functionality verification to test eCall testc ase 11.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WI will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 11.3.2	eCall Only mode / T3445 / eCall inactivity procedure / Removal of eCall only restriction after a call to URI for test service / MSD transfer

11.3.2.1	Test Purpose (TP)

(1)

with { UE is in the state EMM-DEREGISTERED.eCALL-INACTIVE}

ensure that {

when { UE is requested to make an eCall to URI for test service}

then { UE sends ATTACH REQUEST message with EPS attach type IE 'combined EPS/IMSI attach'}

}

(2)

with { UE receives ATTACH ACCEPT message with EPS attach result 'combined EPS/IMSI attach' and

IMS voice over PS sessions is supported }

ensure that {

when { UE is in the EMM-REGISTERED.NORMAL-SERVICE state and an initial IMS registration is performed }

then { UE transmits an initial INVITE request for ecall to uri for test services }

}

(3)

with { UE receives an RRCConnectionRelease message and enters in RRC_IDLE state }

ensure that {

when { SS sends paging request for IMS MT speech call }

then {the UE answers the paging request for IMS MT speech call }

}

(4)

with { UE receives an RRCConnectionRelease message and enters in RRC_IDLE state }

ensure that {

when { UE starts T3445 for 12Hours and the periodic tracking area updating timer T3412 expires }

then { UE sends TRACKING AREA UPDATE REQUEST message with EPS update type = ’Periodic updating' }

}

(5)

with { UE is in RRC_IDLE state }

ensure that {

when { eCall Inactivity timer T3445 expires }

then { The UE performs Detach procedure }

}

(6)

with { UE subjected to national regulations in CEN EN 17184 UE has an ongoing ecall to uri for test services }

ensure that {

when { SS sends SIP INFO message for MSD }

then { the UE sends SIP INFO message including MSD }

}

(7)

with { UE subjected to national regulations in CEN EN 17184 having previously established eCall to URI for test services }

ensure that {

when { SS requests for tranfer of MSD during an incoming IMS call }

then { The UE sends 200 OK and performs MSD transfer }

}

11.3.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.3.1.2.1, 5.5.4, 10.2 and TS 24.229 clauses 5.1.6.11.3, 5.1.6A

[TS 24.301 clause 5.3.1.2.1]

The signalling procedure for the release of the NAS signalling connection is initiated by the network.

In S1 mode, when the RRC connection has been released, the UE shall enter EMM-IDLE mode and consider the NAS signalling connection released.

If the UE is configured for eCall only mode as specified in 3GPP TS 31.102 [17] then:

-	if the NAS signalling connection that was released had been established for eCall over IMS, the UE shall start timer T3444; and

-	if the NAS signalling connection that was released had been established for a call to an HPLMN designated non-emergency MSISDN or URI for test or terminal reconfiguration service, the UE shall start timer T3445.

[TS 24.301 clause 5.5.4]

The eCall inactivity procedure is applicable only to a UE configured for eCall only mode as specified in 3GPP TS 31.102 [17]. The procedure shall be started when timer T3444 or timer T3445 expires or is found to have already expired in any EMM-REGISTERED substate except substates EMM-REGISTERED.PLMN-SEARCH or EMM-REGISTERED.NO-CELL-AVAILABLE. The UE shall then perform the following actions:

-	stop other running timers (e.g. T3411, T3412);

-	if the UE is currently registered to EPS services only, perform a detach procedure for EPS services only;

-	if the UE is currently registered for both EPS services and non-EPS services, perform a combined detach procedure for EPS services and non-EPS services;

-	delete any GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs, and KSI; and

-	enter EMM-DEREGISTERED.eCALL-INACTIVE state.

[TS 24.301 clause 10.2]

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| T3444 | 12 hours | All except EMM-NULL | UE configured for eCall only mode enters EMM-IDLE mode after an eCall over IMS | Removal of eCall only restriction | Perform eCall inactivity procedure as described in subclause 5.5.4 |
| T3445 | 12 hours | All except EMM-NULL | UE configured for eCall only mode enters EMM-IDLE mode after a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service | Removal of eCall only restriction | Perform eCall inactivity procedure as described in subclause 5.5.4 |

[TS 24.229 clause 5.1.6.11.3]

[Rel-19]

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2020 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

[TS 24.229 clause 5.1.6A]

[Rel-19]

This clause specifies a feature which applies to UEs subject to national regulations in CEN EN 17184:2024 [300]. If the upper layers request establishment of an IMS call of the test eCall type while in limited service state, the request shall be rejected. Otherwise, theUE shall follow normal voice call setup to attempt the test eCall. If the CS domain is selected, the UE shall attempt a test eCall setup as a non-emergency services call using appropriate access technology specific procedures.

If the IM CN subsystem is selected and the UE is not currently registered, the UE shall perform an initial registration, as described in subclause 5.1.1.2. The UE shall then apply voice domain selection as specified in 3GPP TS 23.221 [6] and 3GPP TS 23.501 [257] and the procedures as specified in subclause 5.1.2A and 5.1.3 to send an INVITE with the following additions:

1)	the UE shall set the Request-URI to a non-emergency service URN (e.g. SIP, Service, Tel URIs) that is defined by a national regulator, PSAP or eCall test provider for testing purposes; and

NOTE 1:	The Request-URI can be set to "urn:service:test.sos.ecall" as specified in RFC 8147 [244] or the URI can be configured in USIM files; see 3GPP TS 31.102 [15C].

2)	the UE shall insert in the INVITE request:

a)	a To header field with the same service URN as in the Request-URI;

b)	a From header field that includes the public user identity or the tel URI associated with the public user identity, as described in subclause 4.2;

c)	a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

d)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

e)	 a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 2:	Further content for the INVITE such as set the test bit in the MSD is defined in CEN EN 15722:2020 [245] and others is defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

2)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful;

…

While a test eCall remains established a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD and the procedures specified in 3GPP TS 26.267 [9C] for transfer of an updated MSD using the audio media stream.

NOTE 3:	For the updated MSD the test bit is set in the MSD according to CEN EN 15722:2020 [245].

After a test eCall has been established and released, successfully or unsuccessfully, a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD during an incoming IMS call when the following conditions are fulfilled:

1)	the incoming IMS call happens within an implementation dependent time period according to the national requirements within the region of eCall service (i.e. for EU region the timer value is 1 hour as defined by the T9 timer in CEN EN 17184:2024 [300]);

2)	the INVITE for the incoming IMS call contains a P-Asserted-Identity header field containing the same non-emergency service URN as included in the Request-URI for the test eCall or as included in a P-Asserted-Identity header field received in a response to the INVITE for the test eCall;

3)	the INVITE for the incoming IMS call contains an Accept header field that includes the value "application/EmergencyCallData.Control+xml"; and

4)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

When conditions 1) to 4) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

NOTE 4:	As defined in CEN EN 17184:2024 [300], the recipient of a test eCall can instigate a callback to the UE to verify UE callback capability and ability to transfer updated MSD.

NOTE 5:	The recipient of a test eCall is encouraged to call back the UE after a shorter period in order to complete the test eCall without a long wait by a test eCall user.

When conditions 1) and 2) are fulfilled, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C].

NOTE 6:	A test eCall recipient can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the recipient, the serving PLMN, the HPLMN or by some other intermediate network.

NOTE 7:	For a trustable and secure identification of an eCall callback and the related MSD transfer between UE and the PSAP (or test center), the PSAP (or test center) is part of the trust domain of the IMS home operator network where the UE under test belongs to.

11.3.2.3	Test description

11.3.2.3.1	Pre-test conditions

System Simulator:

-	Cell A is configured according to Table 6.3.2.2-1 in [18] and is the serving cell.

UE:

-	the eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

Preamble:

-	the UE is in state Switched OFF (state 1) according to [18].

11.3.2.3.2	Test procedure sequence

Table 11.3.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Wait for 60s for the UE to enter EMM-DEREGISTERED.eCALL-INACTIVE state | - | - | - | - |
| 3 | An eCall to URI for test service is initiated by dialling the eCall Test Number in the EFFDNURI field of the USIM (tel:123456). (See Note 1) | - | - | - | - |
| 3A | Check: Does the UE transmit a RRCConnectionRequest message with ' establishmentCause' set to ' mo-Signalling ' | ---> | -RRCConnectionRequest | 1 | P |
| 3B | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 4 | Check: Does the UE sends an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach"? | --> | ATTACH REQUEST | 1 | P |
| 5-13 | Steps 5-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 14 | The SS transmits an ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message and SS assigns value of 186 minutes for the timer T3412. | <-- | ATTACH ACCEPT | - | - |
| 15 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to the event described in step 16 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS THEN in parallel to the event described in step 16 below the behaviour in table 11.3.2.3.2-2 occurs. | - | - | - | - |
| 16 | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| - | EXCEPTION: Steps 17A1 to 17A5b1 describe behaviour that depends on test case requirements and UE behaviour; the "lower case letter" identifies the different branches. | - | - | - | - |
| 17A1 – 17A3 | IF MULTI_PDN = TRUE THEN Steps 1 – 3 of the generic procedure for UE establishing additional PDN connectivity as specified subclause 4.5A.16 takes place | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 17A4 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| - | EXCEPTION: IF ADD_IMS THEN in parallel to the event described in step 17A4 below the behaviour in table 11.3.2.3.2-2 occurs. | - | - | - | - |
| 17A4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| - | EXCEPTION: Steps17A5a1 to 17A5b1 describe behaviour that depends on UE capabilities/configuration; the "lower case letter" identifies a step sequence that takes place if one of those is supported/configured. | - | - | - | - |
| 17A5a1 | IF pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in steps 9 – 12 of TS 36.508 clause 4.5A.17 | - | - | - | - |
| 17A5b1 | ELSE the non-IMS PDN shall be released as specified in steps 10 -12 of TS 36.508 clause 4.5A.18 | - | - | - | - |
| 18-28 | Void | - | - | - | - |
| 29-31 | The UE performs Steps 23-25 of the generic test procedure as described in TS 36.508 sub clause 4.5A.27.3 | - | - | - | - |
| 31A-31B | Steps 2 – 3 of generic procedure for eCall establishment as defined in TS 34.229-1 [35] annex C.47 take place. | - | - | - | - |
| - | EXCEPTION: Steps 31Ca1 to 31Ca4 describes behaviour that take place if UE supports pc_EPS_IMS_eCall_CEN17184. | - | - | - | - |
| 31Ca1-31Ca2 | Steps 4-5 of generic procedure for eCall establishment as defined in TS 34.229-1 [35] annex C.47 take place. | - | - | - | - |
| 31Ca3 | Check: Does UE send SIP INFO message with MSD update? | --> | SIP INFO | 6 | P |
| 31Ca4 | Step 7 of generic procedure for eCall establishment as defined in TS 34.229-1 [35] annex C.47 takes place. | - | - | - | - |
| 32-35 | Release eCall to URI for test service using the generic procedure described in TS 34.229-1 [35] sub clause C.33. | - | - | - | - |
| 36 | Void | - | - | - | - |
| 37 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 38 | The SS waits 1 minute. | - | - | - | - |
| 39-62 | Steps 1 to 24 of the generic test procedure for IMS MT speech call specified in TS 36.508 sub clause 4.5A.7.3. | - | - | 3 | P |
| 63-66 | Generic test procedure for MT release of IMS call as described in annex C.33 of TS 34.229-1 [35] takes place. | - | - | - | - |
| 67 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| - | EXCEPTION: Steps 67Aa1 to 67Aa31 describes behaviour that takes place if UE supports pc_EPS_IMS_eCall_CEN17184. | - | - | - | - |
| 67Aa1-67Aa23 | Steps 1 to 23A of the generic test procedure for IMS MT speech call specified in TS 36.508 subclause 4.5A.7.3 take place. | - | - | - | - |
| 67Aa24 | Check: Does UE send 200 OK for INVITE message? | --> | 200 OK | 7 | P |
| 67Aa25 | Step 24 of the generic test procedure for IMS MT speech call specified in TS 36.508 subclause 4.5A.7.3 takes place. |  |  |  |  |
| 67Aa26-67Aa27 | Steps 4-5 of generic procedure for eCall establishment as defined in TS 34.229-1 [35] annex C.47 take place. | - | - | - | - |
| 67Aa28 | Check: Does UE send SIP INFO message with MSD update? | --> | SIP INFO | 7 | P |
| 67Aa29 | Step 7 of generic procedure for eCall establishment as defined in TS 34.229-1 [35] annex C.47 takes place. | - | - | - | - |
| 67Aa30 | Generic test procedure for MT release of IMS call as described in annex C.33 of TS 34.229-1 [35] takes place. | - | - | - | - |
| 67Aa31 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| - | EXCEPTION: Steps 68 to 71 are repeated every 186 minutes (expiry of T3412) until T3445 expires. | - | - | - | - |
| 68 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? (See Note 2) | --> | TRACKING AREA UPDATE REQUEST | 4 | P |
| 69 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 70 | The UE sends TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 71 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 72 | Check: Does the UE transmit a DETACH REQUEST message at expiry of T3445? | --> | DETACH REQUEST | 5 | P |
| 73 | The SS responds with DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 74 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| Note 1: The request to originate an eCall to URI for test service may be performed by MMI or AT command. Note 2: Timer T3412 is reset and started with its initial value, when the UE changes from EMM-CONNECTED to EMM-IDLE mode so the timer T3412 should be started after step 67 and T3445 should be started after step 37. |  |  |  |  |  |

Table 11.3.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Steps 4-7 of the generic procedure for IMS registration as defined in TS 34.229-1 [35] annex C.2 take place. | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 2 below the behaviour in table 11.3.2.3.2-3 occurs. | - | - | - | - |
| 2 | Steps 8-11 of the generic procedure for IMS registration as defined in TS 34.229-1 [35] annex C.2 take place. | - | - | - | - |

Table 11.3.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an INVITE request message as defined in step 1 of TS 34.229-1 [35] annex C.47. | --> | INVITE | 2 | P |

11.3.2.3.3	Specific message contents

Table 11.3.2.3.3-1: Message SystemInformationBlockType1 (Preamble)

Derivation path: 36.508 table 4.4.3.2-3 Condition eCalloverIMS

Table 11.3.2.3.3-2: Message ATTACH ACCEPT (step 14, Table 11.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach result | '010'B | ''Combined EPS/IMSI attach'' |  |
| T3412 value |  |  |  |
| Unit | '010' | ''value is incremented in multiples of decihours'' |  |
| Timer value | '11111' | ''186 minutes'' |  |
| GUTI | GUTI-1 |  |  |
| EPS network feature support | '0000 0011'B | IMS voice over PS session in S1 mode is supported emergency bearer services in S1 mode supported |  |

Table 11.3.2.3.3-3: Void

Table 11.3.2.3.3-3A: INVITE (Step 1 Table 11.3.2.3.2-3)

| Derivation path: 34.229-1 [35] table in annex A.2.1 Condition A25 |  |
| --- | --- |
| Header/param | Value/remark |
| Call-Info url purpose | Any URL EmergencyCallData.eCall.MSD |
| Message-body | The following SDP types and values. Session description: - - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: AMR codec (AMR/8000 and/or AMR-WB/16000) shall be present in the media attributes, optionally including channel number "/1". -boundary value (as provided in SIP hdr Content-Type) Content-Type: application/EmergencyCallData.eCall.MSD Content-ID: same URL as in Call-Info header Content-Disposition: by-reference;handling=optional MSD in ASN.1 PER encoding -boundary value (as provided in SIP hdr Content-Type) |

Table 11.3.2.3.3-3B: 200 OK for INVITE (Step 2 TS 34.229-1 [35] annex C.47, in parallel to steps 29-31 Table 11.3.2.3.2-1)

| Derivation path: TS 34.229-1 [35] table in annex A.3.1 Condition A12 |  |
| --- | --- |
| Header/param | Value/remark |
| Message-body | --boundary1 Content-Type: application/EmergencyCallData.eCall.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE"/> </EmergencyCallData.control> --boundary1 The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1] - b=AS:37 - b=RS:0 - b=RR:0 Attributes for media: - a=rtpmap: (payload type) AMR/8000/1 or AMR-WB/16000/1 [Note 1] [Note2] - a=fmtp: (format) mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Note 1: The value for fmt, payload type and format is copied from step 1. Note 2: If UE included AMR-WB/16000 in step 1, SS uses AMR-WB/16000/1. Otherwise SS uses AMR/8000/1. |

Table 11.3.2.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 68, Table 11.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '011'B | ''Periodic updating'' |  |
| Old GUTI | GUTI-1 |  |  |

Table 11.3.2.3.3-4A: Message TRACKING AREA UPDATE ACCEPT (step 69, Table 11.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3412 value |  |  |  |
| Unit | '010' | ''value is incremented in multiples of decihours'' |  |
| Timer value | '11111' | ''186 minutes'' |  |
| EPS network feature support | '0000 0011'B | IMS voice over PS session in S1 mode is supported emergency bearer services in S1 mode supported |  |

Table 11.3.2.3.3-5: Message DETACH REQUEST (Step 72, Table 11.3.2.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 | combined EPS/IMSI detach |  |
| Switch off | 0 | normal detach |  |
| GUTI or IMSI | GUTI-1 |  |  |

Table 11.3.2.3.3-6: INVITE (Step 67Aa9, Table 11.3.2.3.2-1)

| Derivation path: TS 34.229-1 [35] Annex A.2.9 Condition A13 |  |
| --- | --- |
| Header/param | Value/remark |
| Supported option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 98 99 100 - b=AS:37 - b=RS:0 - b=RR:2000 Attributes for media: - a=rtpmap:97 AMR-WB/16000/1 - a=fmtp:97 mode-change-capability=2; max-red=220 - a=rtpmap: 98 telephone-event/16000 - a=fmtp: 98 0-15 - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-change-capability=2; max-red=220 - a=rtpmap: 100 telephone-event/8000 - a=fmtp: 100 0-15 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

Table 11.3.2.3.3-7: 200 OK (Step 67Aa24, Table 11.3.2.3.2-1)

Derivation path: TS 34.229-1 [35] Annex A.3.1 Condition A24

<End of modified section>