3GPP TSG-RAN5 Meeting #106	R5-250478

Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3681 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TBD parameters RLM OOS in DRX and non-DRX mode for UE operating on a cell with less than 5MHz bandwidth test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | # NR_FR1_lessthan_5MHz_BW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR SA FR1 RLM OOS test cases for less than 5 MHz has TBD power parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TBD power parameters have been updated in the test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would be incomplete in spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1.13, 6.5.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-25xxxxx |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.5	Signaling characteristics

### 6.5.1	Radio link monitoring

-------- Start of change 1--------

-------- Unchanged Clauses Skipped --------

#### 6.5.1.13	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test tolerance analysis and measurement uncertainty are missing

6.5.1.13.1	Test purpose

The purpose of this test is to verify that the UE supporting less than 5 MHz bandwidth and a long DRX cycle properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell operating on a 3MHz channel bandwidth. This test will partly verify the NR cell radio link monitoring requirements as specified in clause 8.1 of TS 38.133 [6].

6.5.1.13.2	Test applicability

This test applies to all types of NR UE Release 18 supporting less than 5 MHz bandwidth and a long DRX cycle.

6.5.1.13.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.5.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.1.13.

6.5.1.13.4	Test description

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure 6.5.1.13.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Figure 6.5.1.13.4-1: SNR variation for out-of-sync testing

6.5.1.13.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.1.13.4.1-1.

Table 6.5.1.13.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 6.5.1.13-1 | FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth |

Configure the test equipment and the DUT according to the parameters in Table 6.5.1.13.4.1-2.

Table 6.5.1.13.4.1-2: Initial conditions for radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.1.13.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.7.4 for TE Part |  |  |

General test parameters as specified in Table 6.5.1.3.4.1-4 with config 1 apply except those specified in Table 6.5.1.13.4.1-3.

1. Message contents are defined in clause 6.5.1.13.4.3.

2. Single Cell is used, which is NR FR1 PCell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

3. Downlink signals for NR cell are initially set up according to Annex C.1.2 and C.1.3.

Table 6.5.1.13.4.1-3: General test parameters for FR1 OOS 12 PRB in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| BWchannel | Config 1 | MHz | 3: NRB,c = 12 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.2 FDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.6 FDD |
| SSB Configuration | Config 1 |  | SSB.13 FR1 |
|  | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 2 |
| In sync transmission parameters | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
| Out of sync transmission parameters | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 |
|  | REG bundle size |  | 6 |
|  | CP length |  | Normal |
|  | Mapping from REG to CCE |  | Distributed |

6.5.1.13.4.2	Test Procedure

Same as 6.5.1.3.4.2

6.5.1.13.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clause 4.6.1 and 7.3.1 with the condition “Short_DCI” and with the following exceptions:

Table 6.5.1.13.4.3-1: Common Exception messages for radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode test requirement for a UE operating on a cell with less than 5MHz mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.5-4 with condition SCS15kHz_FDD Table H.3.5-9 Table H.3.7-1 with condition DRX.3 |

Table 6.5.1.13.4.3-2: RLF-TimersAndConstant

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| n310 | n1 |  |  |
| n311 | n1 |  |  |
| t311 | ms1000 |  |  |
| } |  |  |  |

Table 6.5.1.13.4.3-3: SIB1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfoSEQUENCE { |  |  |  |
| q-RxLevMin | -53 | -106 is actual value in dBm (-53 * 2 dBm) | dBm/15kHz |
| } |  |  |  |
| } |  |  |  |

6.5.1.13.5	Test Requirement

Cell-specific test parameters as specified in Table 6.5.1.3.5-1 apply for this test except those specified in Table 6.5.1.13.5-1. Test requirements specified in 6.5.1.3.5 apply to this test.

Table 6.5.1.13.5-1: Cell specific test parameters for FR1 OOS 12 PRB in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 |

-------- End of change 1--------

-------- Start of change 2--------

#### 6.5.1.14	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test tolerance analysis and measurement uncertainty are missing

6.5.1.14.1	Test purpose

The purpose of this test is to verify that the UE supporting less than 5 MHz properly detects the out-of-sync and in sync for monitoring downlink radio link quality of the PCell operating on a 3MHz channel bandwidth. This test will partly verify the FR1 radio link monitoring requirements in TS 38.133 [6] clause 8.1.2.

6.5.1.14.2	Test applicability

This test applies to all types of NR UE from Release 18 onwards for a UE operating on a cell with less than 5 MHz bandwidth.

6.5.1.14.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.5.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.1.14 and 8.1.2.

6.5.1.14.4	Test description

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure 6.5.1.14.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 1.

Figure 6.5.1.14.4-1: SNR variation for out-of-sync testing

6.5.1.14.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.1.14.4.1-1.

Table 6.5.1.14.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 6.5.1.14-1 | FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth |

Configure the test equipment and the DUT according to the parameters in Table 6.5.1.14.4.1-2.

Table 6.5.1.14.4.1-2: Initial conditions for radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode for a UE operating on a BW less than 5 MHz BW

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.1.14.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.7.4 for TE Part |  |  |

1. Message contents are defined in clause 6.5.1.14.4.3.

2. Single Cell is used, which is NR FR1 PCell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

3. General test parameters as specified in Table 6.5.1.14.4.1-3 with config 1 apply except those specified in Table 6.5.1.14.4.1-3.

4. Downlink signals for NR cell are initially set up according to Annex C.1.2 and C.1.3.

Table 6.5.1.14.4.1-3: General test parameters for FR1 OOS 15 PRB in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| BWchannel | Config 1 | MHz | 3: NRB,c = 15 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.3 FDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.7 FDD |
| SSB Configuration | Config 1 |  | SSB.13 FR1 |
|  | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 3 |
|  | Aggregation level | CCE | 4 |
| In sync transmission parameters | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |
|  | Number of Control OFDM symbols |  | 3 |
|  | Aggregation level | CCE | 8 |
| Out of sync transmission parameters | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 |
|  | REG bundle size |  | 6 |
|  | CP length |  | Normal |
|  | Mapping from REG to CCE |  | Non-Distributed |

6.5.1.14.4.2	Test Procedure

Same as 6.5.1.1.4.2.

6.5.1.14.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clause 4.6.1 and 7.3.1 with condition “Short_DCI” and with the following exceptions:

Table 6.5.1.14.4.3-1: Common Exception messages for radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode test requirement for a UE operating on less than 5 MHz BW

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, GAP_NEEDED Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier center of High range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-1 Table H.3.1-6 with conditions gapUE and RLM Table H.3.5-4 with condition SCS15kHz_FDD Table H.3.5-9 |

Table 6.5.1.14.4.3-2: RLF-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| n310 | n1 |  |  |
| t311 | ms1000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

Table 6.5.1.14.4.3-3: SIB1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 | -106 is actual value in dBm (-53 * 2 dBm) | dBm/15kHz |
| } |  |  |  |
| } |  |  |  |

6.5.1.14.5	Test Requirement

Cell specific test parameters as specified in Table 6.5.1.14.5-1 apply except those specified in Table 6.5.1.14.5-1. Test requirements specified in clause 6.5.1.1.5 apply to this test.

Table 6.5.1.14.5-1: Cell specific test parameters for FR1 PCell

| Parameter |  |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |  |
| SNR on RLM-RS |  | Config 1 | dB | 1 | -7 | -15 |  |

-------- End of change 2--------