3GPP TSG-RAN5 Meeting #106	R5-250491

Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3691 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-DC SA FR2 conditional handover including target MCG and target SCG test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | # NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC test case has been missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | The above test case and cell configuration mapping have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case would be missing in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3, 7.3.3.5(new), E.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.3	RRC_CONNECTED state mobility

### 7.3.3	Conditional handover

#### 7.3.3.0	Minimum conformance requirements

----- Start of change 1st ------

##### 7.3.3.0.3	Minimum conformance requirements for NR-DC conditional handover including target MCG and target SCG

[TS 38.133, clause 6.1.6.2]

The requirements in this clause are applicable to conditional handover with target SCG from NR-DC to NR-DC. The requirements in this clause are only applicable to:

-	FR1-FR2 NR-DC to FR1-FR2 NR-DC.

This clause defines requirements for the delay within which the UE shall be able to handover from NR cell to NR cell and change NR PSCell in the meantime.

When the UE receives a RRC message implying conditional handover including target MCG in FR1 and target SCG in FR2,

-	The UE shall be ready to start the transmission of the new uplink PRACH channel of the target PCell within DCHOwithPSCell_PCell ms from the end of the last TTI containing the RRC command, and

-	The UE shall be capable of transmitting PRACH preamble towards the target PSCell no later than DCHOwithPSCell_PSCell ms from the end of the last TTI containing the RRC command.

Where:

-	DCHOwithPSCell_PCell equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [13] plus the interruption time stated in clause 6.1.6.2.1 in TS 38.133 [6].

-	DCHOwithPSCell_PSCell is the PSCell change in clause 6.1.6.2.2 in TS 38.133 [6]

[TS 38.133, clause 6.1.6.2.1] CHO with PSCell – PCell handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [13].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel of the target PCell within DCHOwithPSCell_PCell seconds from the end of the last TTI containing the RRC command.

DCHOwithPSCell_PCell =TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [13].

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover.

Tmeasure is the measurement time stated in clause 6.1.4.2.2 in TS 38.133 [6].

Tprocessing is the SW processing time needed by UE, including RF warm up period.

For FR1-FR2 NR-DC to FR1-FR2 NR-DC, Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell stated in clause 6.1.4.2.4 in TS 38.133 [6].

T∆ is time for fine time tracking and acquiring full timing information of the target cell stated in clause 6.1.4.2.4 in TS 38.133 [6].

Tmargin is time for SSB post-processing stated in clause 6.1.4.2.4 in TS 38.133 [6].

TCHO_execution is the conditional execution preparation time in clause 6.1.4.2.3 in TS 38.133 [6].

[38.133, clause 6.1.6.2.2] CHO with PSCell – PSCell change delay

The requirements in this section shall apply for PSCell change during handover with PSCell from NR DC to NR-DC.

When handover with PSCell from NR-DC to NR-DC is commanded, the PSCell change time shall be less than DHOwithPSCell_PSCell:

-	DCHOwithPSCell_PSCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms

Where:

-	TRRC_delay TEvent_DU Tmeasure TCHO_execution are the same as defined in clause 6.1.6.2.1 in TS 38.133 [6]. Tsearch_PSCell, T∆_PSCell and TPSCell_ DU are the same as defined in clause 6.1.5.4.2 in TS 38.133 [6].

-	Tprocessing is the SW processing time needed by UE, including RF warm up period.

For FR1-FR2 NR-DC to FR1-FR2 NR-DC, Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

-	Tsearch_PCell _Conditional is the time for obtaining the timing reference of target PCell. If SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, Tsearch_PCell _Conditional = TΔ + Tmargin, where TΔ and Tmargin are specified in clause 6.1.5.4.1 in TS 38.133 [6]. Otherwise, Tsearch_PCell _Conditional = 0 ms.

The Trs definition from clause 8.9.2 in TS 38.133 [6] is modified as following for requirements in this section:

-	Trs is the SMTC periodicity of the target NR cell if target PSCell is unknown and SMTC configuration of target unknown PSCell is present in either targetcellSMTC-SCG-r16 or reconfigurationWithSync, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this section is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

PSCell known and unknown condition is as defined in clause 8.9.2 in TS 38.133 [6]

----- End of change 1st------

----- Unchanged Clauses Skipped------

----- Start of change 2nd------

#### 7.3.3.5	NR SA FR2 NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is missing.

-	Message contents are TBD.

- 	Test procedure shall require an update.

7.3.3.5.1	Test purpose

The purpose of this test is to verify the requirements of CHO including target MCG and target SCG in NR-DC requirements specified in clause 6.1.6.2 in TS 38.133 [6]. This test verifies the requirements for inter-frequency FR1-FR1 conditional handover and intra-frequency FR2-FR2 PSCell change.

7.3.3.5.2	Test applicability

This test applies to all types of NR-DC UE from Release 17 onwards and supporting conditional handover.

7.3.3.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.3.3.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.3.5.

7.3.3.5.4	Test description

7.3.3.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 7.3.3.5.4.1-1.

Table 7.3.3.5.4.1-1: NR Supported test configurations for CHO with PSCell change from NR-DC to NR-DC

| Config | Description |
| --- | --- |
| 1 | Source PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Source PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | Source PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Source PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | Source PCell: FR1 NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target PCell: FR1 NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Source PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.3.3.5.4.1-2.

Table 7.3.3.5.4.1-2: Initial conditions for NR SA FR2-FR2 conditional handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.3.4.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR2 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

Table 7.3.3.5.4.1-3: General test parameters for PCell FR1-FR1 Inter frequency conditional handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 3 |  |
| Final condition | Active cell |  | Cell 3 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.7.3.3.5.1-4: General test parameters Intra-frequency FR2-FR2 PSCell change

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 2 |  |
|  | Neighbouring cell |  | Cell 4 |  |
| Final condition | Active cell |  | Cell 4 |  |
| A4-Offset |  | dBm | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

7.3.3.5.4.2	Test procedure

The test scenario comprises four NR cells, source PCell(Cell 1) and source PSCell(Cell 2), target PCell(Cell 3), target PSCell(Cell 4).Cell 1 and Cell 3 are on radio channel 1 in FR1.Cell 2 and Cell 4 are on radio channel 2 in FR2. The supported test configurations are given in Table 7.3.3.5.4.1-1. Test parameters are given in Tables Table 7.3.3.5.4.1-3 and Table 7.3.3.5.4.1-4 below.

The test consists of two successive time periods, with time durations of T1, T2 respectively.

At the start of time duration T1, the UE shall be connected to Cell 1 on radio channel 1 and Cell 2 on radio channel 2. The UE does not have any timing information of cell 3 and cell 4. NR shall configure a message implying conditional handover including target MCG in FR1 and target SCG in FR2 to Cell 3 during T1, at a time earlier than TRRC before the beginning of T2. UE shall be able to do inter-frequency conditional handover from cell 1 to cell 3 and intra-frequency PSCell change from cell 2 to cell 4.

No interruption shall be observed in time period T1. where,

-	TRRC = 16ms, is the RRC procedure delay defined in clause 12 in TS 38.331 [13].

At the start of T2, Cell 3 and Cell 4 become detectable and the condition for conditional PCell handover is met. During T2, the UE performs measurement on Cell 3 and evaluates the execution condition and starts handover procedure when execution condition is satisfied.

The UE shall start to transmit the PRACH to Cell 3 less than Tmeasure + Tinterrupt + TCHO_execution from the start of T2 and the interruption during T2 shall not exceed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin.

The UE shall start to transmit the PRACH to Cell 4 less than Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 from the start of T2.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRCReconfiguration message. Cell 1 and Cell 2 is the active cell. Set Cell 3 and Cell 4 physical cell identity to the initial physical cell identity.

2.	Set the parameters according to T1 in Table 7.3.3.5.5-1 and Table 7.3.3.5.5-2. Propagation conditions are set according to Annex C clause C.2.2. T1 starts. The SS starts continuously scheduling the UE to perform DL reception in every DL slot on Cell 1, Cell 2 and monitoring corresponding ACK/NACK feedback sent by the UE.

3.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) and Cell 4 physical cell identity = ((current cell 4 physical cell identity + 1) mod 1008).

4.	The SS shall transmit an RRCReconfiguration message with conditionalReconfiguration on Cell 1 to configure CHO execution condition and measurement gap pattern #0 for the UE.

5.	The UE shall transmit an RRCReconfigurationComplete message.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.3.5.5-1 and Table 7.3.3.5.5-2. T2 starts.

7.	If

- the UE transmits the PRACH preambles to Cell 3 less than Tmeasure + TCHO_execution + Tinterrupt ms from the beginning of time period T2,

- The UE shall start to transmit the PRACH to Cell 4 less than Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 from the start of T2.

and

- no longer than X consecutive ACK/NACK DTXs are observed by the SS from the start of T2 to the instant the UE transmits the first PRACH preamble, where

-	X = 8∙Tinterrupt for all test configurations

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T2 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync to cause UE handover back from cell 3 to cell 1 and PSCell change from cell 4 to cell 2.

9.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, Cell 2, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1 and Cell 2.

10.	Repeat steps 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.3.3.5.4.3	Message Contents

TBD

7.3.3.5.5	Test Requirements

7.3.3.5.5.1	Test Requirements for NR Conditional Handover

Table 7.3.3.5.5-1: Cell specific test parameters for PCell FR1-FR1 Inter frequency conditional handover

| Parameter |  |  | Unit | Cell 1 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  | -98 |  |
|  | Config 3 |  |  | -95 |  | -95 |  |
|  |  |  | dB | 4+TT | 4+TT | -Infinity | 5+TT |
|  |  |  | dB | 4+TT | 4+TT | -Infinity | 5+TT |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94+TT | -94+TT | -Infinity | -93+TT |
|  | Config 3 |  | dBm/SCS | -91+TT | -91+TT | -Infinity | -90+TT |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59+TT | -64.59+TT | -70.05+TT | -63.85+TT |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49+TT | -58.49+TT | -63.94+TT | -57.75+TT |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 3 less than Tmeasure + Tinterrupt +TCHO_execution from the start of T2. where:

-	For power class 1 = 6720+47+10ms=6777 ms.

-	For power class 2, 3 and 4 = 4160+47+10ms =4217ms.

-	Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms=25+20+2 = 47ms, is the interruption time.

This gives a total of 6777 ms for power class 1 UE and 4217 ms for power class 2/3/4 UE.

The interruption during T2 shall not exceeed Tinterrupt = 47ms.

The rate of correct handovers observed during repeated tests shall be at least 90%.

7.3.3.5.5.2	Test Requirements for NR PSCell change

Table 7.3.3.5.5-2: Cell specific test parameters for Intra-frequency FR2-FR2 PSCell change

| Parameter |  | Unit | Cell 2 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  | Rough |  | Rough |  |
| AoA setup |  |  | Setup 1 as defined in A.3.15 |  |  |  |
| NR RF Channel Number |  |  | 1 |  | 1 |  |
| Duplex mode |  |  | TDD |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  |  | 66 |  |  |  |
| DRx Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  | SR3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | CR3.1 TDD |  |  |  |
| Control Channel RMC |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  | O P. 1 |  |  |  |
| SMTC Configuration |  |  | SMTC pattern 1 |  |  |  |
| SSB Configuration |  |  | SSB. 3 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 120 kHz |  |  |  |
| PRACH configuration |  |  | FR2 PRACH configuration 1 |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  |  |  |
| BWP configuraiton | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 |  |  |  |
| Note2 |  | dBm/SCS | -95.7 |  |  |  |
|  |  | dB | 6+TT | -1.8+TT | -Infinity | 0+TT |
|  |  | dB | 6+TT | 6+TT | -Infinity | 7+TT |
| IoNote3 |  | dBm/ BW | -59.7+TT | -56.7+TT | -59.7+TT | -56.7+TT |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 4 less than Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 from the start of T2.

-	For power class 1 = 6720+10+25+20+10ms+2= 6792 ms.

-	For power class 2, 3 and 4 = 4160+25+20+10+2ms = 4232ms.

This gives a total of 6792 ms for power class 1 UE and 4232 ms for power class 2/3/4 UE.

----- End of change 2nd------

----- Skipped Unchanged Clauses ------

----- Start of change 3rd------

# E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for NR/5GC FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.2.4 | NR SA FR2 2-step non-contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.3.1 | NR SA FR2-FR2 RRC connection release with redirection | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.3.5 | NR SA FR2-FR2 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.3.3 | NR SA FR2 intra-frequency NES triggering conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.3.4 | NR SA FR2-FR1 inter-frequency NES triggering conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.3.5 | NR SA FR2 NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC | NR Cell 1 | NR Cell 2 | NR Cell 3 | NR Cell 12 |  |
| 7.3.5.1 | NR SA FR2 RACH-based Intra-frequency LTM PSCell switch from FR2 to FR2 | NR Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 7.4.1.1 | NR SA FR2 UE transmit timing accuracy | NR Cell 1 |  |  |  |  |
| 7.4.1.3 | NR SA FR2 UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 | NR Cell 1 |  |  |  |  |
| 7.4.3.1 | NR SA FR2 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |
| 7.5.1.1 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.2 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.3 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |

----- End of change 3rd------