3GPP TSG-RAN5 Meeting #106	R5-250514

# Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0356 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add NG.114 PS data off |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Applicability for PS data off is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PS data off applicabilty and NG.114 relation have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability for PS data off with NG.114 context will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of next modified section>

### A.4.5	Additional information

Table A.12: Additional information

| Item | Additional information | Reference | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Void |  |  |  |  |  |
| 2 | Void |  |  |  |  |  |
| 3 | Void |  |  |  |  |  |
| 4 | UE capable of being configured to initiate Dedicated PDP Context | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiatesDedicatedPDPContext |  |
| 5 | UE capable of being configured to initiate P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaPCO |  |
| 6 | Void |  |  |  |  |  |
| 7 | UE capable of being configured to initiate P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaDHCPv6 |  |
| 8 | UE supports P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaPCO |  |
| 9 | Void |  |  |  |  |  |
| 10 | UE supports P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaDHCPv6 |  |
| 11 | Void |  |  |  |  |  |
| 12 | UE capable of initiating a bidirectional voice session over IMS | 24.229 [10], 5.1.6 | o | Rel-5 | pc_BidirecVoiceOverIMS |  |
| 13 | Void |  |  |  |  |  |
| 14 | Void |  |  |  |  |  |
| 15 | Void |  |  |  |  |  |
| 16 | UE Supports " IPv6 address with embedded IPv4 address" in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 17 | UE Supports IPv4 address in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 18 | Void |  |  |  |  |  |
| 19 | UE supports UI capable of showing user notification for Message Waiting Indication | 24.173 [55], Annex F | o | Rel-7 |  |  |
| 20 | Void |  |  |  |  |  |
| 21 | Void |  |  |  |  |  |
| 22 | Void |  |  |  |  |  |
| 23 | Void |  |  |  |  |  |
| 24 | UE supports no reply timer setting | 24.604 [68], 4.9.1.4 | o | Rel-8 |  |  |
|  |  | IR.92 [83], 2.3.8 | m |  |  |  |
| 25 | UE supports sending DTMF events over RTP | 26.114 [56], Annex G | o | Rel-7 |  |  |
|  |  | IR.92 [83], 3.3 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 26 | Void |  |  |  |  |  |
| 27 | UE is capable of obtain ing location information | 24.229 [10] 4.7, 36.509 [79] 4.1, 5.5.2, 34.109 [78] 5.4.2 | o | Rel-9 | pc_IMS_Geolocation |  |
| 28 | Void |  |  |  |  |  |
| 29 | UE supports emergency speech call over 1xRTT | C.S0005-E [74] | o |  | pc_CS_Em_Call_in_1xRTT |  |
| 30 | Void |  |  |  |  |  |
| 31 | UE supports end-to-access-edge media security using SDES | 24.229 [10], 4.2.B.2 | o | Rel-9 |  |  |
| 32 | UE supports video media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | o | Rel-13 | pc_IMS_Video_FeatureTag |  |
|  |  | IR.94 [75], 2.2 | m |  |  |  |
|  |  | NG.114 [99] | m NOTE 1 | Rel-15 |  |  |
| 33 | UE supports Communication Hold during emergency call | 23.167 [76], 7.1.1 | o | Rel-9 | pc_CommunicationHold_DuringEmergencyCall |  |
| 34 | UE indicates g.3gpp.srvcc-alerting media feature tag in INVITE request or 180 (Ringing) response | 24.237 [77], 12.2.3B.1 | o | Rel-10 | pc_IMS_SRVCCAlert |  |
| 35 | Void |  |  |  |  |  |
| 36 | UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request | 24.237 [77] 6A.2.2.2 | o | Rel-12 | pc_BSRVCC |  |
| 37 | UE indicates OMA-TS-XDM_MO-V1_1-20080627-A.doc, section 5.2.8 "Node: /<X>/ AAUTHNAME" is configured | 24.623 [81], Annex B.2 | o | Rel-8 | pc_XCAP_UsernameIsConfiguredInUE |  |
| 38 | The UE use the default public user identity received in P-Associated -URI header in 200 OK for REGISTER as XCAP User Identity (XUI) | IR.92 [83] | m |  | pc_XCAP_XUIisDefaultPublicUserId |  |
|  |  | 24.623 [81] | o | Rel-8 |  |  |
| 39 | UE has the method that support IMS deregistration | 24.229 [10], 5.1.1.6 | o | Rel-5 | pc_IMS_Deregistration |  |
| 40 | UE supports Cs to PS SRVCC | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCC |  |
| 41 | UE supports Cs to PS SRVCC in alerting state | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCAlert |  |
| 42 | UE supports Cs to PS SRVCC and the MSC server assisted mid-call feature | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCMidCall |  |
| 43 | Void |  |  |  |  |  |
| 44 | Void |  |  |  |  |  |
| 45 | UE supports early media | 24.229 [10], 5.1.3 | o | Rel-8 | pc_EarlyMedia |  |
| 46 | UE indicates g.3gpp.accesstype media feature tag in REGISTER | 24.237 [77], 6.2.2 | c1 | Rel-11 |  |  |
| 47 | Void |  |  |  |  |  |
| 48 | Void |  |  |  |  |  |
| 49 | The UE uses XCAP without PDN connection in WLAN | IR.51 [84], 4.6 | o | Rel-11 | pc_WLAN_XCAP_without_PDN | If PICS is true, Wi-Fi access without PDN connection applies. Otherwise, EPC-integrated WLAN applies |
| 50 | The UE uses XCAP in fixed broadband access |  | o | Rel-12 | pc_fixed_broadband_XCAP | In the context of the present specification, a UE supporting fixed broadband, A.18/3, shall set this PICS to true. |
| 51 | The UE uses same public user Identity for From/To as in initial REGISTER | 24.229 [10] | o | Rel-8 | pc_Same_IMPU |  |
|  |  | IR.92 [83], 2.2.1 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 52 | UE reattempts IMS emergency service without registration after rejection of emergency registration. (For Rel-14, this is applicable only when the rejection indicates an anonymous emergency call support) | 23.167 [76], 4.1 | o | Rel-9 | pc_IMS_after_EmReg_rejection |  |
|  |  |  | m | Rel-14 |  |  |
| 53 | Void |  |  |  |  |  |
| 54 | UE supports IMS eCall type of emergency services over EPS | 24.229 [10], 5.1.6.11, L.2.2.6.2 | o | Rel-14 | pc_EPS_IMS_eCall_Capable |  |
| 55 | UE supports IMS eCall Only type of emergency services over EPS | 24.229 [10], 5.1.6.11, L.2.2.6.2 | o | Rel-14 | pc_EPS_IMS_eCall_Only |  |
| 56 | UE supports audio media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | m | Rel-13 | pc_IMS_Audio_FeatureTag |  |
|  |  | IR.92 [83], 2.2.1, 2.2.4 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 57 | UE supports Session Timer | 24.229 [10], 5.1.2A, 5.2.7.2, 5.2.7.3, 5.4.5.3 | o | Rel-6 | pc_IMS_Session_Timer |  |
|  |  | IR.92 [83], 2.2.8 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 58 | UE supports enabling/disabling SMS over IMS | 27.007 [97], 8.72 | o | Rel-15 | pc_Enable_Disable_SMSoverIMS |  |
| 59 | UE supports disabling preconditions | 24.229 [10], 5.1.5A | o | Rel-15 | pc_IMS_Disable_Preconditions |  |
| 60 | UE uses Session Timer for its benefit alone | 24.229 [10] | o | Rel-15 | pc_IMS_Session_Timer_own_benefit |  |
|  |  | NG.114 [99] | o |  |  |  |
|  |  | RFC 4028 [86] cl 7.2 | o |  |  |  |
| 61 | UE supports usage of re-INVITE for session refresh | 24.229 [10] | o | Rel-15 | pc_IMS_Session_Timer_reINVITE |  |
|  |  | NG.114 [99] | o |  |  |  |
|  |  | RFC 4028 [86] cl 7.2 | o |  |  |  |
| 62 | Void |  |  |  |  |  |
| 63 | UE supports IMS eCall type of emergency services over 5GS | 24.229 [10], 5.1.6.11, U.2.2.6.2 | o | Rel-16 | pc_5GS_IMS_eCall_Capable |  |
| 64 | UE supports IMS eCall Only type of emergency services over 5GS | 24.229 [10], 5.1.6.11, U.2.2.6.2 | o | Rel-16 | pc_5GS_IMS_eCall_Only |  |
| 65 | UE supports IMS emergency call over WLAN | 24.229 [10], 5.1.6, R.2.2.6 | o | Rel-13 | pc_WLAN_IMS_EmergencyCall |  |
| 66 | UE supports P-CSCF_RESELECTION_SUPPORT IKEv2 attribute (in untrusted non-3GPP access network) | 24.229 [10], 5.1.6, R.2.2.1C | o | Rel-13 | pc_P_CSCF_Reselection_Support_IKEv2 |  |
| 67 | UE supports PS data off | 24.229 [10] | o | Rel-15 | pc_PS_data_off |  |
|  |  | NG.114 [98] | m NOTE 2 |  |  |  |
| Conditions/Options |  |  |  |  |  |  |
| c1: IF [73] A.4.4-1/32 THEN m ELSE o - - SC UE indicates accesstype. |  |  |  |  |  |  |
| NOTE 1: As an exception to NG.114 support of MTSI Video Call is optional. |  |  |  |  |  |  |
| NOTE 2: As an exception to NG.114 support of PS data off is optional, |  |  |  |  |  |  |

<End of modified section>