3GPP TSG-RAN5 Meeting #106	R5-250522

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5419 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT test case 22.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Table 22.2.3.3.1, Access Technology Identifier is defined as NB-IoT for the Remaining mandatory entries, which is incorrect as the in the Value column it is defined as default which means All technologies as per TS 34.108, clause 8.3.2.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the Access Technology Identifier as it is not needed which is inline with all the other entried of SIM profile in other NB-IoT test cases in the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 22.2.3	NB-IoT / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer

22.2.3.1	Test Purpose (TP)

(1)

with { UE configured with “MinimumPeriodicSearchTimer“ and camped on an VPLMN cell and cells of a higher priority PLMN available }

ensure that {

when { the higher priority PLMN search timer T stored in the USIM or the default value for T is less than the MinimumPeriodicSearchTimer }

then { T shall be set to the MinimumPeriodicSearchTimer. After the first attempt to the higher priority PLMN cell is made, the UE shall not use a value for T that is less than the MinimumPeriodicSearchTimer. When the T expires the UE selects and camps on a cell of the highest priority PLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user }

}

22.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1 and 4.4.3.3.1. Unless otherwise stated these are Rel-13 requirements.

[TS 23.122, clause 4.4.3.1]

At switch on, or following recovery from lack of coverage, the MS selects the registered PLMN or equivalent PLMN (if
it is available) using all access technologies that the MS is capable of and if necessary (in the case of recovery from lack
of coverage, see subclause 4.5.2) attempts to perform a Location Registration.

…

If there is no registered PLMN, or if registration is not possible due to the PLMN being unavailable or registration
failure, the MS follows one of the following two procedures depending on its PLMN selection operating mode. At
switch on, if the MS provides the optional feature of user preferred PLMN selection operating mode at switch on then
this operating mode shall be used. Otherwise, the MS shall use the PLMN selection mode that was used before
switching off.

…

NOTE 1: If successful registration is achieved, then the current serving PLMN becomes the registered PLMN and
the MS does not store the previous registered PLMN for later use.

[TS 23.122, clause 4.4.3.3.1]

If the MS is in a VPLMN, the MS shall periodically attempt to obtain service on its HPLMN (if the EHPLMN list is not present or is empty) or one of its EHPLMNs (if the EHPLMN list is present) or a higher priority PLMN/access
technology combinations listed in "user controlled PLMN selector" or "operator controlled PLMN selector" by scanning in accordance with the requirements that are applicable to i), ii) and iii) as defined in the Automatic Network Selection Mode in subclause 4.4.3.1.1. In the case that the mobile has a stored "Equivalent PLMNs" list the mobile shall only select a PLMN if it is of a higher priority than those of the same country as the current serving PLMN which are stored in the "Equivalent PLMNs" list. For this purpose, a value of timer T may be stored in the SIM. The interpretation of the stored value depends on the radio capabilities supported by the MS:

For an MS that does not only support any of the following or a combination of EC-GSM-IoT or Category M1 or Category NB1(as defined in 3GPP TS 36.306 [54]): T is either in the range 6 minutes to 8 hours in 6 minute steps or it indicates that no periodic attempts shall be made. If no value for T is stored in the SIM, a default value of 60 minutes is used for T.

For an MS that only supports any of the following or a combination of EC-GSM-IoT or Category M1 or Category NB1(as defined in 3GPP TS 36.306 [54]): T is either in the range 2 hours to 240 hours, using 2 hour steps from 2 hours to 80 hours and 4 hour steps from 84 hours to 240 hours, or it indicates that no periodic attempts shall be made. If no value for T is stored in the SIM, a default value of 72 hours is used.

If the MS is configured with the MinimumPeriodicSearchTimer as specified in 3GPP TS 24.368 [50] or 3GPP TS 31.102 [40], the MS shall not use a value for T that is less than the MinimumPeriodicSearchTimer. If the value stored in the SIM, or the default value for T (when no value is stored in the SIM), is less than the MinimumPeriodicSearchTimer, then T shall be set to the MinimumPeriodicSearchTimer

22.2.3.3	Test description

22.2.3.3.1	Pre-test conditions

System Simulator:

-	Three multi-PLMN cells as specified in TS 36.508 clause 8.4.1.2 are configured broadcasting PLMNs as indicated in Table 22.2.3.3.1-1.

-	System information combination 3 as defined in TS 36.508 [18] clause 8.1.4.3.1 is used.

-	The PLMNs are identified in the test by the identifiers in Table 22.2.3.3.1-1.

Table 22.2.3.3.1–1: PLMN identifiers

| Ncell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is registered to PLMN1 before it is switched off.

-	The UE is configured with a value of MinimumPeriodicSearchTimer set to 120 minutes

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 22.2.3.3.1-2.

Table 22.2.3.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 2 | Default PLMN2 Remaining mandatory entries use default values | Default All specified |
| EFOPLMNwACT | 1 | PLMN1 Remaining defined entries use default values | All specified |
| EFHPLMNwAcT | 1 | PLMN4 | NB-IoT |
| EFUST |  | Services 20, 42, 43, 74 and 96 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 [120 minutes] |  |
| EFNASCONFIG |  | MinimumPeriodicSearchTimer set to [120 minutes] |  |

Preamble:

-	The UE is made to camp on Ncell 12 and then Switched OFF (State 1).

22.2.3.3.2	Test procedure sequence

Table 22.2.3.3.2-1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”,“T2” etc are applied at the points indicated in the Main behaviour description in Table 22.2.3.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508.

Table 22.2.3.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 12 | Ncell 13 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | “Off” | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 8.3.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | “Off” | -85 | -85 | Power level “Off” is defined in TS 36.508 Table 8.3.2.2.1-1 |
| T2 | NRS EPRE | dBm/15kHz | -85 | -85 | -85 | Power level “Off” is defined in TS 36.508 Table 8.3.2.2.1-1 |

Table 22.2.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 22.2.3.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3-15b1 | Steps 2 to 14b1 of the registration procedure described in TS 36.508 subclause 8.1.5.2 are performed on Ncell 12. | - | - | - | - |
| 16 | The SS transmits an RRCConnectionRelease-NB message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 17 | Check: Is PLMN1 indicated by the UE? | - | - | 1 | P |
| 18 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 13 after 120 seconds, but before [120mins] (Note 1) from power on? | --> | RRCConnectionRequest-NB | 1 | P |
| 19-23 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 8.1.5A.5 are performed on Ncell 13. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 24 | Check: Is PLMN2 indicated by the UE? | - | - | 1 | P |
| 25 | SS adjusts cell levels according to row T2 of table 22.2.1.3.2-1 | - | - | - | - |
| 26 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 1 after [120 mins] from step 18 | --> | RRCConnectionRequest-NB | 1 | P |
| 27-31 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 8.1.5A.5 are performed. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 32 | Check: Is PLMN4 indicated by the UE? | - | - | 1 | P |
| Note 1: Timers in Step 18 and 26 are derived from the value defined by the MinimumPeriodicSearchTimer. |  |  |  |  |  |

22.2.3.3.3	Specific message contents

None