3GPP TSG-RAN5 Meeting #106	R5-250523

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5420 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT SENSE test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, SENSE-UEContest |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per TS 31.102, clause 4.2.115 - Operator controlled signal threshold per access technology: … Coding: - Operator controlled signal threshold per access technology value for a specified access technology is coded on 1 byte using hexadecimal coding. The Operator controlled signal threshold per access technology value is to be interpreted as Dezibel Milliwatt (dBm). NOTE 2: For instance the value "-105dBm" is coded as '69' in hexadecimal, the value "-80dBm" is coded as '50' in hexadecimal. If signal threshold is configured as 'FF' then ME shall ignore this value. Based on above clause the coding for EFOCST should be ‘5B’ for the threshold value of -91dBm. However it is incorrectly coded as ‘58’ in tables 22.2.14.3.1-2, 22.2.15.3.1-2 and 22.2.15.3.1-3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the coding value of EFOCST to ‘5B’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.14, 22.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 22.2.14	NB-IoT / SENSE/ PLMN selection of RPLMN, HPLMN, UPLMN, OPLMN and Other PLMN / Automatic mode

22.2.14.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and RPLMN, HPLMN, UPLMN ,OPLMN cells available and signal level enhanced network selection is applicable }

ensure that {

when { UE is switched on }

then { UE selects a cell of a PLMN, if received signal quality of this PLMN is equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM }

}

(2)

with { UE camped on a VPLMN cell and cells of a higher priority PLMN available, received signal quality of the highest priority PLMN is lower than the "Operator controlled signal threshold per access technology" stored in the USIM and received signal quality of the second highest priority PLMN is equal to or greater than the "Operator controlled signal threshold per access technology"" stored in the USIM }

ensure that {

when { higher priority PLMN search timer T expires }

then { UE selects and camps on a cell of the second highest priority PLMN }

}

(3)

with { UE in Automatic network selection mode and HPLMN, OPLMN and Other PLMN cells available but the received signal quality from none of the candidate PLMN(s) or PLMN/access technology combination(s) is equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM }

ensure that {

when { UE returns to coverage }

then { UE stops applying signal level enhanced network selection and selects a cell of the highest priority PLMN }

}

22.2.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1, 4.4.3.1.1 and 4.4.3.3.1. Unless otherwise stated these are Rel-18 requirements.

[TS 23.122, clause 4.4.3.1]

a)	if

-	signal level enhanced network selection is not applicable (see clause 3.11); or

-	the MS has stopped applying signal level enhanced network selection according to requirement v) of clause 4.4.3.1.1;

then the MS selects the registered PLMN or equivalent PLMN (if it is available) using all access technologies that the MS is capable of without considering the "Operator controlled signal threshold per access technology" stored in the USIM; or

b)	if:

-	signal level enhanced network selection is applicable (see clause 3.11); and

-	the received signal quality from a candidate PLMN/access technology combination comprising of a registered PLMN or an equivalent PLMN (if it is available) is equal to or greater than the "Operator controlled signal threshold per access technology" of the access technology configured in the USIM.

the MS shall select the registered PLMN or equivalent PLMN (if it is available) and the access technology for which the received signal quality is equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM. If for the registered PLMN or equivalent PLMN there are two or more access technologies for which the received signal quality is equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM, it is up to implementation which access technology is selected by the MS.

[TS 23.122, clause 4.4.3.1.1]

When following the above procedure the following requirements apply:

…

v)	In i), ii), iii), and v), if:

-	signal level enhanced network selection is applicable (see clause 3.11); and

-	the received signal quality of the candidate PLMN/access technology combination is lower than the threshold value indicated for the access technology in the "Operator controlled signal threshold per access technology" stored in the USIM.

the MS shall not consider the PLMN(s) in i) and PLMN/access technology combination(s) in ii), iii) and v) as selection candidate. If the received signal quality from none of the candidate PLMN(s) or PLMN/access technology combination(s) is equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM, the MS shall stop applying signal level enhanced network selection and repeat the network selection procedure as specified in clause 4.4.3.1.

[TS 23.122, clause 4.4.3.3.1]

The attempts to access the HPLMN or an EHPLMN or higher priority PLMN shall be as specified below:

…

j)	In steps i), ii) and iii) of clause 4.4.3.1.1, if signal level enhanced network selection is applicable (see clause 3.11 and step d) of clause 4.4.3.5), the MS shall only select a PLMN, if the received signal quality of the candidate PLMN/access technology combination is equal to or greater than the "Operator controlled signal threshold per access technology".

[TS 23.122, clause 3.11]

Signal level enhanced network selection is optionally supported by the home operator.

Signal level enhanced network selection applies only to NB-IoT, GERAN EC-GSM-IoT and Category M1 or M2 of E-UTRA. An MS supporting any, or a combination, of NB-IoT, GERAN EC-GSM-IoT and Category M1 or M2 of E-UTRA shall apply signal level enhanced network selection if the following conditions are fulfilled:

1)	The MS is in automatic PLMN selection mode;

2)	The MS supports the "Operator controlled signal threshold per access technology" as specified in 3GPP TS 22.011 [19];

3)	The MS is configured for using signal level enhanced network selection as specified in 3GPP TS 24.368 [50];

4)	The MS is configured for using signal level enhanced network selection as specified in 3GPP TS 31.102 [40]; and

5)	The "Operator controlled signal threshold per access technology" is configured in the USIM as specified in 3GPP TS 31.102 [40].

NOTE 1:	The usage of the "Operator controlled signal threshold per access technology" is intended only for IoT stationary devices (see 3GPP TS 22.011 [19]).

NOTE 2:	"Operator controlled signal threshold per access technology" is not expected to be supported by non-IoT devices.

The HPLMN can configure the MS with an "Operator controlled signal threshold per access technology" stored in the USIM (see 3GPP TS 31.102 [40]), which consists of one or more entries, each containing:

a)	a home operator controlled signal threshold; and

b)	an access technology.

The "Operator controlled signal threshold per access technology" is specific for a certain access technology and when applicable, applies to all allowable PLMNs with the corresponding access technology combination.

22.2.14.3	Test description

22.2.14.3.1	Pre-test conditions

System Simulator:

-	Four multi-PLMN Ncells as specified in TS 36.508 clause 8.1.4.1.2 are configured broadcasting PLMNs as indicated in Table 22.2.14.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 22.2.14.3.1-1.

Table 22.2.14.3.1-1: PLMN identifiers

| Ncell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |
| 3 | PLMN3 | 001 | 31 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is registered to PLMN1 before it is switched off.

-	System information combination 3 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in NB-IoT cells.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 22.2.14.3.1-2.

-	The UE is configured for using SENSE as specified in 3GPP TS 24.368 which could be done by MMI or AT command.

Table 22.2.14.3.1-2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 | PLMN1 Remaining mandatory entries use default values | E-UTRAN |
| EFOPLMNwACT | 1 | PLMN2 Remaining defined entries use default values | E-UTRAN |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFUST |  | Services 20, 42, 43, 74 and 148 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (2 hours) |  |
| EFOCST |  | 1(SENSE enabled by operator) 5B’H(Operator controlled signal threshold for NB-IoT is -91dBm) | E-UTRAN |

Preamble:

-	The UE is made to camp on Ncell 12 and then Switched OFF (State 1-NB).

22.2.14.3.2	Test procedure sequence

Table 22.2.14.3.2-1 shows the Ncell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”, “T2” etc are applied at the points indicated in the Main behaviour description in Table 22.2.14.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 22.2.14.3.2-1: Ncell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 12 | Ncell 13 | Ncell 3 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | “Off” | -85 | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | -97 | -97 | -85 | ”Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | NRS EPRE | dBm/15kHz | -97 | -85 | -85 | ”Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T3 | NRS EPRE | dBm/15kHz | -97 | ”Off” | -97 | -97 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 22.2.14.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts Ncell levels according to row T1 of table 22.2.14.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 13? | --> | RRCConnectionRequest-NB | 1 | P |
| 4-15 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 13. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 16 | SS adjusts Ncell levels according to row T2 of table 22.2.14.3.2-1 | - | - | - | - |
| 17 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 12 after 120 seconds, but before 132 mins (Note 1, Note 2) from power on? | --> | RRCConnectionRequest-NB | 2 | P |
| 18-22 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 8.1.5A.5 are performed on Ncell 12. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 23 | SS adjusts Ncell levels according to row T3 of table 22.2.14.3.2-1 | - | - | - | - |
| 24 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 1? | --> | RRCConnectionRequest-NB | 3 | P |
| 25-29 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 8.1.5A.5 are performed on Ncell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| Note 1: Following attempts to access the HPLMN/EHPLMN/higher priority PLMN in VPLMN is operator specific setting (Refer to TS 23.122 Rel-13). Hence, window between 120s to T+Tolerance is being used, where the high priority PLMN search timer T defined by EFHPPLMN. Note 2: Tolerance of 12min is added to allow time for the UE to find the proper PLMN |  |  |  |  |  |

22.2.14.3.3	Specific message contents

None

### 22.2.15	NB-IoT / SENSE/ PLMN selection of RPLMN or (E)HPLMN / Automatic mode

22.2.15.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and signal level enhanced network selection is applicable and UE is fitted with a USIM containing the EHPLMN list and the USIM indicates RPLMN or (E)HPLMN should be selected and received signal quality of RPLMN is equal to or greater than the "Operator controlled signal threshold per access technology" and received signal quality of EHPLMN is lower than the "Operator controlled signal threshold per access technology" }

ensure that {

when { UE is switched on }

then { UE selects a cell of the RPLMN }

}

(2)

with { UE in Automatic network selection mode and signal level enhanced network selection is applicable and UE is fitted with a USIM containing the EHPLMN list and the USIM indicates RPLMN or (E)HPLMN should be selected and received signal quality of RPLMN and EHPLMN are equal to or greater than than the "Operator controlled signal threshold per access technology" }

ensure that {

when { UE is switched on }

then { UE selects a cell of the RPLMN or EHPLMN }

}

(3)

with { UE in Automatic network selection mode and signal level enhanced network selection is applicable and UE is fitted with a USIM not containing or containing empty EHPLMN list and the USIM indicates RPLMN or (E)HPLMN should be selected and received signal quality of RPLMN and VPLMN are equal to or greater than than the "Operator controlled signal threshold per access technology" and received signal quality of HPLMN is lower than the "Operator controlled signal threshold per access technology" }

ensure that {

when { UE is switched on }

then { UE selects a cell of the RPLMN }

}

(4)

with { UE in Automatic network selection mode and signal level enhanced network selection is applicable and UE is fitted with a USIM not containing or containing empty EHPLMN list and the USIM indicates RPLMN or (E)HPLMN should be selected and received signal quality of RPLMN, HPLMN and VPLMN are equal to or greater than the "Operator controlled signal threshold per access technology" }

ensure that {

when { UE is switched on }

then { UE selects a cell of the RPLMN or HPLMN }

}

22.2.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1. Unless otherwise stated these are Rel-18 requirements.

[TS 23.122, clause 4.4.3.1]

EXCEPTION: As an alternative option to this, if the MS is in automatic network selection mode and it finds coverage of an EHPLMN, the MS may register to that EHPLMN and not return to the registered PLMN or equivalent PLMN. If the EHPLMN list is not present or is empty, and the HPLMN is available, the MS may register on the HPLMN and not return to the registered PLMN or equivalent PLMN. The operator shall be able to control by SIM configuration whether an MS that supports this option is permitted to perform this alternative behaviour. If signal level enhanced network selection is applicable (see clause 3.11), the MS may register to that EHPLMN or HPLMN only over an access technology for which the received signal quality is equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM.

…

22.2.15.3	Test description

22.2.15.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN cells as specified in TS 36.508 clause 8.1.4.1.2 are configured broadcasting PLMNs as indicated in Table 22.2.15.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 22.2.15.3.1-1.

Table 22.2.15.3.1-1: PLMN identifiers

| Ncell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is registered to PLMN1 with USIM A inserted before it is switched off.

-	System information combination 3 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in NB-IoT cells.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.8.3.1-2 and Table 6.1.1.8.3.1-3 will be used.

-	The UE is configured for using SENSE as specified in 3GPP TS 24.368 which could be done by MMI or AT command.

Table 22.2.15.3.1-2: USIM A configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFEPSLOCI |  | PLMN1 |  |
| EFUST |  | Service n°71, n°74, n°85 and n°148 are "available" |  |
| EFEHPLMN | 1 | PLMN2 |  |
| EFLRPLMNSI |  | 01 |  |
| EFOCST |  | 1’H(SENSE enabled by operator) 5B’H(Operator controlled signal threshold for E-UTRAN is -91dBm) | E-UTRAN |

Table 22.2.15.3.1-3: USIM B configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFEPSLOCI |  | PLMN1 |  |
| EFUST |  | Service n°74, n°85 and n°148 is "available" |  |
| EFEHPLMN |  | Empty |  |
| EFLRPLMNSI |  | 01 |  |
| EFOCST |  | 1’H (SENSE enabled by operator) 5B’H(Operator controlled signal threshold for E-UTRAN is -91dBm) | E-UTRAN |

Preamble:

-	The UE is made to camp on Ncell 12 and then Switched OFF (State 1).

22.2.15.3.2	Test procedure sequence

Table 22.2.15.3.2-1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”, “T2” etc are applied at the points indicated in the Main behaviour description in Table 22.2.15.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 22.2.15.3.2-1: Ncell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 12 | Ncell 13 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | “Off” | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | -85 | -85 | -97 |  |
| T2 | NRS EPRE | dBm/15kHz | -85 | -85 | -85 |  |
| T3 | NRS EPRE | dBm/15kHz | -97 | -85 | -85 |  |
| T4 | NRS EPRE | dBm/15kHz | -85 | -85 | -85 |  |

Table 22.2.15.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts Ncell levels according to row T1 of table 22.2.15.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 12? | --> | RRCConnectionRequest-NB | 1 | P |
| 4-15 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 12. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 16a1 to 16b1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 16a1-16a4 | If pc_SwitchOnOff then switch off procedure defined in TS 36.523-3 Table 10.3.2.1-1 Steps 2a1-2a4 is performed. | - | - | - | - |
| 16b1 | Else power off the UE |  |  |  |  |
| 17 | SS adjusts Ncell levels according to row T2 of table 22.2.15.3.2-1 | - | - | - | - |
| 18 | Power on the UE. | - | - | - | - |
| 19 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 12 or Ncell 13? | --> | RRCConnectionRequest-NB | 2 | P |
| 20-31 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 12 or Ncell 13. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 32a1 to 32b1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 32a1-32a4 | If pc_SwitchOnOff or pc_USIM_Removal then switch off procedure defined in TS 36.523-3 Table 10.3.2.1-1 Steps 2a1-2a4 is performed. | - | - | - | - |
| 32b1 | Else power off the UE. |  |  |  |  |
| 33 | The SS adjusts Ncell levels according to row T0 of table 22.2.15.3.2-1. | - | - | - | - |
| 34 | The UE is brought back to operation with USIM B inserted. | - | - | - | - |
| 35-46 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 12. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 47a1 to 47b1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 47a1-47a4 | If pc_SwitchOnOff then switch off procedure defined in TS 36.523-3 Table 10.3.2.1-1 Steps 2a1-2a4 is performed. | - | - | - | - |
| 47b1 | Else power off the UE. |  |  |  |  |
| 48 | SS adjusts Ncell levels according to row T3 of table 22.2.15.3.2-1 | - | - | - | - |
| 49 | Power on the UE. | - | - | - | - |
| 50 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 12? | --> | RRCConnectionRequest-NB | 3 | P |
| 51-62 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 12. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 63a1 to 63b1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 63a1-63a4 | If pc_SwitchOnOff then switch off procedure defined in TS 36.523-3 Table 10.3.2.1-1 Steps 2a1-2a4 is performed. | - | - | - | - |
| 63b1 | Else power off the UE. |  |  |  |  |
| 64 | SS adjusts Ncell levels according to row T4 of table 22.2.15.3.2-1 | - | - | - | - |
| 65 | Power on the UE. | - | - | - | - |
| 66 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 12 or Ncell 1? | --> | RRCConnectionRequest-NB | 4 | P |
| 67-78 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 12 or Ncell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

22.2.15.3.3	Specific message contents

None