3GPP TSG-RAN5 Meeting #106	R5-250526

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3697 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of minimum requirements and new test case 7.3.4.1 for FR2 LTM PCell switch from FR2 to FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add minimum requirements for FR2 LTM PCell switch TC - NR SA FR2 LTM PCell Switch test case 7.3.4.1 has been missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case and cell configuration mapping for the test case have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete and test case would be missing in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4.0 (new), 7.3.4.1 (new), E.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.522 CR 0548 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of change 1--------

7.3.4	LTM PCell Switch

7.3.4.0	Minimum conformance requirements

7.3.4.0.1	Minimum conformance requirements for NR FR2 LTM PCell Switch

Same as section 6.3.4.0.1

7.3.4.1	NR SA FR2 RACH based Intra-frequency PCell switch from FR2 to FR2

Editor's Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

The test is incomplete for UE power classes other than PC3

The test is incomplete for test frequencies > 40.8 GHz

The test is incomplete in following aspects:

- Some parameters are in brackets.

- TT analysis is missing.

7.3.4.1.1	Test purpose

To verify the UE's ability to perform NR FR2-NR FR2 RACH-based intra frequency PCell switch specified in 38.133 [6] clause 6.3.1, including cell switch delay and interruption length, for both with and without early TCI state activation.

7.3.4.1.2	Test applicability

This test applies to all types of NR UEs supporting NR LTM for MCG from Release 18 onwards.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

7.3.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.3.4.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.4.1.

7.3.4.1.4	Test description

7.3.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 7.3.4.1.4.1-1.

Table 7.3.4.1.4.1-1: Intra-frequency cell switch from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.3.4.1.4.1-2.

Table 7.3.4.1.4.1-2: Initial conditions for Intra-frequency cell switch from FR2 to FR2

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.4.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 7.3.4.1.4.1-3.

2.	Message contents are defined in clause 7.3.4.1.4.3.

3.	Two cells are deployed in the test, which are FR2 PCell (Cell 1) and FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The power levels and settings are according to Annex C.1.2 and C.1.3 for this test.

Table 7.3.4.1.4.1-3: General test parameters for Intra-frequency cell switch from FR2 to FR2

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 2 |  |  |  |  |
| A3-Offset |  | dB | [-15] |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 0.3 s |  |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | Candidate DLorJoint TCI.State.0 | Candidate DLorJoint TCI.State.2 | Candidate DLorJoint TCI.State.1 | Candidate DLorJoint TCI.State.3 | As specified in clause A.10B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | Candidate DLorJoint TCI.State.1 | Candidate DLorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | Candidate UL TCI.State.0 | N/A | Candidate UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.2 |  |  |  |  |

7.3.4.1.4.2	Test procedure

Two cells are deployed in the test, which are FR2 PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. Test configurations are given in table 7.3.4.1.4.1-1.

Both cell switch delay and interruption length are tested by using the parameters in table 7.3.4.1.4.1-3, and 7.3.4.1.5-1.

The test consists of four successive time periods, with time durations of T1, T2, T3 and T4, respectively.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 7.3.4.1.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	SS shall transmit an RRCReconfiguration message to configure event A3 triggered measurement reporting on the intra-frequency carrier of Cell 2.

4.	UE shall transmit RRCReconfigurationComplete message.

5.	If the UE transmit an A3 message report, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.4.1.5-1. T2 starts.

6.	At the start of T2, the SS shall transmit an RRCReconfiguration message with LTM-Candidate-r18 for Cell2, and SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on the candidate cell (Cell 2) in PUCCH format 2.

7.	UE shall transmit RRCReconfigurationComplete message.

8.	If the UE transmits a valid L1-RSRP result of Cell2:

-	Test 1A and 1B: The SS shall switch the power setting from T2 to T3 as specified in Table 7.3.4.1.5-1. T3 starts.

-	Test 2A and 2B: T3 is skipped. The SS shall switch the power setting from T2 to T4 as specified in Table 7.3.4.1.5-1. T4 starts, go to step10.

9.	At the start of T3, the SS shall transmit a Candidate Cell TCI States Activation MAC-CE (Refer to TS 38.321 [12], clauses 5.18.36, 6.1.3.76) of Cell2. 50ms after the candidate cell TCI state activation MAC-CE transmission, the SS shall switch the power setting from T3 to T4 as specified in Table 7.3.4.1.5-1. T4 starts.

candidate cell TCI activation MAC-CE:

-	Test 1A: CandidateTCI-State#1 is activated.

-	Test 1B: CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

10.	The start of T4 is the instant when the last TTI containing the LTM cell switch command MAC CE(Refer TS 38.321 [12], clauses 6.1.3.75) is sent by Cell1 to the UE.

cell switch MAC-CE:

-	Contention-Free Random-Access Resources are indicated and the field of Timing Advance Command is set to FFF

-	Test 1A: CandidateTCI-State#2 is indicated.

-	Test 1B: CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated

-	Test 2A: CandidateTCI-State#1 is indicated.

-	Test 2B: CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated

11.	If the UE transmits the uplink PRACH channel to Cell 2 less than DLTM from the beginning of time period T4 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.T4 ends.

12.	After T4 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync, which includes the release of LTM-Config-r18, to cause UE handover back to Cell 1.

13.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1

14.	Power off Cell2 and set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 3) mod1008) for next iteration of the test procedure loop. Upon setting the new physical cell identity for Cell 2, power on Cell 2 to ensure Cell 2 is available for the following test steps.

15.	Repeat steps 2-14 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.3.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.3.4.1.4.3-1: Common Exception messages (step 3)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and no GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.3 FR2, SMTC.1 and Synchronous cells Table H.3.1-4 with A3-offset = [-15dB] Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ |

Table 7.3.4.1.4.3-2: Common Exception messages (step 6)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.12.1 with Condition LTM L1-RSRP Table H 3.12-2 with Condition LTM L1-RSRP Table H.3.12-3 for Cell 2 and Condition Candidate config and TCI_INFO Table H.3.12.4 Table H.3.12-5 with pucch-ResourceId = 8 Table H.3.12-6, H.3.12-7 and H.3.12-13 with SSB index = 0 Table H.3.12-8 Table H.3.12-9 for Cell 2 Table H.3.12-10 with Condition Joint AND Early DL Sync for Test 1A Table H.3.12-10 with Condition Seperate for Test 1B and Test 2B Table H.3.12-10 with Condition Joint for Test 2A Table H.3.12-11 |

Table 7.3.4.1.4.3-3 RACH-ConfigCommon for Contention Free Random Access

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric specified in TS 38.508-1 [14] Table 7.3.1-14 with condition FR2 |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n16 |  |  |
| } |  |  |  |
| rsrp-ThresholdSSB | RSRP_51 |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| 0 |  |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 120 |  |  |
| } |  |  |  |

Table 7.3.4.1.4.3-4: RACH-ConfigDedicated for Contention Free Random Access

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 1 entry |  |  |
| ssb[1] | 0 |  |  |
| ra-PreambleIndex[1] | 40 |  |  |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.3.4.1.5	Test requirements

Table 7.3.4.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.3.4.1.5-1: Cell specific test parameters for NR FR2-FR2 Intra frequency cell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 ~ T4 | T1 ~ T4 |
| NR RF Channel Number |  |  |  | 1 | 1 |
| Assumption for UE beamsNote 6 |  |  |  | Rough | Rough |
| AoA setup |  |  |  | Setup 1 as defined in A.9.1 |  |
| Duplex mode |  |  |  | TDD |  |
| TDD configuration |  |  |  | [TDDConf.3.1] |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |
| PDSCH Reference |  |  |  | SR3.1 TDD |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |
| CP length |  |  |  | Normal |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 |  |
| PRACH configuration |  |  |  | PRACH.6 FR2 |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7+TT |  |
| Note2 |  |  | dBm/SCS | -95.7+TT |  |
|  |  |  | dB | -1.8+TT | 0+TT |
|  |  |  | dB | 6+TT | 7+TT |
| SSB_RP |  |  | dBm/SCS | -89.7+TT | -88.7+TT |
| IoNote3 |  |  | dBm/ 95.04MHz | -56.7+TT | -56.7+TT |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 in no later than DLTM from the beginning of time period T4. The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3ms and is specified in 38.133 [6] clause 6.3.1.2.

TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms and is specified in 38.133 [6] clause 6.3.1.2.1

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B

-	TLTM-IU_=20ms

-	TLTM-RRC-processing =10 ms if UE does not support ltm-FastProcessingConfig-r18, otherwise TLTM-RRC-processing =0ms

-	TLTM-processing =10 ms if the UE supports ltm-FastUE-Processing-r18 capability and UE reports 10 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing =15 ms if the UE supports ltm-FastUE-Processing-r18 capability and UE reports 15 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing =20 ms if the UE does not support ltm-FastUE-Processing-r18 capability.

----- End of change 1------

----- skipped unchanged clauses ------

----- Start of change 2 ------

E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for NR/5GC FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 | NR SA FR2 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | NR Cell 1 | NR Cell 23 |  |  |  |
| … |  |  |  |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.4.1 | NR SA FR2 RACH based Intra-frequency PCell switch from FR2 to FR2 | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.4.1.1 | NR SA FR2 UE transmit timing accuracy | NR Cell 1 |  |  |  |  |
| 7.4.1.3 | NR SA FR2 UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 | NR Cell 1 |  |  |  |  |
| 7.4.3.1 | NR SA FR2 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |
| … |  |  |  |  |  |  |
| 7.7.7.1.1 | NR SA FR2 CSI-RSRP measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 7.7.7.2.1 | NR SA FR2-FR2 CSI-RSRP measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |

----- End of change 2------