3GPP TSG-RAN5 Meeting #106	R5-250535

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3621 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PIXIT for SL PDCP Reordering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Tejet, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PDCP layer in sidelink communication should provide reordering function, but this function cannot be tested under the current conditions. The analysis is as follows: Firstly, different from other scenarios, the specifications specify that the t-reordering of sidelink communication depends on UE implementation, as described in TS 38.323 clause 7.3: “c) t-Reordering The duration of the timer is configured by upper layers TS 38.331 [3], except for the case of NR sidelink communication or sidelink SRB4. For NR sidelink communication or sidelink SRB4, the t-Reordering timer is determined by the UE implementation. This timer is used to detect loss of PDCP Data PDUs as specified in clause 5.2.2. If t-Reordering is running, t-Reordering shall not be started additionally, i.e. only one t-Reordering per receiving PDCP entity is running at a given time.” Secondly, there is no related definition to configure the sidelink parameter t-Reordering in current specifications. The above conditions mean that the System Simulator cannot configure or obtain the t-reordering value of sidelink UE via standardized signaling messages, and thus the reordering function of the SL PDCP layer cannot be tested. Therefore, a PIXIT of sidelink PDCP reordering timer is introduced to enable UE to claim the t-reordering value configured by itself. By setting the value in the test procedure. the reordering function of SL PDCP layer can be tested. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new PIXIT of SL PDCP Reordering has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reordering function of PDCP in sidelink communication will not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-1 CR 4848 TS/TR 38.523-2 CR 0554 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.2	E-UTRA and NR PIXIT

Table 9.2-1: E-UTRA and NR PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_ENDC_BandCombination | ENDC_BandCombination_Type | DC_1A_n28A |  | EN-DC Band Combination |
| px_ENDC_CA_BandCombination | ENDC_CA_BandCombination_Type | DC_1A_n28A_n78A |  | EN-DC CA Band Combination |
| px_ENDC_SecondaryBandCombination | ENDC_BandCombination_Type | DC_1A_n77A |  | Secondary EN-DC Band Combination |
| px_IPv4_Address_Other | charstring |  |  | Other IPv4 Address (see NOTE 4) |
| px_IPv6_Address_Other | charstring |  |  | Other IPv6 Address (see NOTE 4) |
| px_IPv4_Address4_NW | charstring |  |  | IPv4 Gateway Address in PDN4 |
| px_IPv6_Address4_NW | charstring |  |  | IPv6 Gateway Address in PDN4 |
| px_IPv4_Address4_UE | charstring |  |  | IPv4 Address connected to PDN4 |
| px_IPv6_Address4_UE | charstring |  |  | IPv6 Address connected to PDN4 |
| px_IPv4_Address5_NW | charstring |  |  | IPv4 Gateway Address in PDN5 |
| px_IPv6_Address5_NW | charstring |  |  | IPv6 Gateway Address in PDN5 |
| px_IPv4_Address5_UE | charstring |  |  | IPv4 Address connected to PDN5 |
| px_IPv6_Address5_UE | charstring |  |  | IPv6 Address connected to PDN5 |
| px_IPv4_Address6_NW | charstring |  |  | IPv4 Gateway Address in PDN6 |
| px_IPv6_Address6_NW | charstring |  |  | IPv6 Gateway Address in PDN6 |
| px_IPv4_Address6_UE | charstring |  |  | IPv4 Address connected to PDN6 |
| px_IPv6_Address6_UE | charstring |  |  | IPv6 Address connected to PDN6 |
| px_MUSIM_PeriodicGapPattern | integer | 0 |  | The ID of a MUSIM periodic gap pattern supported by the UE |
| px_NEDC_BandCombination | NEDC_BandCombination_Type | DC_n28A_3A |  | NE-DC Band Combination |
| px_NEDC_SecondaryBandCombination | NEDC_BandCombination_Type | DC_n28A_39A |  | Secondary NE-DC Band Combination |
| px_NR_CA_BandCombination | NR_CA_BandCombination_Type | CA_n1A_n3A |  | NR CA Band Combination with 2CC |
| px_NR_CA_3CC_BandCombination | NR_CA_3CC_BandCombination_Type | CA_n257H |  | NR CA Band Combination with 3CC |
| px_NR_DC_BandCombination | NR_DC_BandCombination_Type | DC_n78A_n257A |  | NR-DC Band Combination |
| px_NR_DC_CA_BandCombination | NR_DC_CA_BandCombination_Type | DC_n78A_n257G |  | NR-DC CA Band Combination |
| px_NR_CipheringAlgorithm | CipheringAlgorithm | nea2 |  | Ciphering Algorithm (see Note 1) |
| px_NR_IntegrityProtAlgorithm | IntegrityProtAlgorithm | nia2 |  | Integrity Algorithm (see Note 1) |
| px_NR_OverlappingNotSupportedBand_MFBI | integer | 1 |  | A not supported NR band that is overlapping with a supported band (px_NR_PrimaryBand). Applied to MFBI test case scenario. |
| px_NR_OverlappingSupportedBand_MFBI | integer | 1 |  | Another supported NR band that is overlapping with a supported band (px_NR_PrimaryBand). Applied to MFBI test case scenario. |
| px_NR_PrimaryBand | integer | 1 |  | NR primary band |
| px_NR_SecondaryBand | integer | 2 |  | NR secondary band. Applied to inter-band and SUL test cases. |
| px_NR_SidelinkBand | integer | 47 |  | NR sidelink band |
| px_NR_RATComb_Tested | NR_RATComb_Tested_Type | NR_UTRA | NR_UTRA, NR_GERAN | This parameter represents the network RAT capability / preference and indicates which, if any is supported, RAT combination is to be tested. |
| px_NR_V2X_ConcurrentOperationBandCombination | NR_V2X_ConcurrentOperationBandCombination_Type | V2X_n71_n47 |  | V2X operating bands for con-current operation |
| px_Sidelink_PDCP_ReorderTimer | Value in ms | ms1500 |  | The value of t-Reordering timer configured by UE |
| NOTE 1: Unless specified otherwise in the test case prose, the null algorithm shall not be used for verification. NOTE 2: Void. NOTE 3: Void. NOTE 4: IP address used for specific IP-address-related test requirements. The only requirements applying to this IP address is that its value shall be different from the values of all other IP address PIXITs (see TS 36.523-3 [12] clause 9) and it shall not be used within more than one PDN (PDU session) within a test case. In the present version of this specification, it is only used within PDN1 NW. |  |  |  |  |