3GPP TSG-RAN5 Meeting #106			R5-250546

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

Agenda item:	5.5.1.13

Source: 	Qualcomm Inc

Title: 	NPDCCH simulation results for NB-NTN scenarios

Document for:	Discussion and Endorsement

## 1.	Introduction

TS 36.521-4 [4] has NPDCCH demodulation test cases with satellite access using legacy TDL models based on requirements defined in [5].

This paper provides the simulation results for NPDCCH NB-NTN test cases.

## 2.	Discussion

Pasting below relevant section of G.3.5 of 36.521-4 [4] which provides the procedure to come up with the minimum test time for NPDSCH and NPDCCH.

### G.3.5.2	Minimum Test Time procedure for PDSCH and PDCCH scenarios with 1% BLER requirement

1.	Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

2.	The minimum test time is determined by the time it took the residual BLER curve to converge within +/- 10% of the target 1% residual BLER (i.e. within 0.9% and 1.1% BLER) across all seeds.

3. 	If the residual BLER does not converge within the +/- 10% of the 1% target limit within a reasonable amount of time for any seed, increase SNR in steps of 0.1 dB and rerun the simulations within the same target BLER limit

4.	The additional delta SNR required to meet the residual BLER convergence limit is the added SNR uncertainty due to finite test time.

5.	This SNR uncertainty due to finite test time would be one of the MU term in the overall MU calculation.

NPDCCH Simulations for below mentioned NB-NTN NPDCCH demod test cases in [4] were run.

Table 8.3.1.2.1.3-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | Stand-alone | 128 | Anchor | R.NB.3 FDD | EPA5 | 1 | 1 | -4.9 | NB1, NB2 |
| 2 (Note 1) | Stand-alone | 1024 | Non-anchor | R.NB.3 FDD | ETU1 | 1 | 1 | -11.4 | NB1, NB2 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |  |

Figure1: NPDCCH BLER Simulations for Test1 of TC 8.3.1.2.1.3-1 (EPA5) at 1% target Pm-dsg SNR, 30 seeds

Figure2: NPDCCH BLER Simulations for Test2 of TC 8.3.1.2.1.3-1 (ETU1) at 1% target Pm-dsg SNR, 30 seeds

For both NPDCCH test1 and test2, the configured SNR had to be much lower than the spec SNR to see some non-zero BLER. It was especially challenging to find a suitable SNR for the ETU1 case since the spec SNR is already every low.

For the EPA5 case (test 1), 500 seconds can be used as the convergence time where all the seeds converge below the 1% BLER threshold.

Proposal 1: For the EPA5 (test 1), use 500 seconds as the minimum test time.

For the ETU1 case (test 2), 650 seconds can be used as the convergence time.

Proposal 2: For the ETU1 (test 2), use 650 seconds as the minimum test time.

Summarizing the convergence time in table 2-1 for the two NPDCCH scenarios

Table 2-1: Convergence time

| Scenario | REP | Rmax | G | Rmax*G | Spec SNR | Convergence time for +/- 10% threshold | Scaling factor for QC sim to acount for RTT | Updated QC convergence time for +/- 5% threshold | QC MNAS (# of TB) | Min Gap between NPDCCH with RTT | Actual GAP between NPDCCH with RTT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.3.1.2.1_test1_EPA5_GSO config_Anchor | 128 | 128 | 2 | 256 | -4.9 | 500 | 1.25 | 625 | 488 | 1129 | 1280 |
| 8.3.1.2.1_test2_ETU1_GSO config_Anchor | 1024 | 1024 | 1.5 | 1536 | -11.4 | 650 | 1 | 650 | 84 | 7102 | 7680 |

Proposal 3: Use the table 2-1 contents to update Annex G.4.6-1 minimum test time in TS 36.521-4

## 3.	Conclusion

Proposal 1: For the EPA5 (test 1), use 500 seconds as the minimum test time.

Proposal 2: For the ETU1 (test 2), use 650 seconds as the minimum test time.

Proposal 3: Use the table 2-1 contents to update Annex G.4.6-1 minimum test time in TS 36.521-4

## 4.		References

[1]	R5-245291, Simulation results to derive min test time for nb-ntn demod test cases, Qualcoomm

[2]	R5-191250, Discussion paper on statistical tput calculation for demod test cases, Qualcomm

[3]	R5-194687, Discussion on minimum test time for PDSCH and PDCCH with more simulation results, Qualcomm

[4]	TS 36.521-4 v18.4.0

[5]	TS 36.102 v18.8.0

[6]	R5-242427, Discussion on minimum test time simulation methods for NB-IoT NTN, Mediatek

[7]	R5-245292, min test time update for nb-ntn demod test cases, Qualcomm, Mediatek India Technology Pvt.