3GPP TSG-RAN5 Meeting #106			R5-250548

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

Agenda item:	5.3.23.9

Source: 	Qualcomm Inc

Title: 	Discussion paper on FR2 PRACH UE beam correspondence

Document for:	Discussion and Endorsement

## 1.	Introduction

In response to the RAN4 LS [1] to RAN5 for the FR2 UE beam correspondence for initial access and RRC_inactive requirements, there has been several discussions in previous RAN5 meetings.

In [4] an alternate method without requiring beam lock test function was proposed. Action point AP#104.24 was created to include evaluating the testability aspects for testing beam correspondence requirement without beamlock function in addition to with beamlock

| AP#104.24 | RF | Check the testability aspects [with beam lock / without beam lock] for testing beam correspondence requirement in IDLE and INACTIVE mode. If the Beam lock function is confirmed not feasible the alternative method is considered. | Qualcomm, Huawei, Apple, Nokia | R5-244538, R5-246054, R5-245019, R5-244945 | RAN5#106 | Open |
| --- | --- | --- | --- | --- | --- | --- |

This paper provides the test procedure for verifying UE beam correspondence during initial access requirement without beamlock function.

## 2.	Discussion

### 2.1 Test configuration and flow without the use of Beamlock

As highlighted in past meetings, below are the key parameters/configuration to ensure a reliable way to verify the UE beam correspondence for initial access requirements.

Enabling multiple PRACH transmissions

Modify the preambleTransMax IE value to a large value (n200) to allow multiple PRACH transmissions.

For each received PRACH transmission, TE shall send a RAR containing Random access preamble identifier that do not match the transmitted random access preamble.

UE computed power reaches Pcmax before PRACH EIRP measurement

Modify the preambleReceivedTargetPower IE to a large value (-60) such that UE computed Tx power reaches Pcmax.

Accumulated PRACH power measurement period is atleast 1ms

Option1:

prach-ConfigurationIndex value 190 corresponds to PRACH format C2. Each RACH is 6 symbol duration with a CP+Seq duration of 41.67 us. This means for 1ms accumulated PRACH measurement period, 24 PRACH is needed.

Option2:

An even longer applicable PRACH format B4 can be configured for the test to reduce the number of PRACH needed to fulfil the 1ms PRACH measurement period. In this case only 10 PRACH is needed to fulfil 1ms accumulated PRACH.

# of SSB and SSB power level

Configuring a single SSB for this test will ensure UE has only one SSB to measure and send its PRACH towards the direction of the SSB.

Below is the test procedure to verify the SA FR2 PRACH beam correspondence requirements without beamlock.

Without UE_beamlock_idle

1.	Place UE in a starting coordinate.

- Page the UE to trigger RACH

3.	UE starts sending PRACH. TE sends RAR containing Random access preamble identifier that do not match the transmitted random access preamble

- TE waits till it collects 200 PRACH from the UE or max [5 seconds].

- TE uses a sliding window of X (where X is the number of RACH needed for total accumulated PRACH measurement period of 1ms. X = 24 for format C2 and X = 10 for format B4) and picks the set of consecutive RACH providing the highest average PRACH power. This is the EIRP PRACH power for this coordinate.

- Continue with above steps for each UE coordinate to complete the spherical scan.

The above approach is implemented in the RAN5 CR R5-250549 for TC 6.6.3.

Proposal 1: Endorse the approach described in this paper for TC 6.6.3 without UE beamlock_idle function.

## 3.	Conclusion

Proposal 1: Endorse the approach described in this paper for TC 6.6.3 without UE beamlock_idle function.

## 4.		References

[1]		R4-2317764, LS on testability issues of beam correspondence requirement for initial access, 	Huawei

[2]		R5-243159, Discussion on FR2 beam correspondence for initial access, Huawei

[3]	TS 38.101-2 v18.7.0

[4]	R5-245297, UE beam correspondence for initial access, Qualcomm