3GPP TSG-RAN5 Meeting #106	R5-250554

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0940 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to wideband CQI reporting inter-cell interference test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_demod_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Note4 is inconsistent in tables 6.2.2.2.2.3.3-1 and 6.2.3.2.2.3.3-1 compared to core spec No clarification regarding interference model for CSI reporting tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated Note4 to align with core spec in tables 6.2.2.2.2.3.3-1 and 6.2.3.2.2.3.3-1 - Fix a typo in Note6 in table 6.2.3.2.2.3.3-1 - Core spec alignment for new clause addition B.6.3 and reference to the same in all inter-cell interference CQI tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1.2.3, 6.2.2.2.2.3, 6.2.3.1.2.3, 6.2.3.2.2.3, B.6.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.101-4 CR 0722 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency R4-2500493 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

###### 6.2.2.1.2.3	2Rx FDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA

6.2.2.1.2.3.1	Test purpose

Verify that the UE is tracking the channel variations and selecting the largest transport format possible based on inter-cell interference mitigation receiver.

6.2.2.1.2.3.2	Test applicability

This test applies to all types of NR UEs and E-UTRAN UEs supporting EN-DC for release 15 and release 16 supporting MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference.

This test applies to all types of release 17 and forward NR UEs and E-UTRAN UEs supporting EN-DC.

6.2.2.1.2.3.3	Minimum conformance requirements

For the parameters specified in Table 6.2.2.1.2.3.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following,

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ where is specified in Table 6.2.2.1.2.3.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.2.1.2.3.3-1: Wideband CQI reporting test with inter-cell interference

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 10 | 10 |
| Duplex Mode |  |  |  | FDD | FDD |
| Subcarrier spacing |  |  | kHz | 15 | 15 |
| SINR |  |  | dB | -2 | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.3 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 | Row 5,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | - |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-1 | - |
| INR (Note 6) |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2×2 | 1×2 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time-synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR is defined in clause B.6.1. |  |  |  |  |  |

<< Skipped sections >>

###### 6.2.2.2.2.3	2Rx TDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA

6.2.2.2.2.3.1	Test purpose

Verify that the UE is tracking the channel variations and selecting the largest transport format possible based on inter-cell interference mitigation receiver.

6.2.2.2.2.3.2	Test applicability

This test applies to all types of NR UEs and E-UTRAN UEs supporting EN-DC for release 15 and release 16 supporting MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference.

This test applies to all types of release 17 and forward NR UEs and E-UTRAN UEs supporting EN-DC.

6.2.2.2.2.3.3	Minimum conformance requirements

For the parameters specified in Table 6.2.2.2.2.3.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following,

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ where is specified in Table 6.2.2.2.2.3.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.2.2.2.3.3-1: Wideband CQI reporting test with inter-cell interference (TDD)

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 40 | 40 |
| Duplex Mode |  |  |  | TDD | TDD |
| Subcarrier spacing |  |  | kHz | 30 | 30 |
| TDD UL-DL pattern |  |  |  | FR1.30-1 | FR1.30-1 |
| SINR |  |  | dB | -2 | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.3 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3(8) | Row 2(8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 10/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 10/9 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 9.5 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-3 |  |
| INR |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2×2 | 1×2 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR corresponds to Cell 2 is defined in clause B.6.1 |  |  |  |  |  |

<< Skipped sections >>

###### 6.2.3.1.2.3	4Rx FDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA

6.2.3.1.2.3.1	Test purpose

Verify that the UE is tracking the channel variations and selecting the largest transport format possible based on inter-cell interference mitigation receiver.

6.2.3.1.2.3.2	Test applicability

This test applies to all types of NR UEs and E-UTRAN UEs supporting EN-DC for release 15 and release 16 supporting MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference.

This test applies to all types of release 17 and forward NR UEs and E-UTRAN UEs supporting EN-DC.

6.2.3.1.2.3.3	Minimum conformance requirements

For the parameters specified in Table 6.2.3.1.2.3.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following,

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ where is specified in Table 6.2.3.1.2.3.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.3.1.2.3.3-1 Wideband CQI reporting test with inter-cell interference

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 10 | 10 |
| Duplex Mode |  |  |  | FDD | FDD |
| Subcarrier spacing |  |  | kHz | 15 | 15 |
| SINR |  |  | dB | -2 | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.3 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 | Row 5,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | - |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-1 | - |
| INR (Note 6) |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2×4 | 1×4 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR is defined in clause B.6.1. |  |  |  |  |  |

<< Skipped sections >>

###### 6.2.3.2.2.3	4Rx TDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA

6.2.3.2.2.3.1	Test purpose

Verify that the UE is tracking the channel variations and selecting the largest transport format possible based on inter-cell interference mitigation receiver.

6.2.3.2.2.3.2	Test applicability

This test applies to all types of NR UEs and E-UTRAN UEs supporting EN-DC for release 15 and release 16 supporting MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference.

This test applies to all types of release 17 and forward NR UEs and E-UTRAN UEs supporting EN-DC.

6.2.3.2.2.3.3	Minimum conformance requirements

For the parameters specified in Table 6.2.3.2.2.3.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following,

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ where is specified in Table 6.2.3.2.2.3.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.3.2.2.3.3-1: Wideband CQI reporting test with inter-cell interference (TDD)

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 40 | 40 |
| Duplex Mode |  |  |  | TDD | TDD |
| Subcarrier spacing |  |  | kHz | 30 | 30 |
| TDD UL-DL pattern |  |  |  | FR1.30-1 | FR1.30-1 |
| SINR |  |  | dB | -2 | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.3 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3(8) | Row 2(8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 10/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 10/9 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 9.5 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-3 |  |
| INR |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2×4 | 1×4 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR corresponds to Cell 2 is defined in clause B.6.1. |  |  |  |  |  |

<< Skipped sections >>

## B.6.2	Interference model for PDSCH requirements

This subclause provides synchronous network deployment interference model for each explicitly modelled interfering cell in the requirement scenario. In each subframe, each interfering cell shall transmit 16 QAM based randomly modulated data over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel. Transmitted physical channels shall include SSB and TRS/CSI-RS as specified in requirements section.

Transmission rank of the interfering cell shall be randomly determined with probabilities as specified in the requirements. The rank configuration will be independent for each interfering cell

For each slot a single precoding matrix for the number of layers υ associated to the selected rank shall be selected randomly from Table 5.2.2.2.1-1 of TS 38.214 [12] with PRB bundling size as given in the requirements.

The generic beamforming model in subclause B.4.1 shall be applied assuming number of antenna ports as specified in the requirement scenario.

Random precoding with selected rank and precoding matrices for each slot shall be applied to 16 QAM randomly modulated layer symbols including the demodulation reference symbols over antenna port 1000 when the rank is one and antenna ports 1000, 1001 when the rank is two. DMRS type 1 with front loaded single symbol and one additional DMRS position with FDM applied between DMRS and data (number of CDM groups without data is 1)

For REs in the control region, random precoding for the number of Tx antenna ports in the requirement scenario shall be applied to QPSK randomly modulated symbols. The EPRE ratio of these REs shall be as defined for PDCCH in Annex C.3.1

## B.6.3	Interference model for CSI reporting requirements

This subclause provides synchronous network deployment interference model for each explicitly modelled interfering cell in the CSI requirement scenario. In each subframe, each interfering cell shall transmit 16 QAM based randomly modulated data over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel. The SSB of the interfering cell shall be non-colliding with the SSB of the serving cell and TRS of the interfering cell shall be colliding with TRS of the serving cell. Interfering cell shall schedule PDSCH in the slot containing CSI-RS, allowing the CSI-IM of the serving cell to detect the interference.

For each slot a single precoding matrix for the number of layers υ associated to the selected rank shall be selected randomly from Table 5.2.2.2.1-1 of TS 38.214 [12] with PRB bundling size as given in the requirements.

The generic beamforming model in subclause B.4.1 shall be applied assuming number of antenna ports as specified in the requirement scenario.

Random precoding with selected rank and precoding matrices for each slot shall be applied to 16 QAM randomly modulated layer symbols including the demodulation reference symbols over antenna port 1000 when the rank is one and antenna ports 1000, 1001 when the rank is two. DMRS type 1 with front loaded single symbol and one additional DMRS position with FDM applied between DMRS and data (number of CDM groups without data is 1)

For REs in the control region, random precoding for the number of Tx antenna ports in the requirement scenario shall be applied to QPSK randomly modulated symbols. The EPRE ratio of these REs shall be as defined for PDCCH in Annex C.3.1. PDCCH of the interfering cell shall be non-colliding with PDCCH of the serving cell.

<< End of changes >>