3GPP TSG-RAN5 Meeting #106	R5-250564

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5534 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PMCH decoding performance test cases for LTE based 5G broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_bands_part2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | LTE based 5G broadcast WI requires adding the test cases to verify PMCH decoding and CAS detection performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new requirements for PMCH decoding for SCS 0.37 kHz and 2.5 kHZ Added new requirements for CAS detection Added RMC for testing PMCH decoding and CAS detection performance Added MBSFN propagation profile for SCS 1.25 KHz, 0.37 kHz, 2.5 kHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the LTE based 5G broadcast work plan will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 (new), A.3.18 (new), B.2.6.2 (new), B.2.6.3 (new), B.2.6.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 10.2_1.5	Test requirement

Same test requirements as in clause 10.2.5 with the following exceptions:

-	Instead of Table 10.2.5-1  use Table 10.2_1.5-1.

Table 10.2_1.5-1: Test requirement

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 4 | 1.4 MHz | R.40 TDD | OP.4 TDD | MBSFN channel model (Table B.2.6-1) | 1x2 low | 1 | 6.7 | ≥1 |

## 10.4	FDD with 5G terrestrial broadcast

### 10.4.0	Applicability of requirements

For all tests in section 10.4 the applicability rules are defined in Table 10.4.0-1 depending on the capabilities of the UE. The requirements in clause 10.4 are applicable to UEs that support 5G terrestrial broadcast.

Table 10.4.0-1: 5G terrestrial broadcast tests applicability

| Tests | Applicable if UE supports at least the following capability |
| --- | --- |
| Table 10.4.1.1-2 test 1 | 5G terrestrial broadcast support of mbms-ScalingFactor0dot37-r16 and timeSeparationSlot4-r16 |
| Table 10.4.1.1-2 test 2 | 5G terrestrial broadcast support of mbms-ScalingFactor0dot37-r16 and timeSeparationSlot2-r16 |
| Table 10.4.1.2-2 test 1 | 5G terrestrial broadcast support of mbms-ScalingFactor2dot5-r16 |
| Table 10.4.2.1-1 test 1 | 5G terrestrial broadcast support of mbms-ScalingFactor0dot37-r16 and 5G terrestrial broadcast support of PBCH repetition in CAS |
| Table 10.4.2.1-1 test 2 | 5G terrestrial broadcast support of mbms-ScalingFactor2dot5-r16 and 5G terrestrial broadcast support of PBCH repetition in CAS |

For the requirements defined in this section, the difference between CRS EPRE and the MBSFN RS EPRE should be set to 16dB for subcarrier spacing as 0.37kHz, 7.8 dB for subcarrier spacing as 2.5kHz because the UE demodulation performance might be different when this condition is not met (e.g. in scenarios where power offsets are present, such as scenarios when reserved cells are present).

### 10.4.1	Minimum requirement for PMCH decoding

#### 10.4.1.1		Minimum requirement with 0.37kHz subcarrier spacing

Editor’s note: This test case is incomplete. Following aspects are pending

- Initial cal setup

- Test procedure

- Message contents

- MU/TT analysis

- Minimum test time

The receive characteristic of MBMS is determined by the BLER.

For the parameters specified in Table 10.4.1.1-1 and Table A.3.8.1-9, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.4.1.1-2.

Table 10.4.1.1-1: Test Parameters for Testing

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Downlink power allocation | ρA | dB | 0 |
|  | ρB | dB | 0 (Note 1) |
|  |  | dB | 0 |
| Noc at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode in PCell |  |  | 1 |
| Subcarrier spacing for MBSFN cell |  | kHz | 0.37 kHz |
| Bandwidth |  | MHz | 10(Note 2) |
| Note 1: PB = 0. Note 2: For both Pcell and Scell. |  |  |  |

Table 10.4.1.1-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | MBSFN RS type | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | N/A | OP.1 FDD | AWGN | 1x1 | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.106-1 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 18.5 | ≥ 2 |
|  | MBMS Dedicated Cell | 6 | R.106-3 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 18.5 | ≥ 2 |
|  | MBMS Dedicated Cell | 7 | R.106-4 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 18.5 | ≥ 2 |
|  | MBMS Dedicated Cell | 8 | R.106-5 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 18.5 | ≥ 2 |
| 2 | PCell | 10 | N/A | N/A | OP.1 FDD | AWGN | 1x1 | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.106-2 FDD | Type 2 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 20.2 | ≥ 2 |
|  | MBMS Dedicated Cell | 6 | R.106-6 FDD | Type 2 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 20.2 | ≥ 2 |
|  | MBMS Dedicated Cell | 7 | R.106-7 FDD | Type 2 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 20.2 | ≥ 2 |
|  | MBMS Dedicated Cell | 8 | R.106-8 FDD | Type 2 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 20.2 | ≥ 2 |

The normative reference for this requirement is TS 36.101 [2] clause 10.4.1.1

##### 10.4.1.1.1	Test purpose

This test verifies the performance of PMCH with 0.37 kHz subcarrier spacing sent on a MBMS dedicated cell with a given SNR for which the average BLER remains below a given reference value.

##### 10.4.1.1.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 17 and forward and supporting 5G terrestrial broadcast and 0.37 kHz subcarrier spacing.

##### 10.4.1.1.3	Test description

10.4.1.1.3.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: As specified per test number in Table 10.2.3-3 as defined in TS 36.508 [7] clause 4.3.1.

FFS

10.4.1.1.3.2	Test procedure

FFS

##### 10.4.1.1.4	Message contents

Message contents are according to TS 36.508 [7] clauses 4.4, 4.6 and 4.7A with following exceptions:

FFS

##### 10.4.1.1.5	Test requirement

For the parameters specified in Table 10.4.1.1-1, Table 10.4.1.1-2, Annex A.3.18.1, and SNR in Table 10.4.1.1.5-1, the value for the BLER in step [TBD] shall be below the test limit in Annex [TBD] for all subtests shown in Table 10.4.1.1.5-1.

Table 10.4.1.1.5-1: Test Requirement

| Test number | Cell | Bandwidth (MHz) | Reference Channel | MBSFN RS type | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | N/A | OP.1 FDD | AWGN | 1x1 | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.106-1 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 18.5 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 6 | R.106-3 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 18.5 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 7 | R.106-4 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 18.5 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 8 | R.106-5 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 18.5 + TT | ≥ 2 |
| 2 | PCell | 10 | N/A | N/A | OP.1 FDD | AWGN | 1x1 | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.106-2 FDD | Type 2 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 20.2 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 6 | R.106-6 FDD | Type 2 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 20.2 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 7 | R.106-7 FDD | Type 2 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 20.2 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 8 | R.106-8 FDD | Type 2 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 20.2 + TT | ≥ 2 |

#### 10.4.1.2		Minimum requirement with 2.5kHz subcarrier spacing

Editor’s note: This test case is incomplete. Following aspects are pending

- Initial cal setup

- Test procedure

- Message contents

- MU/TT analysis

- Minimum test time

The receive characteristic of MBMS is determined by the BLER.

For the parameters specified in Table 10.4.1.2-1 and Table A.3.18.1, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.4.1.2-2.

Table 10.4.1.2-1: Test Parameters for Testing

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Downlink power allocation | ρA | dB | 0 |
|  | ρB | dB | 0 (Note 1) |
|  |  | dB | 0 |
| Noc at antenna port |  | dBm/15kHz | Noc at antenna port |
| PDSCH transmission mode in PCell |  |  | 1 |
| Subcarrier spacing for MBSFN cell |  | kHz | 2.5 kHz |
| Bandwidth |  | MHz | 10(Note 2) |
| Note 1: PB = 0. Note 2: For both Pcell and Scell. |  |  |  |

Table 10.4.1.2-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x2 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.107 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 | ≥ 2 |
|  | MBMS Dedicated Cell | 6 | R.107-1 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 | ≥ 2 |
|  | MBMS Dedicated Cell | 7 | R.107-2 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 | ≥ 2 |
|  | MBMS Dedicated Cell | 8 | R.107-3 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 | ≥ 2 |

The normative reference for this requirement is TS 36.101 [2] clause 10.4.1.2

##### 10.4.1.2.1	Test purpose

This test verifies the performance of PMCH with 2.5 kHz subcarrier spacing sent on a MBMS dedicated cell with a given SNR for which the average BLER remains below a given reference value.

##### 10.4.1.2.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 17 and forward and supporting 5G terrestrial broadcast and 2.5 kHz subcarrier spacing.

##### 10.4.1.2.3	Test description

10.4.1.2.3.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: As specified per test number in Table 10.2.3-3 as defined in TS 36.508 [7] clause 4.3.1.

FFS

10.4.1.2.3.2	Test procedure

FFS

##### 10.4.1.2.4	Message contents

Message contents are according to TS 36.508 [7] clauses 4.4, 4.6 and 4.7A with following exceptions:

FFS

##### 10.4.1.2.5	Test requirement

For the parameters specified in Table 10.4.1.2-1, Table 10.4.1.2-2, Annex A.3.18.1, and SNR in Table 10.4.1.2.5-1, the value for the BLER in step [TBD] shall be below the test limit in Annex [TBD] for all subtests shown in Table 10.4.1.2.5-1.

Table 10.4.1.2.5-1: Test requirement

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x2 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.107 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 6 | R.107-1 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 7 | R.107-2 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 8 | R.107-3 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |

### 10.4.2	Minimum requirement for CAS detection

#### 10.4.2.1		Minimum requirement for PBCH detection

Editor’s note: This test case is incomplete. Following aspects are pending

- Initial cal setup

- Test procedure

- Message contents

- MU/TT analysis

- Minimum test time

For the parameters specified in Table 8.6.1-1, the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 10.4.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 10.4.2.1-1: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-bch (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x1 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.23-1 | N/A | AWGN | 1x1 low | 1 | -7.2 | ≥ 2 |
| 2 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x2 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.23-1 | N/A | EVA 162Hz | 1x2 low | 1 | -6.7 | ≥ 2 |

##### 10.4.2.1.1	Test purpose

This test verifies the PBCH performance with a given SNR for which the average probability of a mis-detected PBCH (Pm-bch) remains below a given reference value.

##### 10.4.2.1.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 17 and forward and supporting 5G terrestrial broadcast and supporting CAS.

##### 10.4.2.1.3	Test description

10.4.2.1.3.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: As specified per test number in Table 10.2.3-3 as defined in TS 36.508 [7] clause 4.3.1.

FFS

10.4.2.1.3.2	Test procedure

FFS

##### 10.4.2.1.4	Message contents

Message contents are according to TS 36.508 [7] clauses 4.4, 4.6 and 4.7A with following exceptions:

FFS

##### 10.4.2.1.5	Test requirement

For the parameters specified in Table 8.6.1-1, Table 10.4.2.1-1, Annex A.3.18.1, and SNR in Table 10.4.2.1.5-1, the value for the average probability of mis-detected PBCH (Pm-bch) shall be below the test limit in Annex [TBD] for all subtests shown in Table 10.4.2.1.5-1.

Table 10.4.2.1.5-1: Test requirement

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x2 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.107 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 6 | R.107-1 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 7 | R.107-2 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 8 | R.107-3 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |

<<several sections skipped>>

#### A.3.17.2	TDD

Table A.3.17.2-1: Reference Channel TDD

| Parameter | Unit |  |  |
| --- | --- | --- | --- |
| Reference channel |  | R.sTTI.10 TDD | R.sTTI.11 TDD |
| Number of transmitter antennas |  | 4 | 2 |
| Channel bandwidth | MHz | 10 | 10 |
| Aggregation level | SCCE | 2 | 8 |
| DCI Format |  | 7-1C | 7-1F |
| Cell ID |  | 0 | 0 |
| Payload (without CRC) | Bits | 34 | 30 |

## A.3.18	Reference Measurement Channels for LTE based 5G broadcast PMCH receiver requirements

### A.3.18.1	SDO

Table A.3.18.1-1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 | R.PMCH.3 | R.PMCH.4 | R.PMCH.5 | R.PMCH.6 |
| PMCH bandwidth | MHz | 6 | 7 | 8 | 6 | 7 | 8 |
| Allocated resource blocks |  | 30 | 35 | 40 | 30 | 35 | 40 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 2152 | 2472 | 2856 | 14112 | 17568 | 18336 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 1 | 1 | 3 | 3 | 3 |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 6120 | 7140 | 8160 | 18360 | 21420 | 24480 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331 [7]. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

Table A.3.18.1-2 Fixed Reference Channel for PMCH Receiver Requirements (2.5 kHz SCS)

| Parameter | PMCH (2.5 kHz SCS) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  | R.PMCH.7 | R.PMCH.8 | R.PMCH.9 | R.PMCH.10 | R.PMCH.11 | R.PMCH.12 |
| PMCH bandwidth | MHz | 6 | 7 | 8 | 6 | 7 | 8 |
| Allocated resource blocks |  | 30 | 35 | 40 | 30 | 35 | 40 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 2152 | 2472 | 2856 | 14112 | 17568 | 19848 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 1 | 1 | 4 | 4 | 5 |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 6480 | 7560 | 8640 | 19440 | 22680 | 25920 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331 [7]. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

Table A.3.18.1-3 Fixed Reference Channel for PMCH Receiver Requirements (1.25 kHz SCS)

| Parameter | PMCH (1.25 kHz SCS) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  | R.PMCH.13 | R.PMCH.14 | R.PMCH.15 | R.PMCH.16 | R.PMCH.17 | R.PMCH.18 |
| PMCH bandwidth | MHz | 6 | 7 | 8 | 6 | 7 | 8 |
| Allocated resource blocks |  | 30 | 35 | 40 | 30 | 35 | 40 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 2152 | 3112 | 3496 | 16416 | 19080 | 21384 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 1 | 1 | 4 | 5 | 5 |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 7200 | 8400 | 9600 | 21600 | 25200 | 28800 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331 [7]. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

Table A.3.18.1-4 Fixed Reference Channel for PMCH Receiver Requirements (0.37 kHz SCS)

| Parameter | PMCH (0.37 kHz SCS) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  | R.PMCH.19 | R.PMCH.20 | R.PMCH.21 | R.PMCH.22 | R.PMCH.23 | R.PMCH.24 |
| PMCH bandwidth | MHz | 6 | 7 | 8 | 6 | 7 | 8 |
| Allocated resource blocks |  | 30 | 35 | 40 | 30 | 35 | 40 |
| Allocated subframes per 40ms (Note 1) |  | 13 | 13 | 13 | 13 | 13 | 13 |
| Modulation |  | QPSK | QPSK | QPSK | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload |  |  |  |  |  |  |  |
| For each slot | Bits | 9258 (MCS 6) | 11064 (MCS6) | 12576 (MCS6) | 57336 (MCS25) | 66592 (MCS25) | 78704 (MCS25) |
| Number of Code Blocks per Sub-Frame (Note 2) |  | 5 | 6 | 6 | 14 | 16 | 18 |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For each slot | Bits | 26760 | 31220 | 35680 | 80280 | 93660 | 107040 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: First subframe of every 40ms is allocated for non-MBMS transmission. Note 2 If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

<<several sections skipped>>

## B.2.6	MBSFN Propagation Channel Profile

Table B.2.6-1 shows propagation conditions that are used for the MBSFN performance requirements in multi-path fading environment in an extended delay spread environment.

Table B.2.6-1: Propagation Conditions for Multi-Path Fading Environments for MBSFN Performance Requirements in an extended delay spread environment

| Extended Delay Spread |  |
| --- | --- |
| Maximum Doppler frequency [5Hz] |  |
| Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 |
| 30 | -1.5 |
| 150 | -1.4 |
| 310 | -3.6 |
| 370 | -0.6 |
| 1090 | -7.0 |
| 12490 | -10 |
| 12520 | -11.5 |
| 12640 | -11.4 |
| 12800 | -13.6 |
| 12860 | -10.6 |
| 13580 | -17.0 |
| 27490 | -20 |
| 27520 | -21.5 |
| 27640 | -21.4 |
| 27800 | -23.6 |
| 27860 | -20.6 |
| 28580 | -27.0 |

### B.2.6.2	Subcarrier spacing 1.25kHz

Table B.2.6.2-1 shows propagation conditions that are used for the MBSFN performance requirements in multi-path fading environment in an extended delay spread environment for subcarrier spacing as 1.25kHz.

Table B.2.6.2-1: Propagation Conditions for Multi-Path Fading Environments for MBSFN Performance Requirements in an extended delay spread environment with subcarrier spacing 1.25kHz

| Extended Delay Spread |  |
| --- | --- |
| Maximum Doppler frequency [5Hz] |  |
| Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 |
| 30 | -1.5 |
| 150 | -1.4 |
| 310 | -3.6 |
| 370 | -0.6 |
| 1090 | -7.0 |
| 49960 | -10 |
| 49990 | -11.5 |
| 50110 | -11.4 |
| 50270 | -13.6 |
| 50330 | -10.6 |
| 51050 | -17.0 |
| 109960 | -20 |
| 109990 | -21.5 |
| 110110 | -21.4 |
| 110270 | -23.6 |
| 110330 | -20.6 |
| 111050 | -27.0 |

### B.2.6.3	Subcarrier spacing 0.37kHz

Table B.2.6.3-1 shows propagation conditions that are used for the MBSFN performance requirements in multi-path fading environment in an extended delay spread environment for subcarrier spacing as 0.37kHz.

Table B.2.6.3-1: Propagation Conditions for Multi-Path Fading Environments for MBSFN Performance Requirements in an extended delay spread environment with subcarrier spacing 0.37kHz

| Extended Delay Spread |  |
| --- | --- |
| Zero Doppler frequency |  |
| Relative Delay [μs] | Relative Mean Power [dB] |
| 0 | -11 |
| 130 | -10 |
| 220 | -4.5 |
| 240 | -3.5 |
| 400 | 0 |
| 520 | -13 |
| 650 | -20 |
| 800 | -25 |

### B.2.6.4	Subcarrier spacing 2.5kHz

Table B.2.6.4-1 shows propagation conditions that are used for the MBSFN performance requirements in multi-path fading environment in an extended delay spread environment for subcarrier spacing as 2.5kHz.

Table B.2.6.4-1: Propagation Conditions for Multi-Path Fading Environments for MBSFN Performance Requirements in an extended delay spread environment with subcarrier spacing 2.5kHz

| Extended Delay Spread |  |
| --- | --- |
| Maximum Doppler frequency [162Hz] |  |
| Relative Delay [ns] | Relative Mean Power [dB] |
| -310 | -3.6 |
| -280 | -1.5 |
| -160 | -1.4 |
| 0 | 0 |
| 60 | -0.6 |
| 780 | -7.0 |
| 49650 | -10 |
| 49680 | -11.5 |
| 49800 | -11.4 |
| 49960 | -13.6 |
| 50020 | -10.6 |
| 50740 | -17.0 |
| 109650 | -20 |
| 109680 | -21.5 |
| 109800 | -21.4 |
| 109960 | -23.6 |
| 110020 | -20.6 |
| 110740 | -27.0 |

<<End of changes>>