3GPP TSG-RAN5 Meeting #106	R5-250572

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4850 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of XR test case 8.1.5.10.7 - UAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case is added to test the UE could send UL Traffic information in UAI message. Normally, UE acquire UL traffic information through interacting between AS layer and Application layer. Application layer is not considered in this test case, so the UL Traffic information is indicated to AS layer by private AT/MMI command, which is based on UE implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new XR test case 8.1.5.10.7 UE Assistance Information / UL Traffic Information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_XR_enh_plus_CT1-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.10.7(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0556 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.5.10.7	UE Assistance Information / UL Traffic Information

8.1.5.10.7.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide UL traffic information and the UL traffic information is available, and UE did not transmit a UEAssistanceInformation message with ul-TrafficInfo since it was configured to provide UL traffic information }

then { the UE transmits a UEAssistanceInformation message }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide UL traffic information and the UL traffic information has changed with respect to the last reporting (UE determines that the information has been updated) and timer T346l is running }

then { the UE does not transmit a UEAssistanceInformation message }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide UL traffic information and the UL traffic information has changed with respect to the last reporting (UE determines that the information has been updated) and timer T346l is not running }

then { the UE transmits a UEAssistanceInformation message }

}

8.1.5.10.7.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clause 5.3.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.7.4.1]

The purpose of this procedure is for the UE to inform the network of:

…

-	UL traffic information; or

…

[TS 38.331, clause 5.7.4.2]

A UE capable of providing UL traffic information shall initiate the procedure when this information is available upon being configured to do so, and upon change of UL traffic information.

…

Upon initiating the procedure, the UE shall:

…

1>	if configured to provide UL traffic information:

2>	if the UE did not transmit a UEAssistanceInformation message with ul-TrafficInfo since it was configured to provide UL traffic information; or

2>	if UL traffic information included in the previous UEAssistanceInformation has changed since the last transmission of the UEAssistanceInformation message containing ul-TrafficInfo for at least one QoS flow for which timer T346l is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UL traffic information.

NOTE 5:	The UE only considers burstArrivalTime to have changed when it changes relative to the periodicity of the Data Burst arrival.

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide UL traffic information according to 5.7.4.2 or 5.3.5.3:

2>	for each PDU session for which the UE intends to provide UL traffic information in this UEAssistanceInformation message:

3>	set pdu-SessionID to the value of the concerned PDU session ID;

3>	if transmission of the UEAssistanceInformation message is initiated to provide UL traffic information according to 5.3.5.3:

4>	stop timer T346l for each QoS flow of this PDU session for which the UE intends to provide UL traffic information in this UEAssistanceInformation message;

3>	for each QoS flow of this PDU session for which timer T346l is not running and for which the UE intends to provide UL traffic information in this UEAssistanceInformation message:

4>	start timer T346l associated to this QoS flow with the timer value set to the value of ul-TrafficInfoProhibitTimer;

4>	set qfi to the value of the concerned QFI;

4>	if the jitter range measurement is available; and

4>	if the UE did not provide jitter range since it was configured to provide UL traffic information, or if the measured jitter range has changed since the last transmission of the UEAssistanceInformation message containing jitterRange:

5>	set jitterRange to the latest measured value of the jitter range;

4>	if the burst arrival time measurement is available; and

4>	if the UE did not provide burst arrival time since it was configured to provide UL traffic information, or if the measured burst arrival time has changed since the last transmission of the UEAssistanceInformation message containing burstArrivalTime:

5>	set burstArrivalTime to the latest measured value of the burst arrival time;

4>	if the traffic periodicity measurement is available; and

4>	if the UE did not provide traffic periodicity since it was configured to provide UL traffic information, or if the measured traffic periodicity has changed since the last transmission of the UEAssistanceInformation message containing trafficPeriodicity:

5>	set trafficPeriodicity to the latest measured value of the traffic periodicity;

4>	if the UE did not provide pdu-SetIdentification since it was configured to provide UL traffic information, or if the information previously provided in pdu-SetIdentification has changed since the last transmission of the UEAssistanceInformation message containing pdu-SetIdentification:

5>	if the UE is able to identify PDU Set(s) for the QoS flow:

6>	set pdu-SetIdentification to true;

5>	else:

6>	set pdu-SetIdentification to false.

4>	if the UE did not provide psi-Identification since it was configured to provide UL traffic information, or if the information previously provided in psi-Identification has changed since the last transmission of the UEAssistanceInformation message containing psi-Identification:

5>	if the UE is able to identify PSI(s) for the QoS flow:

6>	set psi-Identification to true;

5>	else:

6>	set psi-Identification to false.

[TS 38.331, clause 5.3.5.9]

1>	if the received otherConfig includes the ul-TrafficInfoReportingConfig:

2>	if ul-TrafficInfoReportingConfig is set to setup:

3>	consider itself to be configured to provide UL traffic information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide UL traffic information and stop all instances of timer T346l, if running;

[TS 38.331, clause 7.1.1]

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T346l (The UE maintains one instance of this timer per QoS flow) | Upon transmitting UEAssistanceInformation message with ul-TrafficInfo for the concerned QoS flow. | Upon releasing ul-TrafficInfoReportingConfig during the connection re-establishment/resume procedures, or upon receiving ul-TrafficInfoReportingConfig set to release. | No action. |

8.1.5.10.7.3	Test description

8.1.5.10.7.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	None

Preamble:

-	The UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.7.3.2	Test procedure sequence

Table 8.1.5.10.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure ul-TrafficInfoReportingConfig. | --> | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | <-- | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Set UL Traffic information for QFI 1. (Note 1) | - | - | - | - |
| 4 | Check: Does the UE transmit an UEAssistanceInformation message including UL-TrafficInfo-r18? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 5 | Change UL Traffic information for QFI 1. (Note 1) | - | - | - | - |
| 6 | Check: Does the UE transmit an UEAssistanceInformation message before T346l expiry? | - | - | 3 | F |
| 7 | SS waits for timer T346l expiry. | - | - | - | - |
| 8 | Check: Does the UE transmit an UEAssistanceInformation message including UL-TrafficInfo-r18? | --> | NR RRC: UEAssistanceInformation | 2 | P |
| Note 1: The request to set or change UL Traffic information is performed by MMI or AT Command. UL Traffic information includes at least one of jitter range, burst arrival time, data burst periodicity and PDU Set and PSI identification. |  |  |  |  |  |

8.1.5.10.7.3.3	Specific message contents

Table 8.1.5.10.7.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.10.7.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |
| ul-TrafficInfoReportingConfig-r18 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pdu-SessionsToReportUL-TrafficInfoList-r18 SEQUENCE (SIZE (1.. maxNrofPDU-Sessions-r17)) OF PDU-SessionToReportUL-TrafficInfo-r18 { | 1 entry |  |  |
| PDU-SessionToReportUL-TrafficInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| pdu-SessionID | The same value as the value set in internet PDU session |  |  |
| qfi-ToReportUL-TrafficInfoList-r18 SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI { | 1 entry |  |  |
| QFI[1] | ’00 0001’B | entry 1 QFI 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-TrafficInfoProhibitTimer-r18 | s30 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.7.3.3-2: UEAssistanceInformation (step 4 and step 8, Table 8.1.5.10.7.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ul-TrafficInfo-r18 SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionUL-TrafficInfo-r18 { | 1 entry |  |  |
| PDU-SessionUL-TrafficInfo-r18[1] SEQUENCE { |  | entry 1 |  |
| pdu-SessionID-r18 | The same value as the value set in internet PDU session |  |  |
| qos-FlowUL-TrafficInfoList-r18 SEQUENCE (SIZE (1..maxNrofQFIs)) OF QOS-FlowUL-TrafficInfo-r18 { | 1 entry |  |  |
| qos-FlowUL-TrafficInfoList-r18[1] SEQUENCE { |  | entry 1 |  |
| qfi-r18 | ’00 0001’B | QFI 1 |  |
| jitterRange-r18 | Not checked |  |  |
| burstArrivalTime-r18 | Not checked |  |  |
| trafficPeriodicity-r18 | Not checked |  |  |
| pdu-SetIdentification-r18 | Present but contents not checked |  | Step 4 |
|  | Not checked |  | Step 8 |
| psi-Identification-r18 | Present but contents not checked |  | Step 4 |
|  | Not checked |  | Step 8 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>