3GPP TSG-RAN WG5 Meeting #106	R5-250587

Athens, Greece, 17th – 21st Feb, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0187 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new PRS aggregation positioning procedure test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support PRS bandwidth aggregation for positioning measurements is one of the objectibve of the work item Expanded and improved NR positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 9.3.4.7 PRS aggregation positioning procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4.7(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0174 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

9.3.4.7	PRS aggregation positioning procedure

9.3.4.7.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {

when { UE receives assistance data and a LPP REQUEST LOCATION INFORMATION message indicating the UE is requested to perform joint measurement across aggregated PFLs }

then { UE sends a PROVIDE LOCATION INFORMATION message containing a location estimate and indicating the measurement is based on aggregated DL-PRS resources across PFLs }

}

9.3.4.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.214 clauses 5.1.6.5.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.214, clause 5.1.6.5.3]

When the UE is expected to perform aggregated measurements for bandwidth aggregation across DL PRS positioning frequency layers, the UE expects to be configured with linkage information, via higher layer parameter nr-DL-PRS-AggregationInfo, between DL PRS resource sets across DL PRS positioning frequency layers. For the linked DL PRS resource sets, the UE is expected to be configured with the same values of dl-PRS-QCL-Info, dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-NumSymbols, dl-PRS-ResourceTimeGap, dl-PRS-ResourceRepetitionFactor, dl-PRS-ResourceSymbolOffset, dl-PRS-MutingBitRepetitionFactor, dl-PRS-SubcarrierSpacing, dl-PRS-CyclicPrefix, dl-PRS-CombSizeN, dl-PRS-ResourcePower, NR-MutingPattern, and NR-DL-PRS-SFN0-Offset, and the UE is expected to be configured with DL PRS resources that maintain uniformly spaced DL PRS RE pattern within a symbol across aggregated DL PRS positioning frequency layers. The UE assumes that DL PRS resources across the linked DL PRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, and the UE may assume phase continuity on the DL PRS resources on same symbol(s); otherwise, the UE does not assume that PRS resources from the linked DL PRS resource sets are linked for bandwidth aggregation.

The UE may be indicated by the network that aggregated DL PRS resource set(s) can be used as the reference for the aggregated DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, and UE Rx-Tx time difference measurements.

The UE may be configured to measure and report, subject to UE capability, up to 4 aggregated DL RSTD measurement(s) per pair of dl-PRS-ID, from a different pair of aggregated DL PRS resources across two or three DL PRS positioning frequency layers. The UE may report up to 4 RSTD measurements associated with different aggregated DL PRS resources per UE Rx TEG per dl-PRS-ID.

The UE may be configured to measure and report, subject to UE capability, up to 4 aggregated UE Rx-Tx time difference measurement(s) from aggregated DL PRS resources across two or three DL PRS positioning frequency layers. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different aggregated DL PRS resources per UE RxTx TEG per dl-PRS-ID.

The UE may be requested via higher layer parameter nr-DL-PRS-JointMeasurementRequestedPFL-List to perform the aggregated DL RSTD measurement(s) or the aggregated UE Rx-Tx time difference measurement(s) across two or three DL PRS positioning frequency layers.

The UE may report via higher layer parameter nr-RSTD-BasedOnAggregatedResources or nr-UE-RxTxTimeDiffBasedOnAggregatedResources in a measurement report whether the aggregated DL RSTD measurement(s) or the aggregated UE Rx-Tx time difference measurement(s) is performed. If any aggregated measurement is performed, the two or three DL PRS positioning frequency layers to be used may also be reported by reporting PRS resource set IDs.

If the UE reports a DL PRS-RSRP or a DL PRS-RSRPP with aggregated DL RSTD measurement(s) or aggregated UE Rx-Tx time difference measurement(s), the DL PRS-RSRP or the DL PRS-RSRPP correspond to the aggregated DL PRS resources across two or three DL PRS positioning frequency layers.

For PRS resources on multiple DL PRS positioning frequency layers (PFLs) linked for aggregation, the channel over which a symbol on one PFL for PRS transmission is conveyed can be inferred from the channel over which the same symbol of another PFL or the aggregated PFL is conveyed.

9.3.4.7.3	Test description

9.3.4.7.3.1	Pre-test conditions

System Simulator:

-	Sub-test 30: NR Cell 1 as specified in 8.2.9 and system information combination NR-1 as defined in TS 38.508-1 [30] clause 4.4.3.1.2.

-	Sub-test 32: NR Cell 1, NR Cell 2 and NR Cell 3 as specified in 8.2.11 and system information combination NR-4 as defined in TS 38.508-1 [30] clause 4.4.3.1.2.

UE:

The UE shall begin the test with no assistance data stored.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	Then the SS sends the RESET UE POSITIONING STORED INFORMATION message to the UE to clear the stored assistance data in the UE.

Related PICS/PIXIT Statements:

-

9.3.4.7.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 9.3.4.7.3.2-1 below:

Table 9.3.4.7.3.2-1: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 30 | UE supporting Multi-RTT (Rel-18 onwards) |
| 32 | UE supporting DL-TDOA (Rel-18 onwards) |

Table 9.3.4.7.3.2-2: Test Configuration

| Test Configuration | Network Deployment Type | Test Implementation |
| --- | --- | --- |
| B | NG-RAN NR |  |

Table 9.3.4.7.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an LPP message of type Request Capabilities. | <-- | DLInformationTransfer (LPP REQUEST CAPABILITIES) | - | - |
| 2 | The UE sends an LPP message of type Provide Capabilities including the UE positioning capabilities. | --> | ULInformationTransfer (LPP PROVIDE CAPABILITIES) | - | - |
| 3 | IF the UE LPP message at step 2 includes an acknowledgment request THEN SS sends a LPP Acknowledgement response. | <-- | DLInformationTransfer (LPP ACKNOWLEDGEMENT) | - | - |
| 4 | The SS sends an LPP message of type Provide Assistance Data. | <-- | DLInformationTransfer (LPP PROVIDE ASSISTANCE DATA) | - | - |
| 5 | The SS sends an RRCReconfiguration message. | <-- | RRCReconfiguration |  |  |
| 6 | The UE sends an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete |  |  |
| 7 | The SS sends an LPP message of type Request Location Information including a request for performing joint measurement across aggregated PFLs. | <-- | DLInformationTransfer (LPP REQUEST LOCATION INFORMATION) | - | - |
| 8 | Check: Does the UE send an LPP message of type Provide Location Information including measurements results with indicating the measurement is based on aggregated DL-PRS resources? | --> | ULInformationTransfer (LPP PROVIDE LOCATION INFORMATION) | 1 | P |
| 9 | IF the UE LPP message at step 8 includes an acknowledgment request THEN SS sends a LPP Acknowledgement response. | <-- | DLInformationTransfer (LPP ACKNOWLEDGEMENT) | - | - |

9.3.4.7.3.3	Specific message contents

Table 9.3.4.7.3.3-1: RESET UE POSITIONING STORED INFORMATION (Preamble)

| Derivation Path: 38.509 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | Sub-test 30: 0 0 0 0 0 1 1 0 Sub-test 32: 0 0 0 0 0 1 1 1 | Sub-test 30: Multi-RTT Sub-test 32: DL-TDOA |  |

Table 9.3.4.7.3.3-2: DLInformationTransfer (step 1, step 3, step 4, step 7 and step 9, Table 9.3.4.7.3.2-3)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.3.4.7.3.3-3 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.7.3.3-3: DL NAS TRANSPORT (DLInformationTransfer, Table 9.3.4.7.3.3-2)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1: Set according to Table 8.4-2 | LPP Request Capabilities. |  |
|  |  | Steps 3 and 9: Set according to Table 9.3.4.7.3.3-4 | LPP Acknowledgement |  |
|  |  | Step 4: Set according to Table 9.3.4.7.3.3-5 | LPP Provide Assistance Data |  |
|  |  | Step 7: Set according to Table 9.3.4.7.3.3-9 | LPP Request Location Information |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.3.4.7.3.3-4: LPP Acknowledgement (DL NAS TRANSPORT, Table 9.3.4.7.3.3-3)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID |  | Not present |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |  |
| ackRequested |  | FALSE |  |  |
| ackIndicator |  | (0..255) | Contains the same value of the sequenceNumber field in steps 2 or 8, Table 9.3.4.7.3.2-3 |  |
| } |  |  |  |  |
| lpp-MessageBody |  | Not present. |  |  |
| } |  |  |  |  |

Table 9.3.4.7.3.3-5: LPP Provide Assistance data (DL NAS TRANSPORT, Table 9.3.4.7.3.3-3)

| Derivation Path: Table 8.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.4-1 with the following exceptions: |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 SEQUENCE { |  |  | Sub-test 30 |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 9.3.4.7.3.3-6 |  |  |
| } |  |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 SEQUENCE { |  |  | Sub-test 32 |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 9.3.4.7.3.3-6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.3.4.7.3.3-6: NR-DL-PRS-AssistanceData (LPP Provide Assistance data, Table 9.3.4.7.3.3-5)

| Derivation Path: Table 8.4.1.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.4.1.6-1 with the following exceptions: |  |  |  |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1 [30] Table 4.6.3-188 |  |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 1 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the NR Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 3 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] | As specified in Table 9.3.4.7.3.3-7 with condition NR cell 1 | entry 1 |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] | As specified in Table 9.3.4.7.3.3-7 with condition NR cell 2 | entry 2 | sub-test 32 |
| NR-DL-PRS-AssistanceDataPerTRP-r16[3] | As specified in Table 9.3.4.7.3.3-7 with condition NR cell 3 | entry 3 | sub-test 32 |
| } |  |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1 [30] Table 4.6.3-188 |  |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 3 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the NR Cell 3 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 3 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] | As specified in Table 9.3.4.7.3.3-7 with condition NR cell 1 | entry 1 |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] | As specified in Table 9.3.4.7.3.3-7 with condition NR cell 2 | entry 2 | sub-test 32 |
| NR-DL-PRS-AssistanceDataPerTRP-r16[3] | As specified in Table 9.3.4.7.3.3-7 with condition NR cell 3 | entry 3 | sub-test 32 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-DL-PRS-AggregationInfo-r18 SEQUENCE (SIZE (1..nrMaxNumPRS-BandWidthAggregation-r18)) OF NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18 { | 3 entries |  |  |
| NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18[1] SEQUENCE (SIZE (2..3)) OF NR-DL-PRS-AggregationElement-r18 { | 2 entries | entry 1 |  |
| NR-DL-PRS-AggregationElement-r18[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 0 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 0 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-DL-PRS-AggregationElement-r18[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 1 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 0 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18[2] SEQUENCE (SIZE (2..3)) OF NR-DL-PRS-AggregationElement-r18 { | 2 entries | entry 2 | sub-test 32 |
| NR-DL-PRS-AggregationElement-r18[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 0 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 1 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-DL-PRS-AggregationElement-r18[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 1 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 1 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18[3] SEQUENCE (SIZE (2..3)) OF NR-DL-PRS-AggregationElement-r18 { | 2 entries | entry 3 | sub-test 32 |
| NR-DL-PRS-AggregationElement-r18[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 0 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 2 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-DL-PRS-AggregationElement-r18[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 1 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 2 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.3.4.7.3.3-7: NR-DL-PRS-AssistanceDataPerTRP-r16 (NR-DL-PRS-AssistanceData, Table 9.3.4.7.3.3-6)

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  | NR Cell 1 |
|  | 1 |  | NR Cell 2 |
|  | 2 |  | NR Cell 3 |
| nr-PhysCellID-r16 | As defined in TS 38.508-1 table 4.4.2-2, Physical layer cell Identity for SIG for NR Cell 1/2/3 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 1/2/3 |  |  |
| mnc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 1/2/3 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 1/2/3 Identifier defined in TS 38.508-1 Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 1/2/3 frequency |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| prs-OnlyTP-r16 | Not present |  |  |
| nr-DL-PRS-ExpectedAoD-or-AoA-r17 | Not present |  |  |
| } |  |  |  |

Table 9.3.4.7.3.3-8: NR-DL-PRS-Info (NR-DL-PRS-AssistanceDataPerTRP-r16, Table 9.3.4.7.3.3-7)

| Derivation Path: Table 8.4.1.6-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.4.1.6-3 with the following exceptions: |  |  |  |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-SequenceID-r16 | 0 |  | NR Cell 1 |
|  | 1 |  | NR Cell 2 |
|  | 2 |  | NR Cell 3 |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 0 |  | NR Cell 1 |
|  | 1 |  | NR Cell 2 |
|  | 0 |  | NR Cell 3 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.3.4.7.3.3-9: LPP Request Location Information (DLInformationTransfer, Table 9.3.4.7.3.3-2)

| Derivation Path: Table 8.4-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 8.4-3 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |  |
| locationInformationType |  | locationMeasurementsPreferred |  |  |
| } |  |  |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 SEQUENCE { |  |  |  | Sub-test 30 |
| nr-DL-PRS-JointMeasurementRequest-r18 SEQUENCE { |  |  |  |  |
| nr-DL-PRS-JointMeasurementRequestedPFL-List-r18 SEQUENCE (SIZE (2..3)) OF INTEGER (0..nrMaxFreqLayers-1-r16) { |  |  |  |  |
| INTEGER[1] |  | 0 |  |  |
| INTEGER[2] |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 SEQUENCE { |  |  |  | Sub-test 32 |
| nr-DL-PRS-JointMeasurementRequest-r18 SEQUENCE { |  |  |  |  |
| nr-DL-PRS-JointMeasurementRequestedPFL-List-r18 SEQUENCE (SIZE (2..3)) OF INTEGER (0..nrMaxFreqLayers-1-r16) { |  |  |  |  |
| INTEGER[1] |  | 0 |  |  |
| INTEGER[2] |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.7.3.3-10: RRCReconfiguration (step 5, Table 9.3.4.7.3.2-3)

| Derivation Path: TS 38.508-1, table 4.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  |  |  |  |
| measConfig |  | MeasConfig as defined in Table 8.3.1-2 |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | Sub-test 30 |
| masterCellGroup |  | CellGroupConfig as defined in Table 8.3.2-2 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.7.3.3-11: ULInformationTransfer (step 2 and step 8, Table 9.3.4.7.3.2-3)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.3.4.7.3.3-12 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.7.3.3-12: UL NAS TRANSPORT (ULInformationTransfer, Table 9.3.4.7.3.3-11)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 2: Set according to Table 9.3.4.7.3.3-13 | LPP Provide Capabilities |  |
|  |  | Step 10: Set according to Table 9.3.4.7.3.3-14 | LPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 1 Table 9.3.4.7.3.2-3) |  |

Table 9.3.4.7.3.3-13: LPP Provide Capabilities (ULInformationTransfer, Table 9.3.4.7.3.3-11)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in the LPP Request Capabilities message in step 1, Table 9.3.4.7.3.2-3 |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement SEQUENCE { |  | Present, or not present |  |  |
| ackRequested |  | TRUE |  |  |
| ackIndicator |  | Not present |  |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideCapabilities |  | Dependent on UE capabilities |  |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities |  |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities |  |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities |  |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.7.3.3-14: LPP Provide Location Information (ULInformationTransfer, Table 9.3.4.7.3.3-11)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in LPP Request Location Information message in step 7, Table 9.3.4.7.3.2-3 |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement SEQUENCE { |  | present, or not present |  |  |
| ackRequested |  | TRUE |  |  |
| ackIndicator |  | Not present |  |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideLocationInformation |  | May be present. Any value acceptable |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 SEQUENCE { |  |  |  | Present for sub-test 30 |
| nr-Multi-RTT-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |  |
| nr-Multi-RTT-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16 { |  | 1 entry |  |  |
| NR-Multi-RTT-MeasElement-r16[1] SEQUENCE { |  |  | entry 1 |  |
| dl-PRS-ID-r16 |  | Not checked |  |  |
| nr-PhysCellID-r16 |  | Not checked |  |  |
| nr-CellGlobalID-r16 |  | Not checked |  |  |
| nr-ARFCN-r16 |  | Not checked |  |  |
| nr-DL-PRS-ResourceID-r16 |  | Not checked |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  | Not checked |  |  |
| nr-UE-RxTxTimeDiff-r16 |  | Present. Any value acceptable |  |  |
| nr-AdditionalPathList-r16 |  | Not checked |  |  |
| nr-TimeStamp-r16 |  | Not checked |  |  |
| nr-TimingQuality-r16 |  | Not checked |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  | Not checked |  |  |
| nr-Multi-RTT-AdditionalMeasurements-r16 |  | Not checked |  |  |
| nr-UE-RxTx-TEG-Info-r17 |  | Not checked |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  | Not checked |  |  |
| nr-los-nlos-Indicator-r17 |  | Not checked |  |  |
| nr-AdditionalPathListExt-r17 |  | Not checked |  |  |
| nr-Multi-RTT-AdditionalMeasurementsExt-r17 |  | Not checked |  |  |
| nr-MeasBasedOnAggregatedResources-r18 |  | true |  |  |
| nr-AggregatedDL-PRS-ResourceInfo-List-r18 |  | Not checked |  |  |
| nr-RSCP-r18 |  | Not checked |  |  |
| nr-PhaseQuality-r18 |  | Not checked |  |  |
| nr-RSCP-AddSampleMeasurements-r18 |  | Not checked |  |  |
| nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 |  | Not checked |  |  |
| nr-NTN-UE-RxTxMeasurements-r18 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nr-NTA-Offset-r16 |  | Not checked |  |  |
| nr-SRS-TxTEG-Set-r17 |  | Not present |  |  |
| nr-UE-RxTEG-TimingErrorMargin-r17 |  | Not present |  |  |
| nr-UE-TxTEG-TimingErrorMargin-r17 |  | Not present |  |  |
| nr-UE-RxTxTEG-TimingErrorMargin-r17 |  | Not present |  |  |
| } |  |  |  |  |
| nr-Multi-RTT-Error-r16 |  | Not present |  |  |
| nr-Multi-RTT-SignalMeasurementInstances-r17 |  | Not present |  |  |
| } |  |  |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  |  |  | Present for sub-test 32 |
| nr-DL-TDOA-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |  |
| dl-PRS-ReferenceInfo-r16 |  | Not checked |  |  |
| nr-DL-TDOA-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16 { |  | 1 entry |  |  |
| NR-DL-TDOA-MeasElement-r16[1] SEQUENCE { |  |  | entry 1 |  |
| dl-PRS-ID-r16 |  | Not checked |  |  |
| nr-PhysCellID-r16 |  | Not checked |  |  |
| nr-CellGlobalID-r16 |  | Not checked |  |  |
| nr-ARFCN-r16 |  | Not checked |  |  |
| nr-DL-PRS-ResourceID-r16 |  | Not checked |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  | Not checked |  |  |
| nr-TimeStamp-r16 |  | Not checked |  |  |
| nr-RSTD-r16 |  | Present. Any value acceptable |  |  |
| nr-AdditionalPathList-r16 |  | Not checked |  |  |
| nr-TimingQuality-r16 |  | Not checked |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  | Not checked |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  | Not checked |  |  |
| nr-UE-Rx-TEG-ID-r17 |  | Not checked |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  | Not checked |  |  |
| nr-los-nlos-Indicator-r17 |  | Not checked |  |  |
| nr-AdditionalPathListExt-r17 |  | Not checked |  |  |
| nr-DL-TDOA-AdditionalMeasurementsExt-r17 |  | Not checked |  |  |
| nr-MeasBasedOnAggregatedResources-r18 |  | true |  |  |
| nr-AggregatedDL-PRS-ResourceInfo-List-r18 |  | Not checked |  |  |
| nr-RSCPD-r18 |  | Not checked |  |  |
| nr-PhaseQuality-r18 |  | Not checked |  |  |
| nr-RSCPD-AddMeasurementSamples-r18 |  | Not checked |  |  |
| nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nr-UE-RxTEG-TimingErrorMargin-r17 |  | Not present |  |  |
| } |  |  |  |  |
| nr-dl-tdoa-LocationInformation-r16 |  | Not checked |  |  |
| nr-DL-TDOA-Error-r16 |  | Not present |  |  |
| nr-DL-TDOA-SignalMeasurementInstances-r17 |  | Not present |  |  |
| nr-DL-TDOA-LocationInformationInstances-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |