3GPP TSG RAN WG5 Meeting #106	R5-250627

# Athens, Greece，17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0082 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test conditions for category M1 NTN out-of-band blocking test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The out-of-band blocking test case has a very long run time for GSO testing. Category M1 NTN out-of-band blocking test case shall align with category NB1 and NB2 NTN to reduce test time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The initial conditions for TC 7.6A.3 have been updated to align with category NB1 and NB2 NTN out-of-band blocking test case to reduce test time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test time will remain unnecessary long. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6A.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.6A.3.4	Test Description

7.6A.3.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for category M1 in clause 5.2A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.6A.3.4.1-1. The details of the downlink and uplink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 7.6A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS36.508 [12] subclause 4.3.1 |  |  | One frequency chosen arbitrarily from low or high range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
| Configuration ID | Downlink Configuration |  |  | Uplink Configuration |  |  |
|  | Modulation | RB allocation |  | Modulation | RB allocation | Narrowband index (Note 1) |
| 1 | QPSK | 4 |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211[3], 5.2.4. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.5 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.6A.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

7.	Optionally, for GSO only device to reduce the test time, preconfigure the UE by any means to deactivate ephemeris consistency check so that the UE accepts NGSO (LEO-600) ephemeris.

8a.	In case the ephemeris consistency check on the UE can be deactivated as described in step 7, the test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] Table 5.6.2.1-2 for NGSO (LEO-600).

8b.	In case the ephemeris consistency check on the UE cannot be deactivated as described in step 7, the test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] Table 5.6.2.1-1 for GSO if UE supports only GSO satellites and Table 5.6.2.1-2 for NGSO (LEO-600) if UE supports only NGSO satellites or both GSO and NGSO satellites. Test system shall send same SIB31 information during the duration of the test as defined in TS 36.508 [12] clause 5.6.3.1.

9.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

10.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.6A.3.4.3.

7.6A.3.4.2	Test Procedure

<< End of changes >>