3GPP TSG-RAN WG5#106	R5-250650

# Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3210 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating 7.4F Maximum input level for shared spectrum channel access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TT is missing in the the test case for f > 5.925GHz |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated editor’s note Added TT for f > 5.925GHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TT will be missing and the test case will be incomplete for f > 5.925GHz |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0554 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Annex F updated in R5-250651 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes >>

## 7.4F	Maximum input level for shared spectrum channel access

Editor’s note:

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

7.4F.1	Test purpose

Maximum input level tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to a g-NodeB.

7.4F.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR standalone shared spectrum channel access.

7.4F.3	Minimum conformance requirements

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4F.3-1.

Table 7.4F.3-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -252 | -25 + 10log10(BWChannel /20)Note 2 | -202 |
|  |  | -273,5 | -27 + 10log10(BWChannel /20)Note 3,5 | -223,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.x for 1024 QAM. |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.4 and 7.4F.

7.4F.4	Test description

7.4F.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 7.4F.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4F.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest Lowest UL / Lowest DL, Lowest UL / Highest DL (NOTE 4) |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Downlink Configuration |  | Uplink Configuration |  |
| Modulation | RB allocation | Modulation | RB allocation |
| CP-OFDM 64 QAM | NOTE 1 | DFT-s-OFDM QPSK | NOTE 2 |
| CP-OFDM 256 QAM | NOTE 1 | DFT-s-OFDM QPSK | NOTE 2 |
| CP-OFDM 1024 QAM | NOTE 1 | DFT-s-OFDM QPSK | NOTE 2 |
| NOTE 1: The specific configuration of downlink RB allocation is defined in Table 7.3F.2.4.1-2. NOTE 2: The specific configuration of uplink RB allocation is defined in Table 7.3F.2.4.1-3. NOTE 3: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 4: Additional test points selected according to asymmetric channel bandwidths specified in clause 5.3.6. DL channel bandwidth shall be selected first. NOTE 5: Testing for 1024 QAM is applicable for UE supporting capabilities 1024QAM (pdsch-1024QAM-FR1). |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 , and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The DL and UL Reference Measurement Channels are set according to Table 7.4F.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.4F.4.3.

7.4F.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.4F.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.4F.4.1-1. Since the UE has no payload data and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.4F.5-1. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.4F.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

7.4F.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

The following exception applies to 1024QAM testing:

Table 7.4F.4.3-1: PDSCH-Config

| Derivation Path: TS 38.508-1 Table 5.4.1-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| mcs-Table-r17 | qam1024 |  |  |
| } |  |  |  |

7.4F.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 with parameters specified in Tables 7.4F.5-1.

Table 7.4F.5-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -252 -TT | -25 + 10log10(BWChannel /20)Note 2 -TT | -202 -TT |
|  |  | -273,5 -TT | -27 + 10log10(BWChannel /20)Note 3,5 -TT | -223,5 -TT |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. |  |  |  |  |

Table 7.4F.5-2: Test Tolerance (Maximum input level)

| f ≤ 3.0GHz | 3.0GHz < f ≤ 5.925GHz | f > 5.925GHz |
| --- | --- | --- |
| 0.7 dB | 1.0 dB | 1.0 dB |

#### << End of changes >>