3GPP TSG-RAN5 Meeting #106	R5-250673

# Athens , Greece, 17th Feb 2024 – 21st Feb 2025

Agenda item:	5.3.23.9

Source: 	Apple

Title: 	Discussion on UE beam lock function for beam correspondence test

Document for:	Discussion and Endorsement

# Introduction

RAN4 has introduced FR2 beam correspondence requirement for initial access and RRC_Inactive[1]. There have been several discussions in RAN5 meetings to define the beam correspondence test method with UE beam lock function.

The goal is to consider the beam lock function during initial access for beam correspondence test.

This paper provides Apple’s view on the test method with the UE beam lock function. It also highlights the disadvantages associated to the ‘test method without UE beam lock function’ for beam correspondence test during initial access and RRC_Inactive.

# Discussion

## On UE Beam Lock Function (UBF) during Initial Access

The intention of this topic is to further discuss and promote the use of Beam Lock function for UE beam lock activation and deactivation during initial access.

Following factor should be considered to allow the usage of UE beam lock function for beam correspondence test during initial access and RRC_Inactive:

- In the TR 38.891 section 6.4[7], it is clearly captured, “RAN4 concludes that dedicated beam lock function for RRC_IDLE state is needed, and the details of new beam lock function design can be left to RAN5.” This shows using beam lock function was the RAN4 conclusion at the time.

- As captured in the TR 38.891 section 6.4[7], UE beam lock function is needed to ensure the reliable and repeatable measurements test results.

- As specified in TS 38.509 section 5.4 for UE Beamlock Function (UBF) [2], the objective of UE to activate beam lock is to lock antenna beam pattern once it has formed a beam towards the base station (SS) during measurement period.

- As specified in TS 38.521-2 Section 6.2.1.2.4.2 for UE maximum output power (MOP) - Spherical coverage [5], SS initiates the requests to activate UE Beamlock Function (UBF) during measurements and initiates the requests to deactivate after completing the measurements.

Observation 1: RAN4 concluded that dedicated UE Beamlock Function (UBF) is needed for RRC_IDLE state for reliable and repeatable test results, as captured in the TR 38.891 section 6.4[7].

Observation 2: UE beamlock test function only locks the UE beam pattern towards SS during measurement period , as defined in TS 38.509 section 5.4 [2] and UE is allowed to handle UL power control as specified in TS 38.521-2 section 6.2.1.2 test procedure [5] for UE maximum output power- Spherical coverage[6].

Observation 3: UE is allowed to find the optimal beam for each grid point before beam lock activation, as specified in TS 38.521-2 section 6.2.1.2 [5] test procedure for UE maximum output power- Spherical coverage.

Based on the above findings and observations, we re-iterate the proposals from [6]:

Proposal 1: To allow the UE beam lock function (UBF) to ensure reliable and repeatable measurement results.

## Disadvantages of test method without UE Beam Lock (UBF)

UE beam lock function is needed to ensure the UE preamble transmission are from UE optimal Tx beam. The test method without considering the UE beam lock function can have following drawbacks for beam correspondence test during initial access and RRC_Inactive[8]:

- Without the UE beam lock function, the UE can switch between fine and course beam. This can cause the fluctuation in PRACH Tx power measurements results.

- Without the UE beam lock function, the UE may attempt to change the beam during the measurement period. Consequently, measurement samples may be obtained from sub-optimal and optimal beams. This may lead to inaccuracies in PRACH Tx power measurement results[8].

- Beam correspondence test without UE beam lock function violates the basic assumption [1] made by RAN4.

Observation 4: There are several disadvantages of defining the test method without using UE beam lock:

- It may lead to unstable test results.

- This approach may result in inaccurate PRACH measurement results due to the preambles from sub-optimal beam during measurement period.

- This violates the fundamental assumption [1] made by RAN4 when deciding the minimum requirement.

# Conclusion

The following observation and proposal were made in this contribution.

Observation 1: RAN4 concluded that dedicated UE Beamlock Function (UBF) is needed for RRC_IDLE state for reliable and repeatable test results, as captured in the TR 38.891 section 6.4[7].

Observation 2: UE beamlock test function only locks the UE beam pattern towards SS during measurement period , as defined in TS 38.509 section 5.4 [2] and UE is allowed to handle UL power control as specified in TS 38.521-2 section 6.2.1.2 test procedure [5] for UE maximum output power- Spherical coverage[6].

Observation 3: UE is allowed to find the optimal beam for each grid point before beam lock activation, as specified in TS 38.521-2 section 6.2.1.2 [5] test procedure for UE maximum output power- Spherical coverage.

Observation 4: There are several disadvantages of defining the test method without using UE beam lock:

- It may lead to unstable test results.

- This approach may result in inaccurate PRACH measurement results due to the preambles from sub-optimal beam during measurement period.

- This violates the fundamental assumption [1] made by RAN4 when deciding the minimum requirement.

Based on the above findings and observations, we re-iterate the proposals from [6]:

Proposal 1: To allow the UE beam lock function (UBF) to ensure reliable and repeatable measurement results.

# References

- R4-2317764, “LS on testability issues of beam correspondence requirements”, RAN4#108-bis, October 9 – October 13, 2023

- TS 38.509, “Special conformance testing functions for User Equipment (UE)”, V17.5.0 (2024-03).

- R5-243823,“Discussion on FR2 beam correspondence for initial access”, RAN4#103, May 20 – May 24, 2024

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment. V18.4.0 (2024-10)

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, V18.2.0 (2024-03)

- R5-244945, “Discussion on FR2 beam correspondence during initial access”, RAN4#104, Aug 19 – Aug 23, 2024

- TR 38.891, “User Equipment (UE) Further enhancements of NR RF requirements for frequency range 2 (FR2) for Rel-18”, V18.1.0 (2024-03)

- R5-247075,“Discussion on FR2 beam correspondence for initial access”, RAN5#105, Nov 18 – Nov 22, 2024