3GPP TSG-RAN5 Meeting #106	R5-250674

# Athens , Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:	5.3.23.9

Source: 	Apple

Title: 	On FR2 beam correspondence PRACH configuration Index

Document for:	Discussion and Endorsement

# Introduction

There have been several discussions in RAN5 meetings regarding the RACH configuration for the UE Conformance testing for FR2 beam correspondence during initial access and RRC_Inactive.

The goal is to suggest the UE RACH configuration index for preamble transmissions and duration required for maximum length of initial access procedure considered ‘premableTranmsMax’ configured to n200.

This paper provides Apple’s view on the RACH configuration parameters for beam correspondence test during initial access and RRC_Inactive.

# Discussion

## On RACH Transmission Configuration IE

The intention of this topic is to propose RACH configuration parameters for beam correspondence test.

PRACH Preamble Format and Configuration Index

It is recommended to use the longest preamble format for PRACH transmission to accelerate the testing as per the RAN4 LS [1].As specified in TS 38.211 Table 6.3.3.2-4 [2], Random access configurations for FR2, the available preamble formats A1, A2, A3, B1, B4, C0, C2 with PRACH duration is within the range from 2 to 12. The longest applicable preamble format for PRACH transmission with PRACH duration 12 is Format B4[1]. The PRACH configuration Index for format B4 is from index# 112 to index# 143. Preamble Index#143 have comparatively higher number of radio frames containing PRACH slots than index#112 for RACH transmission occasion.

UE Timing duration required for initial access procedure

The timing analysis is performed to analyse the time duration required for UE to transmit 200 preambles during initial access, the maximum value of premableTransmax counter. UE declares RACH failure upon expiry of this counter. Following assumptions are used for PRACH configuration timing analysis:

- PRACH configuration index: 143 ,132 and 115

- TDD slot configuration pattern [7DS8U] is used based on UL RMC as specified in TS 38.521-2 Table A.2.3-

- ra-response window: sl1, sl2, sl4, sl8 and sl10

- preambleTransMaxCounter is configured to maximum value: 200

Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum, and for FR2-NTN and paired spectrum.

Timing analysis for Preamble Format B4 Indexes # 143, 123 and 115:

| Parameter | Value |  |
| --- | --- | --- |
| PRACH Configuration Index | 143 |  |
| PRACH slot per radio frame based on slot pattern | 5 |  |
| Number of radio frame containing 5 PRACH slots | 1 |  |
| maximum configured preambleTransmax counter | 200 |  |
| slots for ra-response window | sl1/sl2/sl4/sl8 | sl10 |
| Total Duration for 200 preamble transmission [sec] | ((200/5)*10ms)= 0.4 | 0.7 |

Table 1 : Index #143

| Parameter | Value |  |
| --- | --- | --- |
| PRACH Configuration Index | 123 |  |
| PRACH slot per radio frame based on slot pattern | 5 |  |
| Number of radio frame containing 5 PRACH slots | 2 |  |
| maximum configured preambleTransmax counter | 200 |  |
| slots for ra-response window | sl1/sl2/sl4/sl8 | sl10 |
| Total Duration for 200 preamble transmission [sec] | ((200/5)*20ms)= 0.8 | 1.3 |

Table 2 : Index #123

| Parameter | Value |  |
| --- | --- | --- |
| PRACH Configuration Index | 115 |  |
| PRACH slot per radio frame based on slot pattern | 5 |  |
| Number of radio frame containing 5 PRACH slots | 8 |  |
| maximum configured preambleTransmax counter | 200 |  |
| slots for ra-response window | sl1/sl2/sl4/sl8 | sl10 |
| Total Duration for 200 preamble transmission [sec] | ((200/5)*80ms)= 3.2 | 5.3 |

Table 3 : Index #115

Consolidated timing analysis for Indexes # 143, 123 and 115:

|  | Total time required for 200 preamble transmission [seconds] |  |
| --- | --- | --- |
| PRACH Configuration Index /ra-response window | s1, sl2, sl4, sl8 | sl10 |
| 143 | 0.4 | 0.7 |  |
| 123 | 0.8 | 1.3 |  |
| 115 | 3.2 | 5.3 |  |

Table 4 : Total time duration

Observation 1: RACH longest preamble format B4 is recommended in RAN4 LS [1].

Observation 2: ra-ResponeWindow sl1/sl2/sl4/sl8 have same duration due to the TDD slot pattern: 7DS8U

Observation 2: As specified in TS 38.211 Table 6.3.3.2-4  [2], PRACH Configuration Index#143 has the maximum number of radio frame containing RACH occasion and will take shortest duration (0.4 second) for 200 preamble transmission counter.

Observation 3: Based on the TDD slot pattern, the PRACH configuration index #123 has ‘5’ RACH occasions slots for UE to send preambles in every 2nd frame, which is adequate compared to other applicable configuration indices.

Observation 4: The duration UE required for PRACH configuration Index#115 for preambleTransMax= n200 is exceeding 3 seconds.

Proposal 1: To configure UE with PRACH configuration index# 143 for beam correspondence test during initial access to minimize the test time.

Proposal 2: To configure UE with ra-response-window= sl8 for beam correspondence test during initial access.

Proposal 3: To use last 8 UL slots of PRACH for 1ms measurement period out of 200 PRACH retransmissions.

# Conclusion

The following observation and proposal were made in this contribution.

Observation 1: RACH longest preamble format B4 is recommended in RAN4 LS [1].

Observation 2: ra-ResponeWindow sl1/sl2/sl4/sl8 have same duration due to the TDD slot pattern: 7DS8U

Observation 2: As specified in TS 38.211 Table 6.3.3.2-4  [2], PRACH Configuration Index#143 has the maximum number of radio frame containing RACH occasion and will take shortest duration (0.4 second) for 200 preamble transmission counter.

Observation 3: Based on the TDD slot pattern, the PRACH configuration index #123 has ‘5’ RACH occasions slots for UE to send preambles in every 2nd frame, which is adequate compared to other applicable configuration indices.

Observation 4: The duration UE required for PRACH configuration Index#115 for preambleTransMax= n200 is exceeding 3 seconds.

Proposal 1: To configure UE with PRACH configuration index# 143 for beam correspondence test during initial access to minimize the test time.

Proposal 2: To configure UE with ra-response-window= sl8 for beam correspondence test during initial access.

Proposal 3: To use last 8 UL slots of PRACH for 1ms measurement period out of 200 PRACH retransmissions.

# References

- R4-2317764, “LS on testability issues of beam correspondence requirements”, RAN4#108-bis, October 9 – October 13, 2023

- TS 38.211,” Physical channels and modulation”, V18.4.0 (2024-09)

- TS 38.331, Radio Resource Control (RRC); Protocol specification, V18.3.0 (2024-09)

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment. V18.4.0 (2024-10)