3GPP TSG-RAN5 Meeting #106	R5-250698

# Athens, Greece, Feb.17th – 21st 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3724 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ATG RRM test case 19.6.4.2.1 - CSIRS L1 RSRP absolute accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add NR ATG test case 19.6.4.2.1 - NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR ATG test case 19.6.4.2.1 - NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.6.4.2.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

19.6.4.2	CSI-RS based L1-RSRP measurements for ATG

19.6.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG

Editor’s Note: This test case is incomplete in following aspect:

-	TT analysis is still missing.

19.6.4.2.1.1	Test purpose

The purpose of this test is to verify that the CSI-RS based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

19.6.4.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.4.2.

19.6.4.2.1.4	Test description

19.6.4.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.2.1.4.1 with following exceptions:

- During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

- During the test, the specific gNB reference location is emulated by test system.

- Table 6.7.4.2.1.4.1-1 is replaced by Table 19.6.4.2.1.4.1-1;

- Table 6.7.4.2.1.4.1-2 is replaced by Table 19.6.4.2.1.4.1-2;

Table 19.6.4.2.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 19.6.4.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 19.6.4.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 19.6.4.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 19.6.4.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

19.6.4.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.2.1.4.2 with following exceptions:

-	Table 6.7.4.2.1.5-1 is replaced by Table 19.6.4.2.1.5-1.

19.6.4.2.1.4.3	Message contents

Same as the message contents given in clause 6.7.4.2.1.4.3.

19.6.4.2.1.5	Test requirement

Same as the test requirements given in clause 6.7.4.2.1.5 with following exceptions:

-	Table 6.7.4.2.1.5-1 is replaced by Table 19.6.4.2.1.5-1.

Table 19.6.4.2.1.5-1: NR Cell specific parameters (Same as Table 6.7.4.2.1.5-1 with following exceptions)

| Parameter | Test configuration | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Propagation Condition | Config 1,2 | - | AWGN+220 Hz | AWGN+220 Hz |
|  | Config 3 | - | AWGN+500 Hz | AWGN+500 Hz |

<End of Change 1>