3GPP TSG-RAN5 Meeting #106	R5-250719

# Athens, Greece, Feb.17th – 21st 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0941 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT analysis for RedCap RRM test cases 16.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon,Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Cell 2 Es/Noc in T2 and T3 in TC 16.3.1.1 is decreased to 11dB in 38.133 v17.12.0. However, the TT analysis for 16.3.1.1 are still performed based on the old 12dB Cell 2 Es/Noc. Hence the TT analysis for 16.3.1.1 needs to be updated accordingly. Based on the updated TT analysis. We purpose to make following change: - To compensate for the 1dB decrease in Cell Es/Noc, change the TT for A3-offset from -1.5dB to -2.5dB |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TT analysis for 16.3.1.1 is updated. - Grouping table is updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, attachments |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 3740, 3741 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

Adding following .zip file to TR 38.903 attachments:

“38.533 16.3.1.1 TT_v2.zip”

Removing following .zip file from TR 38.903 attachments:

“38.533 16.3.1.1 TT.zip”

<End of Change 1>

<Start of Change 2>

8	Grouping of test cases defined in TS 38.533

Table 8-1: Grouping of FR1 test cases defined in Clauses 4, 4A, 6, 8, 16 and 18 of TS 38.533

| Group | Test Case Numbers | .zip file name | Comments |
| --- | --- | --- | --- |
| SCell_Activation_01 | 4.5.3.1 4.5.3.2 4.5.3.3 6.5.3.1 6.5.3.2 6.5.3.3 10.3.3.1 10.3.3.2 10.3.3.3 11.4.3.1 11.4.3.2 11.4.3.3 13.2.2.1 13.2.2.2 13.2.2.3 | “38.533 4.5.3.1+4.5.3.2+6.5.3.1+6.5.3.2 TT v6.zip” | “2 Inter Frequency NR Cells, 3 time periods, Various number of sub-tests, No fading” |
| <Unchanged contents are omitted> |  |  |  |
| Intra_Freq_HO_Known_Target | 6.3.1.1 16.3.1.2 | “38.533 6.3.1.1+16.3.1.2 TT.zip” | “2 Intra-Freq NR Cells, 3 Time Periods, No Fading” Note: The spreadsheet “16.3.1.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Intra_Freq_HO_Known_Target_1Rx_RedCap | 16.3.1.1 | “38.533 16.3.1.1 TT_v2.zip” | “2 Intra-Freq NR Cells, 3 Time Periods, No Fading” |
| Intra_Freq_HO_Unknown_Target | 6.3.1.2 16.3.1.3 16.3.1.4 | “38.533 6.3.1.2+16.3.1.4 TT.zip” | “2 Intra-Freq NR Cells, 2 Time Periods, No Fading” Note: The spreadsheet “16.3.1.4-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| <Unchanged contents are omitted> |  |  |  |
| InterRAT_re-selection_LTE_Serving_RedCap | 18.1.1.1 | “38.533 18.1.1.1 TT.zip” | “1 NR Cell, 1 LTE serving cell, 3 time periods, no fading, 2Rx RedCap UE” |

<End of Change 2>