3GPP TSG-RAN5 Meeting #106	R5-250726

# Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3743 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial Correction to NR SA FR1 handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton International INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The description of this clause differs from that of other sections in Test cases of NR SA FR1 handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To align the description of reference clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect clause description will lead to confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7, 6.3.1.8, 6.3.1.9, 6.3.1.10, 6.3.1.11, 6.3.1.12, 6.3.1.18, 6.3.3.1, 6.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.3.1.7	NR SA FR1 synchronous DAPS handover

6.3.1.7.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 intra frequency DAPS handover requirements in synchronous scenario specified in TS 38.133 [6] clause 6.1.3.2.

## << Unchanged sections omitted >>

6.3.1.7.5	Test requirements

Table 6.3.1.7.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.3.1.7.5-1: Cell specific test parameters for NR SA FR1 synchronous DAPS handover

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 – T5 | T1 | T2 – T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |
| reportConfigType |  |  |  | periodic |  | N/A |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  | N/A |  |
| CSI reporting periodicity |  | Config 1,2 | slot | 5 |  | N/A |  |
|  |  | Config 3 |  | 10 |  | N/A |  |
| CSI reporting offset |  | Config 1,2 | slot | 3 |  | N/A |  |
|  |  | Config 3 |  | 5 |  | N/A |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |
|  |  |  | dB | 8 | -2.41 | -Infinity | 1.36 |
|  |  |  | dB | 8 | 8 | -Infinity | 10 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 | -Infinity | -88 |
|  | Config 3 |  | dBm/SCS | -87 | -87 | -Infinity | -85 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -57.67 | -61.41 | -57.67 |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -51.56 | -55.31 | -51.56 |
| Propagation condition |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than Dhandover1 from the beginning of time period T3. The rate of correct handovers observed during repeated tests shall be at least 90%, where:

Dhandover1= TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin

-	TRRC_procedure= 10 ms, is the RRC procedure delay specified in TS 38.331 [13] clause 12;

-	Tsearch = 0 ms for known target cell, is the time required to search the target cell specified in 38.133 [6] clause 6.1.1.2.2;

-	TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell specified in TS 38.133 [6] clause 6.1.1.2.2.

-	Tprocessing = 20 ms, is the time for UE processing specified in TS 38.133 [6] clause 6.1.1.2.2.

-	T∆ = 20 ms, is the time for fine time tracking and acquiring full timing information of the target cell specified in TS 38.133 [6] clause 6.1.1.2.2.

-	Tmargin = 2 ms, is the time for SSB post-processing specified in TS 38.133 [6] clause 6.1.1.2.2.

This gives a total of 72 ms.

After successful RACH to cell 2 and until the start of time period T4, UE shall be able to receive PDSCH alternatively from cell 1 and cell 2. UE is not expected to transmit UL to both cell 1 and cell 2 in the same TTI.

The UE shall release Cell 1 less than Dhandover2 from the beginning of time period T4, where:

Dhandover2= TRRC_procedure + Tinterrupt2

-	TRRC_procedure= 10 ms, is the RRC procedure delay specified in TS 38.331 [13] clause 12;

-	Tinterrupt2 = 1 ms for sync intra-frequency DAPS handover, is the allowed interruption length during Dhandover2 as in TS 38.133 [13] clause 6.1.3.2.2;

This gives a total of 11 ms.

UE shall not report CSI to Cell 1 during T5.

6.3.1.8	NR SA FR1 asynchronous DAPS handover

6.3.1.8.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 intra-frequency DAPS handover requirements in asynchronous scenario specified in TS 38.133 [6] clause 6.1.3.2.

## << Unchanged sections omitted >>

6.3.1.8.5	Test requirements

Table 6.3.1.8.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.3.1.8.5-1: Cell specific test parameters for NR SA FR1 asynchronous DAPS handover

| Parameter |  |  |  |  | Unit |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | T1 | T2 – T5 | T1 | T2 – T5 |
| NR RF Channel Number |  |  |  |  |  |  | 1 |  |  |  |
| Duplex mode |  |  | Config 1 |  |  |  | FDD |  |  |  |
|  |  |  | Config 2,3 |  |  |  | TDD |  |  |  |
| TDD configuration |  |  | Config 1 |  |  |  | Not Applicable |  |  |  |
|  |  |  | Config 2 |  |  |  | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 |  |  |  | TDDConf.2.1 |  |  |  |
| BWchannel |  |  | Config 1,2 |  | MHz |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 |  |  |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  |  | Config 1 |  | MHz |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 |  |  |  | 40: NRB,c = 106 |  |  |  |
| DRX Cycle |  |  |  |  | ms |  | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  |  | CR.2.1 TDD |  |  |  |
| TRS configuration |  |  | Config 1 |  |  |  | TRS.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  |  |  | TRS.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OP.1 |  |  |  |
| CSI-RS CSI |  | Config 1 |  |  |  | CSI-RS.1.1 FDD |  |  |  |  |
| configuration for |  | Config 2 |  |  |  | CSI-RS.1.1 TDD |  |  |  |  |
| reporting |  | Config 3 |  |  |  | CSI-RS.2.1 TDD |  |  |  |  |
| reportConfigType |  |  |  |  |  | periodic |  |  | N/A |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  | N/A |  |
| CSI reporting |  | Config 1,2 |  | slot |  | 5 |  |  | N/A |  |
| periodicity |  | Config 3 |  |  |  | 10 |  |  | N/A |  |
| CSI reporting offset |  | Config 1,2 |  | slot |  | 3 |  |  | N/A |  |
|  |  | Config 3 |  |  |  | 5 |  |  | N/A |  |
| SMTC Configuration |  |  |  |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  |  | Config 1,2 |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3 |  |  |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 |  | kHz |  | 15 |  |  |  |
|  |  |  | Config 3 |  |  |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | Config 1,2 |  | kHz |  | 15 |  |  |  |
|  |  |  | Config 3 |  |  |  | 30 |  |  |  |
| PRACH configuration |  |  |  |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration |  |  | Initial DL BWP |  |  |  | DLBWP.0.1 |  |  |  |
|  |  |  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  |  |  |
|  |  |  | Initial UL BWP |  |  |  | ULBWP.0.1 |  |  |  |
|  |  |  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  |  | dBm/15kHz |  | -98 |  |  |  |
| Note2 | Config 1,2 |  |  |  | dBm/SCS |  | -98 |  |  |  |
|  | Config 3 |  |  |  |  |  | -95 |  |  |  |
|  |  |  |  |  | dB |  | 8 | -2.41 | -Infinity | 1.36 |
|  |  |  |  |  | dB |  | 8 | 8 | -Infinity | 10 |
| SSB_RP | Config 1,2 |  |  |  | dBm/SCS |  | -90 | -90 | -Infinity | -88 |
|  | Config 3 |  |  |  |  |  | -87 | -87 | -Infinity | -85 |
| IoNote3 | Config 1,2 |  |  |  | dBm/ 9.36MHz |  | -61.41 | -57.67 | -61.41 | -57.67 |
|  | Config 3 |  |  |  | dBm/ 38.16MHz |  | -55.31 | -51.56 | -55.31 | -51.56 |
| Propagation condition |  |  |  |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

Same test requirements as described in section 6.3.1.7.5 except:

The UE shall release Cell 1 less than Dhandover2 from the beginning of time period T4, where:

Dhandover2= TRRC_procedure + Tinterrupt2

-	TRRC_procedure= 10 ms, is the RRC procedure delay specified in TS 38.331 [13] clause 12;

-	Tinterrupt2 = 2 slots for test configuration 6.3.1.8-1/2 and Tinterrupt2 = 3 slots for test configuration 6.3.1.8-3, is the allowed interruption length during Dhandover2 as in TS 38.133 [13] clause 6.1.3.2.2;

This gives a total of 10 ms + 2 slots for test configuration 6.3.1.8-1/2 and 10 ms + 3 slots for test configuration 6.3.1.8-3.

6.3.1.9	NR SA FR1 Intra-band inter-frequency synchronous DAPS handover

6.3.1.9.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 intra-band inter-frequency DAPS handover requirements in synchronous scenario specified in TS 38.133 [6] clause 6.1.3.2.

## << Unchanged sections omitted >>

6.3.1.9.5	Test requirements

Table 6.3.1.9.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.3.1.9.5-1: Cell specific test parameters for NR SA FR1 Intra-band inter-frequency  synchronous DAPS handover

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2-T5 | T1 |  |  | T2-T5 |
| NR RF Channel Number |  |  |  | 1 |  |  | 2 |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |  |
| reportConfigType |  |  |  |  |  |  |  | periodic |  |  |
| reportQuantity |  |  |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity |  | Config 1,2 | slot | 5 |  |  |  | 5 |  |  |
|  |  | Config 3 |  | 10 |  |  |  | 10 |  |  |
| CSI reporting offset |  | Config 1,2 | slot | 3 |  |  |  | 3 |  |  |
|  |  | Config 3 |  | 5 |  |  |  | 5 |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 11.7 |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 11.7 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -Infinity |  | -86.3 |  |
|  | Config 3 |  | dBm/SCS | -86.99 | -86.99 |  | -Infinity |  | -83.29 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -61.41 |  | -70.05 |  | -58.06 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -55.31 |  | -63.94 |  | -51.96 |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than Dhandover1 from the beginning of time period T3. The rate of correct handovers observed during repeated tests shall be at least 90%, where:

Dhandover1= TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin

- TRRC_procedure= 10 ms, is the RRC procedure delay specified in 38.331 [13] clause 12;

- Tsearch = 0 ms for known target cell, is the time required to search the target cell specified in 38.133 [6] clause 6.1.1.2.2;

- TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell specified in TS 38.133 [6] clause 6.1.1.2.2.

- Tprocessing = 20 ms, is the time for UE processing specified in TS 38.133 [6] clause 6.1.1.2.2.

- T∆ = 20 ms, is the time for fine time tracking and acquiring full timing information of the target cell specified in TS 38.133 [6] clause 6.1.1.2.2.

- Tmargin = 2 ms, is the time for SSB post-processing specified in 38.133 [6] clause 6.1.1.2.2.

This gives a total of 72 ms.

During T3 UE is allowed to cause Tinterrupt1 interruption to cell 1. Tinterrupt1 is defined in TS 38.133 [6] clause 6.1.3.2.2 Table 6.1.3.2.2-2. When UE is transmitting PRACH preamble to cell 2, interruption to cell 1 is allowed.

During T4 UE is allowed to cause Tinterrupt2 interruption to cell 1. Tinterrupt2 is defined in TS 38.133 [6] clause 6.1.3.2.2 Table 6.1.3.2.2-6.

UE shall finish cell 1 release in T4 and shall not send any CSI reports to cell 1 during T5.

6.3.1.10	NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover

6.3.1.10.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 intra-band inter-frequency DAPS handover requirements in asynchronous scenario specified in TS 38.133 [6] clause 6.1.3.2.

## << Unchanged sections omitted >>

6.3.1.10.5	Test requirements

Table 6.3.1.10.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.3.1.10.5-1: Cell specific test parameters for NR SA FR1 Intra-band inter-frequency  asynchronous DAPS handover

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 -T5 |  |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -98 |  |  |  |
|  |  |  | dB | 8 | 8 | -Infinity | 11.7 |
|  |  |  | dB | 8 | 8 | -Infinity | 11.7 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 | -Infinity | -86.3 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -61.41 | -70.05 | -58.06 |
| Propagation condition |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than Dhandover1 from the beginning of time period T3. The rate of correct handovers observed during repeated tests shall be at least 90%, where:

Dhandover1= TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin

-	TRRC_procedure= 10 ms, is the RRC procedure delay specified in TS 38.331 [13] clause 12;

-	Tsearch = 0 ms for known target cell, is the time required to search the target cell specified in 38.133 [6] clause 6.1.1.2.2;

-	TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell specified in TS 38.133 [6] clause 6.1.1.2.2.

-	Tprocessing = 20 ms, is the time for UE processing specified in TS 38.133 [6] clause 6.1.1.2.2.

-	T∆ = 20 ms, is the time for fine time tracking and acquiring full timing information of the target cell specified in TS 38.133 [6] clause 6.1.1.2.2.

-	Tmargin = 2 ms, is the time for SSB post-processing specified in TS 38.133 [6] clause 6.1.1.2.2.

This gives a total of 72 ms.

During T3 UE is allowed to cause Tinterrupt1 interruption to cell 1. Tinterrupt1is defined in TS 38.133 [6] clause 6.1.3.2.2 Table 6.1.3.2.2-2. When UE is transmitting PRACH preamble to cell 2, interruption to cell 1 is allowed.

During T4 UE is allowed to cause Tinterrupt2 interruption to cell 1. Tinterrupt2 is defined in TS 38.133 [6] clause 6.1.3.2.2 Table 6.1.3.2.2-6.

UE shall finish cell 1 release in T4 and shall not send any CSI reports to cell 1 during T5.

6.3.1.11	NR SA FR1 Inter-band inter-frequency synchronous DAPS handover

6.3.1.11.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 inter-band inter-frequency DAPS handover requirements in synchronous scenario specified in TS 38.133 [6] clause 6.1.3.2.

## << Unchanged sections omitted >>

6.3.1.11.5	Test requirements

Table 6.3.1.11.5-1, Table 6.3.1.11.5-2 defines the primary level settings including test tolerances for all tests.

Table 6.3.1.11.5-1: Cell specific test parameters for NR SA FR1 Inter-band inter-frequency  synchronous DAPS handover (Cell 1)

| Parameter |  |  |  |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | T1 | T2 – T5 |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  | Config 1,4,7 |  |  |  | FDD |  |
|  |  | Config 2,3,5,6,8,9 |  |  |  | TDD |  |
| TDD configuration |  | Config 1,4,7 |  |  |  | Not Applicable |  |
|  |  | Config 2,5,8 |  |  |  | TDDConf.1.1 |  |
|  |  | Config 3,6,9 |  |  |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1,4,7 |  |  | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2,5,8 |  |  |  | 10: NRB,c = 52 |  |
|  |  | Config 3,6,9 |  |  |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1,4,7 |  |  | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2,5,8 |  |  |  | 10: NRB,c = 52 |  |
|  |  | Config 3,6,9 |  |  |  | 40: NRB,c = 106 |  |
| TRS configuration |  | Config 1,4,7 |  |  |  | TRS.1.1 FDD |  |
|  |  | Config 2,5,8 |  |  |  | TRS.1.1 TDD |  |
|  |  | Config 3,6,9 |  |  |  | TRS.1.2 TDD |  |
| DRX Cycle |  |  |  |  | ms | Not Applicable |  |
| PDSCH Reference |  | Config 1,4,7 |  |  |  | SR.1.1 FDD |  |
| measurement channel |  | Config 2,5,8 |  |  |  | SR.1.1 TDD |  |
|  |  | Config 3,6,9 |  |  |  | SR.2.1 TDD |  |
| CORESET Reference |  | Config 1,4,7 |  |  |  | CR.1.1 FDD |  |
| Channel |  | Config 2,5,8 |  |  |  | CR.1.1 TDD |  |
|  |  | Config 3,6,9 |  |  |  | CR.2.1 TDD |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |
| CSI-RS configuration |  |  | Config 1,4,7 |  |  | CSI-RS.1.1 FDD |  |
| for CSI reporting |  |  | Config 2,5,8 |  |  | CSI-RS.1.1 TDD |  |
|  |  |  | Config 3,6,9 |  |  | CSI-RS.2.1 TDD |  |
| reportConfigType |  |  |  |  |  | periodic |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |
| CSI reporting | Config 1,2,4,5,7,8 |  |  |  | slot | 5 |  |
| periodicity | Config 3,6,9 |  |  |  |  | 10 |  |
| CSI reporting offset | Config 1,2,4,5,7,8 |  |  |  | slot | 3 |  |
|  | Config 3,6,9 |  |  |  |  | 5 |  |
| SMTC Configuration |  |  |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1,2,4,5,7,8 |  |  |  | SSB.1 FR1 |  |
|  |  | Config 3,6,9 |  |  |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5,7,8 |  |  | kHz | 15 kHz |  |
|  |  | Config 3,6,9 |  |  |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,4,5,7,8 |  |  | kHz | 15 kHz |  |
|  |  | Config 3,6,9 |  |  |  | 30 kHz |  |
| PRACH configuration |  |  |  |  |  | FR1 PRACH configuration 2 |  |
| BWP |  | Initial DL BWP |  |  |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  |  |  | DLBWP.1.3 |  |
|  |  | Initial UL BWP |  |  |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  |  |  | ULBWP.1.3 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 |  |  |  |  | dBm/15kHz | -98 | -98 |
| Note2 |  |  |  | Config 1,2,4,5,7,8 | dBm/SCS | -98 | -98 |
|  |  |  |  | Config 3,6,9 |  | -95 | -95 |
|  |  |  |  |  | dB | 4 | 4 |
|  |  |  |  |  | dB | 4 | 4 |
| SSB_RP |  |  |  | Config 1,2,4,5,7,8 | dBm/SCS | -94 | -94 |
|  |  |  |  | Config 3,6,9 | dBm/SCS | -91 | -91 |
| IoNote3 |  |  |  | Config 1,2,4,5,7,8 | dBm/ 9.36MHz | -64.59 | -64.59 |
|  |  |  |  | Config 3,6,9 | dBm/ 38.16MHz | -58.49 | -58.49 |
| Propagation condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table 6.3.1.11.5-2: Cell specific test parameters for NR SA FR1 Inter-band inter-frequency  synchronous DAPS handover (Cell 2)

| Parameter |  |  |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 - T5 |
| NR RF Channel Number |  |  |  |  | 2 |  |
| Duplex mode | Config 1,2,3 |  |  |  | FDD |  |
|  | Config 4,5,6,7,8,9 |  |  |  | TDD |  |
| TDD configuration | Config 1,2,3 |  |  |  | Not Applicable |  |
|  | Config 4,5,6 |  |  |  | TDDConf.1.1 |  |
|  | Config 7,8,9 |  |  |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,3 |  |  | MHz | 10: NRB,c = 52 |  |
|  | Config 4,5,6 |  |  |  | 10: NRB,c = 52 |  |
|  | Config 7,8,9 |  |  |  | 40: NRB,c = 106 |  |
| BWP BW | Config 1,2,3 |  |  | MHz | 10: NRB,c = 52 |  |
|  | Config 4,5,6 |  |  |  | 10: NRB,c = 52 |  |
|  | Config 7,8,9 |  |  |  | 40: NRB,c = 106 |  |
| TRS configuration | Config 1,2,3 |  |  |  | TRS.1.1 FDD |  |
|  | Config 4,5,6 |  |  |  | TRS.1.1 TDD |  |
|  | Config 7,8,9 |  |  |  | TRS.1.2 TDD |  |
| DRx Cycle |  |  |  | ms | Not Applicable |  |
| PDSCH Reference | Config 1,2,3 |  |  |  | SR.1.1 FDD |  |
| measurement channel | Config 4,5,6 |  |  |  | SR.1.1 TDD |  |
|  | Config 7,8,9 |  |  |  | SR2.1 TDD |  |
| CORESET Reference | Config 1,2,3 |  |  |  | CR.1.1 FDD |  |
| Channel | Config 4,5,6 |  |  |  | CR.1.1 TDD |  |
|  | Config 7,8,9 |  |  |  | CR2.1 TDD |  |
| OCNG Patterns |  |  |  |  | OCNG pattern 1 |  |
| CSI-RS configuration |  | Config 1,4,7 |  |  | CSI-RS.1.1 FDD |  |
| for CSI reporting |  | Config 2,5,8 |  |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3,6,9 |  |  | CSI-RS.2.1 TDD |  |
| SMTC Configuration |  |  |  |  | SMTC pattern 1 |  |
| SSB Configuration | Config 1,2,3,4,5,6 |  |  |  | SSB.1 FR1 |  |
|  | Config 7,8,9 |  |  |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,3,4,5,6 |  |  | kHz | 15 kHz |  |
|  | Config 7,8,9 |  |  |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,3,4,5,6 |  |  | kHz | 15 kHz |  |
|  | Config 7,8,9 |  |  |  | 30 kHz |  |
| PRACH configuration |  |  |  |  | FR1 PRACH configuration 2 |  |
| BWP | Initial DL BWP |  |  |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.3 |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.3 |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  |  |  | dBm/15kHz | -98 | -98 |
| Note2 |  |  | Config 1,2,3,4,5,6 | dBm/SCS | -98 | -98 |
|  |  |  | Config 7,8,9 |  | -95 | -95 |
|  |  |  |  | dB | -Infinity | 4 |
|  |  |  |  | dB | -Infinity | 4 |
| SSB_RP |  |  | Config 1,2,3,4,5,6 | dBm/SCS | -Infinity | -94 |
|  |  |  | Config 7,8,9 | dBm/SCS | -Infinity | -91 |
| IoNote3 |  |  | Config 1,2,3,4,5,6 | dBm/ 9.36MHz | -70.05 | -64.59 |
|  |  |  | Config 7,8,9 | dBm/ 38.16MHz | -63.94 | -58.49 |
| Propagation condition |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than Dhandover1 from the beginning of time period T3. The rate of correct handovers observed during repeated tests shall be at least 90%, where:

Dhandover1= TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin

-	TRRC_procedure= 10 ms, is the RRC procedure delay specified in TS 38.331 [13] clause 12;

-	Tsearch = 0 ms for known target cell, is the time required to search the target cell specified in 38.133 [6] clause 6.1.1.2.2;

-	TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell specified in TS 38.133 [6] clause 6.1.1.2.2.

-	Tprocessing = 20 ms, is the time for UE processing specified in TS 38.133 [6] clause 6.1.1.2.2.

-	T∆ = 20 ms, is the time for fine time tracking and acquiring full timing information of the target cell specified in TS 38.133 [6] clause 6.1.1.2.2.

-	Tmargin = 2 ms, is the time for SSB post-processing specified in TS 38.133 [6] clause 6.1.1.2.2.

This gives a total of 72 ms.

The UE shall complete to release Cell 1 less than Dhandover2 from the beginning of time period T4. During Dhandover2, the interruption on Cell 2 shall not exceed Tinterrupt2 as defined in TS 38.133 [6] Table 6.1.3.2.2-7 for synchronous DAPS HO.

The handover delay Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [13].

6.3.1.12	NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover

6.3.1.12.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 inter-band inter-frequency DAPS handover requirements in asynchronous scenario specified in TS 38.133 [6] clause 6.1.3.2.

## << Unchanged sections omitted >>

6.3.1.12.5	Test requirements

Table 6.3.1.12.5-1, Table 6.3.1.12.5-2 defines the primary level settings including test tolerances for all tests.

Table 6.3.1.12.5-1: Cell specific test parameters for NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover (Cell 1)

| Parameter |  |  |  |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | T1 | T2 – T5 |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  | Config 1,4,7 |  |  |  | FDD |  |
|  |  | Config 2,3,5,6,8,9 |  |  |  | TDD |  |
| TDD configuration |  | Config 1,4,7 |  |  |  | Not Applicable |  |
|  |  | Config 2,5,8 |  |  |  | TDDConf.1.1 |  |
|  |  | Config 3,6,9 |  |  |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1,4,7 |  |  | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2,5,8 |  |  |  | 10: NRB,c = 52 |  |
|  |  | Config 3,6,9 |  |  |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1,4,7 |  |  | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2,5,8 |  |  |  | 10: NRB,c = 52 |  |
|  |  | Config 3,6,9 |  |  |  | 40: NRB,c = 106 |  |
| TRS configuration |  | Config 1,4,7 |  |  |  | TRS.1.1 FDD |  |
|  |  | Config 2,5,8 |  |  |  | TRS.1.1 TDD |  |
|  |  | Config 3,6,9 |  |  |  | TRS.1.2 TDD |  |
| DRX Cycle |  |  |  |  | ms | Not Applicable |  |
| PDSCH Reference |  | Config 1,4,7 |  |  |  | SR.1.1 FDD |  |
| measurement channel |  | Config 2,5,8 |  |  |  | SR.1.1 TDD |  |
|  |  | Config 3,6,9 |  |  |  | SR.2.1 TDD |  |
| CORESET Reference |  | Config 1,4,7 |  |  |  | CR.1.1 FDD |  |
| Channel |  | Config 2,5,8 |  |  |  | CR.1.1 TDD |  |
|  |  | Config 3,6,9 |  |  |  | CR.2.1 TDD |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |
| CSI-RS configuration |  |  | Config 1,4,7 |  |  | CSI-RS.1.1 FDD |  |
| for CSI reporting |  |  | Config 2,5,8 |  |  | CSI-RS.1.1 TDD |  |
|  |  |  | Config 3,6,9 |  |  | CSI-RS.2.1 TDD |  |
| reportConfigType |  |  |  |  |  | periodic |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |
| CSI reporting | Config 1,2,4,5,7,8 |  |  |  | slot | 5 |  |
| periodicity | Config 3,6,9 |  |  |  |  | 10 |  |
| CSI reporting offset | Config 1,2,4,5,7,8 |  |  |  | slot | 3 |  |
|  | Config 3,6,9 |  |  |  |  | 5 |  |
| SMTC Configuration |  |  |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1,2,4,5,7,8 |  |  |  | SSB.1 FR1 |  |
|  |  | Config 3,6,9 |  |  |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5,7,8 |  |  | kHz | 15 kHz |  |
|  |  | Config 3,6,9 |  |  |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,4,5,7,8 |  |  | kHz | 15 kHz |  |
|  |  | Config 3,6,9 |  |  |  | 30 kHz |  |
| PRACH configuration |  |  |  |  |  | FR1 PRACH configuration 2 |  |
| BWP |  | Initial DL BWP |  |  |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  |  |  | DLBWP.1.3 |  |
|  |  | Initial UL BWP |  |  |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  |  |  | ULBWP.1.3 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 |  |  |  |  | dBm/15kHz | -98 | -98 |
| Note2 |  |  |  | Config 1,2,4,5,7,8 | dBm/SCS | -98 | -98 |
|  |  |  |  | Config 3,6,9 |  | -95 | -95 |
|  |  |  |  |  | dB | 4 | 4 |
|  |  |  |  |  | dB | 4 | 4 |
| SSB_RP |  |  |  | Config 1,2,4,5,7,8 | dBm/SCS | -94 | -94 |
|  |  |  |  | Config 3,6,9 | dBm/SCS | -91 | -91 |
| IoNote3 |  |  |  | Config 1,2,4,5,7,8 | dBm/ 9.36MHz | -64.59 | -64.59 |
|  |  |  |  | Config 3,6,9 | dBm/ 38.16MHz | -58.49 | -58.49 |
| Propagation condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table 6.3.1.12.5-2: Cell specific test parameters for NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover (Cell 2)

| Parameter |  |  |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 - T5 |
| NR RF Channel Number |  |  |  |  | 2 |  |
| Duplex mode | Config 1,2,3 |  |  |  | FDD |  |
|  | Config 4,5,6,7,8,9 |  |  |  | TDD |  |
| TDD configuration | Config 1,2,3 |  |  |  | Not Applicable |  |
|  | Config 4,5,6 |  |  |  | TDDConf.1.1 |  |
|  | Config 7,8,9 |  |  |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,3 |  |  | MHz | 10: NRB,c = 52 |  |
|  | Config 4,5,6 |  |  |  | 10: NRB,c = 52 |  |
|  | Config 7,8,9 |  |  |  | 40: NRB,c = 106 |  |
| BWP BW | Config 1,2,3 |  |  | MHz | 10: NRB,c = 52 |  |
|  | Config 4,5,6 |  |  |  | 10: NRB,c = 52 |  |
|  | Config 7,8,9 |  |  |  | 40: NRB,c = 106 |  |
| TRS configuration | Config 1,2,3 |  |  |  | TRS.1.1 FDD |  |
|  | Config 4,5,6 |  |  |  | TRS.1.1 TDD |  |
|  | Config 7,8,9 |  |  |  | TRS.1.2 TDD |  |
| DRx Cycle |  |  |  | ms | Not Applicable |  |
| PDSCH Reference | Config 1,2,3 |  |  |  | SR.1.1 FDD |  |
| measurement channel | Config 4,5,6 |  |  |  | SR.1.1 TDD |  |
|  | Config 7,8,9 |  |  |  | SR2.1 TDD |  |
| CORESET Reference | Config 1,2,3 |  |  |  | CR.1.1 FDD |  |
| Channel | Config 4,5,6 |  |  |  | CR.1.1 TDD |  |
|  | Config 7,8,9 |  |  |  | CR2.1 TDD |  |
| OCNG Patterns |  |  |  |  | OCNG pattern 1 |  |
| CSI-RS configuration |  | Config 1,4,7 |  |  | CSI-RS.1.1 FDD |  |
| for CSI reporting |  | Config 2,5,8 |  |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3,6,9 |  |  | CSI-RS.2.1 TDD |  |
| SMTC Configuration |  |  |  |  | SMTC pattern 1 |  |
| SSB Configuration | Config 1,2,3,4,5,6 |  |  |  | SSB.1 FR1 |  |
|  | Config 7,8,9 |  |  |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,3,4,5,6 |  |  | kHz | 15 kHz |  |
|  | Config 7,8,9 |  |  |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,3,4,5,6 |  |  | kHz | 15 kHz |  |
|  | Config 7,8,9 |  |  |  | 30 kHz |  |
| PRACH configuration |  |  |  |  | FR1 PRACH configuration 2 |  |
| BWP | Initial DL BWP |  |  |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.3 |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.3 |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  |  |  | dBm/15kHz | -98 | -98 |
| Note2 |  |  | Config 1,2,3,4,5,6 | dBm/SCS | -98 | -98 |
|  |  |  | Config 7,8,9 |  | -95 | -95 |
|  |  |  |  | dB | -Infinity | 4 |
|  |  |  |  | dB | -Infinity | 4 |
| SSB_RP |  |  | Config 1,2,3,4,5,6 | dBm/SCS | -Infinity | -94 |
|  |  |  | Config 7,8,9 | dBm/SCS | -Infinity | -91 |
| IoNote3 |  |  | Config 1,2,3,4,5,6 | dBm/ 9.36MHz | -70.05 | -64.59 |
|  |  |  | Config 7,8,9 | dBm/ 38.16MHz | -63.94 | -58.49 |
| Propagation condition |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than Dhandover1 from the beginning of time period T3. The rate of correct handovers observed during repeated tests shall be at least 90%, where:

Dhandover1= TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin

- TRRC_procedure= 10 ms, is the RRC procedure delay specified in TS 38.331 [13] clause 12;

- Tsearch = 0 ms for known target cell, is the time required to search the target cell specified in 38.133 [6] clause 6.1.1.2.2.

- TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell specified in TS 38.133 [6] clause 6.1.1.2.2.

- Tprocessing = 20 ms, is the time for UE processing specified in TS 38.133 [6] clause 6.1.1.2.2.

- T∆ = 20 ms, is the time for fine time tracking and acquiring full timing information of the target cell specified in TS 38.133 [6] clause 6.1.1.2.2.

- Tmargin = 2 ms, is the time for SSB post-processing specified in TS 38.133 [6] clause 6.1.1.2.2.

This gives a total of 72 ms.

The UE shall complete to release Cell 1 less than Dhandover2 from the beginning of time period T4. During Dhandover2, the interruption on Cell 2 shall not exceed Tinterrupt2 as defined in TS 38.133 [6] Table 6.1.3.2.2-6 for asynchronous DAPS HO.

The handover delay Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [13].

## << Unchanged sections omitted >>

6.3.1.13 to 6.3.1.17

6.3.1.18	NR SA FR1 handover with unknown target cell operating with 12 PRB SSB bandwidth

6.3.1.18.1	Test purpose

To verify the UEs ability to perform the NR FR1-NR FR1 intra-frequency handover requirements for unknown target cell operating with 12 PRB SSB bandwidth as specified in TS 38.133[6] clause 6.1.1.2.

## << Unchanged sections omitted >>

6.3.3.1	NR SA FR1 conditional handover

6.3.3.1.1	Test purpose

To verify the requirement for the NR FR1 intra-frequency conditional handover requirements specified in TS 38.133 [6] clause 6.1.4.2

## << Unchanged sections omitted >>

6.3.3.1.5	Test requirements

Table 6.3.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.3.3.1.5-1: Cell specific test parameters for NR SA FR1 conditional handover

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 8 | -2.41 |  | -Infinity |  | 1.36 |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 10 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -Infinity |  | -88 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -Infinity |  | -85 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -57.67 |  | -61.41 |  | -57.67 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -50.56 |  | -55.31 |  | -50.56 |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

The rate of correct handovers observed during repeated tests shall be at least 90%.

The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt from the start of T2. where:

- Tmeasure = 800 ms, is the measurements time specified in TS 38.133 [6] clause 6.1.4.2.2.

- TCHO_execution = 10 ms, is the conditional execution preparation time specified in TS 38.133 [6] clause 6.1.4.2.2.

- Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms, is the interruption time specified in TS 38.133 [6] 6.1.4.2.4.

- Tprocessing = 20 ms, is time for UE processing;

- TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell;

- T∆ = 20 ms, is time for fine time tracking and acquiring full timing information of the target cell;

- Tmargin = 2 ms, is time for SSB post-processing.

This gives a total of 872 ms

The interruption during T2 shall not exceeed Tinterrupt = 62ms.

6.3.3.2	NR SA FR1-FR1 conditional handover

6.3.3.2.1	Test purpose

To verify the requirement for the NR conditional FR1-NR FR1 inter-frequency conditional handover requirements specified in TS 38.133 [6] clause 6.1.4.2.

## << Unchanged sections omitted >>

6.3.3.2.5	Test requirements

Table 6.3.3.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.3.3.2.5-1: Cell specific test parameters for NR SA FR1-FR1 conditional handover

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| Gap pattern ID |  |  |  | gp0 |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 6.70 |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 6.70 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 |  | -Infinity |  | -91.30 |  |
|  | Config 3 |  | dBm/SCS | -91 | -91 |  | -Infinity |  | -88.30 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -62.51 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -56.40 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

The rate of correct handovers observed during repeated tests shall be at least 90%.

The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt from the start of T2. where:

-	Tmeasure = 920 ms, is the measurements time specified in TS 38.133 [6] clause 6.1.4.2.2.

-	TCHO_execution = 10 ms, is the conditional execution preparation time specified in TS 38.133 [6] clause 6.1.4.2.2.

-	Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms, is the interruption time specified in TS 38.133 [6] 6.1.4.2.4.

-	Tprocessing = 20 ms, is time for UE processing;

-	TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell;

-	T∆ = 20 ms, is time for fine time tracking and acquiring full timing information of the target cell;

-	Tmargin = 2 ms, is time for SSB post-processing.

This gives a total of 992 ms

The interruption during T2 shall not exceed Tinterrupt = 62ms.

## < End of changes >>