3GPP TSG-RAN5 Meeting #106	R5-250739

# Athens, Greece, Feb 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3223 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test cases 6.5L.3.2 OOBE for inter-band CA with Tx Diversity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test cases 6.5L.3.2 out of band emissions for inter-band CA with Tx Diversity are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the new test cases 6.5L.3.2 out of band emissions for inter-band CA with Tx Diversity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned test cases will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5L.3.2.0(new), 6.5L.3.2.1(new), 6.5L.3.2.2(new), 6.5L.3.2.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5L.3.1.5	Test requirements

Same test requirement as 6.5G.1 for the band supporting Tx Diversity and 6.5.1 for the other band.

6.5L.3.2	Out of band emission for inter-band UL CA with Tx Diversity

6.5L.3.2.0	Minimum conformance requirements

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the out of band emission requirement is defined per component carrier while both component carriers are active. The requirements specified in clauses 6.5.2.1 and 6.5.2.2 shall apply for the component carrier without Tx Diversity and the requirements specified in clause 6.5G.2 shall apply for the component carrier configured with Tx Diversity. If for some frequency spectrum emission masks of component carriers overlap, then spectrum emission mask allowing higher power spectral density applies for that frequency. If for some frequency a component carrier spectrum emission mask overlaps with the channel bandwidth of another component carrier, then the emission mask does not apply for that frequency.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5L.3.2.

6.5L.3.2.1	Spectrum emission mask for inter-band UL CA with Tx Diversity

6.5L.3.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth.

6.5L.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.5L.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5L.3.2.0.

6.5L.3.2.1.4	Test description

This test is covered by clauses 6.5.2.2 Spectrum emission mask and 6.5G.2.2 Spectrum emission mask for Tx Diversity.

6.5L.3.2.1.5	Test requirements

Same test requirement as 6.5G.2.2 for the band supporting Tx Diversity and 6.5.2.2 for the other band.

6.5L.3.2.2	Additional spectrum emission mask for inter-band UL CA with Tx Diversity

6.5L.3.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.5L.3.2.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.5L.3.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5L.3.2.0.

6.5L.3.2.2.4	Test description

This test is covered by clauses 6.5.2.3 Additional spectrum emission mask and 6.5G.2.3 Additional spectrum emission mask for Tx Diversity.

6.5L.3.2.2.5	Test requirements

Same test requirement as 6.5G.2.3 for the band supporting Tx Diversity and 6.5.2.3 for the other band.

6.5L.3.2.3	Adjacent channel leakage ratio for inter-band UL CA with Tx Diversity

6.5L.3.2.3.1	NR ACLR for inter-band UL CA with Tx Diversity

6.5L.3.2.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5L.3.2.3.1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.5L.3.2.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5L.3.2.0.

6.5L.3.2.3.1.4	Test description

This test is covered by clauses 6.5.2.4.1 NR ACLR and 6.5G.2.4.1 NR ACLR for Tx Diversity.

6.5L.3.2.3.1.5	Test requirements

Same test requirement as 6.5G.2.4.1 for the band supporting Tx Diversity and 6.5.2.4.1 for the other band.

6.5L.3.2.3.2	UTRA ACLR for inter-band UL CA with Tx Diversity

6.5L.3.2.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5L.3.2.3.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.5L.3.2.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5L.3.2.0.

6.5L.3.2.3.2.4	Test description

This test is covered by clauses 6.5.2.4.2 UTRA ACLR and 6.5G.2.4.2 UTRA ACLR for Tx Diversity.

6.5L.3.2.3.2.5	Test requirements

Same test requirement as 6.5G.2.4.2 for the band supporting Tx Diversity and 6.5.2.4.2 for the other band.

<End of Changes>