3GPP TSG-RAN5 Meeting #106	R5-250740

# Athens, Greece, Feb 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3224 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.5L.3.4 for inter-band CA with Tx Diversity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 6.5L.3.4 Transmit intermodulation for inter-band CA with Tx Diversity are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the new test case 6.5L.3.4 Transmit intermodulation for inter-band CA with Tx Diversity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case 6.5L.3.4 will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5L.3.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5L.3.4	Transmit intermodulation for inter-band UL CA with Tx Diversity

6.5L.3.4.1	Test purpose

To verify that the UE transmit intermodulation does not exceed the described value in the test requirement.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

6.5L.3.4.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.5L.3.4.3	Minimum conformance requirements

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the transmit intermodulation requirement specified in clause 6.5.4 shall apply for the component carrier without Tx Diversity and the transmit intermodulation requirement specified in Table 6.5G.4 shall apply for the component carrier configured with Tx Diversity with all component carriers active.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5L.3.4.

6.5L.3.4.4	Test description

This test is covered by clauses 6.5.4 Transmit intermodulation and 6.5G.4 Transmit intermodulation for Tx Diversity.

6.5L.3.4.5	Test requirements

Same test requirement as 6.5G.4 for the band supporting Tx Diversity and 6.5.4 for the other band.

<End of Changes>