3GPP TSG-RAN5 Meeting #106	R5-250754

# Athens, Greece, 17th - 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1508 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclination correction in orbital format GSO ephemeris files |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is observed that some of the values corresponding to inclination-r17 are out the limits with respect to the specification 3GPP TS 36.331: |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inclination-r17 values out of range corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Failure in the configuration of the IoT NTN ephemeris if wrong inclination-r17 values are used within the satellite orbit. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.2.1, 8.4.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.905 CR 0280 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << START OF CHANGES 1>>>

#### 7.6.2.1	Ephemeris for elevation angle 30º

All eMTC NTN RRM test cases defined in section 14 in TS 36.521-3 [34], unless otherwise stated, shall use ephemeris configuration in tables 7.6.2.1-1a to 7.6.2.1-3b, corresponding to an elevation angle of 30º for the type of satellite under test, keeping such information constant throughout the duration of the test:

-	In case UE supports only NGSO satellites or both GSO and NGSO satellites, ephemeris values in Table 7.6.2.1-3a or 7.6.2.1-3b shall be used.

-	Tables 7.6.2-4a to 7.6.2-4d shall only be used if the test case under test is only applicable to GEO satellites.

Table 7.6.2.1-1a: SystemInformationBlockType31 – eMTC NTN Serving cell Ephemeris Information (stateVectors format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -17129898 |  |  |  |  |  |
| positionY-r17 |  | 27505094 |  |  |  |  |  |
| positionZ-r17 |  | 1413939 |  |  |  |  |  |
| velocityVX-r17 |  | 116 |  |  |  |  |  |
| velocityVY-r17 |  | 378 |  |  |  |  |  |
| velocityVZ-r17 |  | -5944 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 7.6.2.1-1b: SystemInformationBlockType31 – eMTC NTN Serving cell Ephemeris Information (orbitalParameters format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |  |  |  |  |
| semiMajorAxis-r17- |  | 8393544277 |  |  |  |  |  |
| eccentricity-r17 |  | 0 |  |  |  |  |  |
| periapsis-r17 |  | 266532360 |  |  |  |  |  |
| longitude-r17 |  | 224998499 |  |  |  |  |  |
| inclination-r17 |  | 2125399 |  |  |  |  |  |
| meanAnomaly-r17 |  | 118496066 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 7.6.2.1-2a: Void

Table 7.6.2.1-2b: Void

Table 7.6.2.1-3a: SystemInformationBlockType31- eMTC NTN Serving cell Ephemeris Information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2935552 |  |  |  |  |  |
| positionY-r17 |  | 4895460 |  |  |  |  |  |
| positionZ-r17 |  | 1188458 |  |  |  |  |  |
| velocityVX-r17 |  | 8506 |  |  |  |  |  |
| velocityVY-r17 |  | -23314 |  |  |  |  |  |
| velocityVZ-r17 |  | 118253 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 7.6.2.1-3b: SystemInformationBlockType31- eMTC NTN Serving cell Ephemeris Information (orbitalParameters format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |  |  |  |  |
| semiMajorAxis-r17- |  | 273626880 |  |  |  |  |  |
| eccentricity-r17 |  | 769300 |  |  |  |  |  |
| periapsis-r17 |  | 198830479 |  |  |  |  |  |
| longitude-r17 |  | 89834718 |  |  |  |  |  |
| inclination-r17 |  | 65473992 |  |  |  |  |  |
| meanAnomaly-r17 |  | 7822790 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 7.6.2.1-4a: SystemInformationBlockType31 – eMTC NTN Serving cell Ephemeris Information (stateVectors format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.035 degrees, one-way delay equal to 127.33 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-4b: SystemInformationBlockType31 – eMTC NTN Serving cell Ephemeris Information orbitalParameters format) for GEO satellites (serving cell 30º elevation angle conditions

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 200160368 |  |  |
| longitude-r17 | 24831613 |  |  |
| inclination-r17 | 5309231 |  |  |
| meanAnomaly-r17 | 66742914 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.035 degrees, one-way delay equal to 127.33 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-4c: SystemInformationBlockType33 (Rel-18)/Test equipment configuration (Rel-17) – eMTC NTN Neighbour cell Ephemeris Information (stateVectors format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -6457459 |  |  |
| positionY-r17 | 31531426 |  |  |
| positionZ-r17 | 4004156 |  |  |
| velocityVX-r17 | -387 |  |  |
| velocityVY-r17 | -66 |  |  |
| velocityVZ-r17 | -104 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-4d: SystemInformationBlockType33 (Rel-18)/Test equipment configuration (Rel-17) – eMTC NTN Neighbour cell Ephemeris Information (orbitalParameters format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 ::= CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 249232221 |  |  |
| longitude-r17 | 228396393 |  |  |
| inclination-r17 | 12328441 |  |  |
| meanAnomaly-r17 | 67231401 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

## <<< END OF CHANGES 1>>>

## << START OF CHANGES 2>>>

#### 8.4.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause FFS in TR36.905 [FFS].

#### 8.4.6.2.0	Assumptions for Ephemeris generation

Same as in section 7.6.2.0.

#### 8.4.6.2.1	Ephemeris for elevation angle 30º

All NB-IoT NTN RRM test cases defined in in section 13 in TS 36.521-3 [34], unless otherwise stated, shall use ephemeris configuration in tables 8.4.6.2.1-1a to 8.4.6.2.1-3b, corresponding to an elevation angle of 30º for the type of satellite under test, keeping such information constant throughout the duration of the test:

-	In case UE supports only NGSO satellites or both GSO and NGSO satellites, ephemeris values in Table 8.4.6.2.1-3a or 8.4.6.2.1-3b shall be used.

-	Tables 8.4.6.2-4a to 8.4.6.2-4d shall only be used if the test case under test is only applicable to GEO satellites.

Table 8.4.6.2.1-1a: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (stateVectors format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -17129898 |  |  |  |  |  |
| positionY-r17 |  | 27505094 |  |  |  |  |  |
| positionZ-r17 |  | 1413939 |  |  |  |  |  |
| velocityVX-r17 |  | 116 |  |  |  |  |  |
| velocityVY-r17 |  | 378 |  |  |  |  |  |
| velocityVZ-r17 |  | -5944 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 8.4.6.2.1-1b: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (orbitalParameters format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |  |  |  |  |
| semiMajorAxis-r17- |  | 8393544277 |  |  |  |  |  |
| eccentricity-r17 |  | 0 |  |  |  |  |  |
| periapsis-r17 |  | 266532360 |  |  |  |  |  |
| longitude-r17 |  | 224998499 |  |  |  |  |  |
| inclination-r17 |  | 2125399 |  |  |  |  |  |
| meanAnomaly-r17 |  | 118496066 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 8.4.6.2.1-2a: Void

Table 8.4.6.2.1-2b: Void

Table 8.4.6.2.1-3a: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2935552 |  |  |  |  |  |
| positionY-r17 |  | 4895460 |  |  |  |  |  |
| positionZ-r17 |  | 1188458 |  |  |  |  |  |
| velocityVX-r17 |  | 8506 |  |  |  |  |  |
| velocityVY-r17 |  | -23314 |  |  |  |  |  |
| velocityVZ-r17 |  | 118253 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 8.4.6.2.1-3b: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (orbitalParameters format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |  |  |  |  |
| semiMajorAxis-r17 |  | 273626880 |  |  |  |  |  |
| eccentricity-r17 |  | 769300 |  |  |  |  |  |
| periapsis-r17 |  | 198830479 |  |  |  |  |  |
| longitude-r17 |  | 89834718 |  |  |  |  |  |
| inclination-r17 |  | 65473992 |  |  |  |  |  |
| meanAnomaly-r17 |  | 7822790 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 8.4.6.2.1-4a: SystemInformationBlockType31-NB – NB-IoT NTN Serving cell Ephemeris Information (stateVectors format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.035 degrees, one-way delay equal to 127.33 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-4b: SystemInformationBlockType31-NB – NB-IoT NTN Serving cell Ephemeris Information orbitalParameters format) for GEO satellites (serving cell 30º elevation angle conditions

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 200160368 |  |  |
| longitude-r17 | 24831613 |  |  |
| inclination-r17 | 5309231 |  |  |
| meanAnomaly-r17 | 66742914 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.035 degrees, one-way delay equal to 127.33 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-4c: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-17) – NB-IoT NTN Neighbour cell Ephemeris Information (stateVectors format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -6457459 |  |  |
| positionY-r17 | 31531426 |  |  |
| positionZ-r17 | 4004156 |  |  |
| velocityVX-r17 | -387 |  |  |
| velocityVY-r17 | -66 |  |  |
| velocityVZ-r17 | -104 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-4d: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-17) – NB-IoT NTN Neighbour cell Ephemeris Information (orbitalParameters format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 249232221 |  |  |
| longitude-r17 | 228396393 |  |  |
| inclination-r17 | 12328441 |  |  |
| meanAnomaly-r17 | 67231401 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

#### 8.4.6.2.2	Ephemeris for NB-IoT NTN RRM UL timing accuracy test cases

Table 8.4.6.2.2-1: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for GSO satellites (minimum Doppler)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -16944523 |  |  |  |  |  |
| positionY-r17 |  | 27362209 |  |  |  |  |  |
| positionZ-r17 |  | 4019393 |  |  |  |  |  |
| velocityVX-r17 |  | -336 |  |  |  |  |  |
| velocityVY-r17 |  | -208 |  |  |  |  |  |
| velocityVZ-r17 |  | 1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |  |  |  |  |

Table 8.4.6.2.2-2: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information for NGSO (LEO-1200) satellites (minimum Doppler)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2771542 |  |  |  |  |  |
| positionY-r17 |  | 4509243 |  |  |  |  |  |
| positionZ-r17 |  | 2455572 |  |  |  |  |  |
| velocityVX-r17 |  | 20837 |  |  |  |  |  |
| velocityVY-r17 |  | -46176 |  |  |  |  |  |
| velocityVZ-r17 |  | 109528 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm. |  |  |  |  |  |  |  |

Table 8.4.6.2.2-3: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information for GSO satellites (maximum positive Doppler)

| Derivation Path: Table clause 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -16989970 |  |  |  |  |  |
| positionY-r17 |  | 27591785 |  |  |  |  |  |
| positionZ-r17 |  | 1413211 |  |  |  |  |  |
| velocityVX-r17 |  | 390 |  |  |  |  |  |
| velocityVY-r17 |  | -65 |  |  |  |  |  |
| velocityVZ-r17 |  | 5945 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to 0.15 ppm |  |  |  |  |  |  |  |

Table 8.4.6.2.2-4: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2654549 |  |  |  |  |  |
| positionY-r17 |  | 4386991 |  |  |  |  |  |
| positionZ-r17 |  | 1594205 |  |  |  |  |  |
| velocityVX-r17 |  | 14581 |  |  |  |  |  |
| velocityVY-r17 |  | -34487 |  |  |  |  |  |
| velocityVZ-r17 |  | 120182 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle |  |  |  |  |  |  |  |

Table 8.4.6.2.2-5: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information for GSO  satellites (maximum negative Doppler)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -17129898 |  |  |  |  |  |
| positionY-r17 |  | 27505094 |  |  |  |  |  |
| positionZ-r17 |  | 1413939 |  |  |  |  |  |
| velocityVX-r17 |  | 116 |  |  |  |  |  |
| velocityVY-r17 |  | 378 |  |  |  |  |  |
| velocityVZ-r17 |  | -5944 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle. |  |  |  |  |  |  |  |

Table 8.4.6.2.2-6: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information for NGSO (LEO-600)  satellites (maximum negative Doppler)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2401021 |  |  |  |  |  |
| positionY-r17 |  | 3830217 |  |  |  |  |  |
| positionZ-r17 |  | 2907041 |  |  |  |  |  |
| velocityVX-r17 |  | 28879 |  |  |  |  |  |
| velocityVY-r17 |  | -61410 |  |  |  |  |  |
| velocityVZ-r17 |  | 105774 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 149.91 degrees, one-way delay equal to 3.66 ms and Doppler equal to -0-19.95 ppm |  |  |  |  |  |  |  |

## <<< END OF CHANGES 2>>>