# 3GPP RAN WG5 Meeting #106							R5-250785

# Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.551 | CR | 0041 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On Annex C.3.5, FR1 Cross-Polarization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Core specification alignment |  |  |  |  |  |  |  |  |
| Summary of change: |  | The additional text further clarify the FR1 Cross-Polarization procedure, ensuring that detructive interference at the CE input won’t occur. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The FR1 Cross-Polarization procedure reccomendation won’t be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGE >>>

## C.3.5	Cross-polarization

This measurement checks how well the measured vertically or horizontally polarized power levels follow expected values. The test setup for cross-polarization is the same as PDP validation in Figure C.3.2-1.

Method of measurement:	Step the emulation and store traces from VNA.

VNA settings for cross-polarization measurements are presented in Table C.3.5-1.

Table C.3.5-1: VNA settings for cross-polarization

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink Centre Frequency in Table C.3.1-1 |
| Span | MHz | 40 |
| Number of traces |  | 1000 |
| Number of points |  | 802 |
| Averaging |  | 1 |

Channel model specification for cross-polarization measurements is presented in Table C.3.5-2.

Table C.3.5-2: Channel model specification for cross-polarization

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Distance between traces in channel model | wavelength (see note 1) | > 2 |
| Channel model |  | As specified in clause C.1 |
| Mobile speed (see note 2) | km/h | 30 |
| NOTE 1: Time [s] = distance [] / MS speed [/s]. MS speed [/s] = MS speed [m /s] / Speed of light [m/s] * Centre frequency [Hz]. NOTE 2: The mobile speed is valid for the Time Domain Alternative method only. |  |  |

Measurement Procedure:

Step the emulation and store traces from VNA. i.e. run the emulation to CIR number 1, pause, measure VNA trace, run the emulation to CIR number 10, pause, measure VNA trace. Continue until 1000 VNA traces are measured. Ensure that the phase relationship at the inputs of the channel emulator do not produce destructive interference.

a.	Use a vertically polarized sleeve dipole to measure the V component.

b.	Use a horizontally polarized (vertically oriented) magnetic loop dipole, or a horizontally polarized sleeve dipole measured in four orthogonal horizontal positions and summed to measure the H component.

Method of measurement result analysis:

Measured VNA traces (frequency responses HV(t,f) and HHt,f are saved into a hard drive. The data is read into, e.g. MATLAB. The frequency responses are averaged in power over time and frequency and the V/H ratio calculated as follows:

PV=tfHV(t,f)2

PH=tfHHt,f2

PV/H[dB]=10log10PVPH

Beam-Specific Block Diagram

It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

Figure C.3.5-1: Setup for Beam-Specific V/H measurements (Beam 1)

Figure C.3.5-2: Setup for Beam-Specific V/H measurements (Beam 2 CDL-C UMa only)

Time Domain Alternative Method:

The power in the Vertical and Horizontal polarizations can also be measured in time domain. The measurement setup for Beam-Specific are presented in Figures C.3.5-3, and C.3.5-4.

Figure C.3.5-3: Setup for Beam-Specific V/H measurements (Beam 1)

Figure C.3.5-4: Setup for Beam-Specific V/H measurements (Beam 2 CDL-C UMa only)

The instruments settings are the same as those in Tables C.3.4-6 and C.3.4-7. The measurement analysis is the same as that of the frequency domain method setting the summation over f to a single point.

The reference V/H-ratios for CDL-C UMa and CDL-C UMi channel model validation are defined in Tables C.3.5-3 and C.3.5-4, respectively.

Table C.3.5-3: Reference V/H-ratios for CDL-C UMa

| UMa C, fc ≤ 2.5 GHz | Beam 1 | Input 1+2: V/H = 0 dB |
| --- | --- | --- |
|  | Beam 2 | Input 3+4: V/H = 0 dB |
| UMa C, fc > 2.5 GHz | Beam 1 | Input 1+2: V/H = 0 dB |
|  | Beam 2 | Input 3+4: V/H = 0 dB |

Table C.3.5-4: Reference V/H-ratios for CDL-C UMi

| UMi C, fc ≤ 2.5 GHz | Beam 1 | Inputs 1+2: V/H = 0 dB |
| --- | --- | --- |
| UMi C, fc > 2.5 GHz | Beam 1 | Inputs 1+2: V/H = 0 dB |

<<< END OF CHANGE >>>