3GPP TSG-RAN5 Meeting #106	R5-250811

# Athens, Greece, Feb 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3761 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of minimum requirements for NES-based conditional handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN4#113 meeting, R4-2420220 was agreed to update minimum requirements for NES-based conditional handover. This document is to align with the RAN4 core requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update minimum requirements for NES-based conditional handover |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The minimum requirements will keep unaligned with core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.0.1, 6.3.3.0.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3.3.0	Minimum conformance requirements

6.3.3.0.1	Minimum conformance requirements for NR FR1 intra-frequency conditional handover

[TS 38.133, clause 6.1.4.2.1]

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [13].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [13].

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover.

-	a condition exists at the measurement reference point which will trigger the conditional handover, or

-	a condition exists at the measurement reference point which will trigger the NES-based conditional handover

Tmeasure is the measurements time stated in TS 38.133 [6] clause 6.1.4.2.2.

TCHO_execution is the conditional execution preparation time in TS 38.133 [6] clause 6.1.4.2.3.

Tinterrupt is the interruption time stated in TS 38.133 [6] clause 6.1.4.2.4.

The NES-based conditional handover delay requirements shall apply if UE receives a RRC message implying NES-based conditional handover before receiving the NES indication in DCI 2-9 command.

UE is not expected to receive a NES indication in DCI 2-9 command before receiving a RRC message implying NES-based conditional handover. When UE receives a RRC message implying NES-based conditional handover but no NES indication in DCI 2-9 command, no NES-based conditional handover requirement is applied. The NES indication is specified in clause 5.5.4 in TS 38.331.

[TS 38.133, clause 6.1.4.2.2]

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For NES-based conditional intra-frequency handover:

-	If UE successfully decodes DCI 2-9 command earlier than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index

-	If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, and the condition of NES-based CHO keeps existing till UE receiving the DCI 2-9 command, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.When TTT or L3 filtering is used an additional delay can be expected.

-	If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, and the condition of NES-based CHO is not met when receiving the DCI 2-9 command, then the measurement time delay equals to the time span from the end of TEvent_DU until a condition keeps existing for TSSB measurement period_intra which will immediately trigger the NES-based conditional handover after DCI 2-9.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used, an additional delay can be expected.

[TS 38.133, clause 6.1.4.2.3]

TCHO_execution is the UE execution preparation time for conditional handover.

For conditional handover, it starts after UE realizes the condition of CHO is met and identity of the target cell is determined.

For NES-based conditional handover,

-	It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9.

-	Otherwise, it starts after UE successfully decodes DCI 2-9.

And TCHO_execution can be up to 10ms.

[TS 38.133, clause 6.1.4.2.4]

The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [8]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [13] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.[TS 38.133, clause 9.2.2]

The requirements in TS 38.133 [6] clause 9.2 apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in  TS 38.133 [6] clauses 10.1.2 for FR1, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133, clause 9.2.5.1]

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in TS 38.133 [6] Table 9.2.5.1-1.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in TS 38.133 [6] Table 9.2.5.1-3.

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in TS 38.133[6] Table 9.2.5.2-1.

CSSFintra: it is a carrier specific scaling factor and is determined

according to CSSFoutside_gap,i in TS 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps.

When intra-frequency SMTC is fully non overlapping with measurement gaps or intra-frequency SMTC is fully overlapping with MGs, Kp=1

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

[TS 38.133, clause 9.2.5.2]

The measurement period for intra-frequency measurements without gaps is as shown in TS 38.133 [6] Table 9.2.5.2-1.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps(FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

[TS 38.133, clause 10.1.2.1.1]

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 is configured.

The accuracy requirements in TS 38.133 [6] Table 10.1.2.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | Depending on band group | -121+ ΔBG_offset | -118+ ΔBG_offset | N/A | -70 |
| 8 | 11 | -6 | Depending on band group | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: For NR operating band groups, ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |

[TS 38.133, annex B.2.2]

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | Depending on band group | -127+ ΔBG_offset | -124+ ΔBG_offset | -6 |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. NOTE 2: For NR operating band groups, ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |

6.3.3.0.2	Minimum conformance requirements for NR FR1 inter-frequency conditional handover

[TS 38.133, clause 6.1.4.2.1]

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [13].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [13].

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover. -	a condition exists at the measurement reference point which will trigger the conditional handover, or

-	a condition exists at the measurement reference point which will trigger the NES-based conditional handover

Tmeasure is the measurements time stated in TS 38.133 [6] clause 6.1.4.2.2.

TCHO_execution is the conditional execution preparation time in TS 38.133 [6] clause 6.1.4.2.3.

Tinterrupt is the interruption time stated in TS 38.133 [6] clause 6.1.4.2.4.

The NES-based conditional handover delay requirements shall apply if UE receives a RRC message implying NES-based conditional handover before receiving the NES indication in DCI 2-9 command.

UE is not expected to receive a NES indication in DCI 2-9 command before receiving a RRC message implying NES-based conditional handover. When UE receives a RRC message implying NES-based conditional handover but no NES indication in DCI 2-9 command, no NES-based conditional handover requirement is applied. The NES indication is specified in clause 5.5.4 in TS 38.331.

[TS 38.133, clause 6.1.4.2.2]

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4.

For NES-based conditional inter-frequency handover:

-	If UE successfully decodes DCI 2-9 command earlier than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, and the condition of NES-based CHO keeps existing till UE receiving the DCI 2-9 command, then the measurement time delay equal to Tidentify_inter_with_index or Tidentify_inter_without_index

-	If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

-	If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, and the condition of NES-based CHO is not met when receiving the DCI 2-9 command, then the measurement time delay equals to the time span from the end of TEvent_DU until a condition keeps existing for TSSB measurement period_inter which will immediately trigger the NES-based conditional handover after DCI 2-9.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used, an additional delay can be expected.

[TS 38.133, clause 6.1.4.2.3]

TCHO_execution is the UE execution preparation time for conditional handover.

For conditional handover, it starts after UE realizes the condition of CHO is met and identity of the target cell is determined.

For NES-based conditional handover,

-	It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9.

-	Otherwise, it starts after UE successfully decodes DCI 2-9.

And TCHO_execution can be up to 10ms.

[TS 38.133, clause 6.1.4.2.4]

The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [8]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [13] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

[TS 38.133, clause 9.3.2]

The requirements in clause 9.3 apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] clauses 10.1.4 for FR1, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.3 for a corresponding Band.

[TS 38.133, clause 9.3.4]

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in 38.133 [6] Table 9.3.4-1.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in 38.133 [6] Table 9.3.4-3.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in 38.133 [6] Table 9.3.5-1.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps.

Table 9.3.4-1: Time period for PSS/SSS detection (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 9.3.4-3: Time period for time index detection (Frequency range FR1)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

[TS 38.133, clause 9.3.5]

When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively,  as shown in table 9.3.5-1 and 9.3.5-2:

Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1)

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

[TS 38.133, clause 10.1.4.1.1]

The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.4.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to TS 38.133 [6] Annex B.2.3 for a corresponding Band for each relevant SSB.

Table 10.1.4.1.1-1: SS-RSRP Inter frequency Absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | Depending on band group | -121+ ΔBG_offset | -118+ ΔBG_offset | N/A | -70 |
| 8 | 11 | -6 | Depending on band group | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Void NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: For NR operating band groups, ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |

[TS 38.133, annex B.2.3]

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | Depending on band group | -125+ ΔBG_offset | -122+ ΔBG_offset | -4 |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. NOTE 2: For NR operating band groups, ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |

<End of Changes>