3GPP TSG-RAN5 Meeting #106	R5-250864

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5434 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition to IoT NTN enhancement test case 22.4.32 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New IoT NTN enhancement test case 22.4.32 need to be added according to work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add IoT NTN enhancement test case 22.4.32. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WP will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.32(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.523-2 CR 1491 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.4.32	NB-IoT / NTN / GNSS validity duration reporting / UE-autonomous GNSS measurement

22.4.32.1	Test Purpose (TP)

(1)

with { UE has acquired SystemInformationBlockType2-NB with gnss-PositionFixDurationReporting present }

ensure that {

when { UE receives RRCConnectionSetup-NB }

then { UE transmits RRCConnectionSetupcomplete-NB with gnss-PositionFixDuration and gnss-validityDuration included }

}

(2)

with { UE has started T390 in RRC_CONNECTED state and gnss-AutonomousEnabled is configured }

ensure that {

when { T390 expires and GNSS position is out-of-date }

then { UE UE transmits a GNSS Validity Duration Report MAC CE upon a new GNSS position becomes valid }

}

(3)

with { UE has started T390 in RRC_CONNECTED state and gnss-AutonomousEnabled is not configured }

ensure that {

when { T390 expires and GNSS position is out-of-date }

then { UE moves to RRC_IDLE state }

}

22.4.32.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331 clause 5.3.3.4, 5.3.3.21, 5.3.3.24 and 5.5.9. Unless otherwise stated these are Rel-18 requirements.

[TS 36.331, clause 5.3.3.4]

1>	set the content of RRCConnectionSetupComplete message as follows:

……

2>	if the UE is connected to NTN:

3>	include gnss-validityDuration in accordance with the remaining time of the GNSS validity duration;

3>	if UE supports GNSS position fix in RRC_CONNECTED and gnss-PositionFixDurationReporting is present in SystemInformationBlockType2(-NB):

4>	include gnss-PositionFixDuration in accordance with the time duration required for the UE to acquire a GNSS position;

……

[TS 36.331, clause 5.3.3.21]

Upon indication that the GNSS position has become out-of-date while in RRC_CONNECTED, the UE shall:

1>	if the UE does not support performing GNSS position fix in RRC_CONNECTED and ul-TransmissionExtensionEnabled is not configured:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

1>	else if ul-TransmissionExtensionEnabled is configured:

2>	if timeAlignmentTimer is configured to be infinity:

3>	start timer T390 with the timer value set to ul-TransmissionExtensionValue;

2>	else:

3>	start timer T390 with the timer value set to the remaining time of timeAlignmentTimer;

1>	else if ul-TransmissionExtensionEnabled is not configured and no indication of network triggered GNSS measurement is received from lower layers:

2>	if gnss-AutonomousEnabled is configured:

3>	perform GNSS measurement using autonomous gaps as specified in clause 5.5.9;

2>	else:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

[TS 36.331, clause 5.3.3.24]

The UE shall:

1>	if timer T390 expires and no indication of network triggered GNSS measurement has been received from lower layers:

2>	if gnss-AutonomousEnabled is configured:

3>	perform GNSS measurement using autonomous gaps as specified in clause 5.5.9;

2>	else:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

[TS 36.331, clause 5.5.9]

For BL UEs or UEs in CE or NB-IoT UEs that are connected to NTN, GNSS measurement can be triggered aperiodically by the GNSS Measurement Command MAC CE (see TS 36.321 [6]), or triggered by the UE autonomously if enabled by the network, or triggered by the UE using available idle periods.

The UE shall:

1>	if an indication to perform GNSS measurement is received from lower layers:

2>	perform GNSS measurement using the measurement gap with a gap length indicated by lower layers, as specified in TS 36.213 [23];

2>	stop timer T390, if running;

1>	if gnss-AutonomousEnabled is configured:

2>	if the gap length is indicated by lower layers:

3>	set the autonomous gap length to the gap length indicated by lower layers;

2>	else:

3>	set the autonomous gap length to the latest reported time duration required for the UE to acquire a GNSS position;

2>	perform GNSS measurement using the autonomous gap starting from T390 expiry if ul-TransmissionExtensionEnabled is configured, otherwise starting from GNSS validity duration expiry;

NOTE:	UE can autonomously start GNSS measurements during available idle periods in RRC_CONNECTED to keep GNSS valid and stop T390 upon indication that a new GNSS position becomes valid. The exact time of starting GNSS measurements during available idle periods is left to UE implementation.

1>	upon starting GNSS measurement:

2>	stop timer T318, if running;

1>	upon indication that a new GNSS position becomes valid:

2>	instruct lower layers to report the remaining GNSS validity duration (see TS 36.321 [6]);

1>	upon indication that GNSS measurement has failed:

2>	if GNSS position is out-of-date; and

2>	if ul-TransmissionExtensionEnabled is not configured or T390 is not running:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

22.4.32.3	Test description

22.4.32.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 is configured according to Table 8.1.4.2-1 and Table 8.1.4.2-2 in TS 36.508 [18].

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in State 1-NB switched off according to TS 36.508 [18].

22.4.32.3.2	Test procedure sequence

Table 22.4.32.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on UE. | - | - | - | - |
| 2 | UE acquires SystemInformationBlockType2-NB. | - | - | - | - |
| 3 | UE transmits an RRCConnectionRequest-NB message. | --> | RRCConnectionRequest-NB | - | - |
| 4 | SS transmits an RRCConnectionSetup-NB message. | <-- | RRCConnectionSetup-NB | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionSetupComplete-NB message? | --> | RRCConnectionSetupComplete-NB | 1 | P |
| 6-20 | The attach procedure is completed and the RRC connection is released by executing steps 5-14b1 of the UE registration procedure in TS 36.508 clause 8.1.5.2.3. | - | - | - | - |
| 21 | Wait till T390 expire upon GNSS position is out of date. | - | - | - | - |
| 22 | Cause UE to change location using MMI or AT commands. | - | - | - | - |
| - | EXCEPTION: Steps 23a1 to 23b1 describe behaviour depending on UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 23a1 | The UE transmits a preamble on NPRACH upon GNSS position becomes valid. | --> | NPRACH Preamble | - | - |
| 23a2 | The SS transmits Random Access Response including UL grant suitable for transmitting GNSS Validity Duration Report MAC CE. | <-- | Random Access Response | - | - |
| 23b1 | The SS transmits an UL grant t suitable for transmitting GNSS Validity Duration Report MAC CE. | <-- | (UL Grant) | - | - |
| 24 | Check: Does UE transmit a MAC PDU to report the remaining GNSS measurement validity duration? | --> | MAC PDU (GNSS Validity Duration Report MAC CE) | 2 | P |
| 25 | The SS transmits RRCConnectionReconfiguration-NB with gnss-AutonomousEnabled not configured. | <-- | RRCConnectionReconfiguration-NB | - | - |
| 26 | UE transmits an RRCConnectionReconfigurationComplete-NB message. | --> | RRCConnectionReconfigurationComplete-NB | - | - |
| 27 | Wait till T390 expire upon GNSS position is out of date. | - | - | - | - |
| 28 | The SS transmits RRC Paging-NB message with UE S-TMSI. | <-- | Paging-NB | - | - |
| 29 | Check: Does the UE transmit an RRCConnectionRequest-NB message? | --> | RRCConnectionRequest-NB | 3 | P |

22.4.32.3.3	Specific message contents

Table 22.4.32.3.3-1: SystemInformationBlockType2-NB (step 2, Table 22.4.32.3.2-1)

| Derivation Path: 36.508 Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| gnss-PositionFixDurationReporting-r18 | true |  |  |
| } |  |  |  |

Table 22.4.32.3.3-2: RRCConnectionSetup-NB (step 4, Table 22.4.32.3.2-1)

| Derivation path: 36.508 table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| mac-MainConfig-r13 CHOICE { |  |  |  |
| explicitValue-r13 SEQUENCE { |  |  |  |
| timeAlignmentTimerDedicated-r13 | infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| gnss-AutonomousEnabled-r18 | true |  |  |
| ul-TransmissionExtensionEnabled-r18 | true |  |  |
| ul-TransmissionExtensionValue-r18 | sf5120 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.32.3.3-3: RRCConnectionSetupComplete-NB (step 5, Table 22.4.32.3.2-1)

| Derivation Path: 36.508 table 8.1.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r13 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| gnss-ValidityDuration-r17 | Any allowed value |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| gnss-PositionFixDuration-r18 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.32.3.3-4:  RCConnectionReconfiguration-NB (step 25, Table 22.4.32.3.2-1)

| Derivation Path: 36.508 table 8.1.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| mac-MainConfig-r13 CHOICE { |  |  |  |
| explicitValue-r13 SEQUENCE { |  |  |  |
| timeAlignmentTimerDedicated-r13 | infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| gnss-AutonomousEnabled-r18 | Not present |  |  |
| ul-TransmissionExtensionEnabled-r18 | true |  |  |
| ul-TransmissionExtensionValue-r18 | sf5120 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]