3GPP TSG-RAN5 Meeting #106	R5-250937

# Athens, Greece, 17th Feb. – 21st Feb. 2025

Title:							MU discussion on FR2 EVM for PUCCH

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

In the previous meeting, discussion on FR2 EVM MU was started for PUCCH [1]. This paper provides our analysis on EVM MU for FR2 PUCCH. Unless otherwise commented, observations and proposals in this paper are for NTC, IFF, max device size ≤ 40 cm.

# 2.	Discussion

To evaluate EVM MU, it is needed to assume the possible minimum UE output level for PUCCH. According to [2] and [3], PUCCH format used in the EVM test case is Format 1 and 3, and RB allocation for these formats is 1@0. From the minimum conformance requirements for MPR, the possible minimum UE output level for PUCCH 1@0 is calculated as below by minimum peak EIRP – (MPR + T(MPR) + MBR). Note that MPR for PUCCH formats 1 and 3 is specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation as defined in 6.2.2.0 of [3], which is MPRWT = 2.0 dB for PC3 and PC5 with CBW ≤ 200 MHz, 3.0 dB for PC3 and PC5 with CBW = 400 MHz, and 6.5 dB for PC1 with CBW ≤ 400 MHz. In addition, for RB allocation ≤ 1.44 MHz such as 1@0, MPRnarrow is applied with values of 2.5 dB for PC3 and PC5, and 14.4 dB for PC1. Finally, MPR is obtained by max(MPRWT, MPRnarrow).

Table 1: Possible minimum UE output level for PUCCH in FR2 EVM test case

| Power class | Frequency range | CBW | Minimum peak EIRP | MPR | T(MPR) | MBR | UE output level |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PC3 | FR2a | ≤ 200 MHz | 22.4 dBm | 2.5 dB | 2 dB | 0.75 dB | 17.15 dBm |
|  |  | 400 MHz |  | 3.0 dB |  |  | 16.65 dBm |
|  | FR2b | ≤ 200 MHz | 20.6 dBm | 2.5 dB |  |  | 15.35 dBm |
|  |  | 400 MHz |  | 3.0 dB |  |  | 14.85 dBm |
| PC1 | FR2a | ≤ 400 MHz | 40 dBm | 14.4 dB | 7 dB | 0 dB | 18.60 dBm |
| PC5 | FR2a | ≤ 200 MHz | 30 dBm | 2.5 dB | 2 dB | 0.7 dB | 24.80 dBm |
|  |  | 400 MHz |  | 3.0 dB |  |  | 24.30 dBm |

Observation 1: For FR2 EVM test case, the possible minimum UE output level for PUCCH is calculated as Table 1.

As agreed in [4], EVM MU is defined as the root sum square (RSS) of the residual EVM due to available SNR and additional TE EVM. For EVM due to available SNR, the above UE output level with polarization mismatch and a TE noise floor assumption of -10.6 dBm/400 MHz for FR2a and -8.5 dBm/400 MHz for FR2b [5] can be used. For additional TE EVM, it is acceptable for us to reuse the values of additional TE EVM for PUSCH at Test ID 4 [5] disregarding the PUCCH waveform, in the same way the MPR applies as explained above. Considering the above assumption, the total EVM MU for PUCCH was calculated in the following table. Furthermore, TT was also calculated as below from the TT formula defined in Table F.3.2-1 of [3] and EVM requirement of 17.5%.

Table 2: Total EVM MU for PUCCH in FR2 EVM test case

| Power class | Frequency range | CBW | UE output level with polarization mismatch | TE noise floor assumption | EVM due to available SNR | Additional TE EVM | Total EVM MU | TT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC3 | FR2a | 50 MHz | 14.14 dBm | -19.6 dBm | 2.05% | 2.50% | 3.23% | 0.00% |
|  |  | 100 MHz |  | -16.6 dBm | 2.90% | 3.26% | 4.36% | 0.00% |
|  |  | 200 MHz |  | -13.6 dBm | 4.10% | 4.61% | 6.17% | 0.00% |
|  |  | 400 MHz | 13.64 dBm | -10.6 dBm | 6.14% | 6.58% | 9.00% | 2.18% |
|  | FR2b | 50 MHz | 12.34 dBm | -17.5 dBm | 3.21% | 3.00% | 4.39% | 0.00% |
|  |  | 100 MHz |  | -14.5 dBm | 4.54% | 4.00% | 6.05% | 0.00% |
|  |  | 200 MHz |  | -11.5 dBm | 6.42% | 5.75% | 8.62% | 2.01% |
|  |  | 400 MHz | 11.84 dBm | -8.5 dBm | 9.62% | 8.00% | 12.51% | 4.01% |
| PC1 | FR2a | 50 MHz | 15.59 dBm | -19.6 dBm | 1.73% | 2.40% | 2.96% | 0.00% |
|  |  | 100 MHz |  | -16.6 dBm | 2.45% | 3.39% | 4.18% | 0.00% |
|  |  | 200 MHz |  | -13.6 dBm | 3.47% | 4.79% | 5.91% | 0.00% |
|  |  | 400 MHz |  | -10.6 dBm | 4.90% | 6.68% | 8.29% | 1.86% |
| PC5 | FR2a | 50 MHz | 21.79 dBm | -19.6 dBm | 0.85% | 2.42% | 2.56% | 0.00% |
|  |  | 100 MHz |  | -16.6 dBm | 1.20% | 3.43% | 3.63% | 0.00% |
|  |  | 200 MHz |  | -13.6 dBm | 1.70% | 4.85% | 5.14% | 0.00% |
|  |  | 400 MHz | 21.29 dBm | -10.6 dBm | 2.54% | 6.85% | 7.31% | 1.47% |

Proposal 1: For FR2 EVM test case, adopt the MTSU/TT values in Table 2 for PUCCH.

# 3.	Conclusion

RAN5 is asked to endorse the following proposal.

Proposal 1: For FR2 EVM test case, adopt the MTSU/TT values in Table 2 for PUCCH.

# 4.	References

[1] R5-247337, “On the MU and TT for SISO and MIMO test cases”, Rohde & Schwarz, RAN5#105

[2] TS 38.508-1, “User Equipment (UE) conformance specification; Part 1: Common test environment”, V18.5.0 (2024-12)

[3] TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, V18.5.0 (2024-12)

[4] R5-221625, “Discussion on MU and TT proposal for FR2 EVM”, Keysight Technologies, RAN5#94-e

[5] TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, V18.5.0 (2024-12)