3GPP TSG-RAN5 Meeting #106 	R5-250957

# Athens, Greece, 17th Feb. 2025 - 21st Feb. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1905 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to DL UL allocation in 7.3B.2 for Rel-17 FR1 inter-band EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Incorrect DL/UL allocations are defined in the test configuration tables of Reference sensitivity for Inter-band EN-DC within FR1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 7.3B.2.3_1.1.4.1-1, DL/UL allocation for the following DC configuration was corrected according to the corresponding minimum conformance requirements in 7.3B.2.0. - DC_2A-66A_n77A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference sensitivity for FR1 inter-band EN-DC will be tested with incorrect DL/UL allocations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3_1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

##### 7.3B.2.3_1.1	Reference sensitivity for EN-DC within FR1 (3 CCs)

Editor's note:	The test requirements section of this test case contains some EN-DC configurations that have no test points defined meaning these cannot be tested.

Editor's note:	For EN-DC configurations with >1NR CC, test points in Table 7.3B.2.3_1.1.4.1-0 only need to be tested if the configuration is a max NR CC of the UE, where detailed execution rules are FFS.

<<Unchanged sections skipped>>

Table 7.3B.2.3_1.1.4.1-1: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions  (three bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  | EN-DC configurations containing the following band combinations: 20-n78: Mid in band 20 and Mid in band 78. |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  |  | Refer to “NR NRB” and “E-UTRA NRB” columns |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | E-UTRA |  |  |  |  | EUTRA/NR |  |  |  |  |  | NR |  |  |  |
|  | Band | Range | NRB |  |  | Band | Range |  | NRB |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc/UL alloc |  | UL MOD | DL MOD |  | UL/DL Ch BW |  | DLalloc/ ULalloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc/UL alloc |
| <<Unchanged sections skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_2A-66A_n77A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 2 | UL 1855 / DL 1935 MHz |  |  | 66 |  | DL 2115 MHz |  |  |  |  | n77 | 3970 MHz |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / All RBs | N/A |  | QPSK |  |  | 5 MHz | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 10 MHz | All RBs / All RBs |
| 211 | 2 | UL 1880 / DL 1960 MHz |  |  | 66 |  | DL 2140 MHz |  |  |  |  | n77 | 3500 MHz |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / All RBs | N/A |  | QPSK |  |  | 5 MHz | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 10 MHz | All RBs / All RBs |
| 3 | 2 | UL 1860 / DL 1940 MHz |  |  | 66 |  | DL 2175 MHz |  |  |  |  | n77 | 3915 MHz |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / All RBs | N/A |  | QPSK |  |  | 5 MHz | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 10 MHz | All RBs / All RBs |
| 411 | 2 | DL 1960 MHz |  |  | 66 |  | UL 1760 / DL 2160 MHz |  |  |  |  | n77 | 3720 MHz |  |  |
|  | N/A | QPSK | 5 MHz | All RBs / 0 | QPSK |  | QPSK |  |  | 5 MHz | All RBs / All RBs | DFT-s-OFDM QPSK | CP-OFDM QPSK | 10 MHz | All RBs / All RBs |
| <<Unchanged sections skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Void Note 2: Void Note 3: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. Note 4: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 6: Test only applicable for to UEs not supporting UE capability singleUL-Transmissionn for the correspondent uplink configuration. Note 7: NR CA fallback to 1CC is sufficient to test, unless both NR cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 8: Test ID with UL harmonic exception avoided. Note 9: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 10: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. NOTE 11: This test ID is for both UL PC3 and PC2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<End of change>>