3GPP TSG-RAN5 Meeting #106	R5-250970

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3444 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Tables for Uu-RelayRLC-ChannelID and SL-RLC-ChannelID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The IEs Uu-RelayRLC-ChannelID and SL-RLC-ChannelID are used for Uu SRBs and DRBs configuration via sidlelink relay, so the their Condition should be SRBs/DRBs, not be SL-SRBs/DRBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Table 4.6.3-202B: Uu-RelayRLC-ChannelConfig, Table 4.6.3-202C: Uu-RelayRLC-ChannelID, Table 4.6.6-27A and Table 4.6.6-30A: SL-SRAP-Config. Formatted the title of - NotificationMessageSidelink with correct style H4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1A, 4.6.3, 4.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

–	NotificationMessageSidelink

Table 4.6.1A-2B: NotificationMessageSidelink

| Derivation Path: TS 38.331 [6], clause 6.6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| NotificationMessageSidelink-r17 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| notificationMessageSidelink-r17 SEQUENCE { |  |  |  |  |
| indicationType-r17 |  | relayUE-Uu-RLF |  | Uu_RLF |
|  |  | relayUE-HO |  | HO |
|  |  | relayUE-CellReselection |  | CellReselection |
|  |  | relayUE-Uu-RRC-Failure |  | Uu_RRC_Failure |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Uu_RLF | Relay UE initiates transmission of the NotificationMessageSidelink message due to Uu RLF |
| HO | Relay UE initiates transmission of the NotificationMessageSidelink message due to reconfiguration with sync |
| CellReselection | Relay UE initiates transmission of the NotificationMessageSidelink message due to cell reselection |
| Uu_RRC_Failure | Relay UE initiates transmission of the NotificationMessageSidelink message due to Uu RRC connection establishment/Resume failure |

<Skip unchanged sections>

–	Uu-RelayRLC-ChannelConfig

Table 4.6.3-202B: Uu-RelayRLC-ChannelConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Uu-RelayRLC-ChannelConfig-r17 ::= SEQUENCE { |  |  |  |
| uu-LogicalChannelIdentity-r17 | LogicalChannelIdentity with condition SRB1 |  | SRB1 |
|  | LogicalChannelIdentity with condition SRB2 |  | SRB2 |
|  | LogicalChannelIdentity with condition DRB1 |  | DRB1 |
| uu-RelayRLC-ChannelID-r17 | Uu-RelayRLC-ChannelID with condition SRB1 |  | SRB1 |
|  | Uu-RelayRLC-ChannelID with condition SRB2 |  | SRB2 |
|  | Uu-RelayRLC-ChannelID with condition DRB1 |  | DRB1 |
| reestablishRLC-r17 | true |  |  |
| rlc-Config-r17 | Not present |  | SRB1 |
|  | RLC-Config with condition AM |  | SRB2,DRB1 |
| mac-LogicalChannelConfig-r17 | LogicalChannelConfig with condition SRB1 |  | SRB1 |
|  | LogicalChannelConfig with condition SRB2 |  | SRB2 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| DRB1 | Establishment of DRB1 |

–	Uu-RelayRLC-ChannelID

Table 4.6.3-202C: Uu-RelayRLC-ChannelID

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Uu-RelayRLC-ChannelID-r17 | n+1 |  | SRBn |
|  | n+4 |  | DRBn |

| Condition | Explanation |
| --- | --- |
| SRBn | Establishment of Sidelink SRBn; n=0..3 |
| DRBn | Establishment of Sidelink DRBn; n=1..16 |

–	SL-RLC-ChannelID

Table 4.6.6-27A: SL-RLC-ChannelID

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RLC-ChannelID-r17 |  | n+1 |  | SRBn |
|  |  | n+4 |  | DRBn |

| Condition | Explanation |
| --- | --- |
| SRBn | Establishment of sidelink SRBn; n=0..3 |
| DRBn | Establishment of sidelink DRBn; n=1..16 |

–	SL-SRAP-Config

Table 4.6.6-30A: SL-SRAP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-SRAP-Config-r17 ::= SEQUENCE { |  |  |  |  |
| sl-LocalIdentity-r17 |  | 1 |  |  |
| sl-MappingToAddModList-r17 |  | Not present |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { |  | 1 entry |  | SRB0 |
| sl-RemoteUE-RB-Identity-r17 CHOICE { |  |  |  |  |
| srb-Identity-r17 |  | 0 |  |  |
| } |  |  |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Uu-RelayRLC-ChannelID using condition SRB0 |  | L2RelayUE |
| sl-EgressRLC-ChannelPC5-r17 |  | Not present |  |  |
| } |  |  |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { |  | 1 entry |  | SRB1 |
| sl-RemoteUE-RB-Identity-r17 CHOICE { |  |  |  |  |
| srb-Identity-r17 |  | 1 |  |  |
| } |  |  |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Uu-RelayRLC-ChannelID using condition SRB1 |  | L2RelayUE |
| sl-EgressRLC-ChannelPC5-r17 |  | Not present |  |  |
| } |  |  |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { |  | 1 entry |  | SRB2 |
| sl-RemoteUE-RB-Identity-r17 CHOICE { |  |  |  |  |
| srb-Identity-r17 |  | 2 |  |  |
| } |  |  |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Uu-RelayRLC-ChannelID using condition SRB2 |  | L2RelayUE |
| sl-EgressRLC-ChannelPC5-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelPC5-r17 |  | SL-RLC-ChannelID using condition SRB2 |  | L2RelayUE, L2RemoteUE |
| } |  |  |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { |  | 1 entry |  | DRB1 |
| sl-RemoteUE-RB-Identity-r17 CHOICE { |  |  |  |  |
| drb-Identity-r17 |  | DRB-Identity using condition DRB1 |  |  |
| } |  |  |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Uu-RelayRLC-ChannelID using condition DRB1 |  | L2RelayUE |
| sl-EgressRLC-ChannelPC5-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelPC5-r17 |  | SL-RLC-ChannelID using condition DRB1 |  | L2RelayUE, L2RemoteUE |
| } |  |  |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { |  | 1 entry |  | DRB2 |
| sl-RemoteUE-RB-Identity-r17 CHOICE { |  |  |  |  |
| drb-Identity-r17 |  | DRB-Identity using condition DRB2 |  |  |
| } |  |  |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Uu-RelayRLC-ChannelID using condition DRB2 |  | L2RelayUE |
| sl-EgressRLC-ChannelPC5-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelPC5-r17 |  | SL-RLC-ChannelID using condition DRB2 |  | L2RelayUE, L2RemoteUE |
| } |  |  |  |  |
| sl-MappingToReleaseList-r1 |  | Not present |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB0 | Establishment of SRB0 |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| DRB1 | Establishment of DRB1 |
| DRB2 | Establishment of DRB2 |
| L2RelayUE | For L2 U2N Relay UE |
| L2RemoteUE | For L2 U2N Remote UE |

<End of Changed Section>