3GPP TSG-RAN5 Meeting #106	R5-250974

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4899 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 12.3.1.1.18 Sidelink Relay / L2 U2N / Remote UE / Sidelink radio link failure related actions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new test case 12.3.1.1.18 Sidelink Relay / L2 U2N / Remote UE / Sidelink radio link failure related actions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 12.3.1.1.18 Sidelink Relay / L2 U2N / Remote UE / Sidelink radio link failure related actions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.1.18(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0565 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.3.1.1.18	Sidelink Relay / L2 U2N / Remote UE / Sidelink radio link failure related actions

12.3.1.1.18.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED and acting as a L2 U2N Remote UE and has sent an RRCReconfigurationSidelink message to the connected L2 U2N Relay UE }

ensure that {

when { UE detecting Sidelink radio link failure on expiring of timer T400}

then { UE initiates the RRC connection re-establishment procedure }

}

12.3.1.1.18.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.8.9.3, 5.8.9.7.1, 5.3.7.2, 5.3.7.3a and 5.3.7.4.

[TS 38.331, clause 5.8.9.3]

The UE shall:

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1>	upon T400 expiry for a specific destination; or

1>	upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination:

2>	consider sidelink radio link failure to be detected for this destination;

2>	release the DRBs of this destination, according to clause 5.8.9.1a.1;

2>	release the SRBs of this destination, according to clause 5.8.9.1a.3;

2>	release the PC5 Relay RLC channels of this destination if configured, in according to clause 5.8.9.7.1;

2>	discard the NR sidelink communication related configuration of this destination;

2>	reset the sidelink specific MAC of this destination;

2>	consider the PC5-RRC connection is released for the destination;

2>	indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2>	if UE is in RRC_CONNECTED:

3>	if the UE is acting as L2 U2N Remote UE for the destination:

4>	initiate the RRC connection re-establishment procedure as specified in 5.3.7.

3>	else:

4>	perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3;

NOTE:	It is up to UE implementation on whether and how to indicate to upper layers to maintain the keep-alive procedure [55].

[TS 38.331, clause 5.8.9.7.1]

The UE shall:

…

1>	if the PC5 Relay RLC channel release was triggered for a specific destination by upper layers as specified in 5.8.9.5 or due to sidelink RLF as specified in 5.8.9.3:

2>	release the RLC entity and the corresponding logical channel associated with the SL-RLC-ChannelID of the specific destination;

[TS 38.331, clause 5.3.7.2]

The UE initiates the procedure when one of the following conditions is met:

…

1>	upon detecting sidelink radio link failure by L2 U2N Remote UE in RRC_CONNECTED, in accordance with clause 5.8.9.3; or

…

NOTE 0:	It is up to UE implementation whether to initiate the procedure while T346g is running.

Upon initiation of the procedure, the UE shall:

…

1>	start timer T311;

…

1>	release sl-L2RemoteUE-Config, if configured;

1>	release the SRAP entity, if configured;

1>	if the UE is acting as L2 U2N Remote UE:

2>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

3>	indicate upper layers to trigger PC5 unicast link release;

3>	perform either cell selection in accordance with the cell selection process as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else (i.e., maintain the PC5 RRC connection):

3>	consider the connected L2 U2N Relay UE as suitable and perform actions as specified in clause 5.3.7.3a;

NOTE 1:	It is up to Remote UE implementation whether to release or keep the current PC5 unicast link.

…

NOTE 2:	For L2 U2N Remote UE, if both a suitable cell and a suitable relay are available, the UE can select either one based on its implementation.

[TS 38.331, clause 5.3.7.3a]

Upon selecting a suitable L2 U2N Relay UE, the L2 U2N Remote UE shall:

1>	indicate to upper layer to trigger the PC5 unicast link establishment with the selected L2 U2N Relay UE, if a new L2 U2N Relay UE is selected;

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	stop the cell selection procedure, if ongoing;

1>	start timer T301;

1>	release the RLC entity for SRB0, if any;

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply the specified configuration of SL-RLC0 as specified in 9.1.1.4;

1> apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

1>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4.

[TS 38.331, clause 5.3.7.4]

The UE shall set the contents of RRCReestablishmentRequest message as follows:

…

1>	if the UE is acting as L2 U2N Remote UE:

2>	establish or re-establish (e.g. via release and add) SL RLC entity for SRB1;

2>	apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1;

2>	apply the default configuration of PDCP as defined in 9.2.1 for SRB1;

2>	apply the default configuration of SRAP as defined in 9.2.5 for SRB1;

…

1>	submit the RRCReestablishmentRequest message to lower layers for transmission.

12.3.1.1.18.3	Test Description

12.3.1.1.18.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 and NR-SS-UE2 operating as L2 U2N Relay UE in RRC_CONNECTED state on the same NR Cell.

-	NR-SS-UE1 and NR-SS-UE2 are synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- -	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.4 and ProSeP using parameter 5G ProSe direct discovery model A as per TS 38.508-1 [4] clause 4.7C.1.

- UE is synchronised on GNSS.

Preamble:

-	The UE is in state 0-A as defined in TS 38.508-1 [4].

12.3.1.1.18.3.2	Test procedure sequence

Table 12.3.1.1.18.3.2-1 illustrates the downlink power levels to be applied for the sidelink power levels to be applied for NR-SS-UEs at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configuration marked "T1" is applied at the points indicated in the Main behaviour description in Table 12.3.1.1.18.3.2-2.

Table 12.3.1.1.18.3.2-1: Time instances of sidelink power level

|  | Parameter | Unit | NR-SS-UE 1 | NR-SS-UE 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | -88 |  |

Table 12.3.1.1.18.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side in TS 38.508-1 Table 4.9.42.2.2-1 using parameter Cast Type (Unicast) and NR-SS-UE1 initiated unicast mode ProSe Direct communication and connected without release is executed. | - | - | - | - |
| 3 | UE is configured by upper layer to release the PC5 Relay RLC channel of DRB1. Note: This step is triggered by MMI or AT command. | - | - | - | - |
| 4 | UE sends an RRCReconfigurationSidelink message to NR-SS-UE1 to indicate PC5 Relay RLC channel of DRB1 release | --> | PC5 RRC: RRCReconfigurationSidelink | - | - |
| 5 | The NR-SS-UE1 waits for 1s (T400 expire). | - | - | - | - |
| 6 | The NR-SS-UEs adjust the power level according to row "T1" in Table 12.3.1.1.18.3.2-1. | - | - | - | - |
| 7 | The procedure for 5G ProSe Layer-2 U2N Relay / Remote UE RRC Reestablishment / Remote UE side in TS 38.508-1 Table Table 4.9.45.2.2-1 is executed on NR-SS-UE2. | - | - | 1 | P |

12.3.1.1.18.3.3	Specific message contents

Table 12.3.1.1.18.3.3-1: RRCReconfigurationSidelink (step 4, Table 12.3.1.1.18.3.2-2)

| Derivation path: TS 38.331 [12], clause 6.6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier-r16 | Not checked |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |
| slrb-ConfigToAddModList-r16 | Not checked |  |  |
| slrb-ConfigToReleaseList-r16 | Not checked |  |  |
| sl-MeasConfig-r16 | Not checked |  |  |
| sl-CSI-RS-Config-r16 | Not checked |  |  |
| sl-ResetConfig-r16 | Not checked |  |  |
| sl-LatencyBoundCSI-Report-r16 | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-DRX-ConfigUC-PC5-r17 | Not checked |  |  |
| sl-LatencyBoundIUC-Report-r17 | Not checked |  |  |
| sl-RLC-ChannelToReleaseListPC5-r17 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelID-r17{ | 1 entry |  |  |
| SL-RLC-ChannelID-r17[1] | SL-RLC-ChannelID using condition DRB1 | entry 1 |  |
| } |  |  |  |
| sl-RLC-ChannelToAddModListPC5-r17 | Not checked |  |  |
| nonCriticalExtension |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>