3GPP TSG-RAN5 Meeting #106	R5-250981

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4904 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case Sidelink Relay / L2 U2N / Relay UE / RRC reconfiguration / PC5 Relay RLC channel configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Sidelink relay test case 12.3.1.2.12 was missing in current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 12.3.1.2.12 Sidelink Relay / L2 U2N / Relay UE / RRC reconfiguration / PC5 Relay RLC channel configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will be incomplete compared to work plan coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.2.12(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0565 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.3.1.2.12	Sidelink Relay / L2 U2N / Relay UE / RRC reconfiguration / PC5 Relay RLC channel configuration

12.3.1.2.12.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED and acting as a L2 U2N Relay UE }

ensure that {

when { UE receives an RRCReconfiguration message to add a PC5 Relay RLC channel }

then { UE sends an RRCReconfigurationSidelink message to remote UE to indicate PC5 Relay RLC channel addition. After receiving RRCReconfigurationCompleteSidelink message from remote UE, UE adds the PC5 Relay RLC channel and sends an RRCReconfigurationComplete message. }

}

(2)

with { UE in NR RRC_CONNECTED and acting as a L2 U2N Relay UE and having established a PC5 Relay RLC channel }

ensure that {

when { UE receives an RRCReconfiguration message to modify the PC5 Relay RLC channel }

then { UE sends an RRCReconfigurationSidelink message to remote UE to indicate PC5 Relay RLC channel modifacation. After receiving RRCReconfigurationCompleteSidelink message from remote UE, UE modifies the PC5 Relay RLC channel and sends an RRCReconfigurationComplete message. }

(3)

with { UE in NR RRC_CONNECTED and acting as a L2 U2N Relay UE and having established a PC5 Relay RLC channel }

ensure that {

when { UE receives an RRCReconfiguration message to release the PCS Relay RLC channel }

then { UE sends an RRCReconfigurationSidelink message to remote UE to indicate PC5 Relay RLC channel modifacation. After receiving RRCReconfigurationCompleteSidelink message from remote UE, UE releases the PC5 Relay RLC channel and sends an RRCReconfigurationComplete message. }

12.3.1.2.12.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.5.1, 5.3.5.5.1, 5.3.5.5.12 and 5.3.5.5.13. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.5.1]

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs/BH RLC channels/Uu Relay RLC channels/PC5 Relay RLC channels, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups, to add/modify/release conditional reconfiguration configuration, to add/modify/release LTM configuration, and to add/modify/release MP configuration. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

RRC reconfiguration to perform reconfiguration with sync includes, but is not limited to, the following cases:

-	reconfiguration with sync and security key refresh, involving RA to the PCell/PSCell, MAC reset, refresh of security and re-establishment of RLC and PDCP triggered by explicit indicators;

-	reconfiguration with sync but without security key refresh, involving RA to the PCell/PSCell, MAC reset and RLC re-establishment and PDCP data recovery (for AM DRB or AM MRB) triggered by explicit indicators;

-	reconfiguration with sync for DAPS and security key refresh, involving RA to the target PCell, establishment of target MAC, and

-	for non-DAPS bearer: refresh of security and re-establishment of RLC and PDCP triggered by explicit indicators;

-	for DAPS bearer: establishment of RLC for the target PCell, refresh of security and reconfiguration of PDCP to add the ciphering function, the integrity protection function and ROHC function of the target PCell;

-	for SRB: refresh of security and establishment of RLC and PDCP for the target PCell;

-	reconfiguration with sync for DAPS but without security key refresh, involving RA to the target PCell, establishment of target MAC, and

-	for non-DAPS bearer: RLC re-establishment and PDCP data recovery (for AM DRB or AM MRB) triggered by explicit indicators.

-	for DAPS bearer: establishment of RLC for target PCell, reconfiguration of PDCP to add the ciphering function, the integrity protection function and ROHC function of the target PCell;

-	for SRB: establishment of RLC and PDCP for the target PCell.

-	reconfiguration with sync for direct-to-indirect path switch or indirect-to-indirect path switch, not involving RA at target side, involving re-establishment of PDCP /PDCP data recovery (for AM DRB) triggered by explicit indicators;

-	reconfiguration with sync for LTM cell switch (without security key refresh), and

-	involving or not involving RA to the target LTM candidate SpCell according to a network indication;

-	MAC reset;

-	depending on a network indication, involving or not involving re-establishment of RLC and PDCP data recovery (for AM DRB).

In (NG)EN-DC and NR-DC, SRB3 can be used for measurement configuration and reporting, for UE assistance (re-)configuration and reporting for power savings, for IP address (re-)configuration and reporting for IAB-nodes, to (re-)configure MAC, RLC, BAP, physical layer and RLF timers and constants of the SCG configuration, to reconfigure PDCP for DRBs associated with the S-KgNB or SRB3, to reconfigure SDAP for DRBs associated with S-KgNB in NGEN-DC and NR-DC, to add/modify/release conditional PSCell change configuration or subsequent CPAC configuration, and to add/modify/release the LTM configuration associated with the SCG (only in NR-DC), provided that the (re-)configuration does not require any MN involvement, and to transmit RRC messages between the MN and the UE during fast MCG link recovery. In (NG)EN-DC and NR-DC, only measConfig, radioBearerConfig, conditionalReconfiguration, ltm-Config (only in NR-DC), bap-Config, iab-IP-AddressConfigurationList, otherConfig, appLayerMeasConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3, except when RRCReconfiguration is received within DLInformationTransferMRDC.

When a clause of 5.3.5 is executed due to an LTM cell switch execution (i.e., as specified in 5.3.5.18.6) or due to a conditional reconfiguration execution for subsequent CPAC (i.e., as specified in 5.3.5.13.8), every appearance of "the received" before RRCReconfiguration message, before a field name, or before an IE name, refers to the RRCReconfiguration message that the UE applies, as specified in 5.3.5.18.6, 5.3.5.13.8, or the field or IE in that RRCReconfiguration message.

[TS 38.331, clause 5.3.5.14]

Upon initiating the procedure, the UE shall:

…

1>	if sl-RLC-ChannelToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	perform PC5 Relay RLC channel release as specified in 5.8.9.7.1;

1>	if sl-RLC-ChannelToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration or RRCSetup:

2>	perform PC5 Relay RLC channel addition/modification as specified in 5.8.9.7.2;

…

[TS 38.331, clause 5.8.9.1.2]

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

…

1>	for each PC5 Relay RLC channel that is to be released due to configuration by sl-ConfigDedicatedNR:

2>	set the SL-RLC-ChannelID corresponding to the PC5 Relay RLC channel in the sl-RLC-ChannelToReleaseListPC5;

1>	for each PC5 Relay RLC channel that is to be established or modified due to receiving sl-ConfigDedicatedNR:

2>	if a PC5 Relay RLC channel is to be established:

3>	assign a new logical channel identity for the logical channel to be associated with the PC5 Relay RLC channel and set sl-MAC-LogicalChannelConfigPC5 in the SL-RLC-ChannelConfigPC5 to include the new logical channel identity;

2>	set the SL-RLC-ChannelConfigPC5 included in the sl-RLC-ChannelToAddModListPC5 according to the received SL-RLC-ChannelConfig corresponding to the PC5 Relay RLC channel, including setting sl-RLC-ChannelID-PC5 to the same value of sl-RLC-ChannelID received in SL-RLC-ChannelConfig;

…

[TS 38.331, clause 5.8.9.7.1]

The UE shall:

1>	if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR:

2>	for each SL-RLC-ChannelID in sl-RLC-ChannelToReleaseList received in sl-ConfigDedicatedNR within RRCReconfiguration, or for each SL-RLC-ChannelID included in the received sl-RLC-ChannelToReleaseListPC5 that is part of the current UE sidelink configuration:

3>	release the RLC entity and the corresponding logical channel associated with the SL-RLC-ChannelID;

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered by end-to-end DRB release as specified in 5.8.9.1a.1.2; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered based on the derivation of PC5 Relay RLC channel configuration for end-to-end sidelink DRB(s), as specified in 5.8.9.7.2:

2>	release the RLC entity and the corresponding logical channel associated with the end-to-end DRB;

1>	if the PC5 Relay RLC channel release was triggered for a specific destination by upper layers as specified in 5.8.9.5, or due to sidelink RLF as specified in 5.8.9.3:

2>	release the RLC entity and the corresponding logical channel associated with the SL-RLC-ChannelID of the specific destination;

[TS 38.331, clause 5.8.9.7.2]

Upon PC5-RRC connection establishment between the L2 U2N Relay UE and L2 U2N Remote UE, the L2 U2N Relay UE shall:

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply RLC specified configuration of SL-RLC0 as specified in clause 9.1.1.4:

1>	apply RLC default configuration of SL-RLC1 as defined in clause 9.2.4 if the L2 U2N Relay UE is in RRC_IDLE/INACTIVE state;

Upon PC5-RRC connection establishment between two UEs for L2 U2U relay operation, the L2 U2U Relay UE and the L2 U2U Remote UE shall:

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply RLC specified configuration of SL-U2U-RLC as specified in clause 9.1.1.4;

For L2 U2U Relay operation in RRC_IDLE/RRC_INACTVE or out of coverage, the PC5 Relay RLC channel addition/modification can be triggered due to the determination of per-hop QoS requirements upon the addition/modification/release of the end-to-end SL DRB(s) as specified in clause 5.8.9.11.3 and 5.8.9.11.4. The source L2 U2U Remote UE and L2 U2U Relay UE derive the configuration for the corresponding PC5 Relay RLC channel based on SIB12/Preconfiguration, as follows:

-	The source L2 U2U Remote UE derives the configuration for the PC5 Relay RLC channel(s) between the source L2 U2U Remote UE and L2 U2U relay UE (i.e. the first hop PC5 Relay RLC channel(s)), by aggregating the QoS profile(s) of the QoS flow(s) with split QoS information of the first hop into a per-SLRB level QoS profile for each end-to-end Sidelink DRB as described in clause 5.8.9.11.4, and considering the SL-RLC-BearerConfig in either sl-RLC-BearerConfigList or sl-RLC-BearerPreConfigList (linked to the SL-RadioBearerConfig which matches the per-SLRB level QoS profile) as the first hop PC5 Relay RLC channel configuration.

-	The L2 U2U Relay UE derives the configuration for the PC5 Relay RLC channel(s) between L2 U2U relay UE and the target L2 U2U Remote UE (i.e. the second hop PC5 Relay RLC channel(s)), by aggregating the QoS profile(s) of the QoS flow(s) with split QoS information of the second hop into a per-SLRB level QoS profile for each end-to-end Sidelink DRB as described in clause 5.8.9.11.3, and considering the SL-RLC-BearerConfig in either sl-RLC-BearerConfigList or sl-RLC-BearerPreConfigList (linked to the SL-RadioBearerConfig which matches the per-SLRB level QoS profile) as the second hop PC5 Relay RLC channel configuration.

The UE shall:

1>	if the PC5 Relay RLC channel addition/modification was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel addition/modification was triggered due to the configuration received within the sl-ConfigDedicatedNR; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel addition/modification was triggered based on the derivation of PC5 Relay RLC channel configuration(s) for end-to-end sidelink DRB(s):

2>	if the current configuration contains a PC5 Relay RLC channel with the received sl-RLC-ChannelID or sl-RLC-ChannelID-PC5; or

2>	if the configuration in SIB12 or SidelinkPreconfigNR has updated, based on which the PC5 Relay RLC channel is derived:

3>	reconfigure the sidelink RLC entity in accordance with the received sl-RLC-Config or sl-RLC-ConfigPC5;

3>	reconfigure the sidelink MAC entity with a logical channel in accordance with the received sl-MAC-LogicalChannelConfig or sl-MAC-LogicalChannelConfigPC5;

2>	else (a PC5 Relay RLC channel with the received sl-RLC-ChannelID or sl-RLC-ChannelID-PC5 was not configured before):

3>	establish a sidelink RLC entity in accordance with the received sl-RLC-Config (in sl-ConfigDedicatedNR, or SIB12, or SidelinkPreconfigNR) or sl-RLC-ConfigPC5;

3>	configure the sidelink MAC entity with a logical channel in accordance with the received sl-MAC-LogicalChannelConfig or sl-MAC-LogicalChannelConfigPC5.

12.3.1.2.12.3	Test Description

12.3.1.2.12.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1.

-	System information combination NR-31 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used.

-	NR-SS-UE

-	NR-SS-UE1 operating as L2 U2N Remote UE.

-	NR-SS-UE1 is synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.4 and ProSeP using condition Monitoring_Restricted_Model_A as per TS 38.508-1 [4] clause 4.7C.1.

- UE is synchronised on GNSS.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4] on NR Cell 1, subclause 4.4A.

12.3.1.2.12.3.2	Test procedure sequence

Table 12.3.1.2.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side in Table 4.9.41.2.2-1 in TS 38.508-1[4] is performed. | - | - | - | - |
| 2 | The SS transmits an RRCReconfiguration message to add a PC5 Relay RLC channel. | <-- | RRC: RRCReconfiguration | - | - |
| 3 | Check: Does the UE send an RRCReconfigurationSidelink message to NR-SS-UE1 to indicate PC5 Relay RLC channel addition? | --> | PC5 RRC: RRCReconfigurationSidelink | 1 | P |
| 4 | NR-SS-UE1 sends an RRCReconfigurationCompleteSidelink message. | <-- | PC5 RRC: RRCReconfigurationCompleteSidelink |  |  |
| 5 | Check: Does the UE send an RRCReconfigurationComplete message to confirm the establishment of the PC5 Relay RLC channel? | --> | RRC: RRCReconfigurationComplete | 1 | P |
| 6 | The SS transmits an RRCReconfiguration message to modify the PC5 Relay RLC channel established at step 5. | <-- | RRC: RRCReconfiguration | - | - |
| 7 | Check: Does the UE send an RRCReconfigurationSidelink message to NR-SS-UE1 to indicate PC5 Relay RLC channel modification? | --> | PC5 RRC: RRCReconfigurationSidelink | 2 | P |
| 8 | NR-SS-UE1 sends an RRCReconfigurationCompleteSidelink message. | <-- | PC5 RRC: RRCReconfigurationCompleteSidelink |  |  |
| 9 | Check: Does the UE send an RRCReconfigurationComplete message to confirm the modification of the PC5 Relay RLC channel? | --> | RRC: RRCReconfigurationComplete | 2 | P |
| 10 | The SS transmits an RRCReconfiguration message to release the PC5 Relay RLC channel modified at step 9. | <-- | RRC: RRCReconfiguration | - | - |
| 11 | Check: Does the UE send an RRCReconfigurationSidelink message to NR-SS-UE1 to indicate PC5 Relay RLC channel release? | --> | PC5 RRC: RRCReconfigurationSidelink | 3 | P |
| 12 | NR-SS-UE1 sends an RRCReconfigurationCompleteSidelink message. | <-- | PC5 RRC: RRCReconfigurationCompleteSidelink |  |  |
| 13 | Check: Does the UE send an RRCReconfigurationComplete message to confirm the release of the PC5 Relay RLC channel? | --> | RRC: RRCReconfigurationComplete | 3 | P |

12.3.1.2.12.3.3	Specific message contents

Table 12.3.1.2.12.3.3-1: RRCReconfiguration (step 2, step 6, step 10, Table 12.3.1.2.12.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  |
| setup | sl-ConfigDedicatedNR |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.3.1.2.12.3.3-2: sl-ConfigDedicatedNR (Table 12.3.1.2.12.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |
| SL-PHY-MAC-RLC-Config-v1700 SEQUENCE { |  |  |  |
| sl-DRX-Config-r17 | Not present |  |  |
| sl-RLC-ChannelToReleaseList-r17 | Not present |  | step 2, step 6 |
| sl-RLC-ChannelToReleaseList-r17 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelID-r17{ |  |  | step 10 |
| SL-RLC-ChannelID-r17[1] | 4 |  |  |
| } |  |  |  |
| sl-RLC-ChannelToAddModList-r17 | Not present |  | step 10 |
| sl-RLC-ChannelToAddModList-r17 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelConfig-r17{ |  |  | step 2, step 6 |
| SL-RLC-ChannelConfig-r17[1] SEQUENCE { |  |  |  |
| sl-RLC-ChannelID-r17 | 4 |  |  |
| sl-RLC-Config-r17 | SL-RLC-Config | Table 12.3.1.2.12.3.3-3 |  |
| sl-MAC-LogicalChannelConfig-r17 | SL-LogicalChannelConfig as defined in TS 38.508-1 [4], Table 4.6.6-12 |  |  |
| sl-PacketDelayBudget-r17 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.3.1.2.12.3.3-3: SL-RLC-Config (Table 12.3.1.2.12.3.3-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RLC-Config-r16 ::= CHOICE { |  |  |  |  |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  |  |
| sl-SN-FieldLengthAM-r16 |  | size12 |  | step 2 |
| sl-SN-FieldLengthAM-r16 |  | size18 |  | step 6 |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.3.1.2.12.3.3-4: RRCReconfigurationSidelink (step 3, step 7, step 11, Table 12.3.1.2.12.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 with condition TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-DRX-ConfigUC-PC5-r17 | Not Present |  |  |
| sl-LatencyBoundIUC-Report-r17 | Not Present |  |  |
| sl-RLC-ChannelToReleaseListPC5-r17 | Not Present |  | step 3, step 7 |
| sl-RLC-ChannelToReleaseListPC5-r17 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelID-r17{ |  |  | step 11 |
| SL-RLC-ChannelID-r17[1] | 4 |  |  |
| } |  |  |  |
| sl-RLC-ChannelToAddModListPC5-r17 | Not Present |  | step 11 |
| sl-RLC-ChannelToAddModListPC5-r17 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelConfigPC5-r17{ |  |  | step 3, step 7 |
| SL-RLC-ChannelConfigPC5-r17[1] SEQUENCE { |  |  |  |
| sl-RLC-ChannelID-PC5-r17 | 4 |  |  |
| sl-RLC-ConfigPC5-r17 CHOICE { |  |  |  |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  |
| sl-SN-FieldLengthAM-r16 | size12 |  | step 3 |
| sl-SN-FieldLengthAM-r16 | size18 |  | step 7 |
| } |  |  |  |
| } |  |  |  |
| sl-MAC-LogicalChannelConfigPC5-r17 SEQUENCE { |  |  |  |
| sl-LogicalChannelIdentity-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.3.1.2.12.3.3-5: RRCReconfigurationCompleteSidelink (step 4, step 8, step 12, Table 12.3.1.2.12.3.3-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1A-4 with condition RX

Table 12.3.1.2.12.3.3-6: RRCReconfigurationComplete (step 5, step 9, step 13, Table 12.3.1.2.12.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-14

<End of Changed Section>