3GPP TSG-RAN5 Meeting #106	R5-250985

Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4908 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 12.3.1.1.20 Sidelink Relay / L2 U2N / Remote UE / Reselection of NR Sidelink U2N Relay UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Added of new test case 12.3.1.1.20 Sidelink Relay / L2 U2N / Remote UE / Reselection of NR Sidelink U2N Relay UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition new test case 12.3.1.1.20 Sidelink Relay / L2 U2N / Remote UE / Reselection of NR Sidelink U2N Relay UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.1.20(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0565 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.3.1.1.20	Sidelink Relay / L2 U2N / Remote UE / Reselection of NR Sidelink U2N Relay UE

12.3.1.1.20.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and acting as a L2 U2N Remote UE and connected to a L2 U2N Relay UE }

ensure that {

when { SL-RSRP of the currently selected NR sidelink U2N Relay UE is below sl-RSRP-Thresh }

then { UE selects the suitable NR sidelink U2N Relay UE }

}

(2)

with { UE in NR RRC_IDLE state and acting as a L2 U2N Remote UE and connected to a L2 U2N Relay UE }

ensure that {

when { UE receives a PROSE DIRECT LINK RELEASE REQUEST message with cause set to “direct connection is not available anymore” }

then { UE responds with a PROSE DIRECT LINK RELEASE ACCEPT message and selects the suitable NR sidelink U2N Relay UE }

}

12.3.1.1.20.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 24.554, clause 8.2.3.2 and TS 38.331, clause 5.8.15.3. Unless otherwise stated these are Rel-17 requirements.

[TS 24.554, clause 8.2.3.2]

The 5G ProSe remote UE shall trigger the UE-to-network relay reselection procedure if one of the following conditions is met:

a)	the UE has received a lower layers indication that the serving 5G ProSe UE-to-network relay UE no longer fulfills the lower layers criteria as specified in 3GPP TS 38.331 [13];

b)	the parameters related to 5G ProSe UE-to-network relay in the configuration parameters for 5G ProSe UE-to-network relay as specified in clause 5.2.5 (e.g., relay service code, User info ID, etc.) have been updated and the serving 5G ProSe UE-to-network relay UE no longer fulfills the conditions specified in clause 8.2.2.2;

c)	the UE has received a PROSE DIRECT LINK ESTABLISHMENT REJECT message from the 5G ProSe UE-to-network relay UE with the PC5 signalling protocol cause value #1 "direct communication to the target UE not allowed";

d)	the UE has received a PROSE DIRECT LINK RELEASE REQUEST message from the 5G ProSe UE-to-network relay UE with the PC5 signalling protocol cause value #1 "direct communication to the target UE not allowed";

e)	the UE has received a PROSE DIRECT LINK RELEASE REQUEST message from the 5G ProSe UE-to-network relay UE with the PC5 signalling protocol cause value #4 "direct connection is not available anymore";

f)	the UE has not received any response from the 5G ProSe UE-to-network relay UE after M consecutive retransmissions of PROSE DIRECT LINK ESTABLISHMENT REQUEST or PROSE DIRECT LINK KEEPALIVE REQUEST messages;

g)	the UE has not received any response from the 5G ProSe UE-to-network relay UE after M consecutive retransmissions of PROSE PC5 DISCOVERY message for UE-to-network relay discovery solicitation used to trigger the PROSE PC5 DISCOVERY message signal strength measurement between the UE and the 5G ProSe UE-to-network relay UE with which the UE has a link established;

NOTE:	The value of M is implementation specific and is less than or equal to the maximum number of retransmissions allowed for PC5 signalling protocol.

h)	the UE has received a PROSE DIRECT LINK ESTABLISHMENT REJECT message from the ProSe UE-to-network relay UE with the cause value #13 "congestion situation";

i)	the UE has received a PROSE DIRECT LINK RELEASE REQUEST message from the ProSe UE-to-network relay UE with the cause value #13 "congestion situation"; or

j)	the UE has received a PROSE DIRECT LINK ESTABLISHMENT REJECT message from the 5G ProSe UE-to-network relay UE with the cause value #15 "security procedure failure of 5G ProSe UE-to-network relay".

In cases c), d), h) and i), the 5G ProSe remote UE shall exclude the 5G ProSe UE-to-network relay UE which sent the message specified in cases c), d), h) or i) from the UE-to-network relay reselection process described below (at least for the indicated back-off time period if provided from the ProSe UE-to-network relay UE in cases h) and i)).

In case j), when the 5G ProSe remote UE has included the UE identity IE set to SUCI in the PROSE DIRECT LINK ESTABLISHMENT REQUEST message, the 5G ProSe remote UE shall exclude the 5G ProSe UE-to-network relay UE which sent the message specified in case j) from the UE-to-network relay reselection process described below.

To conduct UE-to-network relay reselection process, the UE shall first initiate one of the following procedures or both depending on UE's configuration parameters for 5G ProSe UE-to-network relay as specified in clause 5.2.5:

a)	monitoring procedure for UE-to-network relay discovery as specified in clause 8.2.1.2.3; or

b)	discoverer procedure for UE-to-network relay discovery as specified in clause 8.2.1.3.1.

After the execution of the above discovery procedure(s), the 5G ProSe remote UE performs the UE-to-network relay selection procedure as specified in clause 8.2.2.

NOTE:	It is left to UE implementation on how UE handles the case when there is no appropriate 5G ProSe UE-to-network relay UE around.

[TS 38.331, clause 5.8.15.3]

A UE capable of NR sidelink U2N Remote UE operation that is configured by upper layers to search for a NR sidelink U2N Relay UE shall:

- if the UE has no serving cell; or

1>	if the RSRP measurement of the cell on which the UE camps (for L2 and L3 U2N Remote UE in RRC_IDLE or RRC_INACTIVE)/ the PCell (for L3 U2N Remote UE in RRC_CONNECTED) is below threshHighRemote within sl-RemoteUE-Config:

2>	if the UE does not have a selected NR sidelink U2N Relay UE; or

2>	if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is available and is below sl-RSRP-Thresh; or

2>	if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is not available, and SD-RSRP of the currently selected U2N Relay UE is below sl-RSRP-Thresh; or

NOTE 1:	U2N Remote UE uses SL-RSRP measurements for relay reselection trigger evaluation when there is data transmission from U2N Relay UE to U2N Remote UE, and it is left to UE implementation whether to use SL-RSRP or SD-RSRP for relay reselection trigger evaluation in case of no data transmission from U2N Relay UE to U2N Remote UE. If SD-RSRP is used, the discovery procedure will be performed between the U2N Remote UE and the selected U2N Relay UE.

2>	if the UE has a selected NR sidelink U2N Relay UE, and upper layers indicate not to use the currently selected NR sidelink U2N Relay UE; or

2>	if the UE has a selected NR sidelink U2N Relay UE, and upper layers request the release of the PC5-RRC connection; or

2>	if the UE has a selected NR sidelink U2N Relay UE, and sidelink radio link failure is detected on the PC5-RRC connection with the current U2N Relay UE as specified in clause 5.8.9.3:

3>	perform NR sidelink discovery procedure as specified in clause 5.8.13 in order to search for candidate NR sidelink U2N Relay UEs;

4>	when evaluating the one or more detected NR sidelink U2N Relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same U2N Relay UE ID and using the sl-FilterCoefficientRSRP in SIB12 (if in RRC_IDLE/INACTIVE), the sl-FilterCoefficientRSRP in sl-ConfigDedicatedNR (if in RRC_CONNECTED) or the preconfigured sl-FilterCoefficientRSRP as defined in 9.3 (out of coverage), before using the SD-RSRP measurement results;

4>	consider a candidate NR sidelink U2N Relay UE for which SD-RSRP exceeds sl-RSRP-Thresh by sl-HystMin has met the AS criteria;

3>	if the UE detects any suitable NR sidelink U2N Relay UE(s):

4>	consider one of the available suitable NR sidelink U2N relay UE(s) can be selected;

NOTE 2:	A candidate NR sidelink U2N Relay UE which meets all AS layer criteria defined in 5.8.15.3 and higher layer criteria defined in TS 23.304 [65] can be regarded as suitable NR sidelink U2N Relay UE by the NR sidelink U2N Remote UE. If multiple suitable NR sidelink U2N Relay UEs are available, it is up to Remote UE implementation to choose one NR sidelink U2N Relay UE. The details of the interaction with upper layers are up to UE implementation.

NOTE 3:	For L2 U2N Remote UEs in RRC_IDLE/INACTIVE and L3 U2N Remote UEs, the cell (re)selection procedure and relay (re)selection procedure run independently. If both suitable cells and suitable NR sidelink U2N Relay UEs are available, it is up to NR sidelink U2N Remote UE implementation to select either a cell or a NR sidelink U2N Relay UE. Furthermore, L3 U2N Remote UE's selection on both cell and NR sidelink U2N Relay UE is also based on UE implementation.

3>	else:

4>	consider no NR sidelink U2N Relay UE to be selected.

12.3.1.1.20.3	Test Description

12.3.1.1.20.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 and NR-SS-UE2 operating as L2 U2N Relay UE in RRC_CONNECTED state on the same NR Cell.

-	NR-SS-UE1 and NR-SS-UE2 are both synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.4 and ProSeP using parameter 5G ProSe direct discovery model A as per TS 38.508-1 [4] clause 4.7C.1.

- UE is synchronised on GNSS.

Preamble:

-	The UE is in state 0-A as defined in TS 38.508-1 [4].

12.3.1.1.20.3.2	Test procedure sequence

Table 12.3.1.1.20.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the NR-SS-UE at various time instants of the test execution. Row marked "T1" and "T2" are to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 12.3.1.1.20.3.2-1: NR-SS-UE configuration changes over time

|  | Parameter | Unit | NR-SS-UE1 | NR-SS-UE2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T1 | SD-RSRP / SL-RSRP | dBm/SCS | -88 | Off |  |
| T2 | SD-RSRP / SL-RSRP | dBm/SCS | -110 | -88 |  |
| T3 | SD-RSRP / SL-RSRP | dBm/SCS | -95 | -88 |  |

Table 12.3.1.1.20.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts NR-SS-UE levels according to row T1 of table 12.3.1.1.20.3.2-1. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The generic procedures in TS 38.508-1 table 4.9.42.2.2-1 are performed on NR-SS-UE1 using parameter Cast Type (Unicast) and UE initiated unicast mode ProSe Direct communication. NOTE: The RRC connection is released. | - | - | - | - |
| 4 | SS adjusts NR-SS-UE levels according to row T2 of table 12.3.1.1.20.3.2-1. | - | - | - | - |
| 5 | The NR-SS-UE2 transmits PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement with RRC container. | <-- | PC5-D: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Announcement) | - | - |
| 6 | Check: Does the UE transmit a PROSE DIRECT LINK ESTABLISHMENT REQUEST message on NR-SS-UE2? | --> | PC5-S: PROSE DIRECT LINK ESTABLISHMENT REQUEST | 1 | P |
| 7-12b2 | Steps 3 to 8b2 of the generic procedure in TS 38.508-1 table 4.9.39.2.2-1 are performed on NR-SS-UE2. | - | - | - | - |
| 13-30a2 | Steps 5 to 22a2 of the generic procedure in TS 38.508-1 table 4.9.42.2.2-1 are performed on NR-SS-UE2. NOTE: The RRC connection is released. | - | - | - | - |
| 31 | SS adjusts NR-SS-UE levels according to row T3 of table 12.3.1.1.20.3.2-1. | - | - | - | - |
| 32 | The NR-SS-UE2 sends a PROSE DIRECT LINK RELEASE REQUEST message | <-- | PC5-S: PROSE DIRECT LINK RELEASE REQUEST | - | - |
| 33 | Check: Does the UE send a PROSE DIRECT LINK RELEASE ACCEPT message on NR-SS-UE2? | --> | PC5-S: PROSE DIRECT LINK RELEASE ACCEPT | 2 | - |
| 34 | The NR-SS-UE1 transmits PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement with RRC container. | <-- | PC5-D: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Announcement) | - | - |
| 35 | Check: Does the UE transmit a PROSE DIRECT LINK ESTABLISHMENT REQUEST message on NR-SS-UE1? | --> | PC5-S: PROSE DIRECT LINK ESTABLISHMENT REQUEST | 2 | P |
| 36-41b2 | Steps 3 to 8b2 of the generic procedure in TS 38.508-1 table 4.9.39.2.2-1 are performed on NR-SS-UE1. | - | - | - | - |
| 42-58a3 | Steps 5 to 21a3 of the generic procedure in TS 38.508-1 table 4.9.42.2.2-1 are performed on NR-SS-UE1. | - | - | - | - |

12.3.1.1.20.3.3	Specific message contents

Table 12.3.1.1.20.3.3-1: PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement (Steps 5 and 34, Table 12.3.1.1.20.3.2-2)

Derivation Path: TS 38.508-1[4], Table 4.7C.2-8 with Condition L2RemoteUE

Table 12.3.1.1.20.3.3-2: PROSE DIRECT LINK ESTABLISHMENT REQUEST (Steps 6 and 35, Table 12.3.1.1.20.3.2-2)

Derivation Path: TS 38.508-1[4], Table 4.7C.3-1 with Condition Tx

Table 12.3.1.1.20.3.3-3: PROSE DIRECT LINK RELEASE REQUEST (Step 32, Table 12.3.1.1.20.3.2-2)

| Derivation Path: TS 38.508-1[4], Table 4.7C.3-4 with Condition Rx |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PC5 signalling protocol cause | '0000 0100'B | direct connection is not available anymore |  |

Table 12.3.1.1.20.3.3-4: PROSE DIRECT LINK RELEASE ACCEPT (Step 33, Table 12.3.1.1.20.3.2-2)

Derivation Path: TS 38.508-1[4], Table 4.7C.3-5 with Condition Tx

<End of Changed Section>