3GPP TSG-RAN5 Meeting #106	R5-251004

# Athens, Greece, February 17 - 21 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3789 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SA FR2-FR2 event-triggered reporting with two concurrent Pre-MGs test case 7.6.17.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR SA FR2-FR2 event-triggered reporting with two concurrent Pre-MGs test case 7.6.17.2 was missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case NR SA FR2-FR2 event-triggered reporting with two concurrent Pre-MGs was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will remain missing and WP cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.17.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

#### 7.6.17.2	NR SA FR2-FR2 event-triggered reporting with two concurrent Pre-MGs

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- Test applicability is missing in 38.522

- Message contents are FFS

- Test procedure is missing

- Minimum conformance requirement is FFS

7.6.17.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event with autonomous activation/deactivation of Pre-MGs within FR2 concurrent gaps. The test will partly verify the TDD inter-frequency cell search requirements in TS 38.133 [6] clause 9.2.6.and 9.3.4. Also, this test will also jointly verify pre-configured measurement gap activation/deactivation delay in TS 38.133 [6] clause 8.19.2.

7.6.17.2.2	Test applicability

This test applies to all types of NR UE from release 18 onwards supporting pre-configuration of MGs in RRC signalling.

7.6.17.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.17.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.17.2.

7.6.17.2.4	Test description

7.6.17.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.17.2.4.1-1

Table 7.6.17.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.6.17.2-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Test environment parameters are given in Table 7.6.17.2.4.1-2.

Table 7.6.17.2.4.1-2: Initial conditions for NR SA FR2-FR2 event-triggered reporting with two concurrent Pre-MGs

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.17.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	The general test parameter settings are set up according to Table 7.6.17.2.4.1-3.

2.	Message contents are defined in clause 7.6.17.2.4.3.

3.	Two cells are deployed in the TBD.

Table 7.6.17.2.4.1-3: General test parameters for intra-frequency event triggered reporting with  autonomous activation/deactivation of Pre-MG

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | Config 1 | PCell (Cell 1) |  |
| Neighbour cell |  | Config 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | Config 1 | 1: Cell 1 , 2: Cell 2 |  |
| SMTC configuration |  | Config 1 | SMTC.1 | Same configuration for MO1 and MO2 |
| A4-Threshold | dBm | Config 1 | -120 |  |
| CP length |  | Config 1 | Normal |  |
| Hysteresis | dB | Config 1 | 0 |  |
| Time To Trigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF |  |
| Gap Pattern Id |  | Config 1 | 13 | For both pre-conifgured gaps |
| Measurement gap offset | ms | Config 1 | 9 for MeasGapId #1 19 for MeasGapId #2 | No overlapping cases |
| Time offset between serving and neighbour cells |  | Config 1 | 3 s | Synchronous cells |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 0.2 |  |
| T3 | s | Config 1 | 10.4 for PC1 and PC5; 6.5 for other PC |  |

7.6.17.2.4.2	Test procedure

TBD

7.6.17.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

TBD

7.6.17.2.5	Test requirements

Table 7.6.17.2.4.1-3, 7.6.17.2.5-1, and Table 7.6.17.2.5-2 define the primary level settings for all tests.

Table 7.6.17.2.5-1: NR Cell specific test parameters for intra-frequency event triggered reporting with autonomous activation/deactivation of Pre-MG

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel | MHz | 100: NPRB,c = 66 |  |  | 100: NPRB,c = 66 |  |  |
| Data PRBs allocated |  | 24 |  |  | 24 |  |  |
| Intial BWP configuration |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| BWP-1 Configuration |  | DLBWP.1.6 ULBWP.1.6 |  |  | N/A |  |  |
| BWP-2 Configuration |  | DLBWP.1.5 ULBWP.1.5 |  |  | N/A |  |  |
| RLM-RS |  | CSI-RS |  |  | N/A |  |  |
| PDSCH RMC configuration |  | SR.3.2 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | CR.3.1 TDD |  |  | N/A |  |  |
| Dedicated CORESET RMC configuration |  | CCR.3.1 TDD |  |  | N/A |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI states |  | TCI.State.2 |  |  | N/A |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  |  | 120 |  |  |
| OCNG Patterns |  | OP.5 |  |  | N/A |  |  |
| cellIndividualOffset | dB | N/A |  |  | 16 |  |  |
| SSB |  | SSB.1 FR2 |  |  | SSB.7 FR2 |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |

Table 7.6.17.2.5-2: NR OTA Cell specific test parameters for intra-frequency event triggered reporting with automous activation/deactivation of Pre-MG

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  | Setup 3 defined in A.3.15.3 |  |  |  |  |  |
|  |  | AoA1 |  |  | AoA2 |  |  |
| Beam assumptionNote 3 |  | Rough |  |  | Rough |  |  |
| Es | dBm/SCS | -89+TT | -89+TT | -89+TT | -infinity | -89+TT | -89+TT |
| BB Note 4 | dB | -0.12+TT | -0.12+TT | -0.12+TT | -Infinity | -0.12+TT | -0.12+TT |
| SSB_RP | dBm/SCS | -89+TT | -89+TT | -89+TT | -infinity | -89+TT | -89+TT |
|  | dBm/95.04 MHz | -64.41+TT | -64.41+TT | -64.41+TT | -Infinity | -64.41+TT | -64.41+TT |
| Time multiplexing of the downlink transmissions from each AoA |  | Defined in figure A.7.6.1.1.1-1 |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

During T1, the UE shall be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the Pre-MG#2 occasions.

During T2 and T3, the UE shall be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the Pre-MG#1 or Pre-MG#2 occasions, starting from the 1 st complete Pre-MG#1 occasion after the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5 ms as defined in clause 8.19.5.

The UE shall send one Event A3 triggered measurement report for measurements on cell 2, with a measurement reporting delay less than Y ms from the beginning of time period T3, where Y is

-	5120 ms for a UE supporting power class 1 and 5,

-	3200 ms for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 % with the confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

/// End of changes