3GPP TSG-RAN5 Meeting #106	R5-2501026

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3231 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PC3 n81 into AMPR TC with UL-MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_UL_MIMO_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PC3 configuration of SUL n81 with UL MIMO in AMPR TC is still missing The PC3 test requirements of n99 with UL MIMO in AMPR TC is error. Need to update. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test applicability for NS_43 and NS_43U. Added test configuration table for NS_43 and NS_43U for PC3 configuraiton n81. Added test requirement table for NS_43 and NS_43U for PC3 configuraiton n81. Updated Table 6.2D.3_1.5-3: UE Power Class 3 test requirements (NS_56) for band n99. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMPR TC for SUL n81 PC3 UL MIMO will be still missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.3_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2D.3_1	UE additional maximum output power reduction for SUL with UL MIMO

6.2D.3_1.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1_1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3_1.2	Test applicability

The requirements of this test apply in test case 6.5D.2.4.2_1 UTRA Adjacent channel leakage ratio for network signalling values NS_05U to all types of NR UE release 17 and forward that support SUL and UL-MIMO.

The requirements of this test apply in test case 6.5D.3_2.3 Additional Spurious Emissions for network signalling value NS_05, NS_05U, NS_43, NS_43U, and NS_56 to all types of NR UE release 17 and forward that support SUL and UL-MIMO.

NOTE:	Test execution is not necessary if 6.5D.2.4.2_1 and 6.5D.3_2.3 are executed.

6.2D.3_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1_1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1_1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4_1.3 apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.3.1.

6.2D.3_1.4	Test description

6.2D.3_1.4.1	Test procedure

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in Table 6.2D.3_1.4.1-1 to Table 6.2D.3_1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.3_1.4.1-1: Test Configuration Table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for NUL carrier |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink Config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 37 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 38 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 39 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 40 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 41 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 42 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 43 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 44 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 45 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 46 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 47 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 48 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 49 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 50 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.4.1-2: Test Configuration Table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: Use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |  |  |
| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1935 | 10 |  |  | A4 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 37 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 38 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 39 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 40 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 41 | 1942.5 | 15 |  |  | A5 |  | 64 QAM | Outer_Full |
| 42 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 43 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 44 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 45 | 1950 | 20 |  |  | A6 |  | 64 QAM | Outer_Full |
| 46 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 47 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 48 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 49 | 1935 | 10 |  |  | A4 |  | 256 QAM | Outer_Full |
| 50 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 51 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 52 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 53 | 1942.5 | 15 |  |  | A5 |  | 256 QAM | Outer_Full |
| 54 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 55 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| 56 | 1950 | 20 |  |  | A6 |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.4.1-3: Test Configuration Table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters for SUL carrier Lowest for non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carriers |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 1 | Low | 5 | N/A for A-MPR testing | N/A | 14 | CP-OFDM | QPSK | Edge_1RB_Left |
| 2 | Low | 5 |  |  | 6 |  | QPSK | Outer_Full |
| 3 | Low | 5 |  |  | 4 |  | QPSK | 21@4 |
| 4 | Low | 5 |  |  | 4 |  | QPSK | 1@4 |
| 5 | Low | 5 |  |  | 14 |  | 16 QAM | Edge_1RB_Left |
| 6 | Low | 5 |  |  | 6 |  | 16 QAM | Outer_Full |
| 7 | Low | 5 |  |  | 4 |  | 16 QAM | 21@4 |
| 8 | Low | 5 |  |  | 4 |  | 16 QAM | 1@4 |
| 9 | Low | 5 |  |  | 14 |  | 64 QAM | Edge_1RB_Left |
| 10 | Low | 5 |  |  | 6 |  | 64 QAM | Outer_Full |
| 11 | Low | 5 |  |  | 4 |  | 64 QAM | 21@4 |
| 12 | Low | 5 |  |  | 4 |  | 64 QAM | 1@4 |
| 13 | Low | 5 |  |  | 14 |  | 256 QAM | Edge_1RB_Left |
| 14 | Low | 5 |  |  | 6 |  | 256 QAM | Outer_Full |
| 15 | Low | 5 |  |  | 4 |  | 256 QAM | 21@4 |
| 16 | Low | 5 |  |  | 4 |  | 256 QAM | 1@4 |
| 17 | Low | 10 |  |  | 12 |  | QPSK | Edge_1RB_Left |
| 18 | Low | 10 |  |  | 8 |  | QPSK | Outer_Full |
| 19 | Low | 10 |  |  | 6 |  | QPSK | 1@3 |
| 20 | Low | 10 |  |  | 6 |  | QPSK | 43@9 |
| 21 | Low | 10 |  |  | 4 |  | QPSK | 1@35 |
| 22 | Low | 10 |  |  | 2 |  | QPSK | 17@35 |
| 23 | Low | 10 |  |  | 5 |  | QPSK | Edge_1RB_Right |
| 24 | Low | 10 |  |  | 5 |  | QPSK | 1@40 |
| 25 | Low | 10 |  |  | 3 |  | QPSK | 8@44 |
| 26 | Low | 10 |  |  | 12 |  | 16 QAM | Edge_1RB_Left |
| 27 | Low | 10 |  |  | 8 |  | 16 QAM | Outer_Full |
| 28 | Low | 10 |  |  | 6 |  | 16 QAM | 1@3 |
| 29 | Low | 10 |  |  | 6 |  | 16 QAM | 43@9 |
| 30 | Low | 10 |  |  | 4 |  | 16 QAM | 1@35 |
| 31 | Low | 10 |  |  | 2 |  | 16 QAM | 17@35 |
| 32 | Low | 10 |  |  | 5 |  | 16 QAM | Edge_1RB_Right |
| 33 | Low | 10 |  |  | 5 |  | 16 QAM | 1@40 |
| 34 | Low | 10 |  |  | 3 |  | 16 QAM | 8@44 |
| 35 | Low | 10 |  |  | 12 |  | 64 QAM | Edge_1RB_Left |
| 36 | Low | 10 |  |  | 8 |  | 64 QAM | Outer_Full |
| 37 | Low | 10 |  |  | 6 |  | 64 QAM | 1@3 |
| 38 | Low | 10 |  |  | 6 |  | 64 QAM | 43@9 |
| 39 | Low | 10 |  |  | 4 |  | 64 QAM | 1@35 |
| 40 | Low | 10 |  |  | 2 |  | 64 QAM | 17@35 |
| 41 | Low | 10 |  |  | 5 |  | 64 QAM | Edge_1RB_Right |
| 42 | Low | 10 |  |  | 5 |  | 64 QAM | 1@40 |
| 43 | Low | 10 |  |  | 3 |  | 64 QAM | 8@44 |
| 44 | Low | 10 |  |  | 12 |  | 256 QAM | Edge_1RB_Left |
| 45 | Low | 10 |  |  | 8 |  | 256 QAM | Outer_Full |
| 46 | Low | 10 |  |  | 6 |  | 256 QAM | 1@3 |
| 47 | Low | 10 |  |  | 6 |  | 256 QAM | 43@9 |
| 48 | Low | 10 |  |  | 4 |  | 256 QAM | 1@35 |
| 49 | Low | 10 |  |  | 2 |  | 256 QAM | 17@35 |
| 50 | Low | 10 |  |  | 5 |  | 256 QAM | Edge_1RB_Right |
| 51 | Low | 10 |  |  | 5 |  | 256 QAM | 1@40 |
| 52 | Low | 10 |  |  | 3 |  | 256 QAM | 8@44 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.4.1-4: Test Configuration Table for NS_43

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier: Use uplink carrier centre frequency (Fc) as specified in test parameters NUL carrier: Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier: Use channel bandwidth (Ch BW) as specified in test parameters NUL carrier: Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | N/A | N/A | CP-OFDM | QPSK | Outer_Full (A1) |
| 2 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 3 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 4 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 5 | 902.5 | 5 |  |  |  | 16 QAM | Outer_Full (A1) |
| 6 | 910 | 10 |  |  |  | 16 QAM | Outer_Full (A5) |
| 7 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left (A6) |
| 8 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full (A6) |
| 9 | 910 | 10 |  |  |  | 64 QAM | Outer_Full (A3) |
| 10 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left (A6) |
| 11 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full (A6) |
| 12 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left (A6) |
| 13 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Table 6.2D.3_1.4.1-5: Test Configuration Table for NS_43U for PC3 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier: Use uplink carrier centre frequency (Fc) as specified in test parameters NUL carrier: Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier: Use channel bandwidth (Ch BW) as specified in test parameters NUL carrier: Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |
| A-MPR test parameters for NS_43U |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | N/A | N/A | CP-OFDM | QPSK | Outer_Full |
| 2 | 910 | 10 |  |  |  | QPSK | Outer_Full |
| 3 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left |
| 4 | 907.5 | 15 |  |  |  | QPSK | Outer_Full |
| 5 | 902.5 | 5 |  |  |  | 16 QAM | Outer_Full |
| 6 | 910 | 10 |  |  |  | 16 QAM | Outer_Full |
| 7 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left |
| 8 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full |
| 9 | 902.5 | 5 |  |  |  | 64 QAM | Outer_Full |
| 10 | 910 | 10 |  |  |  | 64 QAM | Outer_Full |
| 11 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left |
| 12 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full |
| 13 | 902.5 | 5 |  |  |  | 256 QAM | Outer_Full |
| 14 | 910 | 10 |  |  |  | 256 QAM | Outer_Full |
| 15 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left |
| 16 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2D.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement channels are set according to the applicable tables 6.2D.3_1.4.1-1 to  6.2D.3_1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.3_1.4.3.

6.2D.3_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.3_1.4.1-1 to Table 6.2D.3_1.4.1-3. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

6.2D.3_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO, with the following exceptions for each network signalling value.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL. All the AdditionalSpectrumEmission in 6.2.3.4.3 are sent in SIB1 as part of supplementaryUplink instead of uplinkConfigCommon.

6.2D.3_1.4.3.1	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_05) |  |  |

6.2D.3_1.4.3.2	Message contents exceptions for network signalling value "NS_05U"

1.	Information element additionalSpectrumEmission is set to NS_05U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05U) |  |  |

6.2D.3_1.4.3.3	Message contents exceptions for network signalling value "NS_56"

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.2D.3_1.4.3.4	Message contents exceptions for network signalling value "NS_43"

1.	Information element additionalSpectrumEmission is set to NS_43. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_43) |  |  |

6.2D.3_1.4.3.5	Message contents exceptions for network signalling value "NS_43U"

1.	Information element additionalSpectrumEmission is set to NS_43U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_43U) |  |  |

6.2D.3_1.5	Test requirement

The measured maximum output power shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable tables from Table 6.2D.3_1.5-1 to Table 6.2D.3_1.5-5. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2C.4. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2D.1_1.3-1 apply.

Table 6.2D.3_1.5-1: UE Power Class 3 test requirements (NS_05) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 2 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 36 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 38 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 39 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 40 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 42 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 43 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 44 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 45 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 46 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 47 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 49 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 50 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.5-2: UE Power Class 3 test requirements (NS_05U) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 2 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 39 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 40 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 41 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 43 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 45 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 46 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 47 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 53 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 55 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.5-3: UE Power Class 3 test requirements (NS_56) for band n99

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 2 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 4 | 23 | 1.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 15.5-TT |
| 5 | 23 | 3 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 6 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 7 | 23 | 3 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 8 | 23 | 2 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 9 | 23 | 3.5 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 10 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 11 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 12 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 13 | 23 | 6.5 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 14 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 15 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 16 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 17 | 23 | 3 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 18 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 19 | 23 | 1.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 20 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 21 | 23 | 1.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 22 | 23 | 3 | 2 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 23 | 23 | 3 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 24 | 23 | 1.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 25 | 23 | 3 | 3 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 26 | 23 | 3 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 27 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 28 | 23 | 2 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 29 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 30 | 23 | 2 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 31 | 23 | 3 | 2 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 31 | 23 | 3 | 5 | 0 | 18 | 5 | 4 | 25+TT | 14-TT |
| 33 | 23 | 2 | 5 | 0 | 18 | 5 | 4 | 25+TT | 14-TT |
| 34 | 23 | 3 | 3 | 0 | 20 | 5 | 4 | 25+TT | 16-TT |
| 35 | 23 | 3.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 36 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 37 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 38 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 39 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 40 | 23 | 3.5 | 2 | 0 | 19.5 | 5 | 4 | 25+TT | 14.5-TT |
| 41 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 14-TT |
| 42 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 14-TT |
| 43 | 23 | 3.5 | 3 | 0 | 19.5 | 5 | 4 | 25+TT | 14.5-TT |
| 44 | 23 | 6.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 45 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 46 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 47 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 48 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 49 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 50 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 51 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 52 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.5-4: UE Power Class 3 test requirements (NS_43) for band n81

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3.0 | 3.5 | 0 |  | 19.5 |  | 5.0 |  | 3 | 25+TT | 14.5-TT |  |
| 2 | 23 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 3 | 23 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 4 | 23 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 5 | 23 | 3.0 | 3.5 | 0 |  | 19.5 |  | 5.0 |  | 3 | 25+TT | 14.5-TT |  |
| 6 | 23 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 7 | 23 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 8 | 23 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 9 | 23 | 3.5 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 10 | 23 | 3.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 11 | 23 | 3.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 12 | 23 | 6.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 13 | 23 | 6.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.5-5: UE Power Class 3 test requirements (NS_43U) for band n81

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 2 | 23 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 3 | 23 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 4 | 23 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 5 | 23 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 6 | 23 | 3.0 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 7 | 23 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 8 | 23 | 3.0 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 9 | 23 | 3.5 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 10 | 23 | 3.5 | 4 | 0 |  | 19 |  | 5.0 |  | 3 | 25+TT | 14.0-TT |  |
| 11 | 23 | 3.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 12 | 23 | 3.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 13 | 23 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25+TT | 11.5-TT |  |
| 14 | 23 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25+TT | 11.5-TT |  |
| 15 | 23 | 6.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| 16 | 23 | 6.5 | 9 | 0 |  | 14 |  | 6.0 |  | 3 | 25+TT | 8-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

<<End of changes>>