3GPP TSG-RAN5 Meeting #106	R5-251031

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3448 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test procedure 4.9.27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the test procedure for adding video to a speech call in 5GC defined in clause 4.9.27, UE initial state is state 3N-A/ NR RRC_CONNECTED. When UE is brought to NR RRC_CONNECTED in Table 4.5.4.2-3, SRB2 is configured in step 7. There is no need to configure SRB2 again in step 1 in cl 4.9.27. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed condition SRB2 in Table 4.9.27.3-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The message contents will not reflect a real network behaviour |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.27.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.9.27	Test procedure for adding video to a speech call in 5GC

4.9.27.1	Scope

The purpose of this procedure is to add video to an established speech call.

4.9.27.2	Procedure description

4.9.27.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 3N-A and registered to the IMS.

4.9.27.2.2	Procedure sequence

Table 4.9.27.2.2-1: IMS video call addition in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND message. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| - | EXCEPTION: Steps 2 and 3 can occur in any order. | - | - | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit a ULInformationTransfer message, an UL NAS TRANSPORT message and an PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |

4.9.27.3	Specific message contents

Table 4.9.27.3-1: PDU SESSION MODIFICATION COMMAND (step 1, Table 4.9.27.2.2-1)

| Derivation Path: Table 4.7.2-9. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  | Reference QoS rule #7 as defined in Table 4.8.2.1-7 using condition IMS_VIDEO and Rule operation code 011. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #6 as defined in Table 4.8.2.3-6 respectively. |  |  |
| Mapped EPS bearer contexts |  | Not Present |  |  |
| Mapped EPS bearer contexts |  |  |  | Interworking_with_EPS |
| Mapped EPS bearer context |  |  |  |  |
| EPS bearer identity |  | The same value as the one specified in the Reference QoS flow referred to from the Reference QoS rule indicated in the IE Authorized QoS rules |  |  |
| Operation code |  | ‘001’B | Create new EPS bearer |  |
| E bit |  | '1'B | Parameters list is included |  |
| Number of EPS parameters |  | ’0001’B | 1 parameter |  |
| Mapped EPS QoS parameters |  | See reference dedicated EPS bearer context #3 in TS 36.508 table 6.6.2-1 |  |  |
| Traffic flow Template |  | See reference dedicated EPS bearer context#3 in TS 36.508 table 6.6.2-1 |  |  |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If the UE has indicated support of S1, then the SS shall include this IE to provide details for the interworking with EPS being supported for a PDU session. This requirement is set up for the purpose of facilitating the test description. It is not mandatory for the Network to support Mapped EPS bearer contexts. |

Table 4.9.27.3-2: RRCReconfiguration (step 1, Table 4.9.27.2.2-1)

| Derivation Path: Table 4.6.1-13 with condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-AddVideo |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-DRB(0,1) | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.9.27.3-3: RadioBearerConfig-AddVideo (Table 4.9.27.3-2)

| Derivation Path: Table 4.6.3-132. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod [1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { | SDAP-Config |  |  |
| defaultDRB | false |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | j | j is allocated according to internal TTCN mapping |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config with condition UM |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |