# Title:	Test Tolerances for FR1 CSI-RS-based beam failure detection and link recovery for Satellite access

Source:	Keysight Technologies

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for FR1 CSI-RS-based beam failure detection and link recovery for satellite access.

The test cases in question, from 38.533, are:

- 14.4.2.3,	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in non-DRX mode for Satellite Access

- 14.4.2.4,	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in DRX mode for Satellite Access

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v17.15.0 Annex A.

#### A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode

##### A.14.4.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell which is served by satellite access node (SAN) and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.14.4.2.3.1-1, A.14.4.2.3.1-2, and below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.14.4.2.3.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0 in the active cell to emulate CSI-RS based beam failure. Figure A.14.4.2.3.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled.

Table A.14.4.2.3.1-1: Supported test configurations for FR1 Pcell

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.14.4.2.3.1-2: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| NTN reference Serving satellite configuration | Config 1 |  |  |  |  | SSC.1 |  |
|  | Config 2 |  |  |  |  | SSC.2 |  |
| Active PCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  | Config 1,2 |  |  |  | FDD |  |
| TDD Configuration |  | Config 1,2 |  |  |  | Not Applicable |  |
| RMSI CORESET Reference Channel |  | Config 1,2 |  |  |  | CR.1.1 FDD | A.3.1.2 |
| Dedicated CORESET Reference Channel |  | Config 1,2 |  |  |  | CCR.1.1 FDD | A.3.1.3 |
| SSB Configuration |  | Config 1,2 |  |  |  | SSB.3 FR1 | A.3.10 |
| SSB Configuration |  | Config 1,2 |  |  |  | SSB. 3 FR1 | A.3.10 |
| SMTC Configuration |  | Config 1,2 |  |  |  | SMTC.1 | A.3.11 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  |  |  | 15 KHz |  |
| PRACH Configuration |  | Config 1,2 |  |  |  | FR1 PRACH configuration 4 | A.3.8.2 |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  |  |  | 0 |  |
| OCNG parameters |  |  |  |  |  | OP.1 | A.3.2.1 |
| CP length |  |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  |  | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  |  | Aggregation level |  |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  |  |  | 1 | N |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  |  | Config 1,2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 and q1 |  |  |  | Config 1,2 |  | CSI-RS.1.2 FDD | A.3.14 |
| CSI-RS configuration for CSI reporting |  |  |  | Config 1,2 |  | CSI-RS.1.1 FDD | A.3.14 |
| TRS configuration |  |  |  | Config 1,2 |  | TRS.1.1 FDD |  |
| CSI-RS-Index assigned as RLM RS |  |  |  | Config 1,2 |  | CSI-RS.1.2 FDD | A.3.14 |
| T310 Timer |  |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  |  | 2 |  |
| T1 |  |  |  |  | s | 0.2 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 0.18 |  |
| T3 |  |  |  |  | s | 0.14 |  |
| T4 |  |  |  |  | s | 0 |  |
| T5 |  |  |  |  | s | 0.08 |  |
| D1 |  |  |  |  | s | 0.04 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Table A.14.4.2.3.1-3 Cell specific test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1,2 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1,2 | dB | -10 | -10 | 10 | 10 | 10 |
| CSI-RS_RP of set q1 | Config 1,2 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 1,2 | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  | NTN-TDLC5-200 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Figure A.14.4.2.3.1-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

##### A.14.4.2.3.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 30+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

# 3. Discussion

The test has 5 time periods: T1, T2, T3, T4 and T5. To analyse the test case it is helpful to show the variation of CSI-RS SNR of set q0 and CSI-RSRP of set q1 versus time, together with the signalled threshold, as shown below:

Figure 3-1: SNR and CSI-RSRP variation for CSI-RS-based beam failure detection and link recovery

According to TS 38.214 clause 4.1, the downlink CSI-RS EPRE can be derived from the SS/PBCH block downlink transmit power given by the parameter ss-PBCH-BlockPower and CSI-RS power offset given by the parameter powerControlOffsetSS provided by higher layers. In this case, rsrp-ThresholdCSI-RS = rsrp-ThresholdSSB+ powerControlOffsetSS and the value of rsrp-ThresholdCSI-RS is equal to rsrp-ThresholdSSB as the powerControlOffsetSS is 0 dB.

Since the SNR level of q0 and the CSI-RSRP value of q1 is constant during T3, T4 and T5, only T1, T2 and T3 are selected in the accompanying spreadsheet to simplify the analysis.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters need to be copied from Tables A.14.4.2.3.1-2 and A.14.4.2.3.1-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. The signalled parameter rsrp-ThresholdSSB is also copied.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In these test cases, during T3, the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1. The criterion is the CSI-RSRP value of set q1 is above the rsrp-ThresholdSSB. So further parameter to calculate is:

- (SSB index 1 CSI-RS_RP – rsrp-ThresholdSSB)

c) Uncertainties

The SS provides 1 NR Cell with fading to the UE. We propose to control the following parameters:

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB averaged over BWConfig

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB for PRBs #RB0-RB274

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc ±0.3 dB for PRBs #RB0-RB274

- Fading profile uncertainty ±0.7 dB

In this test the UE measures the CSI-RSRP over specific Physical Resource Block (PRB) numbers # RB0 to # RB274. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition the UE measurement accuracies need to be considered, it has some inaccuracy which could affect the test verdict.

For NR NTN CSI-RS based L1-RSRP measurement accuracy. The UE measurement accuracy is defined in the following extract from TS 38.133:

#### 10.1.19C.2	CSI-RS based L1-RSRP accuracy requirements

##### 10.1.19C.2.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-RSRP in this clause apply to all CSI-RS resources of the serving cell configured for L1-RSRP measurement.

The accuracy requirements in Table 10.1.19C.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-5 [39] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.19.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	Valid information for the SAN serving the target cell has been provided.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.19C.2.1-1.

Table 10.1.19C.2.1-1: CSI-RS based L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz |  |  |
| [5] | [9.5] | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -70 |
| [8.5] | [11.5] | -3 | NR_FDD_SAB_FR1_A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2A. |  |  |  |  |  |  |  |

To ensure this absolute CSI-RSRP accuracy, the conditions of Io, CSI-RS Es/Iot in Table 10.1.19C.2.1-1, and conditions of CSI-RS_RP and CSI-RS Es/Iot in section Annex B.2.19.2 need to be fulfilled.

### B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band for satellite access.

The conditions are defined in Table B.2.19.2-1 for FR1 NR cells.

Table B.2.19.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1 for satellite access

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_SAB_FR1_A | -124 | -121 | -118 | -3 |
| NOTE 1: NR operating band groups for satellite access are defined in clause 3.5.2A. |  |  |  |  |  |

All the side conditions mentioned above refer to Annex B.2.19.2, which is already considered.

In summary, during T3 the SSB Es/Iot of set q1 shall be above -3dB, the tightest requirement for Io of set q1 shall be between -121dBm/15kHz (-93.05dBm/9.36MHz) and -50dBm/BWchannel for band group NR_FDD_SAB_FR1_A, and the tightest requirement for SSB_RP shall be above -124dBm/15kHz for band groups NR_FDD_SAB_FR1_A.

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 SS-RSRP, so the sensitivity factor is 1.000 during T1, T2 and T3. However, the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

Generally, we assume that the CSI-RS of set q0 does not overlap with the CSI-RS of set q1 in the time domain for the BFR testing, so the Es of set q0 would cause no change to q1 Es/Iot and the sensitivity factor is not considered.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE measurement accuracies where relevant. This process follows the superposition principle. Uncertainties are calculated separately for T1, T2 and T3.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE reporting accuracies are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE reporting accuracies. If all the uncertainties were combined root-sum square, the resulting smaller test limits could case a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

CSI-RS of set q0

During T1, T2 and T3, the Io of set q0 meet the Io side condition < -50dBm.

During T1, the Es/Iot of set q0 shall be above 5dB according to table A.14.4.2.3.1-3. Considering the uncertainties, the CSI-RS of set q0 doesn’t meet the requirement of the Es/Iot > 5dB. The Es/Iot is therefore increased by an amount sufficient to achieve Es/Iot > 5dB.

During T2, the Es/Iot of set q0 shall be above -3dB according to table A.14.4.2.3.1-3. Considering the uncertainties, the CSI-RS of set q0 doesn’t meet the requirement of the Es/Iot > -3dB. The Es/Iot is therefore increased by an amount sufficient to achieve Es/Iot > -3dB.

During T3, the Es/Iot of set q0 shall be below -12dB according to table A.14.4.2.3.1-3. Considering the uncertainties, the CSI-RS of set q0 doesn’t meet the requirement of the Es/Iot < -12dB. The Es/Iot is therefore decreased by an amount sufficient to achieve Es/Iot < -12dB.

CSI-RS of set q1

During T1, T2 and T3, the Io of set q1 meet the Io side condition < -50dBm.

During T1 and T2, the L1-RSRP of set q1 shall be below the rsrp-ThresholdSSB. The UE measures L1-RSRP of set q1, and compares to the relevant threshold. The uncertainties of UE Measured (SSB index 1 CSI-RS_RP – rsrp-ThresholdSSB) don’t meet the requirement of the UE Measured (SSB index 1 CSI-RS_RP – rsrp-ThresholdSSB) < 0dB. The Es/Iot of set q1 is therefore decreased by an amount sufficient to achieve (SSB index 1 CSI-RS_RP – rsrp-ThresholdSSB) < 0dB.

During T3, the L1-RSRP of set q1 shall be above the rsrp-ThresholdSSB. The UE measures L1-RSRP of set q1, and compares to the relevant threshold. The uncertainties of UE Measured (SSB index 1 CSI-RS_RP – rsrp-ThresholdSSB) don’t meet the requirement of the UE Measured (SSB index 1 CSI-RS_RP – rsrp-ThresholdSSB) > 0dB. The Es/Iot of set q1 is therefore increased by an amount sufficient to achieve (SSB index 1 CSI-RS_RP – rsrp-ThresholdSSB) > 0dB.

During T3, the CSI-RS_RP of set q1 meet the CSI-RS_RP side condition > -124dBm/15kHz and the Es/Iot side condition > -3dB.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

CSI-RS of set q0

For CSI-RS of set q0 during T1, the Es/Iot is increased by an amount sufficient to achieve Es/Iot > 5dB. The actual offset required is 0.8 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

For CSI-RS of set q0 during T2, the Es/Iot is increased by an amount sufficient to achieve Es/Iot > -3dB. The actual offset required is 0.8 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

For CSI-RS of set q0 during T3, the Es/Iot is decreased by an amount sufficient to achieve Es/Iot < -12dB. The actual offset required is -0.8 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

CSI-RS of set q1

For CSI-RS of set q1 during T1/T2, the Es/Iot is decreased by an amount sufficient to achieve (SSB index 1 CSI-RS_RP – rsrp-ThresholdSSB) < 0dB. The actual offset required is -0.2 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

For CSI-RS of set q1 during T3, the Es/Iot is increased by an amount sufficient to achieve (SSB index 1 CSI-RS_RP – rsrp-ThresholdSSB) > 0dB. The actual offset required is 0.2 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

g) Check controlled parameters Min/Max

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

No other BW/SCS configuration is defined.

# 6. Treatment of other test cases

TS 38.533 also contains one similar test case 14.4.2.4 in clause 14.4.2. Since the test parameters are identical with that of 14.4.2.3, the same level uncertainties and method of deciding Test Tolerances can be used for 14.4.2.4.