3GPP TSG-RAN5 Meeting #106	R5-251062

# Athens, Greece, 17th - 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.551 | CR | 0049 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | External Cable Verification Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, ETS-Lindgren |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MASC test cannot be performed without a verification test procedure to demonstrate that the impact of an external cable, e.g., charging and/or automation cable, is negligible. In RAN5#105, the following actions/preliminary agreements from R5-248007. The last action related to the pass/fail limit and the impact on MU are incorporated in this CR. Editor’s note: The following aspects are either missing or not yet determined: - Test procedure update to enable testing with external power source in FR2 is FFS. Until such test procedure is defined, the FR2 MIMO OTA test procedure shall be based on stand-alone battery power. - Identify the need for testing with a charging cable, e.g., stability of testing using test automation, test time improvements - Pass/Fail limit of [0.5]dB in verification procedure and corresponding impact on MU |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a verification test procedure in Clause G.4. The goal for the verification check is to make sure that the sensitivities with and without external cable, e.g., charging and/or automation cable, for three different ranges are within [0.5]dB. Checking the sensitivity in each range (A: yellow, B: green, C: cyan) illustrated below would increase confidence that none of the antenna modules was covered by the external cable. If only one test point was verified, there is a residual risk that all but one module could be covered by the external cable. The associated MU for using the external cable has been taken into account as well. Since the verification test is performed on test data that is used to determine the final verdict, the pass/fail test limit of [0.5]dB is directly associated to the MU. As such, a new MU element must be added that applies when MASC testing is performed with an external cable; the MTSU shall not take this MU increase into account but solely rely on the standard battery (as it was done in other cases for FR2 SISO) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MASC test cannot be performed with an external cable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.3, G.4, B.2.15, H.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR to 38.508-2 in R5-251196 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

# E.3	Operating voltage

For FR1 MIMO OTA, all test cases shall be performed in the normal voltage condition with the DUT operated in stand-alone battery powered mode. No extreme voltage testing is required. It is recommended to start testing with a fully charged battery and conclude and/or pause testing before the battery has completely lost its charge.

For FR2 MIMO OTA, all test cases shall be performed in the normal voltage condition with the DUT operated in stand-alone battery powered mode or external power source. No extreme voltage testing is required. As outlined in Clause G.4, it shall be demonstrated that the impact of external cable to device performance is negligible compared to stand-alone battery powered mode. It is recommended to start testing with a fully charged battery and conclude and/or pause testing before the battery has completely lost its charge.

<<< Skip Unchanged Sections >>>

# G.4	Use of External Cable

Editor’s note: The following aspect is not yet determined:

-	The pass/fail limit of the verification test to enable testing with external cable, e.g., charging or automation cable, in FR2, which has a direct impact on MU, could be finalized based on the maximum acceptable limit by industry and/or empirically

The DUT may be tested with an external cable for any band and channel if declared by the vendor and proper cable routing instructions are provided, as outlined in Table A.4.3.9-15 [12].

The following verification tests shall be performed in each band that utilized the external cable:

1. Identify test points in test point ranges A, B, and C defined in Table G.4-1 that yielded the best sensitity within each range when performed with the external cable, i.e., PA,cable, PB,cable, PC,cable after the executing test procedure described in 7.1.2.4.2 with a external cable.

Table G.4-1: Verification test points for external cable

| Test Point Range | f | q | Test Poins in Test Point Range, see Table 7.1.2.4.2 |
| --- | --- | --- | --- |
| A | 0°≤f<120° | 45°≤q<135° | 7, 9, 14, 17, 20, 24, 30, 31 |
| B | 120°≤f<240° |  | 8, 12, 13, 16, 18, 25, 26, 27 |
| C | 240°≤f<360° |  | 10, 11, 15, 19, 21, 22, 23, 28, 29 |

2. For each of the test points in ranges A, B, and C identified in step 1, perform sensitivity measurements with the standalone battery (with the external cable removed), i.e., PA,standalone, PB, standalone, PC, standalone

3. Determine whether all sensitivities performed in ranges A, B, and C are within [0.5]dB, i.e.,

|PA,cable – PA,standalone| ≤ [0.5]dB	(4.1)

|PB,cable – PB,standalone| ≤ [0.5]dB	(4.2)

|PC,cable – PC,standalone| ≤ [0.5]dB	(4.3)

4. If the all 3 conditions in step 3 are met, the MASC results performed with the external cable are applicable for each band. However, if any of the conditions in step 3 are not met, the MASC test procedure in Clause 7.1.2.4.2 shall be repeated with the standalone battery, i.e., without the external cable, for the respective band.

If an external cable was used, it is recommended to follow the test sequences as outlined below:

i. Perform the MASC test, Clause 7.1.2.4.2, while following the guidelines of the vendor declaration outlined in Table A.4.3.9-15 [12].

ii. Remove the external cable with as few changes to the device and test setup as possible.

iii. Perform the verification tests above, Steps 1 through 4,  after the conclusion of the MASC tests, step i, and the removal of the external cable, step ii

<<< Skip Unchanged Sections >>>

# B.2	Measurement error contribution descriptions for MPAC

## B.2.1	Mismatch for measurement process

This term comes from the mismatch between the system input cables connecting to the base station simulator output port. For more information, see clause A.4.2.1 of 3GPP TS 38.561 [8].

## B.2.2	Measurement distance uncertainty

The cause of this uncertainty contributor is due to the reduction of distance between the measurement antenna and the DUT. Given that 1.2 m is defined as the minimum range length for FR1 MPAC system and the device is not offset from the axis(es) of rotation, this term could be set as 0 dB.

## B.2.3	Quality of quiet zone

The quality of the quiet zone procedure characterizes the quiet zone performance of the anechoic chamber, specifically the effect of reflections within the anechoic chamber including any positioners and support structures. For FR1 quality of quiet zone measurements, reference antennas of sleeve dipole or magnetic loop are used. For more information, see clauses A.3.7 and A.4.2.8 of 3GPP TS 38.561 [8] with the exception that only the phi-axis ripple test is performed for MIMO OTA.

The ripple test shall be repeated when the RF/propagation conditions inside the chamber have changed, e.g., the chamber has been disassembled and reassembled, portions of the absorber been replaced, measurement antennas/probes been replaced, positioning system been replaced, etc.

## B.2.4	Base Station simulator

gNB emulator is used to drive a signal to the channel emulator and then to the device under test. Generally there occurs uncertainty contribution from absolute level accuracy, non-linearity and frequency characteristic of the gNB emulator.

For practical reasons, in a case that a VNA is used as a calibration equipment, gNB emulator is connected to the system after the calibration measurement is performed by the VNA. Hence, the uncertainty on the absolute level of gNB emulator (transmitter device) cannot be assumed as systematic. This uncertainty will be determined from the manufacturer's datasheet and the distribution used shall match that provided in the datasheet which are commonly quoting MUs/accuracies with a "95 % confidence level" and/or a "coverage factor of 2". In the absence of a declared distribution in the datasheet, the rectangular distribution should be used. Furthermore, the uncertainty of the non-linearity is included in the absolute level uncertainty.

## B.2.5	Channel Emulator

The channel emulator is also working as a signal source in the NR MIMO OTA system, therefore there occurs uncertainty contribution from absolute level accuracy, non-linearity, frequency characteristic and stability of the channel emulator. These uncertainty contributions shall be taken from the manufacturer's data sheet.

## B.2.6	Amplifier uncertainties

Any components in the setup can potentially introduce measurement uncertainty. It is then needed to determine the uncertainty contributors associated with the use of such components. For the case of external amplifiers, the following uncertainties should be considered but the applicability is contingent to the measurement implementation and calibration procedure:

-	Stability:

-	An uncertainty contribution comes from the output level stability of the amplifier. Even if the amplifier is part of the system for both measurement and calibration, the uncertainty due to the stability shall be considered. This uncertainty can be either measured or determined by the manufacturers' data sheet for the operating conditions in which the system will be required to operate.

-	Linearity:

-	An uncertainty contribution comes from the linearity of the amplifier since in most cases calibration and measurements are performed at two different input/output power levels. This uncertainty can be either measured or determined by the manufacturers' data sheet.

-	Noise Figure:

-	When the signal goes into an amplifier, noise is added so that the SNR at the output is reduced with regard to the SNR of the signal at the input. This added noise introduces error on the signal which affects the Error Rate of the receiver thus the EVM (Error Vector Magnitude). An uncertainty can be calculated through the following formula:

-	Where SNR is the signal to noise ratio in dB at the signal level used during the sensitivity measurement.

-	Mismatch:

-	If the external amplifier is used for both stages, measurement and calibration the uncertainty contribution associated with it can be considered systematic and constant -> 0dB. If it is not the case, the mismatch uncertainty at its input and output shall be either measured or determined by the method described in 3GPP TR 38.901 [5].

-	Gain:

-	If the external amplifier is used for both stages, measurement and calibration the uncertainty contribution associated with it can be considered systematic and constant -> 0 dB. If it is not the case, this uncertainty shall be considered.

## B.2.7	Random uncertainty

This contribution is used to account for all the unknown, unquantifiable, etc. uncertainties associated with the measurements.

Random uncertainty MU contributions are normally distributed. The random uncertainty term, by definition, cannot be measured, or even isolated completely. A fixed value of 0.4 dB is suggested for TRMS measurements to include a digital error rate uncertainty and is aligned with the random uncertainty for TRS for NR FR1 (3GPP TS 38.561 [8]).

## B.2.8	Throughput measurement: output level step resolution

When output power of the communication tester is swept to reach the throughput target that defines the sensitivity threshold, the final power step resolution represents an asymmetric uncertainty contribution that can be corrected since this uncertainty is device and test system independent. The lab shall correct the TRMS results by -0,5 output level step resolution of the final power step search and note the correction in the test report. If the alternate DL signal power search procedure with linearization, detailed in clause A.2.3 is applied to assess the DL signal power at each grid point, no correction of TRMS shall be applied and no additional MU shall be considered.

## B.2.9	Signal flatness

For wireless technologies with wide channel bandwidths, the test system might not have a flat frequency response across the entire channel. While the range calibration corrects for any variation of frequency response as a function of the centre frequency of the channel, the broadband delivered to the test zone will be a function of the entire channel bandwidth as opposed to just the centre frequency. For more information, see clause A.4.2.14 of 3GPP TS 38.561 [8].

## B.2.10	Mismatch for calibration process

During calibration stage, there will be impendence mismatch between the various RF cables and components used within the system. Standing waves are created by the reflections between any two components and uncertainty in the signal level will be generated.  In general, three mismatch for calibration process should be considered:

-	Loopback cable path: This item comes from the mismatch between the reference cable and the loopback cable during the loopback cable measurement step.

-	System input path: This item comes from the mismatch between the loopback cable and the system input cable (generally the output cable after BS simulator). The reflectivity of the source output port is measured at the end of the loopback cable connecting to the system input cable.

-	Reference antenna: This item comes from the mismatch between the VNA input port and the reference antenna. The reflectivity of the VNA input port is measured at the end of the reference cable connecting to the reference antenna.

For more information, see clause A.4.2.1 of 3GPP TS 38.561 [8].

## B.2.11	Reference antenna positioning misalignment

This contribution originates from reference antenna alignment and pointing error. In this measurement if the maximum gain directions of the reference antenna and the receiving antenna are aligned to each other, this contribution can be considered negligible and therefore set to zero.

## B.2.12	Total Uncertainty of the Network Analyzer

This contribution originates from all uncertainties involved transmission magnitude measurement (including drift and frequency flatness) with a network analyser. will be determined from the manufacturer's datasheet which is now commonly quoting MUs/accuracies with a "95 % confidence level" and/or a "coverage factor of 2"and the distribution used shall match that provided in the datasheet. In the absence of a declared distribution in the datasheet, the rectangular distribution should be used.

## B.2.13	Uncertainty of an absolute gain of the calibration antenna

The calibration antenna only appears in calibration phase (Stage 1). Therefore, the gain uncertainty has to be taken into account.

This uncertainty shall come from a calibration report (which is now commonly quoting MUs/accuracies with a "95 % confidence level" and/or a "coverage factor of 2") with traceability to a National Metrology Institute with measurement uncertainty budgets generated following the guidelines outlined in internationally accepted standards. In the absence of a declared distribution in the present document, the rectangular distribution should be used.

## B.2.14	Offset of the Phase Center of the Reference Antenna

During range reference measurement, if a directional antenna is used, the uncertainty in the accuracy of positioning its phase center on the axis of rotation will directly generate an uncertainty in this part of the measurement. In practical measurement, sleeve dipoles and loops are used for FR1 calibration, then the uncertainty of this element should be 0 dB, since the phase center offset is negligible.

## B.2.15	Use of External Cable (FR2 Only)

When an external cable, e.g., charging or automation cable, is used for FR2 MIMO OTA testing, the procedure in G.4 verifies that select results with and without external cable are within a fixed pass/fail limit. The overall MU, however, needs to take this pass/fail limit into account if the external cable is used for testing.

<<< Skip Unchanged Sections >>>

## H.1	MU Budget for FR2 3D-MPAC system

Editor’s note: This clause defines the Preliminary Measurement Uncertainty (MU) budget for FR2 3D-MPAC system based on 38.151 section B.2.5, as shown here in Table H.1-1. The final FR2 3D-MPAC MU Budget is FSS.

Table H.1-1: Measurement uncertainty budget for FR2 3D-MPAC system

| UID | Description of uncertainty contribution | Example value (26.5GHz≤f≤29.5GHz) | Example value (37GHz ≤f≤40GHz) | Distribution of the probability | Std Uncertainty (26.5GHz≤f≤29.5GHz) [dB] | Std Uncertainty (37GHz ≤f≤40GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 1.30 | TBD | Actual | [1.30] | TBD |
| 2 | Measure distance uncertainty | 0.00 | TBD | Rectangular | [0.00] | TBD |
| 3 | Quality of quiet zone | 0.6 | TBD | Actual | [0.6] | TBD |
| 4 | Base Station simulator | 2.9 | TBD | Normal | [1.45] | TBD |
| 5 | Channel Emulator -absolute value -stability -linearity | 1.5dB 0.5dB 0.4dB | TBD | Actual (normal- power; rect-stability) | [0.84] | TBD |
| 6 | Amplifier uncertainties | 2.10 | TBD | Normal | [1.05] | TBD |
| 7 | Random uncertainty | 0.50 | TBD | Normal | [0.25] | TBD |
| 8 | Throughput measurement: output level step resolution | 0.25 | TBD | Rectangular | [0.14] | TBD |
| 9 | DUT sensitivity drift | 0.2 | 0.2 | Rectangular | [0.12] | TBD |
| 10 | Signal flatness | 0.00 | 0.00 | Normal | [0.00] | TBD |
| 11 | Use of External Cable (Note 1) | [0.5] | [0.5] | Actual | [0.5] | TBD |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | 0.00 | TBD | U-Shaped | [0.00] | TBD |
| 12 | Reference antenna positioning misalignment | 0.01 | TBD | Rectangular | [0.01] | TBD |
| 13 | Quality of quiet zone | 0.60 | TBD | Actual | [0.60] | TBD |
| 14 | Total uncertainty of the Network Analyzer | 0.73 | TBD | Normal | [0.37] | TBD |
| 15 | Uncertainty of an absolute gain of the calibration antenna | 0.60 | TBD | Normal | [0.30] | TBD |
| 16 | Offset of the Phase Center of the Reference Antenna | 0.00 | TBD | Rectangular | [0.00] | TBD |
| Total Expanded Uncertainty, U, with 95% Confidence Interval |  |  |  |  | [5.05] | TBD |
| NOTE 1: For MTSU derivation purpose, this value is set to 0.0 (no external cable) |  |  |  |  |  |  |

<<< END OF CHANGES >>>