3GPP TSG-RAN WG5 Meeting # 106	R5-251067

# Athens, Greece, February 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0573 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on 4.1.1 for applicability of RF SA FR1 conformance test cases for 4Rx Refsens |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_NR_bands_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The applicability of RF SA FR1 conformance test cases for 4Rx Refsens is missing and should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the applicability of RF SA FR1 conformance test cases for 4Rx Refsens. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability of RF SA FR1 conformance test cases for 4Rx Refsens will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

4.1.1	FR1 standalone conformance test cases

Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-1 [1]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power | Rel-15 | C001l | UEs supporting 5GS FR1 and not supporting RedCap and not supporting eRedCap | D001a | PC1 PC2 PC3 | TC 6.2.1 is skipped if TC 6.2G.1 is executed. |  |
| 6.2.2 | UE maximum output power reduction | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5.2.4.1 is executed. Skip TC 6.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.3 or 6.5B.2.3.3.1 has been executed. TC 6.2.2 is skipped if TC 6.2G.2 is executed. |  |
| 6.2.3 | UE additional maximum output power reduction | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 PC1.5 | Test execution is not necessary if TS 38.521-1 TC 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed. Skip TC 6.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.3 has been executed or TS 38.521-3 TCs 6.5B.2.3.2, 6.5B.2.3.3.2 and 6.5B.4.3 have been executed. TC 6.2.3 is skipped if TC 6.2G.3 is executed. |  |
| 6.2.4 | Configured transmitted power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 PC1.5 | TC 6.2.4 is skipped if TC 6.2G.4 is executed. |  |
| 6.2A.1.1 | UE maximum output power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA (NOTE 1) Intra-band non-contiguous CA (NOTE 1) |  |  |
| 6.2A.2.1 | UE maximum output power reduction for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA: PC2 (NOTE 1), PC3 (NOTE 1) | Test execution is not necessary if TS 38.521-1 TC 6.5A.2.4.1.1 is executed. |  |
| 6.2A.3.1 | UE additional maximum output power reduction for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA (NOTE 1) | Test execution is not necessary if TS 38.521-1 TC 6.5A.2.3 and 6.5A.3.3 are executed. |  |
| 6.2A.4.1 | Configured transmitted power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA (NOTE 1) |  |  |
| 6.2B.1.1 | UE maximum output power for NR-DC | Rel-16 | C004a | UEs supporting 5GS FR1 and NR-DC | E015b |  |  |  |
| 6.2B.2.1 | UE maximum output power reduction for NR-DC | Rel-16 | C004a | UEs supporting 5GS FR1 and NR-DC | E015b | PC1 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5B.2.4 is executed. |  |
| 6.2B.3.1 | UE additional maximum output power reduction for NR-DC | Rel-16 | N/A | No requirements in TS 38.521-1 TC 6.5B.2.3 Additional Spectrum Emission mask for NR-DC and TS 38.521-1 TC 6.5B.3.3 Additional Spurious Emissions for NR-DC. |  |  |  |  |
| 6.2B.4.1 | Configured transmitted power level for NR-DC | Rel-16 | C004a | UEs supporting 5GS FR1 and NR-DC | E015b |  |  |  |
| 6.2C.1 | Configured transmitted power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.2C.3 | UE maximum output power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.2C.4 | UE maximum output power reduction for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 Test execution is not necessary if TS 38.521-1 TC 6.5C.2.4.1 is executed. |  |
| 6.2C.5 | UE additional maximum output power reduction for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.2D.1 | UE maximum output power for UL MIMO | Rel-15 | N/A | No test point applicable to Rel-15 UE | D022 | PC1.5 PC2 PC3 |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.2D.1_1 | UE maximum output power for SUL with UL MIMO | Rel-17 | C179b | UEs supporting 5GS FR1 and SUL and ULFPTx | D024 |  |  |  |
| 6.2D.2 | UE maximum output power reduction for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 | PC1.5 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.4.1 is executed. |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.2D.2_1 | UE maximum output power reduction for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 | PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.4.1_1 is executed. |  |
| 6.2D.3 | UE additional maximum output power reduction for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 | PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.3, 6.5D.2.4.2 and 6.5D.3.3 are executed. |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.2D.PC2 PC33_1 | UE additional maximum output power reduction for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.2D.4 | Configured transmitted power for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 | PC1.5 PC2 PC3 |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.2D.4_1 | Configured transmitted power for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
|  |  | Rel-17 | C179b | UEs supporting 5GS FR1 and SUL and ULFPTx |  |  |  |  |
| 6.2E.1.2 | UE maximum output power for V2X / concurrent operation | Rel-16 | FFS | UEs supporting 5GS FR1 and NR sidelink and Simultaneous transmission of uplink and sidelink | FFS |  |  |  |
| 6.2E.2.1 | UE maximum output power reduction for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 | PC3 PC2 | Test execution is not necessary if TS 38.521-1 TC 6.5E.2.4.1 is executed. |  |
| 6.2E.2.1D | UE maximum output power reduction for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 | PC3 PC2 |  |  |
| 6.2E.2.2 | UE maximum output power reduction for V2X / concurrent operation | FFS | FFS | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR sidelink and Simultaneous transmission of uplink and sidelink | FFS | Inter-band concurrent operation Intra-band concurrent operation: PC3 (NOTE 1), PC2 (NOTE 1) |  |  |
| 6.2E.3.1 | UE additional maximum output power reduction for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 | PC3 PC2 | Test execution is not necessary if TS 38.521-1 TC 6.5E.2.3.1 is executed. |  |
| 6.2E.3.1D | UE additional maximum output power reduction for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 | PC3 PC2 | Test execution is not necessary if TS 38.521-1 TC 6.5E.2.3.1D is executed. |  |
| 6.2E.3.2 | UE maximum output power reduction for V2X / concurrent operation | Rel-16 | N/A | No test case details are specified in TS 38.521-1. |  |  |  |  |
| 6.2E.4.1 | Configured transmitted power for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 | PC3 PC2 |  |  |
| 6.2E.4.1D | Configured transmitted power for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 | PC3 PC2 |  |  |
| 6.2E.4.2 | Configured transmitted power for V2X / concurrent operation | Rel-16 | FFS | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR sidelink and Simultaneous transmission of uplink and sidelink | FFS | Inter-band concurrent operation Intra-band concurrent operation: PC3, PC2 |  |  |
| 6.2F.1 | UE maximum output power for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.2F.2 | UE maximum output power reduction for shared spectrum access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.2F.3 | UE additional maximum output power reduction for shared spectrum access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.2F.4 | Configured transmitted power for shared spectrum access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.2G.1 | UE maximum output power for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.2G.2 | UE maximum output power reduction for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5G.2.3.1 is executed. |  |
| 6.2G.3 | UE additional maximum output power reduction for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5G.2.2 and 6.5G.3.3 are executed. |  |
| 6.2G.4 | Configured transmitted power for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.2H.1.2 | UE maximum output power reduction for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5H.1.2.1 is executed. |  |
| 6.2H.1.3 | UE additional maximum output power reduction for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5H.1.2.3 is executed. |  |
| 6.2H.1.4 | Configured transmitted power for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.2H.3.1 | UE maximum output power for inter-band UL CA with UL MIMO | Rel-17 | C328 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with UL MIMO. | E045 | PC1.5 PC2 PC3 |  |  |
| 6.2I.1 | UE maximum output power for RedCap | Rel-17 | C177 | RedCap UEs supporting 5GS FR1 | D001 |  |  |  |
| 6.2I.2 | UE maximum output power for eRedCap | Rel-18 | C372 | eRedCap UEs supporting 5GS FR1 | D001 |  |  |  |
| 6.2I.3 | Void |  |  |  |  |  |  |  |
| 6.2I.4 | Void |  |  |  |  |  |  |  |
| 6.2J.1 | UE maximum output power for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.2J.2 | Configured transmitted power for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.2L.3.1 | UE maximum output power for inter-band UL CA with Tx Diversity | Rel-17 | C329 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with Tx Diversity. | E046 | PC1.5 PC2 PC3 |  |  |
| 6.3.1 | Minimum output power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | TC 6.3.1 is skipped if TC 6.3G.1 is executed. |  |
| 6.3.3.2 | General ON/OFF time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.3.3.2 if UE supports NSA and TS 38.521-3 TC 6.3B.3.1 or 6.3B.3.2 or 6.3B.3.3 has been executed. TC 6.3.3.2 is skipped if TC 6.3G.3.1 is executed. |  |
| 6.3.3.4 | PRACH time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.3.3.4 if UE supports NSA and TS 38.521-3 TC 6.3B.4.1 or 6.3B.4.2 or 6.3B.4.3 has been executed. TC 6.3.3.4 is skipped if TC 6.3G.3.2 is executed. |  |
| 6.3.3.6 | SRS time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | TC 6.3.3.6 is skipped if TC 6.3G.3.3 is executed. |  |
| 6.3.4.2 | Absolute power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | TC 6.3.4.2 is skipped if TC 6.3G.4.1 is executed. |  |
| 6.3.4.3 | Relative power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | TC 6.3.4.3 is skipped if TC 6.3G.4.2 is executed. |  |
| 6.3.4.4 | Aggregate power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | TC 6.3.4.4 is skipped if TC 6.3G.4.3 is executed. |  |
| 6.3A.1.1 | Minimum output power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.3A.3.1 | Transmit ON/OFF time mask for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.3A.3.2 | Time mask for switching between two uplink carriers | Rel-16 | C051 | UEs supporting 5GS FR1 and Inter-band CA (2UL CA) and dynamic 1Tx-2Tx UL Tx switching | E019 |  |  |  |
| 6.3A.3.3 | Time mask for switching between two uplink carriers with two transmit antenna connectors | Rel-17 | C051b | UEs supporting 5GS FR1 and Inter-band CA (2UL CA) and dynamic 2Tx-2Tx UL Tx switching | E019a |  |  |  |
| 6.3A.3.4 | Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors (3UL CA) | Rel-17 | C051d | UEs supporting 5GS FR1 and inter-band/intra-band CA (3UL CA) and dynamic 1Tx-2Tx UL Tx switching. | E019b |  |  |  |
| 6.3A.3.5 | Time mask for switching between two uplink bands with two transmit antenna connectors (3UL CA) | Rel-17 | C051f | UEs supporting 5GS FR1 and inter-band/intra-band CA (3UL CA) and dynamic 2Tx-2Tx UL Tx switching. | E019c |  |  |  |
| 6.3A.4.1.1 | Absolute power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.3A.4.2.1 | Relative power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.3A.4.3.1 | Aggregate power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.3C.1 | Minimum output power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.3C.3.1 | General transmit ON/OFF time mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.3C.3.2 | General transmit ON/OFF time mask for switching between two uplink carriers | Rel-16 | C178 | UEs supporting 5GS FR1 and SUL and dynamic UL 1Tx-2Tx switching | E044 |  | NOTE 2 |  |
| 6.3C.3.3 | Time mask for switching between two uplink carriers with two transmit antenna connectors | Rel-17 | C178a | UEs supporting 5GS FR1 and SUL and dynamic UL 2Tx-2Tx switching | E044a |  |  |  |
| 6.3C.3.4 | Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors | Rel-17 | C178b | UEs supporting 5GS FR1 and SUL with intra-band contiguous CA and dynamic UL 1Tx-2Tx switching | E044b |  |  |  |
| 6.3C.3.5 | Time mask for switching between two uplink bands with two transmit antenna connectors | Rel-17 | C178c | UEs supporting 5GS FR1 and SUL with intra-band contiguous CA and dynamic UL 2Tx-2Tx switching | E044c |  |  |  |
| 6.3C.3.6 | Time mask for switching across three uplink bands (3CC) | Rel-18 | C346 | UEs supporting 5GS FR1 and SUL with inter-band CA and dynamic UL switching across three bands | E049 |  |  |  |
| 6.3C.3.6_1 | Time mask for switching across three uplink bands (more than 3CC) | Rel-18 | C347 | UEs supporting 5GS FR1 and SUL with intra-band + inter-band CA and dynamic UL switching across three bands | E049 |  |  |  |
| 6.3C.4.1 | Absolute power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.3C.4.2 | Relative power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.3C.4.3 | Aggregate power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.3D.1 | Minimum output power for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.3D.1_1 | Minimum output power for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.3D.3 | Transmit ON/OFF time mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.3D.3_1 | Transmit ON/OFF time mask for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.3D.4.1 | Absolute power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.3D.4.1_1 | Absolute power tolerance for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.3D.4.2 | Relative power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.3D.4.2_1 | Relative power tolerance for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.3D.4.3 | Aggregate power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 | PC1.5 PC2 PC3 |  |  |
| 6.3D.4.3_1 | Aggregate power tolerance for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.3E.1.1 | Minimum output power for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.3E.1.1D | Minimum output power for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.3E.3.1 | Transmit ON/OFF time mask for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.3E.3.1D | Transmit ON/OFF time mask for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.3E.4.1 | Absolute power tolerance for V2X | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.3F.1 | Minimum output power | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.3F.4.2 | Absolute power tolerance for shared spectrum access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.3F.4.3 | Relative power tolerance for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.3F.4.4 | Aggregate power tolerance for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.3G.1 | Minimum output power for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.3.1 | General ON/OFF time mask for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.3.2 | PRACH time mask for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.3.3 | SRS time mask for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.4.1 | Absolute power tolerance for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.4.2 | Relative power tolerance for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.4.3 | Aggregate power tolerance for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3H.1.1 | Minimum output power for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.3H.1.3 | Transmit ON/OFF time mask for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.3J.1 | Minimum output power for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.3.3 | PRACH time mask for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.3.4 | SRS time mask for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.4.1 | Absolute power tolerance for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.4.2 | Relative power tolerance for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.4.3 | Aggregate power tolerance for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.4.1 | Frequency error | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.4.1 if UE supports NSA and TS 38.521-3 TC 6.4B.1.1 or 6.4B.1.2 or 6.4B.1.3 has been executed. TC 6.4.1 is skipped if TC 6.4G.1 is executed. |  |
| 6.4.2.1 | Error vector magnitude | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.4.2.1 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.1 or 6.4B.2.2.1 or 6.4B.2.3.1 has been executed. TC 6.4.2.1 is skipped if TC 6.4G.2.1 is executed. |  |
| 6.4.2.1a | Error Vector Magnitude including symbols with transient period | Rel-16 | C156 | UEs supporting 5GS FR1 AND Band supporting enhancedUL-TransientPeriod | D001a |  |  |  |
| 6.4.2.2 | Carrier leakage | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.4.2.2 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.2 or 6.4B.2.2.2 or 6.4B.2.3.2 has been executed. TC 6.4.2.2 is skipped if TC 6.4G.2.2 is executed. |  |
| 6.4.2.3 | In-band emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.4.2.3 if UE supports NSA and TS 38.521-3 TC 6.4B.2.2.3 or 6.4B.2.3.3 has been executed. TC 6.4.2.3 is skipped if TC 6.4G.2.3 is executed. |  |
| 6.4.2.4 | EVM equalizer spectrum flatness | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.4.2.4 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.4 or 6.4B.2.2.4 or 6.4B.2.3.4 has been executed. TC 6.4.2.4 is skipped if TC 6.4G.2.4 is executed. |  |
| 6.4.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK | Rel-15 | C050 | UEs supporting 5GS FR1 Power Class 3 and pi/2-BPSK modulation scheme for power boosting in FR1 | D017a |  |  |  |
|  |  | Rel-16 | C111 | UEs supporting 5GS FR1 and pi/2-BPSK modulation scheme and low PAPR DMRS | D001a |  |  |  |
| 6.4.2.6 | Phase continuity requirements for DMRS bundling | Rel-17 | C337 | UEs supporting 5GS FR1, dmrs-BundlingPUCCH-Rep-r17 and either dmrs-BundlingPUSCH-multiSlot-r17 or dmrs-BundlingPUSCH-RepTypeA-r17 or dmrs-BundlingPUSCH-RepTypeB-r17 | D001a |  | NOTE 1 |  |
| 6.4A.1.1 | Frequency error for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA and not supporting Tx Diversity) | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA |  |  |
| 6.4A.2.1.1 | Error vector magnitude for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA and not supporting Tx Diversity) | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA |  |  |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA and not supporting Tx Diversity) | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA(NOTE 1) |  |  |
| 6.4A.2.3.1 | In-band emissions for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA and not supporting Tx Diversity) | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA |  |  |
| 6.4C.1 | Frequency error for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.4C.2.1 | Error vector magnitude for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.4C.2.2 | Carrier leakage for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.4C.2.3 | In-band emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.4C.2.4 | EVM equalizer spectrum flatness for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.4C.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK for SUL | Rel-16 | C112 | UEs supporting 5GS FR1 and SUL and pi/2-BPSK modulation scheme and low PAPR DMRS | D003 |  | NOTE 2 |  |
| 6.4D.1 | Frequency error for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.1_1 | Frequency error for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.2.1 | Error vector magnitude for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.2.1_1 | Error Vector Magnitude for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.2.2 | Carrier leakage for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.2.2_1 | Carrier leakage for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.2.3 | In-band emissions for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.2.3_1 | In-band emissions for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.2.4 | EVM equalizer spectrum flatness for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.2.4_1 | EVM equalizer spectrum flatness for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.3 | Time alignment error for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.3_1 | Time alignment error for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.4 | Requirements for coherent UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |  |
| 6.4E.1.1 | Frequency error for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.4E.1.1D | Frequency error for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.4E.2.2.1 | Error Vector Magnitude for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.4E.2.2.1D | Error Vector Magnitude for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.4E.2.3.1 | Carrier leakage for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.4E.2.3.1D | Carrier leakage for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.4E.2.4.1 | In-band emissions for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |  |
| 6.4E.2.4.1D | In-band emissions for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  | NOTE 1 |  |
| 6.4E.2.5.1 | EVM equalizer spectrum flatness for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |  |
| 6.4E.2.5.1D | EVM equalizer spectrum flatness for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  | NOTE 1 |  |
| 6.4F.1 | Frequency error | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.4F.2.1 | Error Vector Magnitude | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.4F.2.2 | Carrier leakage | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.4G.1 | Frequency error for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.4G.2.1 | Error vector magnitude for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.4G.2.2 | Carrier leakage for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.4G.2.3 | In-band emissions for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting | D030 | PC1.5 PC2 PC3 |  |  |
| 6.4G.2.4 | EVM equalizer spectrum flatness for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.4H.1.1 | Frequency error for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.4H.1.2.1 | Error Vector Magnitude for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.4H.1.2.2 | Carrier leakage for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.4H.1.2.3 | In-band emissions for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.4H.1.3 | Time alignment error for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.4H.1.4 | Coherent UL MIMO for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 | NOTE 1 |  |
| 6.5.1 | Occupied bandwidth | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.5.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.2 or 6.5B.1.3 has been executed. TC 6.5.1 is skipped if TC 6.5G.1 is executed. |  |
| 6.5.2.2 | Spectrum emission mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 | Skip TC 6.5.2.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.2.1 or 6.5B.2.3.1 has been executed. TC 6.5.2.2 is skipped if TC 6.5G.2.1 is executed. |  |
| 6.5.2.3 | Additional spectrum emission mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 | Skip TC 6.5.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.2 has been executed. TC 6.5.2.3 is skipped if TC 6.5G.2.2 is executed. |  |
| 6.5.2.4.1 | NR ACLR | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 | Skip TC 6.5.2.4.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.3.1 has been executed. TC 6.5.2.4.1 is skipped if TC 6.5G.2.3.1 is executed. |  |
| 6.5.2.4.2 | UTRA ACLR | Rel-15 | C001a | UEs supporting 5GS FR1 PC3 | D001a |  | Skip TC 6.5.2.4.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.3.2 has been executed. TC 6.5.2.4.2 is skipped if TC 6.5G.2.3.2 is executed. |  |
| 6.5.3.1 | General spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.5.3.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.1.1 or 6.5B.3.2.1 or 6.5B.3.3.1(non-exception requirements) has been executed. TC 6.5.3.1 is skipped if TC 6.5G.3.1 is executed. |  |
| 6.5.3.2 | Spurious emissions for UE co-existence | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.5.3.2 if UE supports NSA and TS 38.521-3 TC 6.5B.3.3.2 (non-exception requirements) has been executed. |  |
| 6.5.3.3 | Additional spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.5.3.3 if UE supports NSA and TS 38.521-3 TC 6.5B.4.3 has been executed. TC 6.5.3.3 is skipped if TC 6.5G.3.3 is executed. |  |
| 6.5.4 | Transmit intermodulation | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.5.4 if UE supports NSA and TS 38.521-3 TC 6.5B.5.3 has been executed. TC 6.5.4 is skipped if TC 6.5G.4 is executed. |  |
| 6.5A.1.1 | Occupied bandwidth for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA: PC2 (NOTE 1), PC3 (NOTE 1) |  |  |
| 6.5A.2.2.1 | Spectrum emission mask for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA |  |  |
| 6.5A.2.3.1 | Additional spectrum emission mask for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA: PC2, PC3Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA (NOTE 1) |  |  |
| 6.5A.2.4.1.1 | NR ACLR for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA: PC2 (NOTE 1), PC3 (NOTE 1) |  |  |
| 6.5A.2.4.2.1 | UTRA ACLR for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.5A.3.1.1 | General spurious emissions for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA Intra-band contiguous CA: PC2, PC3 |  |  |
| 6.5A.3.2.1 | Spurious emissions for UE co-existence for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA | NOTE 3 Skip the testing if UE supports NSA and the same UL DC configuration and TS 38.521-3 TC 6.5B.3.3.2 has been executed |  |
| 6.5A.3.3.1 | Additional spurious emissions for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA: PC2, PC3Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA (NOTE 1) |  |  |
| 6.5A.4.1 | Transmit intermodulation for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA |  |  |
| 6.5C.1 | Occupied bandwidth for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.2.2 | Spectrum emission mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.2.3 | Additional spectrum emission mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.2.4.1 | NR ACLR for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.2.4.2 | UTRA ACLR for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.3.1 | General spurious emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.3.2 | Spurious emissions for UE co-existence for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 NOTE 3 Skip the testing if UE supports NSA and the same UL DC configuration and TS 38.521-3 TC 6.5B.3.3.2 has been executed |  |
| 6.5C.3.3 | Additional spurious emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.4 | Transmit intermodulation for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5D.1 | Occupied bandwidth for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.1_2 | Occupied bandwidth for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.2.2 | Spectrum emission mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.2.2_1 | Spectrum emission mask for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
|  |  | Rel-17 | C179b | UEs supporting 5GS FR1 and SUL and ULFPTx |  |  |  |  |
| 6.5D.2.3 | Additional spectrum emission mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.2.4.1 | NR ACLR for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.2.4.1_1 | NR ACLR for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.2.4.2 | UTRA ACLR for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.2.4.2_1 | UTRA ACLR for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.3.1 | General spurious emissions for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.3.2 | Spurious emissions for UE co-existence for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.3.3 | Additional spurious emissions for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.3_1.1 | General spurious emissions for UL MIMO (Rel-16 onward) | Rel-16 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.3_1.2 | Spurious emission for UE co-existence for UL MIMO (Rel-16 onward) | Rel-16 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.3_1.3 | Additional spurious emissions for UL MIMO (Rel-16 onward) | Rel-16 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.3_2.1 | General spurious emissions for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.3_2.2 | Spurious emissions for UE co-existence for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.3_2.3 | Additional spurious emissions for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.4 | Transmit intermodulation for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.4_1 | Transmit intermodulation for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5E.1.1 | Occupied bandwidth for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.1.1D | Occupied bandwidth for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.2.2.1 | Spectrum emission mask for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.2.2.1D | Spectrum emission mask for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.2.2.2 | Spectrum emission mask for V2X / concurrent operation | FFS | FFS | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR sidelink and Simultaneous transmission of uplink and sidelink | FFS | Inter-band concurrent operation: PC3 |  |  |
| 6.5E.2.3.1 | Additional Spectrum emission mask for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.2.3.1D | Additional Spectrum emission mask for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.2.4.1 | Adjacent channel leakage ratio for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.2.4.1D | Adjacent channel leakage ratio for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  | NOTE 1 |  |
| 6.5E.2.4.2 | Adjacent channel leakage for V2X / concurrent operation | FFS | FFS | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR sidelink and Simultaneous transmission of uplink and sidelink | FFS | Inter-band concurrent operation: PC3 |  |  |
| 6.5E.3.1.1 | General spurious emissions for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.3.1.1D | General spurious emissions for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.3.2.1 | Spurious emissions for UE co-existence for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |  |
| 6.5E.3.2.1D | Spurious emissions for UE co-existence for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.3.2.2 | Spurious emissions for UE co-existence for V2X / concurrent operation | FFS | FFS | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR sidelink and Simultaneous transmission of uplink and sidelink | FFS |  |  |  |
| 6.5E.3.3.1 | Additional spurious emissions requirements for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.4.1 | Transmit intermodulation for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.4.1D | Transmit intermodulation for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5F.1 | Occupied bandwidth for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.5F.2.2 | Spectrum emission mask for operation with shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.5F.2.4 | Adjacent channel leakage ratio for operation with shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.5F.2.4.2 | Shared spectrum channel access ACLR with additional requirement for NS_29 | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.5F.3.1 | General spurious emissions | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.5F.3.3 | Additional spurious emissions for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.5F.4 | Transmit intermodulation for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.5G.1 | Occupied bandwidth for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.2.1 | Spectrum emission mask for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.2.2 | Additional spectrum emission mask for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.2.3.1 | NR ACLR for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.2.3.2 | UTRA ACLR for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC3 |  |  |
| 6.5G.3.1 | General spurious emissions for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.3.2 | Spurious emissions for UE co-existence for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.3.3 | Additional spurious emissions for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.4 | Transmit intermodulation for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5H.1.1 | Occupied bandwidth for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.2.1 | Spectrum emission mask for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.2.2 | Additional spectrum emission mask for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.2.3 | NR ACLR for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.3.1 | General spurious emissions for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.3.2 | Spurious emissions for UE co-existence for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.3.3 | Additional spurious emissions for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.4 | Transmit intermodulation for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.3.3.1 | General spurious emissions for inter-band UL CA with UL MIMO | Rel-17 | C328 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with UL MIMO. | E045 | PC1.5 PC2 PC3 |  |  |
| 6.5H.3.3.2 | Spurious emissions for UE co-existence for inter-band UL CA with UL MIMO | Rel-17 | C328 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with UL MIMO. | E045 | PC1.5 PC2 PC3 |  |  |
| 6.5J.1 | Occupied bandwidth for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.5J.3.1 | General spurious emissions for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.5L.3.3.1 | General spurious emissions for inter-band UL CA with Tx Diversity | Rel-17 | C329 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with Tx Diversity. | E046 | PC1.5 PC2 PC3 |  |  |
| 6.5L.3.3.2 | Spurious emissions for UE co-existence for inter-band UL CA with Tx Diversity | Rel-17 | C329 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with Tx Diversity. | E046 | PC1.5 PC2 PC3 |  |  |
| 7 | Receiver Characteristics |  |  |  |  |  |  |  |
| 7.3.2 | Reference sensitivity power level | Rel-15 | C001l | UEs supporting 5GS FR1 and not supporting RedCap | D001 | 2Rx 4Rx 8Rx PC1.5 PC2 PC3 |  |  |
| 7.3A.1 | Reference sensitivity power level for 2DL CA without exception | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 | 2Rx 4Rx PC2 PC3 |  |  |
| 7.3A.1_1 | Reference sensitivity power level for 2DL CA exceptions | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 E015a E015 | 2Rx 4Rx PC2 PC3 |  |  |
| 7.3A.2 | Reference sensitivity power level for 3DL CA without exceptions | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 | 2Rx 4Rx PC2 PC3 |  |  |
| 7.3A.2_1 | Reference sensitivity power level for 3DL CA exceptions | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 E015 | 2Rx 4Rx PC2 PC3 |  |  |
| 7.3A.3 | Reference sensitivity power level for 4DL CA | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 | 2Rx 4Rx PC2 PC3 |  |  |
| 7.3C.2 | Reference sensitivity power level for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |  |
| 7.3D.2 | Reference sensitivity power level for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.3D.2_1 | Reference sensitivity power level for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.3E.2 | Reference sensitivity for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |  |
| 7.3F.2 | Reference sensitivity for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.3I.2 | Reference sensitivity power level for RedCap | Rel-17 | C177 | RedCap UEs supporting 5GS FR1 | D001 |  |  |  |
| 7.3I.3 | Reference sensitivity power level for eRedCap | Rel-18 | C372 | eRedCap UEs supporting 5GS FR1 | D001 |  |  |  |
| 7.3J.2 | Reference sensitivity power level for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.4 | Maximum input level | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.4 if UE supports NSA and TS 38.521-3 TC 7.4B.2 or 7.4B.3 has been executed. |  |
| 7.4A.1 | Maximum input level for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.4A.2 | Maximum input level for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.4A.3 | Maximum input level for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |  |
| 7.4A.4 | Maximum input level for CA (5DL CA) | Rel-16 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | Skip TC 7.4A.4 if UE supports NSA and TS 38.521-3 TC 7.4B.3_1.4 has been executed. |  |
| 7.4D | Maximum input level for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.4D_1 | Maximum input level for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.4F | Maximum input level for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 7.4J | Maximum input level for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.5 | Adjacent channel selectivity | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | NOTE 1 Skip TC 7.5 if UE supports NSA and TS 38.521-3 TC 7.5B.2 or 7.5B.3 has been executed. |  |
| 7.5A.1 | Adjacent channel selectivity for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.5A.2 | Adjacent channel selectivity for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.5A.3 | Adjacent channel selectivity for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |  |
| 7.5A.4 | Adjacent channel selectivity for CA (5DL CA) | Rel-16 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | Skip TC 7.5A.4 if UE supports NSA and TS 38.521-3 TC [FFS] has been executed. |
| 7.5D | Adjacent channel selectivity for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.5D_1 | Adjacent channel selectivity for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.5F.1 | Adjacent channel selectivity for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.5J | Adjacent channel selectivity for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.6.2 | In-band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.6.2 if UE supports NSA and TS 38.521-3 TC 7.6B.2.2 or 7.6B.2.3 has been executed. |  |
| 7.6.3 | Out-of-band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |  |
| 7.6.4 | Narrow band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D004 |  | Skip TC 7.6.4 if UE supports NSA and TS 38.521-3 TC 7.6B.4.2 or 7.6B.4.3 has been executed. |  |
| 7.6A.2.1 | In-band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.6A.2.2 | In-band blocking for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.6A.2.3 | In-band blocking for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  | Skip TC 7.6A.2.3 if UE supports NSA and TS 38.521-3 TC 7.6B.2.3_1.3 has been executed. |  |
| 7.6A.3.1 | Out-of-band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.6A.3.2 | Out-of-band blocking for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.6A.3.3 | Out-of-band blocking for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |  |
| 7.6A.3.4 | Out-of-band blocking for CA (5DL CA) | Rel-15 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | NOTE 1 |  |
| 7.6A.4.1 | Narrow band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.6A.4.2 | Narrow band blocking for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.6A.4.3 | Narrow band blocking for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  | Skip TC 7.6A.4.3 if UE supports NSA and TS 38.521-3 TC 7.6B.4.3_1.3 has been executed. |  |
| 7.6A.4.4 | Narrow band blocking for CA (5DL CA) | Rel-15 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | NOTE 1 |  |
| 7.6C.2 | In-band blocking for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |  |
| 7.6C.2_1 | Inband Blocking for SUL with DL CA | Rel-17 | C078 | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) | E031 |  |  |  |
| 7.6C.3 | Out-of-band blocking for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |  |
| 7.6C.3_1 | Out-of-band blocking for SUL with DL CA | Rel-17 | C078 | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) | E031 |  |  |  |
| 7.6D.2 | In-band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.6D.2_1 | In-band blocking for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.6D.3 | Out-of-band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.6D.3_1 | Out-of-band blocking for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.6D.4 | Narrow band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.6D.4_1 | Void |  |  |  |  |  |  |  |
| 7.6E.2.1 | In-band blocking for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 7.6E.3.1 | Out-of-band blocking for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 7.6F.2.1 | In-band blocking for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.6F.3.1 | Out-of-band blocking for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.6J.2 | In-band blocking for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.6J.3 | Out-of-band blocking for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.7 | Spurious response | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |  |
| 7.7A.1 | Spurious response for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.7A.2 | Spurious response for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.7A.3 | Spurious response for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |  |
| 7.7A.4 | Spurious response for CA (5DL CA) | Rel-15 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | NOTE 1 |  |
| 7.7C | Spurious response for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |  |
| 7.7D | Spurious response for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.7D_1 | Spurious response for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.7E.1 | Spurious response for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 7.7F.1 | Spurious response for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.7J | Spurious response for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.8.2 | Wide band Intermodulation | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.8.2 if UE supports NSA and TS 38.521-3 TC 7.8B.2.2 or 7.8B.2.3 has been executed. |  |
| 7.8A.2.1 | Wide band Intermodulation for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.8A.2.2 | Wide band Intermodulation for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.8A.2.3 | Wide band Intermodulation for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |  |
| 7.8D.2 | Wide band Intermodulation for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.8D.2_1 | Wide band Intermodulation for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.8E.2.1 | Wide band Intermodulation for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 7.8F.2 | Wide band Intermodulation for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.8J.2 | Wide band intermodulation for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.9 | Spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.9 if UE supports NSA and TS 38.521-3 TC 7.9B.1 or 7.9B.2 or 7.9B.3 has been executed. |  |
| 7.9A.1 | Spurious emissions for CA (2DL CA) | Rel-15 | C005 | UEs supporting 5GS FR1 and inter-band 2DL CA with a DL-only band | E002 |  |  |  |
| 7.9J | Spurious emissions for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-1. NOTE 2: The test case is optional for Rel-17 RedCap UE implementing SUL. NOTE 3: Uplink configurations CA_nA-nB, DC_A_nB, DC_B_nA, SUL_A-nD and SUL_B-nC can be noted as same UL CA configuration, where band nC and nD are the corresponding SUL bands with same UL frequency range of band (n)A and (n)B |  |  |  |  |  |  |  |  |

Table 4.1.1-1a: Void

Table 4.1.1-1b: Void

Table 4.1.1-1c: Void

==============================================================

<< End of changes >>