3GPP TSG-RAN WG5 Meeting # 106	R5-251077

# Athens, Greece, February 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1911 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on Ch7 for EN-DC refsens test requirement handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the discussion in R5-251074, it is noted that for the Refsens test requirements for EN-DC, both UL configuration and DL channel bandwidths should be considered in Table 7.3B.2.3.5-0. Furthermore, since there is no information of “Interference order” such as “ULx/DLy”, “IMDx” and “ACLRx”, etc both in the table of test setting and the table of test requirements, to make it clearer to the test users it is suggested to add a column of “Interference order” in Table 7.3B.2.3.5-0 for information. To avoid confusion of using TT in EN-DC Refsens test, the following rules are suggested to be applied. - For Refsens on E-UTRA bands, the format of “xx” is to be used. - For Refsens on NR bands, the format of “xx+TT” is to be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the template for EN-DC Refsens test requirements in Table 7.3B.2.3.5-0. - Correct the Refsens test requirements for the following PC3 EN-DC configurations with two bands in Table 7.3B.2.3.5-4. - DC_3A_n8A - DC_8A_n20A - DC_20_n8 - DC_28A_n77A, DC_28A_n78A - Correct the Refsens test requirements for the following PC3 EN-DC configurations with three bands in Table 7.3B.2.3_1.1.5-1. - DC_1A-3A_n28A - DC_1A-7A_n28A - DC_1A-3A_n78A, DC_1A-3C_n78A - DC_1A_n3A-n78A - DC_1A-28A_n3A - DC_1A_n28A-n78A - DC_2A_n5A-n77A - DC_3A-20A_n28A - DC_3A_n28A-n78A - DC_3A-41A_n28A - DC_7A_n28A-n78A - DC_13A_n2A-n77A - DC_20A_n28A-n78A - DC_28A_n7A-n78A - DC_66A_n2A-n77A - DC_66A_n5A-n77A - Other editorial corrections in Refsens test requirement tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The REFSENS tests for some EN-DC configurations will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.5, 7.3B.2.3_1.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-0, Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-0: Reference sensitivity test requirement for PC3 EN-DC

| EN-DC configuration | Test ID | E-UTRA/NR band | UL/DL CBW (MHz) | REFSENS test requirement (dBm) | Exception type | Interference order |
| --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n41A | 1 | 39 | 20 | REFSENS | Cross band isolation | >ACLR2 |
|  |  | n41 | 10 | -91.5+TT |  |  |
|  | 2 | 39 | 5 | -97.7 | Cross band isolation | >ACLR2 |
|  |  | n41 | 100 | REFSENS |  |  |
| DC_40A_n41A | 1 | 40 | 5 | -67.9 | Cross band isolation | ACLR2 |
|  |  | n41 | 100 | REFSENS |  |  |
| DC_26A_n41A | 1 | 26 | 5 | REFSENS | UL harmonic | UL3/DL1 |
|  |  | n41 | 100 | -81.8+TT | interference | direct-hit |
|  | 1a | 26 | 5 | REFSENS |  |  |
|  |  | n41 | 10 | -81.1+TT |  |  |
|  | 2 | 26 | 5 | REFSENS | UL harmonic exception | N/A |
|  |  | n41 | 10 | REFSENS | avoided |  |
|  | 3 | 26 | 5 | -81.2 | dual uplink operation | IMD3 |
|  |  | n41 | 10 | REFSENS |  |  |
|  | 4 | 26 | 5 | -72.5 | harmonic mixing | UL1/DL3 |
|  |  | n41 | 10 | REFSENS |  |  |
|  | 5 | 26 | 15 | -69.5 |  |  |
|  |  | n41 | 15 | REFSENS |  |  |
|  | 6 | 26 | 15 | REFSENS | harmonic mixing | N/A |
|  |  | n41 | 15 | REFSENS | avoiding |  |
| NOTE 1: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1a and Table 7.3.2.5-1b of TS 38.521-1 [8] for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b of TS 38.521-1 [8] for 4 antenna port NR band. |  |  |  |  |  |  |

<< next change >>

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n8A | 1 | N/A | -93.3 | - | - | - | - | IMD4 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -91.3 | - | - | - | - | IMD4- | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | -91.3 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n66A | 2 | N/A | -77.3 | - | - | - | - | IMD3 |  |
|  | n66 | 15 | REFSENS | - | - | - | - | N/A |  |
| DC_2A_n66A | 2 | N/A | REFSENS | - | - | - | - | N/A |  |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 |  |
| DC_2A_n77A. DC_2A_n77(2A) | 2 | N/A | -71.3 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A DC_2A_n77(2A) DC_2A-2A_n77(2A) | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -89.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n78A | 2 | N/A | -71.3 | - | - | - | - | IMD23 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_2A_n78A | 2 | N/A | -89.3 | - | - | - | - | N/A | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | IMD43 | TDD |
| DC_3A_n1A | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -74.0+TT | - | - | - | - | IMD3 | FDD |
| DC_3A_n5A | 3 | N/A |  | -89.3 | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -74.0+TT | - | - | - | - | IMD23 | FDD |
| DC_3A_n7A | 3 | N/A | REFSENS | - | - | - | - | - |  |
|  | n7 | 15 | - | -84.6 +TT5 | - | - | - | - |  |
| DC_3A_n8A | 3 | N/A | - | REFSENS | - | - | - | N/A | FDD |
|  | n8 | 15 | -89.0+TT | - | - | - | - | IMD43 | FDD |
|  | 3 | N/A | - | -86.9 | - | - | - | IMD5 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_3A_n41A | 3 | N/A | -88.1 | - | - | - | - | IMD4 | FDD |
|  | n41 |  | REFSENS | - | - | - | - |  |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -70.3 | - | - | - | - | IMD23 | FDD |
|  |  |  | -70.67 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n66A | 5 | N/A | -67.3 | - | - | - | - | IMD23 | FDD |
|  | n66 |  | REFSENS | - | - | - | - | N/A | FDD |
| DC_5A_n77A8e | 5 | N/A | -89 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 5 | N/A | -91.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n78A | 5 | N/A | -89.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_7A_n3A | 7 | N/A | - | -81.3 | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_7A_n5A | 7 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -86.0+TT | - | - | - | - | IMD33 | FDD |
| DC_7A_n66A | 7 | N/A | - | -79.3 | - | - | - | IMD4 | FDD |
|  | n66 | 15 | REFSENS | - |  | - | - | N/A | TDD |
| DC_7A_n77A | 7 | N/A | -90.2 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n1A | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -94.0 + TT | - | - | -- | - | IMD4 | FDD |
| DC_8A_n3A | 8 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n3 | 15 | - | REFSENS | - | - | - | N/A | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | - | -87.4+ TT | - | - | - | IMD5 | FDD |
| DC_8A_n20A | n20 | 15 | -72.0+TT | - | - | - | - | IMD3 | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 8 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_8A_n41A | 8 | N/A | -84.2 | - | - | - | - | IMD33 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | N/A | -88.0 | - | - | - | - | IMD4 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_12A_n77A DC_12A_n77(2A) | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_12_n78 | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_14A_n77A DC_14A_n77(2A) | 14 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_18A_n77A | 18 | N/A | -90.9 | - | - | - | - | IMD49 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 18 | N/A | -94.8 | - | - | - | - | IMD59 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_18A_n78A | 18 | N/A | -90.9 | - | - | - | - | IMD410 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_19A_n78A | 19 | N/A | -85.7 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_20A_n3A | 20 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | -93.0 +TT | - | - | - | - | IMD4 | FDD |
|  | 20 | N/A | -87.3 | - | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20_n7 | 20 | N/A | -84.3 | - | - | - | - | IMD3 | FDD |
|  | n7 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_20_n8 | 20 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | N/A | -74.6 | - | - | - | - | IMD44 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n28A | 21 | N/A | -96.8 | - | - | - | - | IMD5 | FDD |
|  | n28 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_21A_n79A | 21 | N/A | -80.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_28A_n77A, DC_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_30A_n77A DC_30A_n77(2A) | 30 | N/A | -90.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_48A_n66A | 48 | N/A | - | REFSENS | - | - | - | N/A | TDD |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 | FDD |
| DC_66A_n2A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n2 | 15 | -78.0+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n2 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n5A | 66 | N/A | -68.8 | - | - | - | - | IMD23 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n25A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n25 | 15 | -76.5+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -75.8 | - | - | - | - | IMD3 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n71A | 66 | N/A | -93.8 | - | - | - | - | IMD4 | FDD |
|  | n71 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) | 66 | N/A | -67.8 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n78A | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_71A_n66A | 71 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n66 | 15 | -94.5 +TT | - | - | - | - | IMD4 | FDD |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: Void NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Void. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 - 4100 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 10: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 - 3800 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-5: Reference sensitivity exceptions due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm)) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order) | Duplex mode |
| DC_1A_n77A | 1 | N/A | -63.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 1 | N/A | -81.5 | - | - | - | - | IMD43 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | REFSENS-17.8 | - | - | - | - | IMD4 |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n77A | 2 | N/A | -65.2 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A |  |  | -65.157 |  |  |  |  |  |  |
| DC_2A_n77C | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A-2A_n77C | 2 | N/A | -78.2 | - | - | - | - | IMD49 | FDD |
|  |  |  | -78.157 |  |  |  |  |  |  |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n41A | 3 | N/A | -77.9 | - | - | - | - | IMD4 | FDD |
|  |  |  | -80.67 | - | - | - | - |  |  |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -64.4 | - | - | - | - | IMD2 | FDD |
|  |  |  | -67.17 | - | - | - | - |  |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -77.8 | - | - | - | - | IMD4 | FDD |
|  |  |  | -80.57 | - | - | - | - |  |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n77A8 | 5 | N/A | -78.7 | - | - | - | - | IMD49 | FDD |
| DC_5A_n77C8 | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_12A_n77A | 12 | N/A | -84.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -80.93 | - | - | - | - | IMD5 | FDD |
| DC_13A_n77C | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_14A_n77A | 14 | N/A | -84.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_19A_n78A | 19 | N/A | -74.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_21A_n79A | 21 | N/A | -65.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | REFSENS |  | - | - | N/A | TDD |
| DC_28A_n77A | 28 | N/A | -78.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_30A_n77A | 30 | N/A | -81.4 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C | 66 | N/A | -64.47 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -87.53 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

7.3B.2.3_1	Reference sensitivity for EN-DC within FR1 (>2 CCs)

7.3B.2.3_1.1	Reference sensitivity for EN-DC within FR1 (3 CCs)

Editor's note:	The test requirements section of this test case contains some EN-DC configurations that have no test points defined meaning these cannot be tested.

Editor's note:	For EN-DC configurations with >1NR CC, test points in Table 7.3B.2.3_1.1.4.1-0 only need to be tested if the configuration is a max NR CC of the UE, where detailed execution rules are FFS.

7.3B.2.3_1.1.1	Test purpose

To verify the ability of UE that support EN-DC configurations to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when no CA exceptions are allowed and single carrier requirements apply whenever possible. A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3B.2.3_1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 3CC EN-DC.

7.3B.2.3_1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

<< next change >>

7.3B.2.3_1.1.5	Test requirement

Reference sensitivity test requirements for EN-DC configurations affected by 3 band 2UL intermodulation interference, are specified in Table 7.3B.2.3_1.1.5-1 and Table 7.3B.2.3_1.1.5-2 with uplink configuration specified in Table 7.3B.2.3_1.1.4.1-1.

Reference sensitivity test requirements for test points in Table 7.3B.2.3_1.1.4.1-1, are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.3_1.1.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for PC3 EN-DC in NR FR1 (three bands)

| EN-DC Configuration | Test ID | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -92.3 | - | - | - |  | IMD5 |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  | 2 | 1 | N/A | -88.3 | - | - | - |  | IMD4 |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
| DC_1A-7A_n28A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 7 | N/A | - | -64.3 | - | - |  | IMD2 |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
| DC_1A-3A_n77A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -64.8 | - | - | - |  | IMD2 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -87.8 | - | - | - |  | IMD4 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | -68.3 | - | - | - | FDD | IMD2 |  |
|  |  | 3 | N/A | REFSENS | - | - | - |  | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-3A_n78A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-3C_n78A |  | 3 | N/A | -65.1 | - | - | - |  | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -96.5 | - | - | - | FDD | IMD5 |  |
|  |  | 3 | N/A | REFSENS | - | - | - |  | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A_n3A-n78A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n3 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | n78 | 15 | - | -67.4+TT | - | - | TDD | IMD2 |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n3 | 15 | -69.1+TT | - | - | - | FDD | IMD2 |  |
| DC_1A-3A_n79A | 1 | 1 | N/A | -95.7 | - | - | - | FDD | IMD5 |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_1A-5A_n78A | 1 | 1 | N/A | -91.0 | - | - | - | FDD | IMD4 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -89.0 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | -81.2 | - | - | - | FDD | IMD3 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -94.2 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-7A_n78A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 7 | N/A | -88.2 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -90.6 | - | - | - | FDD | IMD4 |  |
|  |  | 7 | N/A | - | REFSENS | - |  | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-18A_n77A | 1 | 1 | N/A | -82.9 | - | - | - | FDD | IMD3 |  |
|  |  | 18 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-19A_n78A | 1 | 1 | N/A | -81.5 | - | - | - | FDD | IMD3 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -94.2 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-19A_n79A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -81.0 | - | - | - | FDD | IMD3 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  | 2 | 1 | N/A | -91.2 | - | - | - | FDD | IMD4 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_1A-20A_n8A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -87.8 | - | - | - | FDD | IMD4 |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-20A_n78A | 1 | 1 | N/A | -79.0 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-20A_n78A | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -93.3 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-21A_n78A | 1 | 1 | N/A | -68.7 | - | - | - | FDD | IMD2 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -95.7 | - | - | - | FDD | IMD5 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -67.8 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -96.4 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-28A_n3A | 1 | 1 | N/A | -88.3 |  | - | - | FDD | IMD4 |  |
|  |  | n3 | 15 | REFSENS |  | - | - | FDD | N/A |  |
|  |  | 28 | N/A | REFSENS |  | - | - | FDD | N/A |  |
| DC_1A_n28A-n78A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -80.1+TT | - | - | TDD | IMD3 |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n28 | 15 | -94.3+TT | - | - | - | FDD | IMD5 |  |
| DC_1A-28A_n78A | 1 | 1 | N/A | -83.6 | - | - | - | FDD | IMD3 |  |
|  |  | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -93.6 | - | - |  | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-41A_n28A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | - | -64.3 | - | - | TDD | IMD2 |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-41A_n77A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -86.3 | - | - | - | TDD | IMD4 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -90.0 | - | - | - | FDD | IMD4 |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -93.7 | - | - | - | TDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-42A_n79A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  |  | 42 | N/A | -93.2 | - | - | - | TDD | IMD5 |  |
| DC_1A_n78A-n79A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n79 | 15 | - | - | - | -73.7+TT | TDD | IMD3 |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -91.2+TT | - | - | TDD | IMD5 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_2A_5A-n77A | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -93.5 | - | - | - | FDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 2 | N/A | -80.8 | - | - | - | FDD | IMD3 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_2A_n5A-n77A |  | n5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | -79.8+TT | - | - | TDD | IMD3 |  |
|  | 1 | 2 | N/A | -81.3 | - | - | - | FDD | IMD3 |  |
| DC_2A-13A_n77A |  | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_2A-14A_n66A | 1 | 2 | N/A | -90.1 | - | - | - | FDD | IMD4 |  |
|  |  | 14 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n66 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_2A-14A_n77A | 1 | 2 | N/A | -80,8 | - | - | - | FDD | IMD3 |  |
|  |  | 14 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_2A-66A_n2A | 1 | 2 | N/A | -77.3 |  |  |  | FDD | IMD3 |  |
|  |  | 66 | N/A | REFSENS |  |  |  | FDD | N/A |  |
|  |  | n2 | 15 | REFSENS |  |  |  | FDD | N/A |  |
| DC_2A-66A_n5A | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -91.6 | - | - | - | FDD | IMD4 |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_2A-66A_n41A | 1 | 2 | N/A | -74.4 | - | - | - | FDD | IMD4 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n41 | 15 | REFSENS | - | - | - | TDD | N/A |  |
|  | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -69.6 | - | - | - | FDD | IMD2 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -88.4 | - | - | - | FDD | IMD4 |  |
| DC_2A-66A_n77A |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -94.8 | - | - | - | FDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 2 | N/A | -65.2 | - | - | - | FDD | IMD2 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-5A_n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -89.0 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -70.3 | - | - | - | FDD | IMD2 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 3 | N/A | -88.3 | - | - | - | FDD | IMD4 |  |
|  |  | 5 |  | [TBD] |  |  | 10.75 |  |  |  |
| DC_3A-7A_n5A | 1 | 3 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | 7 | N/A | - | - 64.3 | - | - | FDD | IMD2XX |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_3A-7A_n28A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 7 | N/A | - | -77.4 | - | - |  | IMD3 |  |
|  | 2 | 7 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 3 | N/A | -70.3 | - | - | - |  | IMD2 |  |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A | 1 | 3 | N/A | -78.7 | - | - | - | FDD | IMD3 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -87.7 | - | - | - | FDD | IMD4 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-8A_n78A | 1 | 3 | N/A | -79.8 | - | - | - | FDD | IMD3 |  |
|  |  | 8 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-19A_n78A | 1 | 3 | N/A | -82.0 | - | - | - | FDD | IMD3 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-19A_n79A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -80.8 | - | - | - | FDD | IMD3 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  | 2 | 3 | N/A | -90.8 | - | - | - | FDD | IMD4 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_3A-20A_n28A | 1 | 3 | N/A | -86.9 | - | - | - | FDD | IMD4 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_3A-20A_n78A | 1 | 3 | N/A | -79.0 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-21A_n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -90.5 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -65.5 | - | - | - | FDD | IMD2 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-21A_n79A | 1 | 3 | N/A | -78.5 | - | - | - | FDD | IMD3 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_3A_n28A-n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -91.3+TT | - | - | TDD | IMD5 |  |
| DC_3A-28A_n78A | 1 | 3 | N/A | -79 | - | - | - | FDD | IMD3 |  |
|  |  | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-40A_n1A | 1 | n1 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 40 | N/A | -91.3 | - | - | - | TDD | IMD5 |  |
| DC_3A-41A_n28A | 1 | 3 | N/A | -70.3 | - | - | - | FDD | IMD2 |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
|  |  | n28 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -69.9 | - | - | - | TDD | IMD2 |  |
|  |  | n28 | 15 | REFSENS | - | - | - | TDD | N/A |  |
|  | 3 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -81.4 | - | - | - | TDD | IMD3 |  |
|  |  | n28 | 15 | REFSENS | - | - | - | TDD | N/A |  |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 41 | N/A | -92 | - | - | - | TDD | IMD5 |  |
|  | 2 | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 3 | N/A | -79.9 | - | - | - | FDD | IMD3 |  |
| DC_3A_n78A-n79A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n79 | 15 | - | - | - | -73.3+TT | TDD | IMD3 |  |
|  | 2 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -91.6+TT | - | - | TDD | IMD5 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_5A-7A_n78A | 1 | 5 | N/A | -89.0 | - | - | - | FDD | IMD4 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 7 | N/A | -67.2 | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 5 | N/A | -67.1 | - | - | - | FDD | IMD2 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 5 | N/A | -94.0 | - | - | - | FDD | IMD5 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_5A-66A_n2A | 1 | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -91.6 | - | - | - | FDD | IMD4 |  |
|  |  | n2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_5A-66A_n77A | 1 | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -85.6 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_7A-8A_n3A | 1 | 7 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | 8 | N/A | -78.3 | - | - | - | FDD | IMD3 |  |
|  |  | n3 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | - | -65.3 | - | - | FDD | IMD2+ IMD3YY |  |
|  |  | 8 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n3 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-20A_n1A | 1 | 7 |  |  | REFSENS |  |  | FDD | N/A |  |
|  |  | 20 |  |  | -88.8 |  |  | FDD | IMD5 |  |
|  |  | n1 |  | REFSENS |  |  |  | FDD | N/A |  |
| DC_7A-20A_n3A | 1 | 7 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | 20 | N/A | - | -82.8 | - | - | FDD | IMD2 |  |
|  |  | n3 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | - | -68.3 | - | - | FDD | IMD2 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n3 | 15 | - | REFSENS | - | - | FDD | N/A |  |
| DC_7A-20A_n8A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -78.9 | - | - | - | FDD | IMD3 |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | -76.2 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 3 | 7 | N/A | -78.5 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-20A_n28A | 1 | 7 | N/A | - | -91.4 | - | - | FDD | IMD5 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-20A_n78A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -65.8 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -93.3 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 7 | N/A | -66.5 | - | - | - | FDD | IMD2 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_7A-28A_n3A | 1 | 7 | N/A | REFSENS |  |  |  | FDD | N/A |  |
|  |  | 28 | N/A | -77.8 |  |  |  | FDD | IMD2 |  |
|  |  | n3 | 15 | REFSENS |  |  |  | FDD | N/A |  |
|  | 2 | 7 | N/A | -79.3 |  |  |  | FDD | IMD3 |  |
|  |  | 28 | N/A | REFSENS |  |  |  | FDD | N/A |  |
|  |  | n3 | 15 | REFSENS |  |  |  | FDD | N/A |  |
| DC_7A-28A_n5A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -93.4 | - | - | - | FDD | IMD5 |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | -91.9 | - | - | - | FDD | IMD5 |  |
|  |  | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-28A_n78A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -89.5 | - | - | - |  | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -94.8 | - | - | - |  | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 7 | N/A | -66.8 | - | - | - | FDD | IMD2 |  |
|  |  | 28 | N/A | REFSENS | - | - | - |  | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_7A_n28A-n78A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -66.1+TT | - | - | TDD | IMD2 |  |
|  | 2 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | -69.7+TT | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n2 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A_n2A-n77A |  | n77 | 15 | - | -78+TT | - | - | TDD | IMD3 |  |
|  | 2 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n2 | 15 | -82+TT | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A-66A_n2A |  | 66 | N/A | -91.3 | - | - | - | FDD | IMD4 |  |
|  |  | n2 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A-66A_n77A |  | 66 | N/A | -81.4 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | FDD | N/A |  |
| DC_14A-66A_n77A | 1 | 13 | N/A | -81.1 | - | - | - | FDD | IMD3 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -85.6 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | REFSENS | REFSENS | - | - | TDD | N/A |  |
| DC_18A-41A_n77A DC_18A-41C_n77A | 1 | 18 | N/A | -95.9 | - | - | - | FDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 1 | 18 | N/A | -95.9 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
| DC_19A-21A_n78A | 1 | 19 | N/A | -80.6 | - | - | - | FDD | IMD3 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 19 | N/A | -86.1 | - | - | - | FDD | IMD4 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_19A-21A_n79A | 1 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -95.5 | - | - | - | FDD | IMD5 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_19A_n78A-n79A | 1 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n79 | 15 | - | - | - | -60.3+TT | TDD | IMD2 |  |
|  | 2 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -67.0+TT | - | - | TDD | IMD2 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_20A_n28A-n78A | 1 | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -87.1+TT | - | - | TDD | IMD4 |  |
|  | 2 | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n28 | 15 | -89.1+TT | - | - | - | FDD | IMD4 |  |
| DC_21A_n78A-n79A | 1 | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n79 | 15 | - | - | - | -59.5+TT | TDD | IMD2 |  |
|  | 2 | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -66.0+TT | - | - | TDD | IMD2 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_28A_n7A-n78A | 1 | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n7 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -66.1+TT | - | - | TDD | IMD2 |  |
|  | 2 | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n7 | 15 | -67.5+TT | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_14A-66A_n2A | 1 | 14 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -91.2 | - | - | - | FDD | IMD4 |  |
|  |  | n2 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 1 | n2 | 15 | -65.9+TT | - | - | - | FDD | IMD2 |  |
| DC_66A_n2A-n77A |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_66A_n5A-n77A |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | -78.7+TT | - | - | TDD | IMD39 |  |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: Void NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 6: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the Band 46. The reference sensitivity should only be verified when this is not the case (the requirements for Band 46 specified in the CA_7A-46A in clause 7.3.1 of TS 36.101 [5] apply). NOTE 7: This band is subject to IMD3 also which MSD is not specified. NOTE 8: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.5-1a: Reference sensitivity exceptions for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| EN-DC Configuration | Test ID | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -58.8 | - | - | - | FDD | IMD23 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-3A_n77A |  | 3 | N/A | -75.8 | - | - | - | FDD | IMD43 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | -62.3 | - | - | - | FDD | IMD23 |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -59.1 | - | - | - | FDD | IMD2 |  |
| DC_1A-3A_n78A |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -81.5 | - | - | - | FDD | IMD5 |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | -74.7 | - | - | - | FDD | IMD5 |  |
| DC_1A-3A_n79A |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 18 | N/A | -83.5 | - | - | - | FDD | IMD5 |  |
| DC_1A-18A_n77A |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -74.3 | - | - | - | FDD | IMD3 |  |
|  |  | 18 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | -72.6 | - | - | - | FDD | IMD3 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-19A_n78A |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -80.8 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -66.0 | - | - | - | FDD | IMD3 |  |
| DC_1A-19A_n79A |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -73.2 | - | - | - | FDD | IMD4 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | -62.7 | - | - | - | FDD | IMD2 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -83.1 | - | - | - | FDD | IMD5 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-21A_n78A |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -61.8 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -84.4 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -74.8 | - | - | - | FDD | IMD4 |  |
| DC_1A-41A_n77A |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -81.7 | - | - | - | FDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A_n78A-n79A |  | n79 | 15 | - | +TT | - | - | TDD | IMD3 |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | +TT | - | - | TDD | IMD5 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-42A_n79A |  | 42 | N/A | -72.2 | - | - | - | TDD | IMD5 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_2A-5A_n77A | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -83.7 | - | - | - | FDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 2 | N/A | -72.5 | - | - | - | FDD | IMD3 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 2 | N/A | -98.0 +TT+24.2 | - | - | - | FDD | IMD3 |  |
| DC_2A-13A_n77A |  | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_2A-14A_n77A | 1 | 2 | N/A | -82.5 | - | - | - | FDD | IMD3 |  |
|  |  | 14 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -99.5 +TT+34.7 | - | - | - | FDD | IMD2 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -99.5 +TT+21.1 | - | - | - | FDD | IMD43 |  |
| DC_2A-66A_n77A |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 2 | N/A | -98.0 +TT+37.6 | - | - | - | FDD | IMD2 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 3 | N/A | -72.1 | - | - | - | FDD | IMD3 |  |
| DC_3A-18A_n77A |  | 18 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 3 | N/A | -70.0 | - | - | - | FDD | IMD3 |  |
| DC_3A-19A_n78A |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -71.8 | - | - | - | FDD | IMD3 |  |
| DC_3A-19A_n79A |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  | 2 | 3 | N/A | -80.1 | - | - | - | FDD | IMD4 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_3A-21A_n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -76.1 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -59.7 | - | - | - | FDD | IMD2 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -89.8 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 3 | N/A | -66.5 | - | - | - | FDD | IMD3 |  |
| DC_3A-21A_n79A |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_3A-41A_n77A |  | 41 | N/A | -84.3 | - | - | - | TDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A_n78A-n79A |  | n79 | 15 | - | -70.5+TT | - | - | TDD | IMD3 |  |
|  | 2 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -70.6+TT | - | - | TDD | IMD5 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_5A-66A_n77A | 1 | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -76.6 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_13A_n2A-n77A | 2 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n2 | 15 | -98.0 +TT+25.0 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A-66A_n77A |  | 66 | N/A | -99.5+ TT+25.3 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | FDD | N/A |  |
| DC_14A-66A_n77A | 1 | 13 | N/A | -72.8 | - | - | - | FDD | IMD3 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -77.4 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | REFSENS | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 19 | N/A | -71.6 | - | - | - | FDD | IMD3 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_19A-21A_n78A |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 19 | N/A | -74.1 | - |  |  | FDD | IMD4 |  |
|  |  | 21 | N/A | REFSENS | - |  |  | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS |  |  | TDD | N/A |  |
|  | 1 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_19A-21A_n79A |  | 21 | N/A | -74.5 | - | - | - | FDD | IMD5 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_19A_n78A-n79A |  | n79 | 15 | - | - | - | -60.5+TT | TDD | IMD2 |  |
|  | 2 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | - | - | -61.0+TT | TDD | IMD2 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_21A_n78A-n79A |  | n79 | 15 | - | -59.7+TT | - | - | TDD | IMD2 |  |
|  | 2 | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -57.0+TT | - | - | TDD | IMD2 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | n2 | 15 | -98.0+ TT+37.6 | - | - | - | FDD | IMD2 |  |
| DC_66A_n2A-n77A |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| NOTE 1: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.5-2: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| EN-DC Configuration | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| NOTE 1: Both of the transmitters shall be set min (+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |  |  |

Test tolerance is the same as given in Table 7.3B.2.3.5-2.

==============================================================

<< End of changes >>