3GPP TSG-RAN WG5 Meeting # 106	R5-251079

# Athens, Greece, February 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3236 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on 7.3.2.5 for Refsens symbol for degradation from PC3 to PC2 on FDD bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_FR1_FDD_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in R5-251075, for the Refsens test requirements for PC2 FDD band when UE supporting Tx diversity or not supporting Tx diversity, symbols in Table 7.3.2.5-2c and 7.3.2.5-2d are specified respectively for each band. To reduce the risk of missing notes for newly introduced bands, a general symbol of “REFSENS” for all bands is suggested to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the symbol “REFSENS_nX” for each band to a general symbol “REFSENS” for all bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The individual symbol of “REFSENS_nX” for each band will be cumbersome and increase the risk of missing notes for newly introduced bands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

7.3.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Tables 7.3.2.5-1a and Tables 7.3.2.5-1b for 2 Rx antenna port, Tables 7.3.2.5-2 a and Tables 7.3.2.5-2b for 4 Rx antenna port, Table 7.3.2.5-2c and Table 7.3.2.5-2d for PC2 UE on FDD bands, Table 7.3.2.5-2e and Table 7.3.2.5-2f for 8 Rx antenna ports, and parameters specified Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3.2.4.1-3.

Table 7.3.2.5-1a: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands for PC3

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS (kHz) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n1 | 15 |  | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT |  | -90.6 +TT | -90.1 +TT | -89.6 +TT | FDD |
|  | 30 |  |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT |  | -90.7 +TT | -90.2 +TT | -89.7 +TT |  |
|  | 60 |  |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT |  | -90.9 +TT | -90.3 +TT | -89.7 +TT |  |
| n2 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | -90.7 +TT | -84.1 +TT |  | -81.5 +TT |  |  | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -92.0 +TT | -90.8 +TT | -84.2 +TT |  | -81.6 +TT |  |  |  |
|  | 60 |  |  | -95.5 +TT | -93.4 +TT | -92.2 +TT | -90.9 +TT | -84.3 +TT |  | -81.7 +TT |  |  |  |
| n3 | 15 |  | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | -89.7 +TT | -88.9 +TT | -86.2+TT | -82.3 +TT | -81.3+TT | -79.7 +TT | FDD |
|  | 30 |  |  | -94.1 +TT | -92.1 +TT | -91.0 +TT | -89.8 +TT | -89.0 +TT | -86.3+TT | -82.4 +TT | -81.4+TT | -79.8 +TT |  |
|  | 60 |  |  | -94.5 +TT | -92.4 +TT | -91.2 +TT | -90.0 +TT | -89.1 +TT | -86.4+TT | -82.6 +TT | -81.5+TT | -79.9 +TT |  |
| n5 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -86.8 +TT | -84.8 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -88.6 +TT | -84.9 +TT |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | -90.7 +TT | -89.9 +TT |  | -88.6 +TT |  | -81.5 +TT | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -92.0 +TT | -90.8 +TT | -90.0 +TT |  | -88.7 +TT |  | -81.5 +TT |  |
|  | 60 |  |  | -95.5 +TT | -93.4 +TT | -92.2 +TT | -91.0 +TT | -90.1 +TT |  | -88.9 +TT |  | -81.5 +TT |  |
| n8 | 15 |  | -97.0 +TT | -93.8 +TT | -91.4 +TT | -85.8 +TT | -83.6+TT | -81.3+TT | -78.4+TT |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -91.7 +TT | -87.2 +TT | -84.7+TT | -81.4+TT | -78.5+TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 |  | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 |  | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 |  | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 |  | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 |  | -100.0 +TT | -96.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 +TT |  |  |  |  |  |  |  |  |  |
| n25 | 15 |  | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT | -89.3 +TT | -82.2 +TT | -81.7 +TT | -79.5 +TT | -77.6+TT |  | FDD |
|  | 30 |  |  | -93.6 +TT | -91.6 +TT | -90.5 +TT | -89.4 +TT | -82.3 +TT | -81.8 +TT | -79.6 +TT | -77.7 +TT |  |  |
|  | 60 |  |  | -94.0 +TT | -91.9 +TT | -90.7 +TT | -89.6 +TT | -82.4 +TT | -81.9 +TT | -79.7 +TT | -77.8 +TT |  |  |
| n26 | 15 | -99.7 +TT | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |  |  |  |  |  |  |
| n28 | 15 | -100.2 +TT | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT |  | -78.5 +TT |  |  |  |  | FDD |
|  | 30 |  |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  | -78.6 +TT |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 |  | -99.0 +TT | -95.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n31 | 15 | -95.7 + TT | -93.5 + TT |  |  |  |  |  |  |  |  |  | FDD |
| n65 | 15 |  | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.6+TT | -94.6+TT | -93.5+TT |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0+TT | -94.9+TT | -93.7+TT |  |  |  |  |  |  |  |
| n66 | 15 |  | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | -92.2 +TT | -91.4 +TT |  | -90.1 +TT |  |  | FDD |
|  | 30 |  |  | -96.6 +TT | -94.6 +TT | -93.5 +TT | -92.3 +TT | -91.5 +TT |  | -90.2 +TT |  |  |  |
|  | 60 |  |  | -97.0 +TT | -94.9 +TT | -93.7 +TT | -92.5 +TT | -91.6 +TT |  | -90.4 +TT |  |  |  |
| n70 | 15 |  | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT |  |  |  |  |  |  |
| n71 | 15 |  | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT | -84.17 +TT -74.88+TT | -82.57 +TT -67.18+TT | -80.77 +TT -64.08 +TT |  |  |  | FDD |
|  | 30 |  |  | -94.3 +TT | -91.9 +TT | -87.4 +TT | -84.27 +TT -74.98 +TT | -82.67 +TT -67.28 +TT | -80.87 +TT -64.18 +TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n72 | 15 | -95.7 + TT | -93.5 + TT |  |  |  |  |  |  |  |  |  | FDD |
| n74 | 15 |  | -99.53 +TT | -96.33 +TT | -94.53 +TT | -93.33 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.63 +TT | -94.63 +TT | -93.53 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.03 +TT | -94.93 +TT | -93.73 +TT |  |  |  |  |  |  |  |
| n85 | 15 | -99.2 +TT | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |
| n100 | 15 | -102.2 +TT | -100.0 +TT |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | -97.29 +TT | -94.0 +TT | -91.6 +TT | -86.9 +TT |  |  |  |  |  |  |  |
| n105 | 30 |  |  | -94.3 +TT | -91.9 +TT | -87.9 +TT |  |  |  |  |  |  | FDD |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n106 | 15 | -99.2 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. NOTE 7: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 8: Applies to UEs that support optional symmetric UL/DL for this BW. NOTE 9: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands for PC3, PC2, PC1.5

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS (kHz) | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)+TT |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)+TT |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 35, 40 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40 | -97.5 + 10log10(NRB/11)+TT |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11)+TT |  |
| n411 | 15 | 5, 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11)+TT |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11)+TT |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11)+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1+TT |  |
|  | 60 | 10 | -97.5+TT |  |
| n54 | 15 | 5 | -100+TT | TDD |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11)+TT |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11)+TT |  |
| n791 | 15 | 10, 20, 40, 50 | -95.8 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 20, 40, 50, 60, 80, 100 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 20, 40, 50, 60, 80, 100 | -96.5 + 10log10(NRB/11) +TT |  |
| n91 | 15 | 5 | -100+TT | FDD |
| n92 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | FDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
| n93 | 15 | 5 | -100+TT | FDD |
| n94 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | FDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24)+TT |  |
| n10911 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) +TT | FDD |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. NOTE 10: FFS. NOTE 11: Applies for DL channels for which channels edges are > 15 MHz away from 2xFUL at 15 kHz SCS and > 30 MHz away from 2xFUL at 30 kHz SCS. In case of UL second harmonic direct hit, the value is modified to -71.9 dBm for all channel bandwidths. |  |  |  |  |

Table 7.3.2.5-2a: Four antenna port Reference sensitivity QPSK PREFSENS FDD bands for PC3

| Operating Band | SCS (kHz) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 |  | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT | -94.6 +TT |  | -93.3 +TT | -92.8 +TT | -92.3 +TT | FDD |
|  | 30 |  |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT | -94.7 +TT |  | -93.4 +TT | -92.9 +TT | -92.4 +TT |  |
|  | 60 |  |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT | -94.8 +TT |  | -93.6 +TT | -93 +TT | -92.4 +TT |  |
| n2 | 15 |  | -100.7 +TT | -97.5 +TT | -95.7 +TT | -94.5 +TT | -93.4 +TT | -86.8 +TT |  | -84.2 +TT |  |  | FDD |
|  | 30 |  |  | -97.8 +TT | -95.8 +TT | -94.7 +TT | -93.5 +TT | -86.9 +TT |  | -83.3 +TT |  |  |  |
|  | 60 |  |  | -98.2 +TT | -96.1 +TT | -94.9 +TT | -93.6 +TT | -87.0 +TT |  | -84.4 +TT |  |  |  |
| n3 | 15 |  | -99.7 +TT | -96.5 +TT | -94.7 +TT | -93.5 +TT | -92.4 +TT | -91.6 +TT | -88.9+TT | -90.3 +TT | -84.0+TT | -82.4 +TT | FDD |
|  | 30 |  |  | -96.8 +TT | -94.8 +TT | -93.7 +TT | -92.5 +TT | -91.7 +TT | -90.0+TT | -90.4 +TT | -84.1+TT | -82.5 +TT |  |
|  | 60 |  |  | -97.2 +TT | -95.1 +TT | -93.9 +TT | -92.7 +TT | -91.8 +TT | -90.1+TT | -90.6 +TT | -84.2+TT | -82.6 +TT |  |
| n56 | 15 |  | -100.4 +TT | -97.2 +TT | -95.4 +TT | -89.2 +TT | -87.2 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.5 +TT | -95.5 +TT | -91 +TT | -87.3 +TT |  |  |  |  |  |  |
| n71 | 15 |  | -100.7 +TT | -97.5 +TT | -95.7 +TT | -94.5 +TT | -93.4 +TT | -92.6 +TT |  | -91.3 +TT |  | -84.2 +TT | FDD |
|  | 30 |  |  | -97.8 +TT | -95.8 +TT | -94.7 +TT | -93.5 +TT | -92.7 +TT |  | -91.4 +TT |  | -84.2 +TT |  |
|  | 60 |  |  | -98.2 +TT | -97.1 +TT | -94.9 +TT | -93.7 +TT | -92.8 +TT |  | -91.6 +TT |  | -84.2 +TT |  |
| n85 | 15 |  | -99.7 +TT | -96.5 +TT | -94.1 +TT | -88.5 +TT | -86.3 +TT | -84 +TT | -81.1 +TT |  |  |  | FDD |
|  | 30 |  |  | -96.8 +TT | -94.4 +TT | -89.9 +TT | -87.4 +TT | -84.1 +TT | -81.2 +TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n86 | 15 |  | -99.4 +TT | -96.2 +TT | -93.8 +TT | -88.2 +TT |  |  | -80.8 +TT |  |  |  | FDD |
|  | 30 |  |  | -96.5 +TT | -94.1 +TT | -89.6 +TT |  |  | -80.9 +TT |  |  |  |  |
| n13 | 15 | -99.7 +TT |  | -96.5 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.8 +TT |  |  |  |  |  |  |  |  |  |
| n206 | 15 |  | -99.4 +TT | -96.2 +TT | -93.4 +TT | -92.2 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.5 +TT | -93.5 +TT | -92.4 +TT |  |  |  |  |  |  |  |
| n265 | 15 | -102.4+TT | -100.27+TT | -97.27+TT | -95.47+TT | -90.3+TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.57+TT | -95.47+TT | -90.4+TT |  |  |  |  |  |  |  |
| n266 | 15 | -102.1 +TT | -99.97 +TT | -96.97 +TT | -95.17 +TT | -90 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.2 +TT | -95.1 +TT | -90.1 +TT |  |  |  |  |  |  |  |
| n286 | 15 | -102.6 +TT | -100.9 +TT | -97.9+ TT | -95.9 +TT | -93.2 +TT |  | -80.9 +TT |  |  |  |  | FDD |
|  | 30 |  |  | -98 +TT | -96 +TT | -93.4 +TT |  | -81 +TT |  |  |  |  |  |
| n25 | 15 |  | -99.2 +TT | -96.0 +TT | -94.2 +TT | -93.0 +TT | -92.0 +TT | -84.9 +TT | -84.4 +TT | -82.2 +TT | -80.3+TT |  | FDD |
|  | 30 |  |  | -96.3 +TT | -94.3 +TT | -93.2 +TT | -92.1 +TT | -85.0 +TT | -84.5 +TT | -82.3 +TT | -80.4 +TT |  |  |
|  | 60 |  |  | -96.7 +TT | -94.6 +TT | -93.4 +TT | -92.3 +TT | -85.1 +TT | -84.6 +TT | -82.4 +TT | -80.5 +TT |  |  |
| n30 | 15 |  | -101.7 +TT | -98.5 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -98.8 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 |  | -102.2 +TT | -99.0 +TT | -97.2 +TT | -96.0 +TT | -94.9 +TT | -94.1 +TT |  | -92.8 +TT |  |  | FDD |
|  | 30 |  |  | -99.3 +TT | -97.3 +TT | -96.2 +TT | -95.0 +TT | -94.2 +TT |  | -92.9 +TT |  |  |  |
|  | 60 |  |  | -99.7 +TT | -97.6 +TT | -96.4 +TT | -95.2 +TT | -94.3 +TT |  | -93.1 +TT |  |  |  |
| n70 | 15 |  | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT |  |  |  |  |  |  |
|  | 60 |  |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT |  |  |  |  |  |  |
| n716 | 15 |  | -99.6 +TT | -96.4 +TT | -94 +TT | -88.4 +TT | -86.68 +TT -77.39 +TT | -85.08 +TT -69.69 +TT | -83.28 +TT -66.59 +TT |  |  |  | FDD |
|  | 30 |  | -96.7 +TT | -94.3 +TT | -89.8 +TT | -86.7 +TT | -85.18 +TT -77.49 +TT | -83.38 +TT -69.79 +TT | -96.78 +TT -66.69 +TT |  |  |  |  |
| n85 | 15 | -99.7 +TT | -101.9 +TT | -96.5 +TT | -86.7 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.8 +TT | -86.8 +TT |  |  |  |  |  |  |  |  |
| n105 | 15 | -99.9 +TT |  | -96.7 +TT | -94.3 +TT | -89.6 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -97 +TT | -94,6 +TT | -90.6 +TT |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for above listed operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2 The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 3: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. NOTE 5: When 4Rx operation is supported by FWA form factor. NOTE 6: When 4Rx operation is supported by handheld UE. NOTE 7: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 8: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 9: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-2b: Four antenna port Reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands for PC3, PC2, PC1.5

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS (kHz) | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+-2.7 TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 35, 40 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) -2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n411 | 15 | 5, 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/52)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11)-2.7 +TT |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11)-2.2+TT |  |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11)-2.2 +TT |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11)-2.2 +TT |  |
| n791 | 15 | 10, 20, 40, 50 | -95.8 + 10log10(NRB/52)-2.2 +TT | TDD |
|  | 30 | 10, 20, 40, 50, 60, 80, 100 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 20, 40, 50, 60, 80, 100 | -96.5 + 10log10(NRB/11)-2.2 +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |

Table 7.3.2.5-2c: Reference Sensitivity for PC2 UE on FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | - | REFSENS | REFSENS | REFSENS |
| n3 | REFSENS+0.5 | REFSENS +0.5 | REFSENS +0.5 | REFSENS +0.5 | REFSENS +0.6 | REFSENS +0.8 | REFSENS +1.1 | REFSENS +1.5 | REFSENS +2.3 | REFSENS +2.8 |
| n7 | REFSENS+0.5 | REFSENS+0.5 | REFSENS+0.5 | REFSENS+0.5 | REFSENS+0.5 | REFSENS+0.5 | - | REFSENS+0.5 | - | REFSENS+2.0 |
| n8 | REFSENS+0.5 | REFSENS+0.7 | REFSENS+0.8 | REFSENS+2.3 | REFSENS+2.8 | REFSENS+3.2 | REFSENS+3.1 | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: void. NOTE 3: void. NOTE 4: void. NOTE 5: void. NOTE 6: The symbol “REFSENS” refers to the two antenna port and four antenna port Reference Sensitivity of the corresponding band in Table 7.3.2.5-1a and Table 7.3.2.5-2a. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-2d: Reference Sensitivity for PC2 UE on FDD bands for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | - | REFSENS | REFSENS | REFSENS |
| n3 | REFSENS +1.4 | REFSENS +1.5 | REFSENS +1.5 | REFSENS +1.5 | REFSENS +1.6 | REFSENS +1.7 | REFSENS +2.8 | REFSENS +5 | REFSENS +5.5 | REFSENS +6.0 |
| n7 | REFSENS+0.9 | REFSENS+1.0 | REFSENS+1.0 | REFSENS+1.0 | REFSENS+1.1 | REFSENS+1.1 | - | REFSENS+1.1 | - | REFSENS+5.3 |
| n8 | REFSENS+1.3 | REFSENS+1.4 | REFSENS+2.1 | REFSENS+5.8 | REFSENS+6.1 | REFSENS+6.5 | REFSENS+7.0 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: void. NOTE 3: void. NOTE 4: void. NOTE 5: The symbol “REFSENS” refers to the two antenna port and four antenna port Reference Sensitivity of the corresponding band in Table 7.3.2.5-1a and Table 7.3.2.5-2a. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-2e: Eight antenna port Reference sensitivity QPSK PREFSENS FDD bands

FFS

Table 7.3.2.5-2f: Eight antenna port Reference sensitivity QPSK PREFSENS TDD bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS (kHz) | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52) -4.0 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) - 4.0 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) - 4.0 +TT |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) - 4.0 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) - 4.0 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) - 4.0 +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. |  |  |  |  |

Table 7.3.2.5-3: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

For the UE which supports CA, SUL or DC band combination, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 a and Table 7.3.2.5-1b shall be increased by the amount given in ΔRIB,c defined in subclause 7.3.3 for the applicable operating bands.

==============================================================

<< End of changes >>