3GPP TSG-RAN WG5 Meeting #106								R5-251107

Athen, GR, 17th – 21nd February, 2025

Agenda item:	5.3.37.2

Source:	CAICT

Title:	Discussion on charging cable for FR2 MIMO OTA

Document for:	Approval

1	Introduction

In RAN5 #105 meeting, it was agreed to identify the need for FR2 MIMO OTA testing with a charging cable in TS38.551 [1]. Given RAN4 has successfully complete the measurement campaign activity for FR2 MIMO OTA in Rel-18 phase, we provide our observations as aligned FR2 OTA labs in this contribution.

2	Observations

2.1 Number of test points

The necessity of charging cables in OTA testing is inherently tied to test duration. Traditional FR2 SISO TX/RX beam peak search procedures require 1106 test points, leading to extended test durations. Long term testing increases the risk of battery depletion, leading to practical discussions on the necessity of charging during testing. By contrast, FR2 MIMO OTA methodologies required measurement points to only 36, achieving a 97% reduction in the number of test points. This small number of testing points fundamentally reduces the risk of battery depletion, making charging cables less necessary.

Observation 1: Unlike the 1106 test points required for FR2 beam peak search, the FR2 MIMO OTA test only requires 36 test points, reducing the risk of battery depletion.

2.2 RAN4 Rel-18 FR2 MIMO OTA Measurement Campaign

The RAN4 Rel-18 FR2 MIMO OTA measurement campaign for band n261 has been successfully completed across aligned labs. As one of the aligned labs, our experience throughout the entire lab alignment and measurement campaign activity phase is that as long as the FR2 device is fully charged before testing, it can support the completion of a test case without the need for recharging during testing.

Offline discussions with other aligned labs confirm this observation aligns with their experiences, indicating consistent device energy sustainability under standardized test durations.

Observation 2: Pre-test full charging suffices for FR2 MIMO OTA test, with no observed need for mid-test recharging across aligned laboratories during lab alignment and measurement campaign activity.

Proposal 1: Take the above observations into account when identifying the necessity of using a charging cable for FR2 MIMO OTA testing, ensuring that the specific conditions and potential impacts on test accuracy are carefully evaluated.

3	Conclusion

This contribution makes the following observations and proposals.

Observation 1: Unlike the 1106 test points required for FR2 beam peak search, the FR2 MIMO OTA test only requires 36 test points, reducing the risk of battery depletion.

Observation 2: Pre-test full charging suffices for FR2 MIMO OTA test, with no observed need for mid-test recharging across aligned laboratories during lab alignment and measurement campaign activity.

Proposal 1: Take the above observations into account when identifying the necessity of using a charging cable for FR2 MIMO OTA testing, ensuring that the specific conditions and potential impacts on test accuracy are carefully evaluated.

References

- R5-248007, Charging Cable Verification Procedure, Keysight Technologies UK Ltd, ETS-Lindgren