3GPP TSG-RAN5 Meeting #106	R5-251117

# Athens, Greece, Feburary 17 – Feburary 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3246 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of 6.4E.2.5 EVM equalizer spectrum flatness for V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NR V2X EVM equalizer spectrum flatnees test cases are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the test configurations, test procedures, test requirements to complete the test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4E.2.5, 6.4E.2.5.1D, 6.4E.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0998 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.4E.2.5	EVM equalizer spectrum flatness for V2X

6.4E.2.5.1	EVM equalizer spectrum flatness for V2X / non-concurrent operation

6.4E.2.5.1.1	Test purpose

The zero-forcing equalizer correction applied in the EVM measurement process (as described in Annex E) must meet a spectrum flatness requirement for the EVM measurement to be valid. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the V2X sidelink allocated block variation in dB of the equalizer coefficients generated by the EVM measurement process. The EVM equalizer spectrum flatness requirement does not limit the correction applied to the signal in the EVM measurement process but for the EVM result to be valid, the equalizer correction that was applied must meet the EVM equalizer spectrum flatness minimum requirements. The basic measurement interval is the same as for EVM.

6.4E.2.5.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.4E.2.5.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4E.2.1.

f

FUL_High

FUL_High – 3(5) MHz

< 4(4) dBp-p

Range 1

Range 2

max(Range 1)-min(Range 2) < 5(6) dB

max(Range 2)-min(Range 1) < 7(10) dB

< 8(12) dBp-p

6.4E.2.5.1.4	Test description

6.4E.2.5.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.4E.2.5.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.4E.2.5.1.4.1-1: Test Configuration Table for PSSCH and PSCCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid and Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | CP-OFDM QPSK |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.4E.2.5.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.4E.2.5.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

6.4E.2.5.1.4.2	Test procedure

1.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

2.	Measure spectrum flatness using Global In-Channel Tx-Test (Annex E). When sidelink transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the measurement interval is reduced by one symbol, accordingly.

6.4E.2.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10.

6.4E.2.5.1.5	Test requirement

Each of the n spectrum flatness functions, shall derive four ripple results in Annex E.4.4.1. The derived results shall not exceed the values in Figure 6.4E.2.5.1.5-1:

For normal conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.4E.2.5.1.5-1 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.4E.2.5.1.5-1).

For normal conditions, the peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 6.4E.2.5.1.5-1. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.4E.2.5.1.5-1).

For extreme conditions, the EVM equalizer spectral flatness shall not exceed the values specified in Table 6.4E.2.5.1.5-2. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.4E.2.5.1.5-1).

Table 6.4E.2.5.1.5-1: Requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 4 + TT (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 8 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each NR frequency band specified in Table 5.5E-1 NOTE 3: Test tolerance TT = 1.4 dB. |  |

Table 6.4E.2.5.1.5-2: Minimum requirements for EVM equalizer spectrum flatness (extreme conditions)

| Frequency range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 4 + TT (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 12 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each NR frequency band specified in Table 5.5-1 NOTE 3: Test tolerance TT = 1.4 dB. |  |

Figure 6.4E.2.5.1.5-1: The test requirements for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated for unshaped modulations (the ETC test requirements are within brackets)

6.4E.2.5.1D	EVM equalizer spectrum flatness for V2X / non-concurrent operation / SL-MIMO

6.4E.2.5.1D.1	Test purpose

The zero-forcing equalizer correction applied in the EVM measurement process (as described in Annex E) must meet a spectrum flatness requirement for the EVM measurement to be valid. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the V2X sidelink allocated block variation in dB of the equalizer coefficients generated by the EVM measurement process. The EVM equalizer spectrum flatness requirement does not limit the correction applied to the signal in the EVM measurement process but for the EVM result to be valid, the equalizer correction that was applied must meet the EVM equalizer spectrum flatness minimum requirements. The basic measurement interval is the same as for EVM.

6.4E.2.5.1D.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

6.4E.2.5.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4E.2.1.

6.4E.2.5.1D.4	Test description

Same test description as in 6.4E.2.5.1 with following exceptions:

For initial conditions:

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

6.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO. The UE is synchronized to GNSS,

For test procedures:

Steps 1 is replaced by:

1.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR with 2-layer MIMO codebook TPMI 0. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

In step 2, the spectrum flatness is measured at each transmit antenna connector respectively.

6.4E.2.5.1D.5	Test requirement

Each of the n spectrum flatness functions, shall derive four ripple results in Annex E.4.4.1 on each antenna connector. The derived results shall not exceed the values in Figure 6.4E.2.5.1D.5-1:

For normal conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.4E.2.5.1D.5-1 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.4E.2.5.1D.5-1).

For normal conditions, the peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 6.4E.2.5.1D.5-1. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.4E.2.5.1D.5-1).

For extreme conditions, the EVM equalizer spectral flatness shall not exceed the values specified in Table 6.4E.2.5.1D.5-2. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.4E.2.5.1D.5-1).

Table 6.4E.2.5.1D.5-1: Requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 4 + TT (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 8 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each NR frequency band specified in Table 5.5E-1 NOTE 3: Test tolerance TT = 1.4 dB. |  |

Table 6.4E.2.5.1D.5-2: Minimum requirements for EVM equalizer spectrum flatness (extreme conditions)

| Frequency range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 4 + TT (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 12 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each NR frequency band specified in Table 5.5-1 NOTE 3: Test tolerance TT = 1.4 dB. |  |

Figure 6.4E.2.5.1D.5-1: The test requirements for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated for unshaped modulations (the ETC test requirements are within brackets)

6.4E.2.5.2	EVM equalizer spectrum flatness for V2X / concurrent operation

6.4E.2.5.2.1	Test purpose

Same test purpose as in 6.4E.2.5.1.1 for concurrent operation.

6.4E.2.5.2.2	Test applicability

No test case details are specified. The In-band emissions requirements for inter-band concurrent operation and intra-band concurrent operation apply and are tested in clauses 6.4.2.4 for NR Uu operation and 6.4E.2.5.1 for NR sidelink option.

< End of Changes >