Title: 	Discussion on test point selection for NR Spurious emission co-existence for NR V2X concurrent operation in FR1

Source: 	Huawei

Introduction

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NR Spurious emission co-existence for NR V2X concurrent operation in FR1.

Discussion

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects. The test case of spurious emission for UE coexistence of CA would be taken as reference.

The minimum requirement for spurious emission co-existence for NR V2X concurrent operation is copied below for reference.

This clause specifies the additional requirements for inter-band con-current V2X operation with the single CC uplink assigned to two NR bands for coexistence with protected bands for the specified simultaneous transmission of the inter-band con-current V2X configurations in Table 6.5E.3.3-1. The intersection of the requirements for the individual bands specified in clause 6.5.3.2 shall also apply for the specified simultaneous transmission of the inter-band con-current V2X. Intersection of a requirement means that both UL or sidelink transmission constituent bands have the same protected band requirement specified and if one or both protected bands have note(s) associated those note(s) also apply.

For the inter-band con-current NR V2X operation, the UE-coexistence requirements in Table 6.5E.3.3.1-1 apply for the corresponding inter-band con-current operation with transmission assigned to both uplink in licensed band and sidelink in Band n47.

NOTE:	For inter-band con-current V2X operation with uplink assigned to NR band and slidelink transmission assigned to NR V2X operating bands, the requirements in Table 6.5E.3.3-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5E.3.3-1 and in clause 6.5.3.2 would be considered to be verified by the measurements verifying the one uplink inter-band con-current UE to UE co-existence requirements.

Table 6.5E.3.3-1: Requirements for inter-band con-current V2X operation

| V2X | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| con-current operating band cofiguration | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_n71A-n47A | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

For the intra-band NR V2X transmission where Uu and SLoverlap in time , the UE-coexistence requirements in Table 6.5A.3.2.1-1 apply for the corresponding intra-band concurrent operation for the both uplink and sidelink transmission in licensed band.

  - Test Environment

The test environment for basic NR FR1 test cases is selected as Normal. The same environment selection can be adopted for V2X.

Proposal 1: Define test environment for Spurious emission co-existence for V2X concurrent operation as Normal.

  - Test Subcarrier Spacing

Test subcarrier spacing is selected as Lowest for Spurious emission for basic test cases in FR1. The same approach can be followed for NR V2X test cases.

Proposal 2: Select Lowest SCS supported for UE for Spurious emission co-existence for V2X concurrent operation in FR1.

  - Test Frequencies

For spurious emission co-existence of inter-band CA, the test frequencies as band combination specific condition. The same approach can be followed for NR V2X test cases.

Proposal 3: The test frequencies are selected as band combination specific condition for Spurious emission co-existence for V2X inter-band concurrent operation in FR1.

For spurious emission co-existence of intra-band CA, the test frequencies are selected as Low range and High range. The same approach can be followed for NR V2X test cases.

Proposal 3a: The test frequencies are Low range and High range for Spurious emission co-existence for V2X intra-band concurrent operation in FR1.

  - Test Channel Bandwidth

Test channel bandwidth are selected Minimum for Spurious emission co-existence for basic test cases in FR1. The same approach can be followed for NR V2X test cases.

Proposal 4: Test Minimum channel bandwidth for Spurious emission co-existence for V2X inter-band concurrent operation in FR1.

For spurious emission co-existence of intra-band CA, the test channel bandwidth are selected as Lowest NRBagg and Highest NRBagg. The same approach can be followed for NR V2X test cases.

Proposal 4a: The test channel bandwidth are Lowest NRBagg and Highest NRBagg for Spurious emission co-existence for V2X intra-band concurrent operation in FR1.

  - Uplink configuration

The uplink configuration for inter-band CA is selected as CP-OFDM QPSK. The same approach can be followed for NR V2X test cases.

Proposal 5: Select CP-OFDM waveform and QPSK modulation to test Spurious emission for NR V2X inter-band concurrent operation in FR1.

The RB allocation is selected as Edge_1RB_Left and Edge_1RB_Right for default configuration in basic FR1 test case. Alternative allocations could be selected based on band combination specific analysis.

Proposal 6: Test Spurious emission for UE coexistence for V2X concurrent operation with Edge_1RB_Left and Edge_1RB_Right as default configuration. Alternative allocations could be selected based on band combination specific analysis.

The modulation type for intra-band CA is selected as CP-OFDM QPSK. The uplink configurations are 1RB_Left+1RB_right and outer_full+outer_full. For V2X the 1RB allocation is not feasible. For simplification only outer_full+outer_full is selected.

Proposal 7: Select CP-OFDM QPSK with outer_full+outer_full allocation to test Spurious emission for NR V2X intra-band concurrent operation in FR1.

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Occupied Bandwidth test case definition:

Proposal 1: Define test environment for Spurious emission co-existence for V2X concurrent operation as Normal.

Proposal 2: Select Lowest SCS supported for UE for Spurious emission co-existence for V2X concurrent operation in FR1.

Proposal 3: The test frequencies are selected as band combination specific condition for Spurious emission co-existence for V2X inter-band concurrent operation in FR1.

Proposal 3a: The test frequencies are Low range and High range for Spurious emission co-existence for V2X intra-band concurrent operation in FR1.

Proposal 4: Test Minimum channel bandwidth for Spurious emission co-existence for V2X inter-band concurrent operation in FR1.

Proposal 4a: The test channel bandwidth are Lowest NRBagg and Highest NRBagg for Spurious emission co-existence for V2X intra-band concurrent operation in FR1.

Proposal 5: Select CP-OFDM waveform and QPSK modulation to test Spurious emission for NR V2X inter-band concurrent operation in FR1.

Proposal 6: Test Spurious emission for UE coexistence for V2X concurrent operation with Edge_1RB_Left and Edge_1RB_Right as default configuration. Alternative allocations could be selected based on band combination specific analysis.

Proposal 7: Select CP-OFDM QPSK with outer_full+outer_full allocation to test Spurious emission for NR V2X intra-band concurrent operation in FR1.

References

[1] R5-180065 , “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] R5-180111, “Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases”, Ericsson, RAN 5 Ad hoc, Munich, Jan. 2018.

[3] R4-1800402, “38.101-1 Implementation of agreed CRs after NR-AH-1801”, Qualcomm Incorporated, RAN4 NR-AH#1801, San Diego, CA, USA, Jan 2018.

[4] TR 38.817-01 v0.3.0, “General aspects for UE RF for NR”.

[5] R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019

[6] 3GPP TS 38.101-1 v16.20.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)