3GPP TSG-RAN5 Meeting #106 	R5-251158

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3819 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to TE connection diagram for RedCap 2RX PRACH test cases 16.3.2.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm India Pvt Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RedCap 2RX PRACH test cases 16.3.2.2.x, TE connection diagram are not correctly defined. Test case needs 1X2 connection but connection diagram is defined for 2X2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated TE connection diagram for RedCap 2RX PRACH test cases 16.3.2.2.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TE connection diagram will remain wrongly configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.2.2.2,16.3.2.2.4,16.3.2.2.6,16.3.2.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

##### 16.3.2.2.2	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE

16.3.2.2.2.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will also verify the requirements in 38.133 [6] Clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model

16.3.2.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.3.2.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clauses A.16.3.2.2.2.

16.3.2.2.2.4	Test description

16.3.2.2.2.4.1	Initial conditions

Same as the initial conditions given in 6.3.2.2.1.4.1 with following exceptions:

-	Table 6.3.2.2.1.4.1-1 is replaced by Table 16.3.2.2.2.4.1-1.

-	Table 6.3.2.2.1.4.1-2 is replaced by Table 16.3.2.2.2.4.1-2.

Table 16.3.2.2.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.3.2.2.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.2.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 16.3.2.2.2.4.1-2: Initial conditions for Contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.3.2.2.2.4.2	Test procedure

Same as the test procedure given in clause 6.3.2.2.1.4.2.

16.3.2.2.2.4.3	Message contents

Same as the message contents given in clause 6.3.2.2.1.4.3.

16.3.2.2.2.5	Test requirement

Same as the test requirements given in clause 6.3.2.2.1.4.2 with following exceptions:

-	Table 6.3.2.2.1.5-1 is replaced by Table 16.3.2.2.2.5-1.

-	The check on the power of the first preamble in Test 2, Test 3, Test 5 and Test 7 is replaced as follows:

-	The power of the first preamble shall be X dBm within the accuracy specified in Table 6.3.2.2.1.5-2, where

-	X = -22 for test configuration 16.3.2.2.2-1, 16.3.2.2.2-3 and 16.3.2.2.2-4,

-	X = -25 for test configuration 16.3.2.2.2-2.

Table 16.3.2.2.2.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 | As defined in A.3 and A.3A, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.10 and A.3.10B |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.10 and A.3.10B |
| Duplex Mode for Cell 1 |  | Config 1,4 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.2.1. |
| PDSCH parameters Note 4 |  | Config 1,4 |  | SR.1.1 FDD | As defined in A.1.1. |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  | Config 3 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 | dBm/15kHz | -95 |  |
|  |  | Config 3 |  | -98 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  | Config 3 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 | dBm/15kHz | -115 |  |
|  |  | Config 3 |  | -118 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1,2,4 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3 |  | -65.38/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | PRACH.1 FR1 | As defined in A.7.1. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

< Unchanged sections omitted >

##### 16.3.2.2.4	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE

16.3.2.2.4.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will also verify the requirements in 38.133 [6] Clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model

16.3.2.2.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards. In addition, Test 2 is applicable to UEs that support CSI-RS based Random Access Preamble which requires UE to support csi-RS-CFRA-ForHO.

16.3.2.2.4.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clauses A.16.3.2.2.4.

16.3.2.2.4.4	Test description

16.3.2.2.4.4.1	Initial conditions

Same as the initial conditions given in 6.3.2.2.2.4.1 with following exceptions:

-	Table 6.3.2.2.2.4.1-1 is replaced by Table 16.3.2.2.4.4.1-1.

-	Table 6.3.2.2.2.4.1-2 is replaced by Table 16.3.2.2.4.4.1-2.

Table 16.3.2.2.4.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.3.2.2.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.2.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.4-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.4-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 16.3.2.2.4.4.1-2: Initial conditions for non-contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.3.2.2.4.4.2	Test procedure

Same as the test procedure given in clause 6.3.2.2.2.4.2.

16.3.2.2.4.4.3	Message contents

Same as the message contents given in clause 6.3.2.2.2.4.3.

16.3.2.2.4.5	Test requirement

Same as the test requirements given in clause 6.3.2.2.2.4.2 with following exceptions:

-	Table 6.3.2.2.2.5-1 is replaced by Table 16.3.2.2.4.5-1.

-	The check on the power of the first preamble in Test 3 and Test 4 is replaced as follows:

-	The power of the first preamble shall be X dBm within the accuracy specified in Table 6.3.2.2.2.5-2, where

-	X = -22 for test configuration 16.3.2.2.4-1, 16.3.2.2.4-3 and 16.3.2.2.4-4,

-	X = -25 for test configuration 16.3.2.2.4-2.

Table 16.3.2.2.4.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 | SSB.1 FR1 | As defined in A.3, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 3 |  | SSB.1 RedCap FR1 | SSB.1 RedCap FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | 2 | Different from the definition in A.3.10B |
| SS/PBCH block index |  |  |  | 0,1 | 0,1 | Different from the definition in A.3.10B |
| CSI-RS Configuration |  | Config 1,2,4 |  | N/A | CSI-RS.1.1 FDD | As defined in A.1.4 |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| Duplex Mode for Cell 1 |  | Config 1 |  | FDD | FDD |  |
|  |  | Config 2,3 |  | TDD | TDD |  |
|  |  | Config 4 |  | HD-FDD | HD-FDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | OP.1 | As defined in A.2.1. |
| PDSCH parameters Note 4 |  | Config 1,4 |  | SR.1.1 FDD | SR.1.1 FDD | As defined in A.3.1.1 |
|  |  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 TDD | CR.1.1 TDD |  |
|  |  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 TDD | CCR.1.1 TDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB | 0 | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| SSB with index 0 |  |  | dB | 3 | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 3 |  | -101 | -101 |  |
|  |  |  | dB | 3 | 3 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 | dBm/15kHz | -95 | -95 |  |
|  |  | Config 3 |  | -98 | -98 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 | -95 |  |
| SSB with index 1 |  |  | dB | -17 | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 3 |  | -101 | -101 |  |
|  |  |  | dB | -17 | -17 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 | dBm/15kHz | -115 | -115 |  |
|  |  | Config 3 |  | -118 | -118 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 | -115 |  |
| Io Note 2 |  | Config 1,2,4 | dBm | -65.3/9.36MHz | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3 |  | -65.38/18.36 MHz | -65.38/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | PRACH.2 FR1 | PRACH.3 FR1 | As defined in A.3.8B.2. |
| Propagation Condition |  |  | - | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 16.3.2.2.6	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE

16.3.2.2.6.1	Test purpose

To verify that the behaviour of the 2-step RA type random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will also verify the requirements in 38.133 [6] Clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model

16.3.2.2.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx and supporting 2-step RACH from Release 17 onwards.

16.3.2.2.6.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clauses A.16.3.2.2.6.

16.3.2.2.6.4	Test description

16.3.2.2.6.4.1	Initial conditions

Same as the initial conditions given in 6.3.2.2.3.4.1 with following exceptions:

-	Table 6.3.2.2.3.4.1-1 is replaced by Table 16.3.2.2.6.4.1-1.

-	Table 6.3.2.2.3.4.1-2 is replaced by Table 16.3.2.2.6.4.1-2.

Table 16.3.2.2.6.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.3.2.2.6-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.2.6-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.6-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.6-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 16.3.2.2.6.4.1-2: Initial conditions for Contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.3.2.2.6.4.2	Test procedure

Same as the test procedure given in clause 6.3.2.2.3.4.2.

16.3.2.2.6.4.3	Message contents

Same as the message contents given in clause 6.3.2.2.3.4.3.

Table 16.3.2.2.6.4.3-1: BWP-UplinkCommon (Void)

16.3.2.2.6.5	Test requirement

Same as the test requirements given in clause 6.3.2.2.3.4.2 with following exceptions:

-	Table 6.3.2.2.3.5-1 is replaced by Table 16.3.2.2.6.5-1.

-	The check on the power of the first preamble in Test 1, Test 2 and Test 3 is replaced as follows:

-	The power of the first preamble shall be X dBm within the accuracy specified in Table 6.3.2.2.3.5-2, where

-	X = -22 for test configuration 16.3.2.2.6-1, 16.3.2.2.6-3 and 16.3.2.2.6-4,

-	X = -25 for test configuration 16.3.2.2.6-2.

-	The check on the power of the first MsgA PUSCH transmission in Test 1, Test 2 and Test 3 is replaced as follows:

-	The power of the first MsgA PUSCH transmission shall be Y dBm within the accuracy specified in Table 6.3.2.2.4.5-2A, where

-	Y = -25 for test configuration 16.3.2.2.6-1, 16.3.2.2.6-2 and 16.3.2.2.6-4,

-	Y = -22 for test configuration 16.3.2.2.6-3.

Table 16.3.2.2.6.5-1: General test parameters

| Parameter |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  |  | Config 1,2,4 |  | SSB.1 FR1 | As defined in A.3, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  |  | Config 3 |  | SSB.1 RedCap FR1 |  |
| Number of SSBs per SS-burst |  |  |  |  | 2 | Different from the definition in A.3 |
| SS/PBCH block index |  |  |  |  | 0,1 | Different from the definition in A.3 |
| Duplex Mode for Cell 1 |  |  | Config 1 |  | FDD |  |
|  |  |  | Config 2,3 |  | TDD |  |
|  |  |  | Config 4 |  | HD-FDD |  |
| TDD Configuration |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  |  | Config 1,4 |  | SR.1.1 FDD | As defined in A.1.1 |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| SSB with index 0 |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  |  | Config 3 |  | -101 |  |
|  |  |  |  | dB | 3 |  |
|  | SS-RSRP Note 3 |  | Config 1,2,4 | dBm/15kHz | -95 |  |
|  |  |  | Config 3 |  | -98 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  |  | Config 3 |  | -101 |  |
|  |  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 |  | dBm/15kHz | -115 |  |
|  |  | Config 3 |  |  | -118 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  | Config 1,2,4 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  |  | Config 3 |  | -65.38/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| MsgA Configuration |  |  |  |  | MsgA.1 FR1 | As defined in A.7A.1 |
| msgA-RSRP-ThresholdSSB |  |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Propagation Condition |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 16.3.2.2.8	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE

16.3.2.2.8.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in 38.133 [6] Clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

16.3.2.2.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx and 2-step RACH from Release 17 onwards.

16.3.2.2.8.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clauses A.16.3.2.2.8.

16.3.2.2.8.4	Test description

16.3.2.2.8.4.1	Initial conditions

Same as the initial conditions given in 6.3.2.2.4.4.1 with following exceptions:

-	Table 6.3.2.2.4.4.1-1 is replaced by Table 16.3.2.2.8.4.1-1.

-	Table 6.3.2.2.4.4.1-2 is replaced by Table 16.3.2.2.8.4.1-2.

Table 16.3.2.2.8.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.3.2.2.8-1 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.8-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 16.3.2.2.8.4.1-2: Initial conditions for Contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.3.2.2.8.4.2	Test procedure

Same as the test procedure given in clause 6.3.2.2.4.4.2.

16.3.2.2.8.4.3	Message contents

Same as the message contents given in clause 6.3.2.2.4.4.3.

Table 16.3.2.2.8.4.3-1: BWP-UplinkCommon (Void)

16.3.2.2.8.5	Test requirement

Same as the test requirements given in clause 6.3.2.2.4.4.2 with following exceptions:

-	Table 6.3.2.2.4.5-1 is replaced by Table 16.3.2.2.8.5-1.

-	The check on the power of the first MsgA PUSCH in Test 1, Test 2 and Test 3 is replaced as follows:

-	The power of the first MsgA PUSCH transmission shall be -22 dBm for test configuration 16.3.2.2.8-1 and -25 dBm for test configuration 16.3.2.2.8-2 within the accuracy specified in Table 6.3.2.2.4.5-2A.

Table 16.3.2.2.8.5-1: General test parameters

| Parameter |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  |  | Config 1 |  | SSB.1 RedCap FR1 | As defined in A.3, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  |  | Config 2 |  | SSB.1 FR1 |  |
| Number of SSBs per SS-burst |  |  |  |  | 2 | Different from the definition in A.3 |
| SS/PBCH block index |  |  |  |  | 0,1 | Different from the definition in A.3 |
| Duplex Mode for Cell 1 |  |  | Config 1 |  | TDD |  |
|  |  |  | Config 2 |  | HD-FDD |  |
| TDD Configuration |  |  | Config 1 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  |  | Config 1 |  | TRS.1.2 TDD |  |
|  |  |  | Config 2 |  | TRS.1.1 FDD |  |
| OCNG Pattern Note 1 |  |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  |  | Config 1 |  | SR.2.1 TDD | As defined in A.1.1 |
|  |  |  | Config 2 |  | SR.1.1 FDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.2.1 TDD |  |
|  |  |  | Config 2 |  | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.2.1 TDD |  |
|  |  |  | Config 2 |  | CCR.1.1 FDD |  |
| NR RF Channel Number |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| SSB with index 0 |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  | Config 2 |  | -98 |  |
|  |  |  |  | dB | 3 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 |  | dBm/15kHz | -95 |  |
|  |  | Config 3 |  |  | -98 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  | Config 2 |  | -98 |  |
|  |  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 |  | Config 1,2,4 | dBm/15kHz | -115 |  |
|  |  |  | Config 3 |  | -118 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  | Config 1 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  |  | Config 2 |  | -65.38/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| MsgA Configuration |  |  |  |  | MsgA.2 FR1 | As defined in A.7A.1 |
| msgA-RSRP-ThresholdSSB |  |  |  |  | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Propagation Condition |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

<< End of Changes >>