# Title:	Test Tolerances for FR1 RLM In-Sync Tests for Satellite Access

# Source:	Qualcomm

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for Radio Link Monitoring In-Sync Test cases for Satellite Access.

The test cases in question, from 38.533, are:

- 14.4.1.2, NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access

- 14.4.1.4, NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access

- 14.4.1.6, NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access

- 14.4.1.8, NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v17.16.0 Annex A:

#### A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode

##### A.14.4.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the SAN PCell. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1C.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. Supported test configurations are shown in table A.14.4.1.2.1-1. The test parameters are given in Tables A.14.4.1.2.1-2, and A.14.4.1.2.1-3 below. There is one cell (Cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.14.4.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms.

The UE shall be provided with the valid information about the SAN serving the each cell in the test before the test.

Table A.14.4.1.2.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.14.4.1.2.1-2: General test parameters for FR1 in-sync testing in non-DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| BWchannel |  | Config 1, 2 | MHz | 10: NRB,c = 52 |
| DL initial BWP configuration |  | Config 1, 2 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  | Config 1, 2 |  | CR.1.1 FDD |
| Dedicated CORESET Reference Channel |  | Config 1, 2 |  | CCR.1.1 FDD |
| SSB Configuration |  | Config 1, 2 |  | SSB.1 FR1 |
| SMTC Configuration |  | Config 1, 2 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  | 15 kHz |
| PRACH Configuration |  | Config 1, 2 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 1000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1, 2 |  |  | CSI-RS.1.1 FDD |
| CSI-RS for tracking | Config 1, 2 |  |  | TRS.1.1 FDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 0.24 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 0.88 |
| D1 |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.14.4.1.2.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
| Propagation condition |  |  | NTN-TDLC5-200 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure A.6.5.1C.2.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Figure A.14.4.1.2.1-1: SNR variation for in-sync testing

##### A.14.4.1.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

# 3. Discussion

A similar analysis can be performed for both SSB-based RLM and CSI-RS-based RLM In-Sync test cases. For simplicity, we will use SSB-based test case 14.4.1.2 in this analysis, and then we will extend it to the rest.

The test has 5 time periods: T1, T2, T3, T4 and T5. To analyse the test case, it is helpful to show the variation of SSB SNR versus time, as shown below:

Figure 3-1: SNR variation for in-sync testing

Observations

During T1 the SNR is above Qin.

During T2 the SNR drops below Qin but remains above Qout.

During T3 the SNR is below Qout.

During T4 the SNR rises above Qout but remains below Qin.

During T5 the SNR is above Qin.

A detailed analysis of the test case is given in section 4 of this document.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.14.4.1.2.1-1, A.14.4.1.2.1-2 and A.14.4.1.2.1-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

c) Uncertainties

The SS provides 1 NR Cell with fading to the UE. We propose to control the following parameters:

- AWGN Absolute power, Noc ±1.5 dB averaged over BWConfig

- AWGN Absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

- Fading profile uncertainty for 2Tx ±0.7 dB

In this test the UE measures the SNR over specific Physical Resource Block (PRB) numbers # RBJ to # RBJ+19. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

Two sets of parameters are therefore given. The set averaged over the configured bandwidth and the set averaged over PRBs # RBJ-RBJ+19 have similar values to those already used for other tests.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition, the maximum input level needs to be within the requirements specified in 38.101-5, which for NR-NTN is less than -40dBm/ChBW:

## 7.4	Maximum input level

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in 3GPP TS 38.101-1 [5] Annex A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1) with parameters specified in Table 7.4-1.

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5, 10, 15, 20 |
| Power in Transmission Bandwidth Configuration3 | dBm | -402 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-2 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in 3GPP TS 38.101-1 [5] Annex A.3.2.3 for 64 QAM. NOTE 3: Power in transmission bandwidth configuration value is rounded to the nearest 0.5dB value. |  |  |

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in Io, so the sensitivity factor is 1.000 during T1 to T5. However, the same error of 1dB in the absolute AWGN level Noc would cause no change to Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE measurement accuracies where relevant. This process follows the superposition principle. Uncertainties are calculated separately for T1 to T5.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

Es/Iot of SSB index 0

During T1, the Es/Iot of SSB0 shall be above 1dB as specified in Table A.14.4.1.2.1-3 to ensure it stays above Qin by the intended margin. Considering the uncertainties, SSB0 doesn’t meet the requirement of Es/Iot > 1dB, so Es/Noc is therefore increased by an amount sufficient to achieve Es/Iot > 1dB.

During T2, the Es/Iot of SSB0 shall be above -7dB as specified in Table A.14.4.1.2.1-3 to ensure it stays above Qout by the intended margin. Considering the uncertainties, SSB0 doesn’t meet the requirement of Es/Iot > -7dB, so Es/Noc is therefore increased by an amount sufficient to achieve Es/Iot > -7dB.

During T3, the Es/Iot of SSB0 shall be below -15dB as specified in Table A.14.4.1.2.1-3 to ensure it stays below Qout by the intended margin. Considering the uncertainties, SSB0 doesn’t meet the requirement of Es/Iot < -15dB, so Es/Noc is therefore decreased by an amount sufficient to achieve Es/Iot < -15dB.

During T4, the Es/Iot of SSB0 shall be below -4.5dB as specified in Table A.14.4.1.2.1-3 to ensure it stays below Qin by the intended margin. Considering the uncertainties, SSB0 doesn’t meet the requirement of Es/Iot < -4.5dB, so Es/Noc is therefore decreased by an amount sufficient to achieve Es/Iot < -4.5dB.

During T5, the Es/Iot of SSB0 shall be above 1dB as specified in Table A.14.4.1.2.1-3 to ensure it stays above Qin by the intended margin. Considering the uncertainties, SSB0 doesn’t meet the requirement of Es/Iot > 1dB, so Es/Noc is therefore increased by an amount sufficient to achieve Es/Iot > 1dB.

During T1 to T5, the Io of set q0 meets the Io side condition < -40dBm. Note that the -50dBm side condition does not apply here, as no measurements covered by the side conditions in TS 38.133 section 10 are being made.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

Es/Iot of SSB index 0

During T1, Es/Noc is increased by an amount sufficient to achieve Es/Iot > 1dB. The actual offset required is 0.8 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

During T2, Es/Noc is increased by an amount sufficient to achieve Es/Iot > -7dB. The actual offset required is 0.8 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

During T3, Es/Noc is decreased by an amount sufficient to achieve Es/Iot < -15dB. The actual offset required is -0.8 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

During T4, Es/Noc is decreased by an amount sufficient to achieve Es/Iot < -4.5dB. The actual offset required is -0.8 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

During T5, Es/Noc is increased by an amount sufficient to achieve Es/Iot > 1dB. The actual offset required is 0.8 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

g) Check controlled parameters Min/Max

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 14.4.1.2 configuration 1 refers to the GSO satellite access configuration. In addition, test 14.4.1.2 also contains another test configuration with identical power levels for NGSO satellite access. There is no test tolerance or measurement uncertainty difference between both cases, so the above analysis can also be leveraged to the NGSO test configuration. There are no other BW/SCS configurations defined.

# 6. Treatment of other test cases

TS 38.533 also contains 3 other similar test cases 14.4.1.4, 14.4.1.6 and 14.4.1.8 in clause 14.4.1. Since the test parameters are identical with that of 14.4.1.2, the same level uncertainties and method of deciding Test Tolerances can be used.