3GPP TSG-RAN5 Meeting #72	R5-xxxx

# USA, Aug 22-26, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR |  | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to spec 34.123-1 for test case 12.4.1.4c procedure 1, test case 12.2.1.5d and test case 12.3.2.8 procedure 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, INTEL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  |  | Date: |  | 2016-08-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | “Routing area update accept” received by UE at step 19b is not integrity protected due to which UE may fail the test case. The step 19b is applicable for only R11 and above UEs. A Rel-11 or higher Operation Mode A UE supporting optionally initiate the RAU procedure in Steps 19a-19c for TC 12.4.1.4c proc1 and same is applicable for TC 12.2.1.5d and TC 12.3.2. proc1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added integrity check before sending “Routing area update accept” message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A compliant device may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.4.1.4c proc1, 12.2.1.5d and 12.3.2.8 proc1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 1.1 Change 1

12.4.1.4c.4	Method of test

Proc 1: Test procedure 1

Initial condition

System Simulator:

Three cells, cell A in MCC1/MNC2/LAC1/RAC1 (RAI-8, Not HPLMN), cell B in MCC1/MNC2/LAC1/RAC2 (RAI-10, Not HPLMN), cell C in MCC2/MNC1/LAC1/RAC1 (RAI-2).
All three cells are operating in network operation mode II.
The SIB1 IE "CN domain specific NAS system information", for the CS Domain, is set to value "00 00" (T3212 value is set to 0 and ATT flag is set to FALSE) in all cells.

The PLMN contains Cell C is equivalent to the PLMN that contains Cell A.

NB: i) Cell C will be mapped to Cell 4 as found in TS 34.108 clause 6.1.4.1.

User Equipment:

The UE has a valid P-TMSI-1, RAI-8.

If the UE is in UE operation mode A, then the UE has been registered in the CS domain.

Related ICS/IXIT statements

Support of PS service 	Yes/No
UE operation mode A 	Yes/No
UE operation mode C 	Yes/No
Switch off on button 		Yes/No
Automatic PS attach procedure at switch on or power on 	Yes/No

Test procedure: The SS rejects a routing area updating with the cause value 'GPRS services not allowed in this PLMN'. The SS checks that the UE performs PLMN selection.

Expected Sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
|  |  |  |  | The following messages are sent and shall be received on cell A. |
| 1 | UE |  |  | The UE is set in UE operation mode C (see ICS). If UE operation mode C not supported, goto step 20. |
| 2 | SS |  |  | The SS is set in network operation mode II. Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the "Non-Suitable cell". Set the cell type of cell C to the "Non-Suitable cell". (see note) |
| 3 | UE |  |  | The UE is powered up or switched on and initiates an attach (see ICS). Cell A is preferred by the UE. |
| 4 | -> |  | ATTACH REQUEST | Attach type = 'GPRS attach' Mobile identity = P-TMSI-1 Old Routing area identity = RAI-8 |
| 4a | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 4b | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 4c | SS |  |  | The SS starts integrity protection. |
| 5 | <- |  | ATTACH ACCEPT | No new mobile identity assigned. P-TMSI and P-TMSI signature not included. Attach result = 'GPRS only attached' Routing area identity = RAI-8 Equivalent PLMNs = MCC2,MNC1 |
|  |  |  |  | The following messages are sent and shall be received on cell B. |
| 6 | SS |  |  | Set the cell type of cell A to the " Suitable neighbour cell ". Set the cell type of cell B to the "Serving cell". (see note) |
| 7 | UE |  |  | Cell B is preferred by the UE. |
| 8 | -> |  | ROUTING AREA UPDATE REQUEST | Update type = 'RA updating' Old Routing area identity = RAI-8 |
| 9 | <- |  | ROUTING AREA UPDATE REJECT | GMM cause = 'GPRS services not allowed in this PLMN' |
| 10 | <- |  | PAGING TYPE1 | Mobile identity = P-TMSI-1 PAGING TYPE1 (used for NW-mode II). Paging order is for PS services. |
| 11 | UE |  |  | No response from the UE to the request. This is checked for 10 seconds. |
| 12 | SS |  |  | Set the cell type of cell B to the "Non-Suitable cell". Set the cell type of cell A to the "Serving cell". (see note) |
| 12a |  |  |  | The following step 13 is only performed for UE operation mode C. |
| 13 | UE |  |  | The UE performs PLMN selection. |
| 14 | UE |  |  | No ATTACH REQUEST or LOCATION UPDATE REQUEST sent to the SS (SS waits 30 seconds). |
| 15 | SS |  |  | Set the cell type of cell A to the "Non-Suitable cell". Set the cell type of cell C to the "Serving cell". (see note) |
| 16 |  |  |  | The following step is only performed for UE operation mode A. |
| 16a | UE |  | Registration on CS | See TS34.108 Parameter mobile identity is IMSI |
| 17 | -> |  | ATTACH REQUEST | Attach type = 'GPRS attach' Mobile identity = IMSI |
| 17a | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 17b | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 17c | SS |  |  | The SS starts integrity protection. |
| 18 | <- |  | ATTACH ACCEPT | Attach result = 'GPRS only attached' Allocated P-TMSI = P-TMSI-1 P-TMSI Signature = P-TMSI-1 signature Routing area identity = RAI-2 Equivalent PLMNs = MCC1,MNC2 |
| 19 | -> |  | ATTACH COMPLETE |  |
| 19 | -> |  | ATTACH COMPLETE |  |
|  |  |  |  | A Rel-11 or higher Operation Mode A UE supporting S1 mode may optionally initiate the RAU procedure in Steps 19a-19c within 10s |
| 19a | -> |  | ROUTING AREA UPDATING REQUEST | P-TMSI-1 signature Routing area identity = RAI-2 |
| 19aa | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 19ab | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 19ac | SS |  |  | The SS starts integrity protection. |
| 19b | <- |  | ROUTING AREA UPDATING ACCEPT | Allocated P-TMSI = P-TMSI-1 P-TMSI Signature = P-TMSI-1 signature Routing area identity = RAI-2 Equivalent PLMNs = MCC1, MNC2 |
| 19c | -> |  | ROUTING AREA UPDATING COMPLETE |  |
| 20 |  |  |  | The UE is set in UE operation mode A (see ICS) and the test is repeated from step 2 to step 19c. |
| NOTE: The definitions for "Non-Suitable cell", "Suitable neighbour cell" and "Serving cell" are specified in TS34.108 clause 6.1 in the relevant to the RAT of the cells "Reference Radio Conditions " subclauses. |  |  |  |  |

## 1.2 Change 2

12.2.1.5d.4	Method of test

Initial condition

System Simulator:

Three cells cell A with MCC1/MNC2/LAC1/RAC1 (RAI-8, Not HPLMN), cell B in MCC1/MNC2/LAC1/RAC2 (RAI-10, Not HPLMN), cell C in MCC2/MNC1/LAC1/RAC2 (RAI-7).
All three cells are operating in network operation mode II (in case of UE operation mode A).

The PLMN contains Cell C is equivalent to the PLMN that contains Cell A.

Sintrasearch and Sintersearch values for cells A, B and C are 20 dB.

NB: i) Cell C will be mapped to Cell 4 as found in TS 34.108 clause 6.1.4.1.

User Equipment:

The UE has a valid P-TMSI-1, RAI-8.

Related ICS/IXIT statements

Support of PS service 	Yes/No
UE operation mode C 	Yes/No
UE operation mode A 	Yes/No (only if mode C not supported)
Switch off on button 	Yes/No
Automatic PS attach procedure at switch on or power on 	Yes/No
UE supports CS call establishment 	Yes/No

Test procedure

The SS rejects a PS attach with the cause value 'GPRS services not allowed in this PLMN'. The SS checks that the UE performs PS attach with attach type = GPRS attach when a new equivalent PLMN is entered.

Expected Sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
|  | SS |  |  | The following messages are sent and shall be received on cell A. |
| 1 | UE |  |  | The UE is set in UE operation mode A OR The UE is set in UE operation mode C (see ICS). |
| 2 | SS |  |  | The SS is set in network operation mode II. Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the " Non-suitable cell ". Set the cell type of cell C to the " Non-suitable cell " (see note) |
| 3 | UE |  |  | The UE is powered up or switched on and initiates an attach (see ICS). Cell A is preferred by the UE. |
| 4 | UE |  | Registration on CS | See TS 34.108 This is applied only for UE in UE operation mode A. Mobile identity = TMSI-1 |
| 5 | -> |  | ATTACH REQUEST | Attach type = 'GPRS attach' Mobile identity = P-TMSI-1 |
| 5a | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 5b | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 5c | SS |  |  | The SS starts integrity protection. |
| 6 | <- |  | ATTACH ACCEPT | Attach result = 'GPRS only attached' Allocated P-TMSI = P-TMSI-1 Routing area identity = RAI-8 Equivalent PLMNs = MCC2,MNC1 |
| 6a | -> |  | ATTACH COMPLETE |  |
| 7 | <- |  | DETACH REQUEST | Detach type = re-attach required |
| 8 | -> |  | DETACH ACCEPT |  |
| 9 | SS |  |  | The SS is set in network operation mode II. Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the " Suitable neighbour cell ". Set the cell type of cell C to the " Non-suitable cell " (see note) |
| 9a | SS |  |  | SS shall wait for 1 second to receive the Attach Request in step 10 on the existing RRC Connection. In case ATTACH REQUEST is not received within 1 second then existing RRC Connection is released. |
| 10 | -> |  | ATTACH REQUEST | Attach type = 'GPRS attach' Mobile identity = P-TMSI-1 |
| 11 | <- |  | ATTACH REJECT | GMM cause = 'GPRS services not allowed in this PLMN' |
| A12 | UE |  |  | If the UE is in UE Operation Mode C Set the cell type of cell A to the " Suitable neighbour cell ". Set the cell type of cell B to the " Suitable neighbour cell ". Set the cell type of cell C to the " Serving cell " The UE performs PLMN selection. |
| B12 | UE |  |  | If the UE is in Operation Mode A The UE initiates an attach automatically, by MMI or by AT command. |
| B12a | UE |  |  | No ATTACH REQUEST sent to SS (SS waits 30 seconds). |
|  |  |  |  | The following messages are sent and shall be received on cell A. This verifies that a UE in operation mode A is still IMSI attached for CS services in cell A |
| B12a.1 | <- |  | PAGING TYPE1 | Mobile identity = TMSI-1 Paging order is for CS services. Paging cause = "Terminating conversational call[UE supports CS call establishment] or Terminating Low Priority Signalling" |
| B12a.2 | -> |  | RRC CONNECTION REQUEST |  |
| B12a.3 | <- |  | RRC CONNECTION SETUP |  |
| B12a.4 | -> |  | RRC CONNECTION SETUP COMPLETE |  |
| B12a.5 | -> |  | PAGING RESPONSE |  |
| B12a.6 | SS |  |  | The SS releases the RRC connection |
| B12b | SS |  |  | The SS is set in network operation mode II. Set the cell type of cell A to the " Suitable neighbour cell ". Set the cell type of cell B to the " Suitable neighbour cell ". Set the cell type of cell C to the " Serving cell " (see note) |
|  |  |  |  | The following messages are sent and shall be received on cell C. |
| B12c |  |  | Registration on CS | Only performed if UE is set to operation mode A. |
| 13 | -> |  | ATTACH REQUEST | Attach type = 'GPRS attach' Mobile identity = IMSI |
| 14 | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 15 | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 16 | SS |  |  | The SS starts integrity protection. |
| 17 | <- |  | ATTACH ACCEPT | Attach result = 'GPRS only attached' Allocated P-TMSI = P-TMSI-2 P-TMSI Signature = P-TMSI-2 signature Routing area identity = RAI-7 Equivalent PLMNs = MCC1,MNC2 |
| 18 | -> |  | ATTACH COMPLETE |  |
|  |  |  |  | A Rel-11 or higher Operation Mode A UE supporting S1 mode may optionally initiate the RAU procedure in Steps 18a-18c within 10s. |
| 18a | -> |  | ROUTING AREA UPDATING REQUEST | P-TMSI-2 signature Routing area identity = RAI-7 |
| 18aa | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 18ab | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 18ac | SS |  |  | The SS starts integrity protection. |
| 18b | <- |  | ROUTING AREA UPDATING ACCEPT | Allocated P-TMSI = P-TMSI-2 P-TMSI Signature = P-TMSI-2 signature Routing area identity = RAI-7 Equivalent PLMNs = MCC1, MNC2 |
| 18c | -> |  | ROUTING AREA UPDATING COMPLETE |  |
| 19 |  |  | Void |  |
| NOTE: The definitions for "Suitable neighbour cell", "Non-suitable cell" and "Serving cell" are specified in TS34.108 clause 6.1 in the relevant to the RAT of the cells "Reference Radio Conditions" subclauses. |  |  |  |  |

## 1.3 Change 3

12.3.2.8.4	Method of test

Proc 1: Test procedure 1

Initial conditions

System Simulator:

Two cells cellA in MCC2/MNC1/LAC1/RAC1(RAI-2, Not HPLMN), cellB in MCC1/MNC2/LAC2/RAC1.
Both cells are operating in network operation mode II.
The PLMN contains Cell B is equivalent to the PLMN that contains Cell A.
The SIB1 IE "CN domain specific NAS system information", for the CS Domain, is set to value "00 00" (T3212 value is set to 0 and ATT flag is set to FALSE) in both cells.

NB: i) Cell B will be mapped to Cell 4 as found in TS 34.108 clause 6.1.4.1.

User Equipment:

The UE has a valid TMSI-1, P-TMSI-1 and RAI-2.

The UE has been registered in the CS domain.

Related ICS/IXIT statement(s)

-	Support of PS service Yes/No.
-	UE operation mode A Yes/No 
-	UE operation mode C Yes/No (only if mode A not supported).
-	Switch off on button Yes/No.
-	Automatic PS attach procedure at switch on or power on Yes/No.

Test procedure

Two cells are configured.

Cell A transmits with higher power so that the UE attempts an attach procedure to cell A.

Cell B is configured as “non-Suitable Cell”.

The UE initiates a PS attach procedure.

The SS sends a PS detach with the cause "GPRS services not allowed in this PLMN".

The SS verifies that the UE does not perform a periodic ROUTING AREA UPDATE procedure in this PLMN after the timer T3312 is expired and does not respond a paging for PS services.

Cell B transmits with high power so that the UE attempts an attach procedure to cell B.

The UE initiates a PS attach procedure.

The SS verifies that the UE performs a periodic ROUTING AREA UPDATE procedure.

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
|  | SS |  |  | The following messages are sent and shall be received on cell A. |
| 1 | UE |  |  | The UE is set in UE operation mode A or C (see ICS). |
| 2 | SS |  |  | Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the " non-Suitable cell " |
| 3 | UE |  |  | The UE is powered up or switched on and initiates an attach (see ICS). Cell A is preferred by the UE. |
| 4 | -> |  | ATTACH REQUEST | Attach type = 'GPRS attach' Mobile identity = P-TMSI-1 Old Routing area identity = RAI-2 |
| 5 | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 6 | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 7 | SS |  |  | The SS starts integrity protection. |
| 8 | <- |  | ATTACH ACCEPT | Attach result = ' GPRS only attached' Allocated P-TMSI = P-TMSI-2 P-TMSI Signature = P-TMSI-2 signature Routing area identity = RAI-2 Equivalent PLMNs = MCC1,MNC2 Periodic RA Update Timer (T3312) = 6minutes |
| 9 | -> |  | ATTACH COMPLETE |  |
| 10 | <- |  | DETACH REQUEST | Detach Type = 're-attach not required' Cause = 'GPRS services not allowed in this PLMN' |
| 11 | -> |  | DETACH ACCEPT |  |
| 12 | SS |  |  | The SS releases the RRC connection. |
| 13 | <- |  | PAGING TYPE1 | Mobile identity = P-TMSI-2 Paging order is for PS services. |
| 14 | UE |  |  | No response from the UE to the request. This is checked for 10 seconds. |
| 15 | UE |  |  | The SS verifies that the UE does not attempt to access the network for T3312. |
|  |  |  |  | The following messages are sent and shall be received on cell B. |
| 16 | SS |  |  | If UE is set in UE operation mode C then Set the cell type of cell A to the "Suitable neighbour cell ". If UE is set in UE operation mode A then Set the cell type of cell A to the "Non Suitable neighbour cell ". Set the cell type of cell B to the "Serving cell " (see note) |
| 17 |  |  |  | Cell B is preferred by the UE. |
| 18 |  |  | Registration on CS | See TS 34.108 This is applied only for UE in UE operation mode A. |
| 19 |  |  |  | The UE initiates an attach automatically (See ICS), by MMI or AT command. |
| 20 | -> |  | ATTACH REQUEST | Attach type = 'GPRS attach' Mobile identity = IMSI |
| 21 | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 22 | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 23 | SS |  |  | The SS starts integrity protection. |
| 24 | <- |  | ATTACH ACCEPT | Attach result = 'GPRS only attached' Allocated P-TMSI = P-TMSI-2 P-TMSI Signature = P-TMSI-2 signature Routing area identity = RAI-9 Equivalent PLMNs = MCC2,MNC1 Periodic RA Update Timer (T3312) = 6minutes |
| 25 | -> |  | ATTACH COMPLETE |  |
| 25a | SS |  |  | The SS releases the RRC connection. |
|  |  |  |  | A Rel-11 or higher Operation Mode A UE supporting S1 mode may optionally initiate the RAU procedure in Steps 25aa-25ac within 10s. |
| 25aa | -> |  | ROUTING AREA UPDATING REQUEST | P-TMSI-2 signature Routing area identity = RAI-9 |
| 25aaa | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 25aab | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 25aac | SS |  |  | The SS starts integrity protection. |
| 25ab | <- |  | ROUTING AREA UPDATING ACCEPT | Allocated P-TMSI = P-TMSI-2 P-TMSI Signature = P-TMSI-2 signature Routing area identity = RAI-9 Equivalent PLMNs = MCC2, MNC1 |
| 25ac | -> |  | ROUTING AREA UPDATING COMPLETE |  |
| 25b | SS |  |  | The SS verifies that the UE does not attempt to access the network for T3312. |
| 26 | SS |  |  | SS checks that the IE "Establishment cause" in the received RRC CONNECTION REQUEST message is set to "Registration". |
| 27 | -> |  | ROUTING AREA UPDATING REQUEST | Update type = 'Periodic updating' Old P-TMSI signature=P-TMSI-2 signature Old Routing area identity = RAI-9 |
| 28 | <- |  | ROUTING AREA UPDATING ACCEPT | No new mobile identity assigned. P-TMSI and TMSI not included. Update result = 'RA updated' Equivalent PLMNs = MCC2,MNC1 |
| NOTE: The definitions for "Suitable neighbour cell", "Non-suitable cell" and "Serving cell" are specified in TS34.108 clause 6.1 in the relevant to the RAT of the cells "Reference Radio Conditions" subclauses. |  |  |  |  |