3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210006

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1458 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR template cs_MTC_SSB_NR_FR1 to update SIB24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to 38.133 cl 9.2.1, when measurement gaps are needed, the UE is not expected to detect SSB which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. Switching time is 0.5ms for frequency range FR1 and 0.25ms for frequency range FR2. Therefore, UE SSB detection is proposed to start earlier to take into account gap starting time + switching time. #draft proseCR would be raised for next RAN5 meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the value of SIB24 periodicityAndOffset_r15 to sf20_r15 = 19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NR5GC_IRAT cell-reselection from EUTRA_IDLE to NR RRC_IDLE |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.508 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| - Template name | cs_MTC_SSB_NR_FR1 |
| --- | --- |
| Reason for change | According to 38.133 cl 9.2.1, when measurement gaps are needed, the UE is not expected to detect SSB which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. Switching time is 0.5ms for frequency range FR1 and 0.25ms for frequency range FR2. Therefore, UE SSB detection is proposed to start earlier to take into account gap starting time + switching time. #draft proseCR would be raised for next RAN5 meeting |
| Summary of change | Change the value of SIB24 periodicityAndOffset_r15 to sf20_r15 = 19 |
| TTCN module | EUTRA38_IRAT_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before change:

template(value) MTC_SSB_NR_r15 cs_MTC_SSB_NR_FR1 :=

{ /* @status    APPROVED (NR5GC_IRAT) */

periodicityAndOffset_r15 := { sf20_r15 := 0 },

ssb_Duration_r15 := sf2

};

After change:

template(value) MTC_SSB_NR_r15 cs_MTC_SSB_NR_FR1 :=

{ /* @status    APPROVED (NR5GC_IRAT) */

periodicityAndOffset_r15 := { sf20_r15 := 19 },

ssb_Duration_r15 := sf2

};