3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210029

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1475 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The TTCN CR R5s201316 change 3 has been wrongly implemented in D20wk50. Watchdog timout should be implemented for second alt Step 13, 17 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the implementation of Watchdog timout after second alt Step 13, 17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_1_2_NR_TestBody1 |
| --- | --- |
| Reason for change | The TTCN CR R5s201316 change 3 has been wrongly implemented in D20wk50. Watchdog timout should be implemented for second alt Step 13, 17 |
| Summary of change | Move the implementation of Watchdog timout after second alt Step 13, 17 |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_1_2_NR_TestBody1( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var NR_SYSTEM_IND v_Received_NR_SysInd; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingUL2; var SubFrameTiming_Type v_TimingUL3; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112(v_NR_PhysicalParameters); var integer v_FrameNum := 0; var integer v_FrameNum2 := 0; var template (present) integer v_SubFrameNum := 0; var integer v_SubFrameNum_Rcvd := 0; var integer v_Slot_Rcvd := 0; var template (present) integer v_PRACH_PreamblesGroupA := (0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21,22,23,24,25,26,27); var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var float v_TimerT2 := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 0.96); var float v_TimerT3 := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 1.920); //@sic R5s200823 sic@ var SubFrameTiming_Type v_TimingDRB; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var boolean v_DRB_Flag := false; var boolean v_PRACH_Flag := false; timer t_Watchdog := 5.0; if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_SubFrameNum := (4,9); } else //FR1 FDD { v_SubFrameNum := (2,6,9); } } else { // FR2 // with SCS =120, 10 subframes with each 8 slots. Hence slots 0 ..39 map to sub frames 0..4 v_SubFrameNum := (1,2,3,4,6,7,8,9); } // Enable SR reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); // configure RACH procedure suitable for steps 4 to 17. f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // @siclog "Step 1" siclog@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,200); // Timing of DL // Configure SS transmission of Timing advance f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_TimingAdvance_CE)); // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Stop); // @siclog "Steps 2" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 700); //Time alignemnet is 750ms // @siclog "Steps 3" siclog@ // PDCP SDU =7B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header + 2B Padding Sub PDU =17B=136 bits (an allowed TB size as per 38.523-3 Annex B) t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //cs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_NR(v_TimingDL), {crs_NR_PDCP_SDU_7B} )); // @siclog "Steps 3" siclog@ // preconfigured at SS to not allocate grants // @siclog "Steps 4" siclog@ alt { [ ] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { repeat; }; [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo(?, v_SubFrameNum), v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd { v_FrameNum := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.SFN.Number; v_SubFrameNum_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Subframe.Number; if (not v_IsFR1) {v_Slot_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Slot.SlotOffset.Numerology3;}; //@sic R5s200043 sic@ // FR2 4,9,14,19,24,29,34,39 //@sic R5-198323 R5s200111 R5-201100 R5-201210 sic@ if ((v_IsFR1 and ischosen(v_NR_DuplexMode.TDD) and (v_FrameNum mod 2) == 1 and (v_SubFrameNum_Rcvd == 4 or v_SubFrameNum_Rcvd == 9)) or //TDD (v_IsFR1 and (not ischosen(v_NR_DuplexMode.TDD)) and (v_FrameNum mod 8) == 1 and (v_SubFrameNum_Rcvd == 2 or v_SubFrameNum_Rcvd == 6 or v_SubFrameNum_Rcvd == 9)) or //FDD //Fr2 (( not v_IsFR1 ) and (v_FrameNum mod 4) == 1 and ((v_SubFrameNum_Rcvd == 0 and v_Slot_Rcvd == 8) // slot 8 or (v_SubFrameNum_Rcvd == 1 and v_Slot_Rcvd == 1) //slot 9 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 2) //slot 18 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 3) //slot 19 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 4) //slot 28 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 5) // slot 29 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 6) // slot 38 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 7) //slot 39 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 0) // slot 48 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 1) //slot 49 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 2) // slot 58 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 3) //slot 59 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 4) // slot 68 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 5) //slot 69 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 6) // slot 78 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 7))//slot 79 )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4 "); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4, wrong frame number used "); } }; [] t_Watchdog.timeout {} //@sic R5s201316 sic@ }; // @siclog "Steps 5" siclog@ //@sic R5-198323 R5s200111 R5-201100 R5-201210 sic@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo((v_FrameNum, v_FrameNum+2, v_FrameNum+4, v_FrameNum+8), v_SubFrameNum), v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd; v_FrameNum2 := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.SFN.Number; v_SubFrameNum_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Subframe.Number; if (not v_IsFR1) {v_Slot_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Slot.SlotOffset.Numerology3;};//@sic R5s200043 sic@ if ((v_IsFR1 and ischosen(v_NR_DuplexMode.TDD) and (v_FrameNum2 == v_FrameNum or v_FrameNum2 == v_FrameNum+2 )and (v_SubFrameNum_Rcvd == 4 or v_SubFrameNum_Rcvd == 9)) or //TDD (v_IsFR1 and (not ischosen(v_NR_DuplexMode.TDD)) and (v_FrameNum2 == v_FrameNum+8) and (v_SubFrameNum_Rcvd == 2 or v_SubFrameNum_Rcvd == 6 or v_SubFrameNum_Rcvd == 9)) or //FDD //FR2 (( not v_IsFR1 ) and (v_FrameNum2 == v_FrameNum or v_FrameNum2 == v_FrameNum+4)and ((v_SubFrameNum_Rcvd == 0 and v_Slot_Rcvd == 8) // slot 8 or (v_SubFrameNum_Rcvd == 1 and v_Slot_Rcvd == 1) //slot 9 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 2) //slot 18 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 3) //slot 19 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 4) //slot 28 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 5) // slot 29 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 6) // slot 38 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 7) //slot 39 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 0) // slot 48 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 1) //slot 49 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 2) // slot 58 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 3) //slot 59 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 4) // slot 68 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 5) //slot 69 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 6) // slot 78 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 7)) //slot 79 )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 "); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5, wrong frame number used "); } // @siclog "Steps 6" siclog@ // preconfigured @ SS // @siclog "Steps 7" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd; v_TimingUL := v_Received_NR_SysInd.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7 "); // @siclog "Step 8" siclog@ // preconfigured @ SS // @siclog "Step 9" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd; v_TimingUL2 := v_Received_NR_SysInd.Common.TimingInfo.SubFrame; if (f_SubFrameTimingDuration_Float(v_TimingUL, v_TimingUL2) < v_TimerT2) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9: TP4 failed, UE didn't send PRACH Preamble while Timer_2 is running"); } // @siclog "Step 10-12, 4-16" siclog@ // preconfigured @ SS // @siclog "Step 13, 17" siclog@ //@sic R5s200823 sic@ alt { [not v_DRB_Flag] DRB.receive (car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, -, {crs_NR_PDCP_SDU_7B}))-> value v_NR_DRB_COMMON_IND { v_TimingDRB := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); v_DRB_Flag := true; repeat; } [not v_PRACH_Flag]SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, v_PRACH_PreamblesGroupA)) -> value v_Received_NR_SysInd { v_TimingUL3 := v_Received_NR_SysInd.Common.TimingInfo.SubFrame; v_PRACH_Flag := true; if (not v_DRB_Flag) { if (f_SubFrameTimingDuration_Float(v_TimingUL2, v_TimingUL3) < v_TimerT3) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); repeat; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13: TP6 failed, UE didn't send PRACH Preamble while Timer_3 is running"); } } else { if ((f_SubFrameTimingDuration_Float(v_TimingUL2, v_TimingUL3) < v_TimerT3) and (f_SubFrameTiming_Duration(v_TimingUL3, v_TimingDRB)>0)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13: TP6 failed, UE didn't send PRACH Preamble while Timer_3 is running"); } } } [v_PRACH_Flag] SYSIND.receive (car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17: UE Repeating the PRACH after Data Transmission"); } }; f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config };

After Change:

function f_TC_7_1_1_1_2_NR_TestBody1( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var NR_SYSTEM_IND v_Received_NR_SysInd; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingUL2; var SubFrameTiming_Type v_TimingUL3; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112(v_NR_PhysicalParameters); var integer v_FrameNum := 0; var integer v_FrameNum2 := 0; var template (present) integer v_SubFrameNum := 0; var integer v_SubFrameNum_Rcvd := 0; var integer v_Slot_Rcvd := 0; var template (present) integer v_PRACH_PreamblesGroupA := (0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21,22,23,24,25,26,27); var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var float v_TimerT2 := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 0.96); var float v_TimerT3 := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 1.920); //@sic R5s200823 sic@ var SubFrameTiming_Type v_TimingDRB; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var boolean v_DRB_Flag := false; var boolean v_PRACH_Flag := false; timer t_Watchdog := 5.0; if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_SubFrameNum := (4,9); } else //FR1 FDD { v_SubFrameNum := (2,6,9); } } else { // FR2 // with SCS =120, 10 subframes with each 8 slots. Hence slots 0 ..39 map to sub frames 0..4 v_SubFrameNum := (1,2,3,4,6,7,8,9); } // Enable SR reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); // configure RACH procedure suitable for steps 4 to 17. f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // @siclog "Step 1" siclog@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,200); // Timing of DL // Configure SS transmission of Timing advance f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_TimingAdvance_CE)); // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Stop); // @siclog "Steps 2" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 700); //Time alignemnet is 750ms // @siclog "Steps 3" siclog@ // PDCP SDU =7B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header + 2B Padding Sub PDU =17B=136 bits (an allowed TB size as per 38.523-3 Annex B) t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //cs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_NR(v_TimingDL), {crs_NR_PDCP_SDU_7B} )); // @siclog "Steps 3" siclog@ // preconfigured at SS to not allocate grants // @siclog "Steps 4" siclog@ alt { [ ] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { repeat; }; [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo(?, v_SubFrameNum), v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd { v_FrameNum := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.SFN.Number; v_SubFrameNum_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Subframe.Number; if (not v_IsFR1) {v_Slot_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Slot.SlotOffset.Numerology3;}; //@sic R5s200043 sic@ // FR2 4,9,14,19,24,29,34,39 //@sic R5-198323 R5s200111 R5-201100 R5-201210 sic@ if ((v_IsFR1 and ischosen(v_NR_DuplexMode.TDD) and (v_FrameNum mod 2) == 1 and (v_SubFrameNum_Rcvd == 4 or v_SubFrameNum_Rcvd == 9)) or //TDD (v_IsFR1 and (not ischosen(v_NR_DuplexMode.TDD)) and (v_FrameNum mod 8) == 1 and (v_SubFrameNum_Rcvd == 2 or v_SubFrameNum_Rcvd == 6 or v_SubFrameNum_Rcvd == 9)) or //FDD //Fr2 (( not v_IsFR1 ) and (v_FrameNum mod 4) == 1 and ((v_SubFrameNum_Rcvd == 0 and v_Slot_Rcvd == 8) // slot 8 or (v_SubFrameNum_Rcvd == 1 and v_Slot_Rcvd == 1) //slot 9 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 2) //slot 18 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 3) //slot 19 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 4) //slot 28 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 5) // slot 29 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 6) // slot 38 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 7) //slot 39 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 0) // slot 48 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 1) //slot 49 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 2) // slot 58 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 3) //slot 59 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 4) // slot 68 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 5) //slot 69 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 6) // slot 78 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 7))//slot 79 )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4 "); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4, wrong frame number used "); } }; //MOVED[] t_Watchdog.timeout {} //@sic R5s201316 sic@ }; // @siclog "Steps 5" siclog@ //@sic R5-198323 R5s200111 R5-201100 R5-201210 sic@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo((v_FrameNum, v_FrameNum+2, v_FrameNum+4, v_FrameNum+8), v_SubFrameNum), v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd; v_FrameNum2 := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.SFN.Number; v_SubFrameNum_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Subframe.Number; if (not v_IsFR1) {v_Slot_Rcvd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame.Slot.SlotOffset.Numerology3;};//@sic R5s200043 sic@ if ((v_IsFR1 and ischosen(v_NR_DuplexMode.TDD) and (v_FrameNum2 == v_FrameNum or v_FrameNum2 == v_FrameNum+2 )and (v_SubFrameNum_Rcvd == 4 or v_SubFrameNum_Rcvd == 9)) or //TDD (v_IsFR1 and (not ischosen(v_NR_DuplexMode.TDD)) and (v_FrameNum2 == v_FrameNum+8) and (v_SubFrameNum_Rcvd == 2 or v_SubFrameNum_Rcvd == 6 or v_SubFrameNum_Rcvd == 9)) or //FDD //FR2 (( not v_IsFR1 ) and (v_FrameNum2 == v_FrameNum or v_FrameNum2 == v_FrameNum+4)and ((v_SubFrameNum_Rcvd == 0 and v_Slot_Rcvd == 8) // slot 8 or (v_SubFrameNum_Rcvd == 1 and v_Slot_Rcvd == 1) //slot 9 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 2) //slot 18 or (v_SubFrameNum_Rcvd == 2 and v_Slot_Rcvd == 3) //slot 19 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 4) //slot 28 or (v_SubFrameNum_Rcvd == 3 and v_Slot_Rcvd == 5) // slot 29 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 6) // slot 38 or (v_SubFrameNum_Rcvd == 4 and v_Slot_Rcvd == 7) //slot 39 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 0) // slot 48 or (v_SubFrameNum_Rcvd == 6 and v_Slot_Rcvd == 1) //slot 49 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 2) // slot 58 or (v_SubFrameNum_Rcvd == 7 and v_Slot_Rcvd == 3) //slot 59 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 4) // slot 68 or (v_SubFrameNum_Rcvd == 8 and v_Slot_Rcvd == 5) //slot 69 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 6) // slot 78 or (v_SubFrameNum_Rcvd == 9 and v_Slot_Rcvd == 7)) //slot 79 )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 "); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5, wrong frame number used "); } // @siclog "Steps 6" siclog@ // preconfigured @ SS // @siclog "Steps 7" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd; v_TimingUL := v_Received_NR_SysInd.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7 "); // @siclog "Step 8" siclog@ // preconfigured @ SS // @siclog "Step 9" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, v_PRACH_PreamblesGroupA))-> value v_Received_NR_SysInd; v_TimingUL2 := v_Received_NR_SysInd.Common.TimingInfo.SubFrame; if (f_SubFrameTimingDuration_Float(v_TimingUL, v_TimingUL2) < v_TimerT2) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9: TP4 failed, UE didn't send PRACH Preamble while Timer_2 is running"); } // @siclog "Step 10-12, 4-16" siclog@ // preconfigured @ SS // @siclog "Step 13, 17" siclog@ //@sic R5s200823 sic@ alt { [not v_DRB_Flag] DRB.receive (car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, -, {crs_NR_PDCP_SDU_7B}))-> value v_NR_DRB_COMMON_IND { v_TimingDRB := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); v_DRB_Flag := true; repeat; } [not v_PRACH_Flag]SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, v_PRACH_PreamblesGroupA)) -> value v_Received_NR_SysInd { v_TimingUL3 := v_Received_NR_SysInd.Common.TimingInfo.SubFrame; v_PRACH_Flag := true; if (not v_DRB_Flag) { if (f_SubFrameTimingDuration_Float(v_TimingUL2, v_TimingUL3) < v_TimerT3) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); repeat; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13: TP6 failed, UE didn't send PRACH Preamble while Timer_3 is running"); } } else { if ((f_SubFrameTimingDuration_Float(v_TimingUL2, v_TimingUL3) < v_TimerT3) and (f_SubFrameTiming_Duration(v_TimingUL3, v_TimingDRB)>0)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13: TP6 failed, UE didn't send PRACH Preamble while Timer_3 is running"); } } } [v_PRACH_Flag] SYSIND.receive (car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17: UE Repeating the PRACH after Data Transmission"); } [] t_Watchdog.timeout {} }; f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config };